Low PAPR RS

| Parameter Name | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- | --- |
| DMRSdownlink-r16 | enabled | new | Per DL BWP | Dedicated | When this parameter is present in DMRS-DownlinkConfig, then the Rel.16 low PAPR DMRS is used instead of the Rel.15 DMRS according to TS 38.211, Clause X |  |
| DMRSuplink-r16 | enabled | new | Per UL BWP | Dedicated | When this parameter is present in DMRS-UplinkConfig, under transformPrecodingDisabled, then the Rel.16 low PAPR DMRS is used for CP-OFDM PUSCH, instead of the Rel.15 DMRS, according to TS 38.211, Clause X |  |
| DMRSuplinkTransformPrecoding-r16 | enabled | new | Per UL BWP | Dedicated | When this parameter is present in DMRS-UplinkConfig, under transformPrecodingEnabled, then the Rel.16 low PAPR DMRS is used for PUSCH with pi/2 BPSK modulation, instead of the Rel.15 DMRS, according to TS 38.211, Clause X |  |
| pi2BPSKscramblingID0 | 0..65535 | new | Per UL BWP | Dedicated | UL DMRS scrambling initialization for pi/2 BPSK DMRS for PUSCH (see TS 38.211, Clause X). When the field is absent the UE applies the value Physical cell ID (physCellId). | Requires DMRSuplinkTransformPrecoding-r16 to be enabled |
| pi2BPSKscramblingID1 | 0..65535 | new | Per UL BWP | Dedicated | UL DMRS scrambling initialization for pi/2 BPSK DMRS for PUSCH (see TS 38.211, Clause X). When the field is absent the UE applies the value Physical cell ID (physCellId). | Requires DMRSuplinkTransformPrecoding-r16 to be enabled |
| DMRSuplinkTransformPrecodingPUCCH-r16 | enabled | new | Per UL BWP | Dedicated | When this parameter is present in PUCCH-Config then the Rel.16 low PAPR DMRS is used for PUCCH Format 3 and 4 with pi/2 BPSK modulation, instead of the Rel.15 DMRS according to TS 38.211, Clause X | The UE must be configured to use pi/2 BPSK for UCI symbols instead of QPSK for PUCCH Format 3 and 4. |

UL FPTX

| Parameter Name | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- | --- |
| ULFPTx | {enabled} | new R16 | per CC or UL BWP | dedicated | gNB configures UE to support UL full power transmission |  |

MU-CSI

| Parameter Name | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- | --- |
| R | 1, 2 | New R16 | Per CC per UE | Dedicated | CQI subband size divided by PMI subband size | Only when codebookType-r16=typeIIr16 OR typeIIr16-PortSelection |
| ParamCombination | 1, 2, 3, 4, 5, 6, 7, 8 | New R16 | Per CC per UE | Dedicated | Supported parameter combination (L,y0,p0,beta) | *) Only when codebookType-r16=typeIIr16 OR typeIIr16-PortSelection **) Values '6' and '8' are only applicable for Rank <= 2 and 32 CSI-RS ports |
| n1-n2-codebookSubsetRestriction-r16 | Set of (N1,N2) values are same as Rel.15. Set of values for CBSR is TBD | Ext R15 (details up to RAN2) | Per CC per UE | Dedicated | (N1,N2) parameter for R16 Type II codebook. Whether to reuse the IE for Type I SP or Type II depends on further agreement and is up to RAN2 | Only when codebookType-r16=typeIIr16 |
| TypeII-ri-Restriction-r16 | BIT STRING (SIZE (4)) | Ext R15 (details up to RAN2) | Per CC per UE | Dedicated | Bitmap for rank restriction (Rel.16 Type II CB supports up to rank 4) | Only when codebookType-r16=typeIIr16 |
| codebookType-r16 | New R16 values: typeIIr16, typeIIr16-PortSelection | Ext R15 (details up to RAN2) | Per CC per UE | Dedicated | Codebook type. Can reuse R15 RRC parameter, but with additional values | n/a |
| TypeII-PortSelection-ri-Restriction-r16 | BIT STRING (SIZE TBD) | Ext R15 (details up to RAN2) | Per CC per UE | Dedicated | Bitmap for rank restriction (Whether Rel.16 Type II Port Selection CB supports up to rank 4 is TBD in RAN1) | Only when codebookType-r16= typeIIr16-PortSelection |
| portSelectionSamplingSize-r16 | 1,2,3,4 | Ext R15 (details up to RAN2) | Per CC per UE | Dedicated | Parameter d in Rel-16 Type II Port Selection CB | Only when codebookType-r16= typeIIr16-PortSelection |

M-TRP

| Parameter Name | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- | --- |
| controlResourceSetToAddModList | SEQUENCE(SIZE (1..5)) OF ControlResourceSet | Extension | Per DL BWP | dedicated | [Agreement] For multi-PDCCH based multi-TRP operation, the maximum number of CORESETs per “PDCCH-config” is 5, according to UE capability. [Agreement] For multi-PDCCH based multi-TRP operation, the maximum number of CORESETs that can be configured with the same TRP (i.e. same higher layer index configured per CORESET (if configured) per “PDCCH-Config”) is up to UE capability, including at least a candidate value of 3. |  |
| controlResourceSetToReleaseList | SEQUENCE(SIZE (1..5)) OF ControlResourceSetId | Extension | Per DL BWP | dedicated | [Agreement] For multi-PDCCH based multi-TRP operation, the maximum number of CORESETs per “PDCCH-config” is 5, according to UE capability. [Agreement] For multi-PDCCH based multi-TRP operation, the maximum number of CORESETs that can be configured with the same TRP (i.e. same higher layer index configured per CORESET (if configured) per “PDCCH-Config”) is up to UE capability, including at least a candidate value of 3. |  |
| HigherLayerIndexPerCORESET | [0:1:M] FFS M>1 | New | per CORESET and Per DL BWP | dedicated | [Agreement] The index to be used to generate separated ACK/NACK codebook is a higher layer signalling index per CORESET |  |
| AddtionalDataScramblingIdentityPDSCH | INTEGER (0..1023) | New or Extension up to RAN2 | Per DL BWP | dedicated | Additional scrambling ID for PDSCHs scheduled by Multiple-DCI [Agreement] At least for eMBB with M-DCI NCJT in order to generate different PDSCH scrambling sequences, support enhancing RRC configuration to configure multiple dataScramblingIdentityPDSCH [Agreement] In case higher layer index per CORESET is configured, for multi-PDCCH based multi-TRP operation, when multiple dataScramblingIdentityPDSCH parameters are configured, each dataScramblingIdentityPDSCH is associated with a higher layer signalling index per CORESET (if configured) and is applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index. | FFS: Whether and how to specify UE behaviour in case the higher layer index per CORESET is not configured. |

MB1+2

| Parameter Name | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- | --- |
| maxNrofSpatialRelationInfos | 64 | extension of R15 | a constant value | a constant value (used for dedicated RRC) | The value is increased to be 64 and used per BWP | n/a |
| maxNrofCandidateBeams | 64 | extension of R15 | a constant value | a constant value (used for dedicated RRC) | The value is increased to be 64 and used per BWP | n/a |
| reportQuantity | 'cri-SINR', 'ssb-Index-SINR' | extension of R15 | configured in CSI-reportConfig | dedicated | parameter to configure UE to report CRI+SINR or SSBRI+SINR | n/a |
| nrofReportedRSForSINR | {n1, n2, n3, n4} | new R16 | configured in CSI-reportConfig | dedicated | number of RS to be reported in a beam report instance for L1-SINR based beam report | n/a |
| rsrp-ThresholdSSBBFR | RSRP-Range | new R16 | Per DL BWP | dedicated | new beam identification threshold | It should be configured when candidateBeamResourceList is configured. FFS: whether candidateBeamResourceList is optionally configured |
| beamFailureDetectionResourceList | sequence (size of (1,…,[maxNrofBeamFailureDetectionResources])) of NZP-CSI-RS-ResourceId | new R16 | Per DL BWP | dedicated | CSI-RS resources for beam failure detection Note: Whether to use an parameter or reuse RadioLinkMonitoringConfig with restriction is up to RAN2. | Only periodic 1-port CSI-RS for BM can be configured |
| [maxNrofBeamFailureDetectionResources] | TBD | new R16 | constant value | n/a. | maximum number of SCell BFD RS per BWP Note: Whether to use an independent parameter or reuse maxNrofFailureDetectionResources is determined by the value to be decided at RAN1 #98b | n/a |
| [New parameter(s) for Msg1 PUCCH transmission for BFRQ - to be determined by RAN2] | [To be determined by RAN2] | new R16 | Per Cell Group | dedicated | dedicated request for SCell BFR. Note: From RAN1 perspective, the parameters related to SR transmission, including pucch-ResourceID, prohibitTimer, transMax and periodicityAndOffset, can be configured for SCell BFR, as SR configuration in Rel-15. How to define this parameter(s) is up to RAN2. | n/a |
| candidateBeamResourceList | sequence (size of (1,…,maxNrofCandidateBeams)) of candidateBeamResource | new R16 | Per DL BWP | dedicated | resource list for SCell new beam identification | n/a |
| candidateBeamResource | structure including: cell, choice{ssb-Index, nzp-csi-RS-ResourceId} | new R16 | Per DL BWP | dedicated | one resource for SCell new beam identification | n/a |
| cell | ServCellIndex | new R16 | Configured in new structure candidateBeamResource | dedicated | serving cell index for RS for new beam identification | n/a |