3GPP TSG RAN WG1 #98		R1-1908007

Prague, CZ, August 26th – 30th, 2019

3GPP TSG-RAN WG2 Meeting #106	R2-1908462

Reno, USA, 13–17 May, 2019

# Title:	Reply LS on SDAP in MTSI

# Response to:	S4-190555/R2-1905538

# Release:	Release 16

# Work Item:	5G_MEDIA_MTSI_ext

Source:	RAN WG2

To:	SA WG4

Cc:	GSMA 5GJA, SA WG2, RAN WG1

Contact Person:

Name:	Jaehyuk Jang

E-mail Address:	jack.jang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

RAN2 would like to thank SA4 for the LS on the usage of SDAP in MTSI and re-usability of delay and error profiles (S4-190555/R2-1905538). RAN2 has discussed the questions and provides SA4 the following feedback (starting with [R2] below):

| WG | Question |
| --- | --- |
| RAN2 | - Will SDAP be configured without headers or configured with headers for a direction or both directions in (1) point-to-point communication or (2) many-to-one communication such as Multi-party Multimedia Conference? [R2]: For point-to-point communication, RAN2 assumes that the SDAP would be configured without header for both directions in the typical MTSI cases as SDAP header would be needed only when more than one QoS flows are multiplexed in a DRB or reflective mapping is enabled. However the use of SDAP header cannot be precluded based on gNB implementation and in other cases. For many-to-one communication, RAN2 is not able to provide an answer as RAN2 is not aware of the detailed characteristics of Multi-party Multimedia Conference. If multiple RTP streams of the Multi-party Multimedia Conference, as described in S4-160725 (R2-164622, S2-163364), are bound to a single QoS flow (subject to SA2 discussion), the same answer for point-to-point communication can be applied. |

| WG | Question |
| --- | --- |
| RAN1/ RAN2 | - SA4 wonders if even in NR, there will be preferred uplink TBSs designed to offer optimized transmissions. If so, SA4 would like to be informed of these preferred TBSs. SA4 would also like to understand the potential penalty, e.g., in transmission efficiency, latency, etc., if the preferred TBSs cannot be matched. [R2]: RAN2 assumes RAN1 would answer this question. |

2. Actions:

To SA WG4 group.

ACTION: 	RAN2 respectfully asks SA4 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #107	26 – 30 August 2019	Prague, Czech Republic

TSG-RAN WG2 Meeting #107bis	14 – 18 October 2019	China