3GPP TSG-RAN WG1 #98b							        R1-1910429

Chongqin, 14th October – 18th October, 2019

Source:	Mitsubishi Electric

Title:	Discussion on UL Timing Advance and RACH

Agenda Item:	7.2.5.3	Uplink timing advance/RACH procedure

Document for: 	Discussion/Decision

# Introduction

RAN#80 approved a new SI on solutions evaluation for NR to support Non-Terrestrial Network [1]. This SI description was slightly revised in RAN#83 [2].

The objectives of the SI for physical-layer are reported as follows:

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

A significant difference of Non-Terrestrial Network (NTN) compared to terrestrial network is the simultaneous presence of a very large propagation delay (up to hundreds of milliseconds) and a very large Doppler shift (up to several SCS) due to the fast moving of (LEO) satellites.

All necessary timing advance procedures rely on an accurate estimation of the delay, in particular during the initial access, despite the large Doppler shift. A joint time and frequency synchronization is needed, that will rely on both the cyclic prefix (CP) and the reference signals [3].

In this contribution, several issues related to DL/UL synchronization, UL Timing Advance (TA) and RACH are discussed.

# DL/UL synchronization

The NTN scenario is characterized with very large time delays combined with large Doppler shifts due to the LEO satellite velocity.

In practice, with a LEO satellite configuration at 600 km, a speed of 7,6 km/s and a carrier frequency of 2.6 GHz, a Doppler shift of 65.9 KHz is obtained, corresponding for example to 4.4 SCS of 15 KHz [3].

However, this value can be reduced thanks to Doppler pre-compensation in LEO/MEO constellations. For example, [4] proposed to operate a “blind” frequency shift compensation on the satellite side: “Based on the satellite ephemeris and the beam layout it is possible to predict precisely the relative radial velocity between the satellite and the center of each beam. Then, the Doppler frequency shift due to satellite mobility is compensated on board as if all the served UEs were at the beam center.”

The resulting maximal residual frequency error values due to both satellite and UE mobility is represented in Table 1, extracted from [4].

Table 1: Maximal residual frequency error values due to both satellite and UE mobility

| Scenario | Satellite altitude [km] | Beam footprint diameter [km] | UE velocity [km/h] | Max Doppler shift residual error due to both satellite and UE mobility normalized by the carrier frequency [ppm] |
| --- | --- | --- | --- | --- |
| C & D | 600 | 200 | 1000 | +/-4,30 |
| C & D | 600 | 200 | 500 | +/-4,22 |
| C & D | 600 | 200 | 0 | +/-4,14 |
| C & D | 1200 | 200 | 1000 | +/-2,1 |
| C & D | 1200 | 200 | 500 | +/-2,05 |
| C & D | 1200 | 200 | 0 | +/-2,01 |

As noted in [5], the original NR-PSS/SSS design was performed taking into account the following target requirements, extracted from [6]:

- Robustness against initial frequency offset up to 5 ppm

- 10 ppm as optional requirement

- Reasonable complexity for NR-PSS/SSS detection

- Good one-shot detection probability at -6 dB received baseband SNR condition with less than 1% false alarm rate

- …

In a terrestrial network, this value of 5 ppm is mainly due to the UE local oscillator inaccuracy.

Then, it was observed in [5] that in order to avoid an initial AFC lock of the order of one second or less during NTN frequency synchronization, the residual Doppler shift after pre-compensation should be in the order of 5 ppm or less with LEO=600 km. This condition is verified under the hypothesis of Table 1.

In [11], we noted that for the DL, the existence of a scenario with a residual Doppler shift after pre-compensation higher than 5 ppm should be investigated.

The above analysis relies on the hypothesis that the maximum beam footprint diameter is equal to 200km. In last RAN1#98 meeting this value seemed controversial and a refined value/justification is expected in the current meeting. In practice, as noticed in the previous table, the maximum relative Doppler value in a beam depends on the footprint parameter but also on the altitude h. As a result, observe that for a given maximum residual Doppler value after pre-correction, the maximum diameter size depends (increases) of the altitude.

Observation 1: For a given maximum residual Doppler value after pre-correction, the maximum diameter size depends (increases) of the altitude.

Proposal 1: For residual Doppler evaluations, set the maximum acceptable footprint diameter as a function of the altitude.

# UL Timing Advance

Initial TA acquisition and indication

In [7], the summary of RAN1#97 discussions on the subject, two options were proposed for the acquisition of the common TA:

      - Option 1: Indication of common TA to all users within the coverage of same beam with following

        - Broadcasting can be considered as baseline for signalling, e.g., via SIB/MIB

        - Additional UE-specific TA indicated in RAR with extension of existing TA range

        - FFS on the reference point   for common TA calculation (e.g., with/without consideration on UE altitude)

      - Option 2: Autonomous acquisition of the common TA at UE with known location and satellite ephemeris:

        - FFS on the impacts on PRACH procedure

        - FFS on how to obtain the common TA, e.g., based on indication of common TA or a common reference distance

        - FFS on how to obtain the UE specific TA, e.g., additional UE-specific TA indicated in RAR or UE calculates its UE specific TA based on known location and satellite ephemeris

As noticed in [11], we cannot assume that all UEs will be always able to estimate their position with a good accuracy.

Therefore, Option 1 should be preferred, and the corresponding common TA indication should be broadcasted. This should not defend GNSS-equipped UEs to improve their acquisition information with position information.

Proposal 2: The common TA is broadcasted to all users within the coverage of same beam.

TA maintenance

As indicated above, we cannot assume that all UES will be always able to estimate their position accurately. Therefore, for TA maintenance, open loop mechanisms cannot always be robust enough. This implies that closed-loop solution should be kept., with indication of the TA adjustment over time.

Proposal 3: For TA maintenance, support at least the closed-loop solution.

# RACH

In RAN1#98 meeting, there have been several discussions on RACH format [7], in particular on the ways to enhance its format to cope with a larger coverage

It has been agreed that the current RACH format/parameters are not well suited for NTN. For example, [9] and [10] made some proposals on how to enhance the current PRACH format.

One key point here is the current PRACH is a Zadoff-Chu sequence. In NR, when discussing a new PSS design, it has been accepted that such Zadoff-Chu sequences suffer from time-frequency ambiguity performance [8], and this led to the new PSS sequence, i.e. a M-sequence. For PRACH, the situation is more difficult, as one need to design a set of orthogonal sequences, in order to distinguish between UEs transmitting simultaneously and having a higher time/frequency uncertainty with respect to the terrestrial case.

Because of this inner weakness of Zadoff-Chu sequences, we doubt that the optimal solution consists in simply modifying some parameters (CP size, subcarrier separation) or by repeating the same sequence. A redesign seems needed.

It is clear that further studies are needed on the subject, in particular for UEs having no GNSS capability.

Proposal 4: Consider study on a new PRACH sequence for NTN.

# Conclusions

Based on the discussion in this contribution, we make the following observations and proposals:

Observation 1: For a given maximum residual Doppler value after pre-correction, the maximum diameter size depends (increases) of the altitude.

Proposal 1: For residual Doppler evaluations, set the maximum acceptable footprint diameter as a function of the altitude.

Proposal 2: The common TA is broadcasted to all users within the coverage of same beam.

Proposal 3: For TA maintenance, support at least the closed-loop solution.

Proposal 4: Consider study on a new PRACH sequence for NTN.

# References

### RP-181370 Study on solutions evaluation for NR to support Non-Terrestrial Network, RAN#80.

- RP-180333 NR-NTN: NR impact area identification, initial downlink synchronization, RAN#79, Chennai, India (Thales).

### R1-1903201 Evaluation of DL synchronization and central frequency tracking for NTN, RAN1#96, February 2019, Huawei.

### R1-1905206 Doppler shift compensation in NTN, RAN1#96b, Thales.

### R1-1904644 Doppler compensation in initial access procedure in NR-NTN, RAN1#96b, Mediatek.

### 3GPP TR 38.802 v14.1.0, Study on New Radio Access Technology Physical Layer Aspects (Release 14), June 2016

### R1-1907782 Draft summary of 7.2.5.3 on UL timing and PRACH, RAN1#98, Prague, CZA, August 26-30, 2019 (ZTE)

### R1-1708160 Remaining details for synchronization signals (Huawei, HiSilicon)

### R1-1909261 RACH Procedure and UL Timing Control for NTN RAN1#98, Prague, CZA, August 26-30, 2019 (Qualcomm)

### R1-1908015 PRACH analysis under NR-NTN scenario RAN1#98, Prague, CZA, August 26-30, 2019 (Mediatek)

### R1-1908674 Discussion on UL Timing Advance and RACH, RAN1#98, Prague, CZ (Mitsubishi)