3GPP TSG RAN1 NB-IoT Ad-Hoc Meeting 	R1-160099

Budapest, Hungary, January 18-20, 2016

Agenda Item:	2.1.2.1

Source:	Huawei, HiSilicon

Title:	PAPR reduction for multi-tone PUSCH transmissions

Document for:	Discussion and decision

Introduction

At RAN#70, the NB-IoT WID [1] was revised based on the agreed WF from RAN1#83 (see [2]) to state:

- For the uplink:

  - Single tone transmissions are supported. Two numerologies should be configurable by the network for single-tone transmission: 3.75 kHz and 15 kHz. A cyclic prefix is inserted. Frequency domain sinc pulse-shaping in the physical layer description.

  - Multi-tone transmissions are supported, based on SC-FDMA with 15 kHz UL subcarrier spacing.

  - FFS: Additional mechanisms for PAPR reduction.

  - The UE shall indicate the support single-tone and/or multi-tone, details to be discussed by RAN WGs

PAPR reduction for NB-IoT uplink has been discussed in e.g. [2], [3] and [4]. Some promising results have been observed for single tone transmissions. However, the PAPR reduction schemes for multi-tone transmissions proposed so far have all come with obvious drawbacks. This document provides some evaluations on the multi-tone PAPR reduction proposal in [3]. Note that whether or not such schemes are needed within the Rel-13 timeframe still needs to be considered.

Analysis of multi-tone PAPR reduction schemes

BW efficiency

For SC-FDMA with PAPR Reduction Filter (PRF, [2~4]), one is trading PAPR reduction with excess BW. The key idea is to transmit M QAM symbols using Q>M subcarriers. PRF has been shown in [3] to significantly reduce the PAPR to, for instance, around 3 dB for QPSK, at the expense of 33% higher excess BW (i.e. with M=24 and Q=32). Specifically, the following post-DFT precoding matrix is used:

,

where

In [3] the above scheme was evaluated assuming 2.5 kHz subcarrier spacing. However, it was agreed in [1] that for multi-tone transmissions, 15 kHz subcarrier spacing is used. Hence it is interesting to understand how the scheme performs with the new subcarrier spacing. Two cases are considered here: a) 3 QAM symbols with a 5-tap filter; b) 4 QAM symbols with a 6-tap filter. The filter coefficients are shown in Table 1.

Table 1 Filter coefficients of PRF

| M | Q | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 5 | 1 | 2 | 3 | 4 | 5 | - |
|  |  | 0.1750 | 0.9846 | 1 | 0.9846 | 0.1750 | - |
| 4 | 6 | 1 | 2 | 3 | 4 | 5 | 6 |
|  |  | 0.3250 | 0.9457 | 1 | 1 | 0.9457 | 0.3250 |

Figure 1 shows the CCDF curves of SC-FDMA and SC-FDMA with PRF, with the simulation assumptions in Annex A. The results confirm that PRF can effectively reduce PAPR also with 15 kHz subcarrier spacing, e.g. 2.6 dB PAPR advantage is achieved compared to SC-FDMA with M=4, Q=6. Again, the disadvantage is that a lot of excess BW (50%) is needed in PRF, significantly reducing the spectral efficiency of SC-FDMA.

Figure 1. PAPR CCDF of SC-FDMA and SC-FDMA with PRF

Proposal 1: PAPR reduction for multi-tone transmission should not significantly reduce the bandwidth efficiency.

Modulation Scheme

Phase rotation was proposed in [5] to facilitate PAPR reduction, e.g. pi/2 for BPSK and pi/4 for QPSK. However, phase rotation, if not combined with other mechanisms, will increase the PAPR in the case of small number of allocated subcarriers (e.g. < 8 subcarriers). As shown in Figure 2, the PAPR increase can be up to 0.8 dB for QPSK with M=4.

Figure 2. PAPR CCDF of SC-FDMA and SC-FDMA with pi/4 QPSK

Observation 1: For small numbers of allocated subcarriers, such as less than 8 subcarriers, phase rotation for QPSK would increase PAPR.

SC-FDMA can be considered as a special PRF with all “1” coefficients, in which case the PAPR cannot be reduced by pi/4 QPSK. However, the PAPR can be significantly reduced if the SRRC PRF is applied as presented in Table 2.

Three modulation schemes, QPSK, pi/4 QPSK and Offset-QAM (OQAM, [6]), are evaluated. As shown in Figure 3, PRF with pi/4 QPSK and OQAM can get 0.2 dB and 1.2 dB advantages respectively, compared to PRF without phase rotation. One can further optimize the PRF coefficients to get a lower PAPR. For instance, Table 2 gives an optimized filter whose PAPR is only 1.3 dB as shown in Figure 3.

Table 2 Filter coefficients of PRF

| M | Q | Filter | Filter coefficients |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 7 | Tap No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  |  | SRRC α= 0.8 | 0.2700 | 0.7200 | 0.9456 | 1.0143 | 0.9456 | 0.7200 | 0.2700 |
|  |  | Opt. Filter | 0.4189 | 0.6200 | 0.8175 | 1.2424 | 0.8175 | 0.6200 | 0.4189 |

Figure 3. PAPR CCDF of PRF with different modulation schemes

Observation 2: The actual PAPR performance of PRF is dependent on the chosen modulation scheme.

Observation 3: For small numbers of allocated subcarriers, a specific PAPR Reduction Filter would be needed for each modulation scheme.

Conclusions

In this document, we discuss several PAPR reduction schemes for multi-tone PUSCH transmissions. Whether or not such schemes are needed within the Rel-13 timeframe still needs to be considered. The following observations and proposals are presented.

Proposal 1: PAPR reduction for multi-tone transmission should not significantly reduce the bandwidth efficiency.

Observation 1: For small numbers of allocated subcarriers, such as less than 8 subcarriers, phase rotation for QPSK would increase PAPR.

Observation 2: The actual PAPR performance of PRF is dependent on the chosen modulation scheme.

Observation 3: For small numbers of allocated subcarriers, a specific PAPR Reduction Filter would be needed for each modulation scheme.

- References

RP-152284, “Revised Work Item: Narrowband IoT (NB-IoT)”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #70, Sitges, Spain, December 7 – 10, 2015.

3GPP TR 45.820 V13.0.0, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, August 2015.

R1-156809, Performance Comparison of PAPR Reduction Schemes, Samsung, 3GPP TSG RAN WG1 Meeting #83, Anaheim, USA, November 15 – 22, 2015.

R1-156627, PAPR Reduction for Uplink of NB-IoT, ZTE, 3GPP TSG RAN WG1 Meeting #83, Anaheim, USA, November 15 – 22, 2015.

R1-156012, NB-LTE – UL Modulation, Ericsson, 3GPP TSG RAN WG1 Meeting #82bis, Malmo, Sweden, October 5 – 9, 2015.

R1-051209, OFDM/OQAM technical overview, 3GPP TSG RAN WG1#42bis, San Diego, USA, October 10 – 14, 2005.

- Simulation Assumptions

The simulation assumptions used in the simulation are listed in Table 3.

Table 3 Assumptions for Link Level Simulations

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Antenna configuration | 1 Tx × 2 Rx |
| Frequency error | F_offset(t) = F_est_error + (F_drift_inactive *T_inactive) + (F_drift_active * t). |
| NB LTE specific frequency error (F_est_error) | Randomly chosen from [-50, 50] Hz |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| Coding scheme | Turbo Code |
| Code rate | 0.35 for M=4, 0.47 for M=3 |
| modulation | QPSK |
| # repetition | 1 |
| # subcarriers | 12 |
| IFFT size | 128 |
| Channel model | TU |
| Doppler shift | 1Hz |

Note: the code rate is chosen such that the same data rate is achieved for different M’s.