3GPP TSG RAN WG1 NR Ad-Hoc Meeting                                                          R1-1700131

Spokane, USA, 16th - 20th January 2017

Source:	ZTE, ZTE Microelectronics

Title:	Beam management for DL Control Channel

Agenda Item:	5.1.2.2

Document for: 	Discussion and Decision

Introduction

In RAN1 #86bis, the following agreements were reached about beam management for control channel [1]:

- Support using same or different beams on control channel and the corresponding data channel transmissions

  - FFS the antenna ports for control channel and the corresponding data channel (e.g., sharing some ports or not)

- Study detailed aspects related to beams/beam pairs indication/reporting involving usage of control and data channels and involving one or more TRPs

In RAN1 #87, the following agreements were reached about beam management for control channel [2]:

- NR supports with and without a downlink indication to derive QCL assumption for assisting UE-side beamforming for downlink control channel reception

  - FFS: details

    - E.g., QCL assumption details

    - E.g., indication signaling (e.g. DCI, MAC CE, RRC, etc.)

    - E.g., beam-related indication for DL control and data channels

In this contribution, we further elaborate the beam management for multi-beam based DL control channel considering UE beamforming.

Discussion

In a multi-beam based communication system such as in above 6GHz bands, beam-based DL control channel transmission procedure should at least contain two stages, i.e. the initial access before the UE is connected and the connected state.

The first stage is DL control channel transmission for initial access, where the signaling from gNB or information feedback from UE can’t reach each other before the UE is connected to the network and the limited beam training results only can be carried by RACH procedure. As a result, the transmission mechanism for DL control channel should be fixed or configured with some pre-specified rules at the initial access before the UE is in the connected state. For example, only one optimal Tx beam is reported by RACH procedure, which would be used for DL control channel transmission from gNB, and the fixed transmission scheme such as SFBC is assumed. The same beam(s) could be assumed for DL control and data channel transmission for simplicity.

Proposal 1: The initial transmission scheme for the DL control channel should be fixed or configured with some pre-specified rules, and its associated beam(s) selected should be specified by RACH procedure.

The second stage is DL control channel transmission for UE being in the connected state, where beam training results reported from UE and signaling from gNB can reach each other, which give the capability of more flexibility for DL control channel transmission. The following aspects should be considered:

- Beam(s) selected for DL control channel among reported beam(s)

Based on certain type of beam measurement RS, a cluster of beam(s) is fed back by UE, which may contain multiple groups of beam corresponding different receive beam respectively. If UE is not knowing the information of the beam group used for DL control channel, the UE can’t tell which receive beam should be used for DL control channel. For example in Figure 1, two beam group, i.e. beam group A and beam group B is reported by UE, for which the correct receive beam is receive beam 1 and receive beam 2 respectively. If beam group B is selected by gNB to transmit DL control channel standing at the scheduling point, while receive beam 1 other than receive beam 2 is selected by UE without transmit beam group information to receive the DL control channel, the receive performance for DL control channel may be degraded or even can not be decoded correctly. As a result, the group of beam used for DL control channel transmission should reach a consensus between gNB and UE, e.g. through certain predefined rules or indication.

Figure 1 Multiple receive beams for the reported beam(s)

Proposal 2: The group of beam selected for DL control channel transmission should be indicated or determined by some predefined rules.

- Transmission scheme(s) determination for DL control channel

Since DL control channel transmission applied in variety of scenarios and use cases should be considered for NR, multiple transmission schemes should be supported for DL control channel, such as transmit diversity, beam/precoder cycling, and so on. UE needs to get the information of transmission scheme used for DL control channel to decode it correctly. Besides, since the transmit beam(s) or the number of transmit beam(s) may be different for different transmission schemes, the information of which can be benefit for UE to select the correct receive beam(s) to receive the DL control channel. As a result, the transmission scheme used for DL control channel should be reach a consensus among gNB and UE, e.g. through indication. Since the DL control channel should be fit for variable of scenarios and use cases, the transmission scheme(s) used for the DL control channel should be configured with some flexibility.

Proposal 3: The transmission scheme(s) used for DL control channel should be configured with some flexibility.

- Beam indication/reporting for DL control and data channel

According to the agreement last meeting, same or different beams for DL control and data channel transmission are supported. As a result, separate beam management procedure, probably including beam training, beam selection, beam reporting and beam indication, may be needed for DL control and data channel.

The same beam training procedure could be used for DL control and data channel to avoid excess resource overhead and time delay. While separate beam selection could be considered for DL control and data channel, due to that the beams or transmission schemes may be different for them. However, separate beam reporting may lead to some redundancy since there are more or less the same beam(s) used for DL control and data channel. A unified reporting including multiple beams or multiple groups of beams with separate subset used for DL control and data channel respectively according to some predefined rules for selecting the subset from the reported beams or certain distinctive identification feedback together with the reported beams. Separate indication for beam information or beam switching seems more meaningful for DL control and data channel since different beam(s) may be used for DL control and data channel and different beam switching point may happen at different time for them. Furthermore, a beam pool containing one or more groups of beams which is shared for DL control and data channel could be determined in a predefined or semi-static way, and the beam information or beam switching for DL control and data channel can be indicated separately within the beam pool, as shown in Figure 2.

Figure 2 Separate beam indication for DL control and data channel within shared beam resource pool.

Proposal 4:

- A shared beam pool can be introduced for DL control and data channel for NR.

- Separate indication for beam information or beam switching for DL control and data channel within the shared beam pool should be supported.

- Configuration for the DL control channel transmission

Different from data channel whose configuration could be indicated by DL control channel dynamically, how the DL control channel is configured needs to be carefully studied. Since the DL control channel should be fit for variable of scenarios and use cases, different beam width or group of beam or transmission scheme may be used for different scenario or use case, thus it is better configure the transmission of DL control channel with some flexibility, e.g. through certain signaling indication. Either MAC or RRC signaling could be considered, and the earlier DL control channel may be used for the configuration for the latter DL control channel. For example that if multi-level DL control channel is supported for NR, the first level of DL control channel can be configured by MAC or RRC signaling, while the second level of it can be indicated through the first level of it dynamically.

Besides, some time unit patterns may be used for DL control channel with different configurations such as different receive beam, different transmission scheme, or different level of DL control channel. The time or frequency patterns can be predefined or configured in a semi-static way. Take an example that there are multiple transmission schemes being for DL control channel transmission in Figure 3, slot pattern 1 containing slot 1, 4 and 7 in each sub-frame is assumed for DL control channel transmission based on Transmission scheme 1, and the remaining slots in each sub-frame is assumed for DL control channel transmission based on Transmission scheme 2.

Figure 3 Slot pattern for different transmission scheme for DL control channel

Proposal 5: The configuration with some flexibility for DL control channel should be carefully studied considering the scheduling limitation for gNB.

- Time unit pattern can be considered for DL control channel transmission, such as different receive beam, different transmission scheme, or different level of DL control channel.

Simulation results

The CDF of received SNR of the following two options with blockage are simulated using link-level models. Detailed simulation assumptions can be found in appendix.

Option 1: Only one best beam pair is reported for DL control channel transmission.

Option 2: Two groups of beam pairs including a transmission one and an alternative one are reported with UE-centric grouping for which more details can be found in [4], and the optimal pair for these two groups is dynamically selected with the objective of maximizing received SNR at each time for DL control channel transmission.

Figure 4 CDF of received SNR

In Figure 4, CDF of received SNR of the above three options is provided for CDL-A and CDL-B channel models with blockage comparing performance without blockage. It can be seen that Option 2 performs better than Option 1 especially for low received SNR, and up to 30dB gain can be achieved. It is because that multiple groups of beam including at least candidate group of beam can be used for against channel blockage. However, in order for the optimal pair of reported multiple groups of beams can be selected for DL control channel transmission every time, the measurement for the reported multiple groups of beam pairs should be maintained.

Observation 1: Multiple groups of beam including transmission group and at least one alternative group are benefit for improving received performance.

Proposal 6: The measurement for the reported multiple groups of beam pairs should be maintained.

Conclusion

In this contribution, some considerations on beam management for control channel are discussed. According to the discussion, the following observations and proposals are made:

Proposal 1: The initial transmission scheme for the DL control channel should be fixed or configured with some pre-specified rules, and its associated beam(s) selected should be specified by RACH procedure.

Proposal 2: The group of beam selected for DL control channel transmission should be indicated or determined by some predefined rules.

Proposal 3: The transmission scheme(s) used for DL control channel should be configured with some flexibility.

Proposal 4:

- A shared beam pool can be introduced for DL control and data channel for NR.

- Separate indication for beam information or beam switching for DL control and data channel within the shared beam poll should be supported.

Proposal 5: The configuration with some flexibility for DL control channel should be carefully studied considering the scheduling limitation for gNB.

- Slot pattern can be considered for DL control channel transmission, such as different receive beam, different transmission, or different level of DL control channel.

Observation 1: Multiple groups of beam including transmission group and at least one alternative group are benefit for improving received performance.

Proposal 6: The measurement for the reported multiple groups of beam pairs should be maintained.

- References

R1-1610511, WF on Beam management for DL control channel, ZTE, ZTE Microelectronics, Intel, Xinwei

R1-1613670, WF on beam management for control and data channel, ZTE, ZTE Microelectronics, ASTRI, Intel, Samsung, LGE

R1-1611421, Beam management for Control Channel, ZTE, ZTE Microelectronics

R1-1611416, Performance evaluation for beam grouping principles, ZTE, ZTE Microelectronics

- Appendix

Table 1: Simulation assumptions

| Parameter | Values |
| --- | --- |
| Scenarios | CDL-A, CDL-B |
| Sectorization | Random AoD offset from -60o to 60o, Random ZoD offset from 90o to 135o, Random AoA offset from -180o to 180o, Random ZoA offset from 45o to 90o |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (4,8,2), (dV,dH) = (0.5, 0.5)λ, 2 RF chains, Antenna modelling based on TR38.900 V2.0.0, i.e. HPBW = 65o, GE,max =8dBi with Am = 30dB |
| UE antenna configuration | Mg = Ng = 1; (M,N,P) = (2, 4, 2), (dV,dH) = (0.5, 0.5)λ, 2 RF chains, Antenna modelling based on TR38.900 V2.0.0, i.e. HPBW = 90o, GE,max =5dBi with Am = 30dB |
| BS and UE Analog codebook | Two dimensional DFT based with oversampling equals one |
| Carrier Frequency | 30 GHz |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 60kHz |
| UE speed | 3km/h |
| Blockage model | Blockage Model A TR38.900 |