3GPP TSG RAN WG1 AH_NR Meeting	              	   R1-1700231

Spokane, USA, 16th - 20th January 2017

Source:	CATT

Title:	Discussion on DMRS design for UL

Agenda Item:	5.1.2.3.2

Document for:	Discussion and Decision

Introduction

In RAN1#86bis, it was agreed that [1]:

- NR Support DFT-S-OFDM based waveform complementary to CP-OFDM waveform, at least for eMBB uplink for up to 40GHz

- FFS additional low PAPR techniques

- CP-OFDM waveform can be used for a single-stream and multi-stream (i.e. MIMO) transmissions, while DFT-S-OFDM based waveform is limited to a single stream transmissions (targeting for link budget limited cases)

- Network can decide and communicate to the UE which one of CP-OFDM and DFT-S-OFDM based waveforms to use

  - Note: both CP-OFDM and DFT-S-OFDM based waveforms are mandatory for UEs

- RAN1 should target for a common framework in designing CP-OFDM and DFT-S-OFDM based waveforms (without compromising CP-OFDM performance/complexity), e.g., control channels, RS, etc.

- Discuss further offline for possible refined evaluation assumptions/methodology for waveform evaluations

In 3GPP RAN 1#87, the following agreements on DMRS have been reached [2]:

- Support variable/configurable DMRS pattern for data demodulation

  - FFS: Time and/or frequency domain density can be configured

  - FFS: RE location can be configured

  - At least one configuration supports front-loaded DMRS pattern

Based on the agreements listed above, in this contribution, we present our considerations on DMRS designs for UL in NR.

DMRS design for UL

In legacy LTE system, considering the efficiency of power amplifier and coverage in UL, DFT-S-OFDM was adopted. This framework has essential influence on the design of physical signal and channels in UL. As mentioned above, it’s agreed that DFT-S-OFDM is to be supported as complementary to CP-OFDM waveform. Furthermore, CP-OFDM waveform can be used for a single-stream and multi-stream (i.e. MIMO) transmissions, while DFT-S-OFDM based waveform is limited to a single stream transmissions (targeting for link budget limited cases). Therefore, it’s reasonable to keep commonality in DMRS design for UL and DL, if both are CP-OFDM based.

Proposal 1: it’s reasonable to keep commonality in DMRS design for UL and DL for CP-OFDM waveform..

In the last meeting, agreement on supporting configurable DMRS pattern for data demodulation has been reached. Based on this agreement, we discussed the DMRS design for DL in one of our company’s contributions [3]. For sake of commonality between UL and DL in DMRS design, the same pattern designed for DL also applies to UL transmission for  CP-OFDM waveform in UL. One motivation of introducing common design between DL and UL is to simplify interference management in dynamic TDD system.

Proposal 2: for DMRS pattern in CP-OFDM waveform based UL, introduce base pattern as basic unit, based on which DMRS patterns with configurable densities in time and frequency domains and flexible time-domain location are achievable.

The design principles based on base pattern are summarized as follows:

- The base patterns presented define the REs occupied for DMRS transmission in one or two adjacent symbols.

- Density in frequency domain of DMRS is adjustable by selecting difference base pattern

- Time domain density of DMRS can be adjusted by configuring the number of repetition for base pattern in one slot

- Front-loaded pattern is supported by placing base pattern right at the beginning of data transmission

- CDM-F is used for rank1-4, while CDM-F+TDM can be used for rank 5-8

- More than 8 orthogonal ports can be supported by extending the base pattern in more symbols

Examples of base patterns and patterns for 7 and 14-symbol slots are shown in Figure 1,2 and 3. It’s noted that, for DL, the first two symbols are assumed to be reserved for control channel. However, depending on detailed design and configuration, the first two symbols are not necessarily to be control channel in UL. Anyway, as the base pattern can be inserted in the first symbol of data transmission, front-loaded RS for UL is still achievable.

Figure.1 DMRS base patterns

Figure.2 DMRS patterns for 7-symbol slot

Figure.3 DMRS patterns for 14-symbol slot

For DMRS design in DFT-S-OFDM waveform based UL, as only rank 1 is supported, re-using of the DMRS structure defined for LTE to the largest extent is preferred. Similar to DL DMRS design based on base pattern, the DMRS-containing symbol same as LTE UL can start with any symbol within a slot, and then the base pattern can be repeated in time domain to fit the required density. In addition, it’s also possible to consider of using the same DMRS patterns for all waveforms.

Proposal 3: for DMRS design in DFT-S-OFDM waveform based UL, reuse the structure of LTE UL DMRS as much as possible.

Proposal 4: it’s also possible to consider of using the same DMRS patterns for all waveforms.

Conclusions

This contribution provides our considerations on UL DMRS design for NR MIMO. Based on the discussion above, we have the following proposal:

Proposal 1: it’s reasonable to keep commonality in DMRS design for UL and DL, if both are CP-OFDM based.

Proposal 2: for DMRS pattern of CP-OFDM based UL, introduce base pattern as basic unit, based on which DMRS patterns with configurable densities in time and frequency domains and flexible time-domain location are achievable.

Proposal 3: for DMRS pattern of DFT-S-OFDM based UL, reuse the structure of LTE UL DMRS as much as possible.

Proposal 4: it’s also possible to consider of using the same DMRS patterns for all waveforms.

- References

- RAN1 #86bis Chairman’s Notes.

- RAN1 #87 Chairman’s Notes.

- R1-1700230, “Discussion on DMRS design for DL”, CATT.