3GPP TSG RAN WG1 NR ad-hoc		 			R1-1700524

Spokane, USA 16th - 20th January 2017

Agenda Item:	5.1.5.2.2

Source: 	LG Electronics

Title: 	Control channel coding scheme for small block length

Document for:	Discussion and decision

- Introduction

In last RAN1 #87 meeting, the polar code was adopted for uplink control channel and became working assumption for downlink control channel [1]. For control channel, it was agreed to evaluate different channel codes based on agreement for simulation assumptions.

In this contribution we evaluate channel coding performances based on the control channel assumptions.

Discussion

The simulation assumption for channel coding performance evaluation for the control channel is described as follows:

Table 1 Simulation assumptions for control channel

| Channel | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK |  |  |  |
| Coding scheme | Repetition | Simplex | Reed-Muller | Polar |
| Decoding algorithm | 1/24, 1/12, 1/6, 1/3, 1/2 |  |  |  |
| Decoding algorithm | ML | ML | FHT | SC |
| Info Block length | 1, 2, 8, 16, 20 |  |  |  |

Based on the agreement in [2], code rate 1/24 is valid for info block length of 1~2bits. For Reed-Muller code, we reuse design rules which are defined in LTE spec. Since block length is small, we assume successive cancellation decoding algorithm to avoid performance degradation due to CRC overhead of CRC-aided list decoding. For the Polar code evaluation, the Gaussian approximation algorithm is used to determine location of frozen bit [3]. To achieve best performance for polar code, the native rate which is close to the target rate is selected for each scenario. For the polar code rate matching, we use QUP (Quasi Uniform Puncturing) pattern for puncturing which is described in [4].

Base on the simulation result, we evaluate the performance of control channel coding schemes. Simulation results are attached in Appendix.

Information: 1-2bits

For the 1-2bits information, we compare the simulation results for repetition, simplex and Reed-Muller codes. In case of 1 bit, repetition code and Reed-Muller code have same encoding / decoding structure. Thus, they show same performance on 1 bit information. As shown in Figure 1, repetition code and Reed-Muller code shows better performance than simplex code. Meanwhile, in case of 2 bits, simplex code shows the best performance for all code rates as shown in Figure 2. Repetition code and Reed-Muller code show similar performance in this bit size. Thus it is preferable to use repetition code for 1 information bit and simplex code for 2 bits information like in LTE

Observation 1: When information bit size is 1, repetition code and Reed-Muller code performs better than simplex code.

Observation 2: When information bit size is 2, simplex code performs better than repetition code and Reed-Muller code.

Proposal 1: Use repetition code when information bit is 1, and use simplex code when information bit is 2.

Information: 8bits

For the 8bits information, we compare the simulation results for Reed-Muller codes and polar codes. In this region, Reed-Muller code outperforms polar code in all cases as shown in Figure 3.

Observation 3: When information bit size is 8, Reed-Muller code outperforms polar code.

Proposal 2: Use Reed-Muller code when information bit is 8.

Information: 16 bits

For the 16 bits information, we compare the simulation results for Reed-Muller code and polar code. Figure 4 shows that polar code has comparable or better performance than polar.

Observation 4: When information bit size is 16, polar code shows comparable or better performance than Reed-Muller code.

Information: 20 bits

For the 20 bits information, we compare the simulation results for Reed-Muller code and polar code. 5 shows that polar code has comparable performance than Reed-Muller code.

Observation 5: When information bit size is 20, polar code shows comparable performance than Reed-Muller code.

- Conclusion

In this contribution, we present performance results to compare control channel coding schemes. The observations and proposal of our contributions are as follows:

Observation 1: When information bit size is 1, repetition code and Reed-Muller code performs better than simplex code.

Observation 2: When information bit size is 2, simplex code performs better than repetition code and Reed-Muller code.

Proposal 1: Use repetition code when information bit is 1, and use simplex code when information bit is 2.

Observation 3: When information bit size is 8, Reed-Muller code outperforms polar code.

Observation 4: When information bit size is 16, polar code shows comparable or better performance than Reed-Muller code.

Observation 5: When information bit size is 20, polar code shows comparable performance than Reed-Muller code.

- Reference

- [1]	TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies

- [2]	Chairman’s Notes RAN1 #86

- [3]	Trifonov, Peter. “Efficient design and decoding of polar codes.” IEEE Transactions on Communications 60.11 (2012): 3221-3227.

- [4]	Chung, Sae-Young, Thomas J. Richardson, and Rüdiger L. Urbanke. “Analysis of sum-product decoding of low-density parity-check codes using a Gaussian approximation.” IEEE Transactions on Information Theory 47.2 (2001): 657-670.

- Appendix: Simulation results

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5