3GPP TSG RAN WG1 Meeting NR Adhoc       		                                               R1-1700860

Spokane, USA 16th - 20th January 2017

Agenda Item:	5.1.3.1

Source:	              NEC

Title:	             Numerology indication for a mixed numerology carrier

Document for:	Discussion/Decision

Introduction

In RAN1#86bis, the frequency domain multiplexing pattern for different numerologies has been agreed as following:

Working assumption:

- Adopt RB grid for FDM as it was agreed in TDM

And the time domain alignment is agreed as:

Agreements:

- Alignment within a subframe

  - Symbol level alignment across different subcarrier spacings with the same CP overhead is assumed within a subframe duration in a NR carrier

  - FFS: Unlicensed spectrum case

- For normal CP family, the following is adopted

  - For Fs = 15 kHz * 2n (n is non-negative integer),

    - Each symbol length (including CP) of 15 kHz equals the sum of the corresponding 2n symbols of Fs

    - Other than the first OS in every 0.5msec, all OFDM symbols within 0.5msec have the same size

    - The first OS in 0.5msec  is longer by 16 Ts (assuming 15 kHz and FFT size of 2048) compared to other OSs

      - 16 Ts is used for CP for the first symbol

  - For Fs = 15 kHz * 2n (n is a negative integer)

    - Each symbol length (including CP) of Fs equals the sum of the corresponding 2-n symbols of 15 kHz

In addition, dynamic eMBB and URLLC service multiplexing is agreed as following:

Agreements:

- From network perspective, multiplexing of transmissions with different latency and/or reliability requirements for eMBB/URLLC in DL is supported by

  - Using the same sub-carrier spacing with the same CP overhead

    - FFS: different CP overhead

  - Using different sub-carrier spacing

    - FFS: CP overhead

  - NR supports both approaches by specification

- NR should support dynamic resource sharing between different latency and/or reliability requirements for eMBB/URLLC in DL

In RAN1#87,  the agreements regarding the dynamic resource allocation of different numerologies are as following:

Agreements:

- NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion.

  - Potential specification impact includes but is not limited to:

    - FFS:CSI-RS measurement

  - FFS: the time and frequency granularity of dynamic resource allocation

    - FFS: If spectrum confinement (filtering, windowing, …) can be dynamically varied or not

With above agreements in mind, UE needs to know the numerology for a specific time-frequency resource for decoding. In this contribution, we mainly focus on the numerology indication and frequency resource allocation in a mixed numerology carrier.

Discussion

Basic unit for numerology indication

Different numerologies have different subcarrier spacings and symbol durations. A PRB with 12 subcarriers is agreed to be the frequency domain unit for scheduling and RS design. And time domain scheduling unit will be selected from multiple symbols, which may be 2, 7, or 14, etc. As a result, for a given subcarrier number and symbol number within a set of time-frequency resource, different numerologies will have different expressions, which can be shown in the following Fig 1.

Fig 1. Time-frequency resource unit for different numerologies

If legacy RS pattern and other design based on the PRB are to be kept, the numerology granularity should be at least a PRB, which is 12 subcarriers in frequency domain and several symbols in time domain. For multiple numerologies, the largest subcarrier spacing should be considered in frequency domain, consequently, a basic numerology variation unit in frequency domain should be the PRB size with the largest subcarrier spacing, which is shown in following Fig 2.

Fig 2. Basic unit for multiple numerologies

For UE perspective, numerology on every basic unit for possible resource allocation should be available before data decoding, then the numerology indication can be fulfilled by several ways:

- Option 1: Indicate numerology for each basic unit;

- Option 2: Divide the basic units to different groups and indicate numerology for each group.

For Option 1, the indication is simple, flexible and straight forward with large signaling overhead. For Option 2, the group pattern needs to be predefined or configured firstly, and overall signaling may be less than Option 1. There can be various grouping patterns for Option 2, such as frequency domain repetition, grouping with restriction on the group numbers.

Guard band may also be necessary depends on the waveform adopted. If guard band is to be reserved, indication of this guard band information is useful for UE reception. The guard band indication can be combined together with the numerology indication, for the reason that with change of numerology of adjacent basic unit, there is a necessity of guard band.

Proposal 1: Define the basic unit for numerology indication.

Time-frequency resource for numerology indication

Although the numerology should be optimized based on the latency requirements of different traffics and impairment of equipment such as phase noise, deployment scenarios such as Doppler shift, etc., the numerology indication information should be carried by the time-frequency resource without numerology change to guarantee reliability. The reference numerology with 15KHz agreed in previous meeting is a good candidate place for this indication for below 6GHz. And for above 6GHz, a non-dynamically changing or static numerology can be used. Semi-static or dynamic information can be carried on the time-frequency resource corresponding to the static or reference numerology.

Proposal 2: The numerology indication information is located at the static or reference numerology.

Resource allocation

UE needs to know the detailed frequency domain resources for UL/DL data communication. With mixed numerology in a single carrier, the situation is more complicated. There are mainly two methods:

- Option a: Two step resource allocation indication based on the basic unit and the PRB within the basic unit.

- Option b: Resource allocation based on the smallest frequency domain PRB size.

For Option a, as the basic unit is a unified concept regardless of the adopted numerology, the basic unit index/indices can be informed to UE firstly, and then the PRB indices within one or more basic units can be indicated as the second step, which is shown in following Fig 3.

Fig 3: Two-step resource allocation

For Option b, the smallest frequency domain PRBs is also a unified concept irrespective of the numerology, which is shown in following Fig 4.

Fig 4. Resource allocation based on the smallest frequency domain PRB

The disadvantage of Option b is that signaling overhead may be large, especially for very large system bandwidth.

Proposal 3: Study the resource allocation for mixed numerology in a single carrier.

Conclusion

In this contribution, we discussed the remaining issues on mixed numerology in a single carrier, and our proposals are as following:

Proposal 1: Define the basic unit for numerology indication.

Proposal 2: The numerology indication information is located at the reference numerology.

Proposal 3: Study the resource allocation for mixed numerology in a single carrier.

- References

RAN1#86bis Chairman notes

RAN1#87 Chairman notes