3GPP TSG RAN WG1 NR Ad-hoc Meeting 	R1-1700906

# Spokane, USA, 16th – 20th January 2017

Agenda item:	5.1.2.1

Source:	Samsung

Title:	Evaluation assumption for advanced receivers based on network coordination

Document for:	Discussion and decision

- Introduction

In RAN1#87, the following agreement on NR network coordination aspects was made [1].

Agreements: - For advanced receivers based on network coordination, system-level simulation are encouraged to be evaluated in NR study item - For system-level simulations, urban macro scenario, dense urban scenario excluding small cells, indoor hotspot scenario, and dense urban scenario including small cells with the same carrier frequency are encouraged to be evaluated in NR study item - Simulation assumptions of TR 38.802 can be a starting point - FFS whether or not to further update the simulation assumptions - Details on additional information assumed in evaluations should be provided by each company R1-1613681 WF on Evaluation Assumptions for Advanced Receivers based on Network Coordination

In this contribution, we will focus on the system-level simulation (SLS) evaluation assumptions and parameters for the performance of advanced receivers based on network coordination.

- Updated proposals on system-level evaluation assumptions

The system-level evaluation assumptions were agreed as above in the last meeting as a first step to verify the performances of advanced receivers. While it will have a massively large antenna arrays with SU/MU-MIMO operations, NR will also have a vast majority of gNBs with a small number of TXRUs, such as two to four TXRUs, where beamforming is not used. Thus, we also need to investigate the same antenna array configuration with smaller number of CSI ports and the other array configurations with smaller number of CSI ports.

Proposal 1: For evaluating advanced receiver schemes based on NR network coordination, it is recommended that the following additional four cases for TRP be included in NR study item:

- (M,N,P,Mg,Ng) = (8,8,2,1,1) with 16 CSI ports

- (M,N,P,Mg,Ng) = (8,4,2,1,1) with 8 CSI ports

- (M,N,P,Mg,Ng) = (8,2,2,1,1) with 4 CSI ports

- (M,N,P,Mg,Ng) = (8,1,2,1,1) with 2 CSI ports.

Updated parameters of system-level simulation evaluations for advanced receivers based on network coordination are provided in Table 1 in Appendix.

- Conclusion

This contribution discussed the SLS evaluation assumptions and parameters for comparing the performances of advanced receivers based on network coordination during the period of the NR study item. Base on the above discussion, our proposal is made as follows:

Proposal 1: For evaluating advanced receiver schemes based on NR network coordination, it is recommended that the following additional four cases for TRP be included in NR study item:

- (M,N,P,Mg,Ng) = (8,8,2,1,1) with 16 CSI ports

- (M,N,P,Mg,Ng) = (8,4,2,1,1) with 8 CSI ports

- (M,N,P,Mg,Ng) = (8,2,2,1,1) with 4 CSI ports

- (M,N,P,Mg,Ng) = (8,1,2,1,1) with 2 CSI ports.

- References

- 3GPP Chairman’s Notes, RAN1#87, Reno, NV, November 2016.

Appendix

Table 1: Parameters of system-level simulation evaluations for advanced receivers based on network coordination

| Parameters | Urban Macro | Dense urban (Single or Dual layer) | Indoor hotspot |
| --- | --- | --- | --- |
| Carrier frequency | 4GHz | Macro layer: 4GHz Small cell layer: 4GHz (co-channel) | 4GHz, 30GHz |
| Subcarrier spacing | 15kHz |  |  |
| System Bandwidth | 20MHz |  |  |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (optional: 7 macro sites, similar to that in SCE SI | Macro Layer: Hexagonal grid, 3 sectors per site, 19 macro sites (optional: 7 macro sites, similar to that in SCE SI) Small cell layer: small cells uniformly random dropping within macro geographical area | Indoor floor: 12 BSs per 120m x 50m |
| ISD | 500m | Macro layer: 200m | 20m |
|  |  | Small cell layer: Randomly |  |
| Number of Small Cell | - | 3,9 (Optional) | Candidate TRP numbers: 3,6,12 |
| Small cell TP dropping | - | According to TR 38.802 | - |
| Minimum distances | 35m | 35m for macro cells and 5m for micro cells | - |
| Channel model | 3D UMa | 3D UMa (Macro layer) and 3D UMi (Micro layer) | ITU InH |
| TP Tx power | 49dBm/20MHz | 44dBm/20MHz | 24dBm/20MHz |
| TP antenna configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (M,N,P,Mg,Ng) = (8,4,2,1,1) (M,N,P,Mg,Ng) = (8,2,2,1,1) (M,N,P,Mg,Ng) = (8,1,2,1,1) (dH,dV) = (0.5, 0.8)λ. | (M,N,P,Mg,Ng) = (8,8,2,1,1) (M,N,P,Mg,Ng) = (8,4,2,1,1) (M,N,P,Mg,Ng) = (8,2,2,1,1) (M,N,P,Mg,Ng) = (8,1,2,1,1) (dH,dV) = (0.5, 0.8)λ. | (4,4,2,1,1) for 4GHz (4,8,2,1,1) for 30GHz |
| TP antenna pattern | According to TR 36.873 |  |  |
| TP antenna height | 25m | 25m for macro cells and 10m for micro cells | 3m |
| UE antenna configuration | Cross Pol (-45/45 degree) |  |  |
| UE antenna pattern | Omnidirectional |  |  |
| UE antenna height | follow 38.802 |  |  |
| UE antenna gain | 0 dBi |  |  |
| UE receiver noise figure | 9 dB |  |  |
| UE Dropping | indoor UE 80%, outdoor UE 20% |  | indoor UE 100% |
| UE speed | 3km/h, 30km/h (follow 38.802) |  | 3km/h (follow 38.802) |
| UE receiver | Baseline for calibration purpose : MMSE-IRC Advanced receiver : advanced receivers can be provided by each company |  |  |
| Association of UE to TP | Baseline: RSRP for intra-frequency |  |  |
| Transmission scheme | closed-loop rank 1 and 2 SU-MIMO with rank adaptation, open-loop rank 1 and 2 SU-MIMO with rank adaptation (optional), other ranks are not precluded. MU-MIMO are not precluded |  |  |
| Coordination cluster size for ideal backhaul | Provided by each company |  |  |
| Coordinated TP measurement set size | Provided by each company |  |  |
| Feedback assumption | Non-ideal CSI-RS/IMR channel/interference estimation (# of CSI ports = 2, 4, 8, 16, 32) |  |  |
| Traffic model | Non full buffer FTP traffic model 1/3, S = 0.1Mbytes (optional) or 0.5Mbytes |  |  |
| Traffic load (Resource utilization) | 20%, 40%, 60%, Optional 80% |  |  |
| RS modelling | Realistic |  |  |
| Channel estimation | Realistic |  |  |
| Overhead modelling | Realistic |  |  |
| Handover margin | 3dB |  |  |
| Backhaul link delay | 0ms, 2ms (optional), 5ms, 50ms (optional). Other values are not precluded. Report by each company |  |  |
| Coordination assumptions | Complexity of coordination / information exchange shall be taken into account |  |  |