3GPP TSG RAN WG1 NR AH Meeting	R1-1700928

# Spokane, USA, 16th – 20th January 2017

Agenda item:	5.1.2.3.1

Source: 	Samsung

Title: 	Discussions on CSI-RS design for NR MIMO

Document for:	Discussion and Decision

Introduction

In RAN1#86bis and RAN1#87, the following agreements on CSI-RS were made [1], [2]:

Agreements: - At least the following RSs are supported for NR downlink - CSI-RS: Reference signal with main functionalities of CSI acquisition, beam management - FFS: RRM measurement Agreements: - Support NR CSI-RS pattern with at least the following properties: - CSI-RS mapped in one or multiple [consecutive] symbols - FFS: At least CSI-RS located at the earlier part of a slot in some cases - FFS: before or after the DM-RS agreed to study in RAN1#85 - FFS: CSI-RS located at other part of a slot - Working assumption: Configurable density of CSI-RS in frequency and/or time domain by UE-specific manner - Working assumption: CSI-RS for NR should support up to 32 ports - FFS: whether or not to have 32 ports codebook - Support at least following configurations of NR CSI-RS - UE-specific configuration to support - Wideband CSI-RS, i.e. from UE perspective, the full bandwidth the UE is configured to operate with - Partial-band CSI-RS, i.e. from UE perspective, part of the bandwidth the UE is configured to operate with - FFS: Different patterns may be used for wideband and subband CSI-RSs Agreements: - CSI-RS configuration for NR includes at least ‘number of antenna ports’ - Configuration can be explicit or implicit - The number of CSI-RS antenna ports can be independently configured for periodic/semi-persistent CSI reporting and aperiodic CSI reporting Working assumption: - The number of antenna ports supported for CSI-RS configuration includes at least the following values - 1,2,4,8,[12],16,[20],[24],[28],32 - RAN1 will update/remove above bracket parts in the next meeting - Study e.g., performance, scenario, RS overhead, RS pattern considering the frame structure, reuse of hardware between NR and LTE Agreements: - NR supports semi-persistent CSI-RS transmissions - Activation(s)/de-activation(s) of CSI-RS resource is triggered dynamically - Note: “dynamically” here can be DCI and/or MAC CE based. FFS details. - NR supports semi-statically configured/re-configured periodic CSI-RS transmissions - FFS: Details on signaling mechanisms Agreements: - NZP CSI-RS resource is defined in NR, as a set of NZP CSI-RS port(s) mapped to a set of REs within a frequency span/a time duration (details FFS) which can be measured at least to derive a CSI - Multiple NZP CSI-RS resources can be configured to UE at least for supporting CoMP and multiple beamformed CSI-RS based operations, where - Each NZP CSI-RS resource at least for CoMP can have different number of CSI-RS ports. - Further study at least the following aspects: - FFS QCL aspects - E.g., set of QCL parameters - E.g., QCL assumptions within a NZP CSI-RS resource, among two or more resources, etc. - FFS whether or not a single NZP CSI-RS resource can be used to derive two or more CSIs - FFS whether or not many NZP CSI-RS resources can be used to derive a single CSI Working assumptions: - For Type I, CSI feedback using a PMI codebook for X CSI-RS ports is supported - Supported values of X are at least 1,2,4,8,12,16,[24],32 - Note: For X=1, Type I does not have PMI feedback - Support for other values of X is not precluded - Note the number of ports in CSI-RS resource configuration may not be the same with the number of ports in the PMI codebook

This contribution provides Samsung’s views on design principles of CSI-RS for NR MIMO, focusing on the highlighted parts above.

Design principle of NR CSI-RS

According to the agreements in RAN1#86bis [1], NR CSI-related setting consists of following three settings; 1) CSI reporting settings, 2) RS settings, 3) CSI measurement settings. Among these three component settings, we will focus on the design principle of the RS setting for CSI-RS in this contribution.

As intensively discussed in RAN1, NR CSI-RS design is desirable to satisfy various functionalities instead of its diverging use cases. For instance, it have been widely studied from FD-MIMO SI to eFD-MIMO WI that the required features at CSI measurement aspects such as wide coverage and high penetration cannot be met by a single type of CSI-RS design. However, streamlining the framework for CSI-RS in LTE-A Pro is a good starting point of NR CSI-RS design. Based on this, we can avoid introducing multiple classes/subclasses and redundant configurations while keep its various features.

Based on the discussions above, we propose to introduce dual-level CSI-RS comprise at least following two types. For the non-UE-specific RS use case for CSI measurement, functionalities analogous to LTE NP CSI-RS and cell-specific BF CSI-RS (both composed of one or more static macro-beams) can be combined into one type, termed the level-1 CSI-RS which is composed of K≥1 CSI-RS resources. Each of the K cell-/group-specific beam(s) represented by each of the K CSI-RS resources can cover relatively wide coverage. As evident, the use cases of the level-1 CSI-RS are the combination of use cases of Rel.13 NP CSI-RS and cell-specific BF CSI-RS. When used for virtual sectorization, a UE can be configured with CRI feedback. In addition, partial-port CSI-RS (CSI-RS subsampling in port-domain) can be implemented by configuring a UE to measure K > 1 CSI-RS resources, each resource representing a part of the larger port array. The second type of CSI-RS, analogous to Rel.13 UE-specific BF CSI-RS, is termed the level-2 CSI-RS. Sharing the same characteristics, level-2 CSI-RS can be composed of UE-specific beams and provides high penetration at relatively narrow coverage than the level-1 CSI-RS.

Since the level-1 CSI-RS are supposed to perform RRM/CSI measurement without (or probably with coarse) priory information on CSI, wideband transmission of CSI-RS would be beneficial. However, in case of level-2 CSI-RS, wideband CSI-RS transmission may not be required always because of the long-term wideband CSI from level-1 CSI-RS. Moreover, partial-band CSI-RS transmission of level-2 CSI-RS will reduce RS overhead for the level-2 CSI-RS that requires UE-specific beam direction and dedicated CSI-RS resource per UE.

Another important use case of level-1 CSI-RS is time/frequency tracking for PDSCH decoding. As widely studied in the past releases, PSS/SSS transmitted in subband might not be enough to provide fine time/frequency synchronization for PDSCH decoding. Therefore, to aid SIB(s) decoding, which probably be transmitted via wider bandwidth than that for MIB, wideband level-1 CSI-RS with restricted cell-specific configuration can be a good candidate for fine time/frequency tracking. In this case, the restricted cell-specific CSI-RS configuration can be informed to UEs via MIB.

Table 1 summarizes the comparison between these two CSI-RS types.

Table 1. Dual-level CSI-RS

| System aspect | Level-1 CSI-RS | Level-2 CSI-RS |
| --- | --- | --- |
| CSI-RS resource usage | Non-UE-specific RS use cases for CSI measurement, RRM measurement, and time/frequency tracking. E.g., one CSI-RS resource per “coverage beam”, each associated with nk (1,2,4,.., 32) CSI-RS ports, K ≥ 1 CSI-RS resources | UE-specific RS use cases for CSI measurement. E.g. one CSI-RS resource per UE with m (2,4,6,8) ports, m ≤ nk |
| CSI-RS “beams” | Cell-/group-specific beam direction(s) with wide coverage | UE-specific beam direction(s) with narrow coverage |
| Transmission bandwidth | Wideband | Partial-band |
| Use cases | - Virtual sectorization - “Partial port” CSI-RS mapping - Time/frequency tracking for PDSCH decoding | - UE-specific beamformed CSI-RS |

As described by Figure 1, a gNB can transmit two first-level CSI-RSs on two cell-specific beams to two UEs. These two cell-specific beams 0 and 1 (hence two level-1 CSI-RSs) can be transmitted simultaneously or multiplexed in time or frequency. The gNB transmits two level-2 CSI-RSs 0.0 and 0.1. The two UE-specific beams utilized for transmitting the two second-level CSI-RSs are derived relative to the first cell-specific beam and hence correspond to subspaces of the cell-specific beam. Since these two UE-specific beams are transmitted along a smaller range of directions, they have higher penetration due to additional beamforming gains. The second UE measures second-level CSI-RS and receives data transmission from the gNB along a data beam 0.1.0. Beamforming vector for the data beam can be formed either from a CSI reporting derived from the second-level CSI-RS (more relevant for FDD) or measuring UL SRS (more relevant for TDD). Multiple CSI-RS resource components per each CSI-RS level can be studied for ease of beam direction acquisition considering operation scenarios according to the various environments such as granularity of virtual sectorization, UE mobility, etc.

Figure 1. NR MIMO operation based on Dual-level CSI-RS

Considering various aspects such as efficient resource utilization and interference management, on-demand transmission of CSI-RS seems to be a primary operational mode. On the other hand, on-demand transmission of CSI-RS may cause burdensome signaling overhead if the UE needs to report CSI only for link maintenance. In this case, periodic CSI-RS transmission might be beneficial for both of gNB and UE complexities. As a hybrid type of both configurations, semi-persistent CSI-RS configurations can be proper in case that moderate data rate and transmission periodicity are required, e.g. for voice service.

Support of forward compatibilities and co-existence with LTE/LTE-A is another important issue in NR. CSI-RS patterns based on small time/frequency granularity might be a good starting point to cover both of forward compatibilities and co-existence with LTE/LTE-A. For instance, given that NR may have large dynamic range in time domain from multiple TTIs and/or numerologies, short time duration of CSI-RS pattern can be good for ease of RS interference management. Therefore, small time granularity such as CSI-RS patterns within one OFDM symbol can be considered. In LTE-A, “aggregable CSI-RS resource” was introduced for larger than 8 ports CSI-RS to provide high flexibility for various purposes such as CSI-RS interference management, CSI-RS port sharing, etc. Analogous to LTE-A, the concept of CSI-RS aggregation can be adopted with new CSI-RS component resource. The CSI-RS component resource is defined as the base unit for CSI-RS aggregation in NR. Considering the need of CSI-RS transmission within one OFDM symbol, it is proposed to define the CSI-RS component resource as two adjacent REs in frequency domain. A gNB can aggregate multiple CSI-RS component resource, taking into account various aspects such as the number of antenna ports in a CSI-RS resource, required CSI-RS RE patterns for efficient CSI-RS interference management, etc. For co-existence with LTE/LTE-A and NR, it is important to secure that one of the subsets of the resulting CSI-RS resource for NR should be included in the CSI-RS resource pool (CSI-RS REs) for LTE/LTE-A to support a proper rate matching of legacy UEs.

Based on above discussions, we propose following CSI-RS design principles for NR:

Proposal: Following CSI-RS design principles should be considered for NR

A dual-level CSI-RS structure includes:

A level-1 CSI-RS to cover ‘non-UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be cell-/group-specific; and the CSI-RS beam(s) can provides wide coverage

Support wideband transmission

Restricted cell-specific configuration is signalled via MIB to support time/frequency tracking for PDSCH decoding

A level-2 CSI-RS to cover ‘UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be UE-specific; and the CSI-RS beam(s) can provides narrow coverage

Support partial-band transmission

Support of high flexibility via small time/frequency granularities

A CSI-RS resource can be aggregated by multiple CSI-RS component resources

A CSI-RS component resource comprises two adjacent REs in frequency domain

Aggregation can be applied for both time and frequency domain

One of the subsets of the resulting CSI-RS resource for NR should be included in the CSI-RS resource pool (CSI-RS REs) for LTE/LTE-A

Conclusions

In this contribution, Samsung’s view of CSI-RS for NR is presented. Based on above discussions, the following proposals are made:

Proposal: Following CSI-RS design principles should be considered for NR

A dual-level CSI-RS structure includes:

A level-1 CSI-RS to cover ‘non-UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be cell-/group-specific; and the CSI-RS beam(s) can provides wide coverage

Support wideband transmission

Restricted cell-specific configuration is signalled via MIB to support time/frequency tracking for PDSCH decoding

A level-2 CSI-RS to cover ‘UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be UE-specific; and the CSI-RS beam(s) can provides narrow coverage

Support partial-band transmission

Support of high flexibility via small time/frequency granularities

A CSI-RS resource can be aggregated by multiple CSI-RS component resources

A CSI-RS component resource comprises two adjacent REs in frequency domain

Aggregation can be applied for both time and frequency domain

One of the subsets of the resulting CSI-RS resource for NR should be included in the CSI-RS resource pool (CSI-RS REs) for LTE/LTE-A

- References

- 3GPP, RAN1#86bis, Chairman’s Notes

- 3GPP, RAN1#87, Chairman’s Notes