3GPP TSG RAN WG1 NR Ad-Hoc Meeting	R1- 1700980

# Spokane, USA, 16th – 20th January 2017

Agenda item:	5.1.5.3

Source: 	Samsung

Title: 	Discussion on Channel Coding for eMBB Data Channel

Document for:	Discussion and Decision

Introduction

In the RAN1 #87 meeting, it was agreed that

Agreement: - UL eMBB data channels: - Working Assumption to adopt flexible LDPC as the single channel coding scheme for small block sizes (to be confirmed unless significant issues are identified by the RAN1 Jan adhoc in relation to performance, implementation complexity and flexibility) - (Note that it is already agreed to adopt LDPC for large block sizes) - DL eMBB data channels: - Adopt flexible LDPC as the single channel coding scheme for all block sizes

In this contribution, we summarized our companion contributions [2-7] to select the channel coding scheme for small block size of UL eMBB data channels for NR.

Discussion

The channel coding scheme for data channel should be selected based on the relevant requirements; performance, flexibility, latency and complexity. To compare of candidate channel coding schemes in fair way, we present the code block error rate performance for each coding scheme when false alarm rates and decoding complexity are almost the same for all codes. In this section, we provide the analysis results taking into account this fair comparison.

  - Performance

In [2], we present the simulation results of the coding performance with the same computational complexity and information block size (<= 1024) for fair comparison. The decoding performance of LDPC and LTE turbo code is obviously improved as the number of iterations increases, however, the rate of improvement could slow due to decoding convergence after some iterations. Furthermore, the decoding complexity increases linearly with the number of iterations for LDPC and turbo codes. The decoding performance of polar code is obviously improved as the number of list increases, however, the decoding complexity increases almost linearly with the number of lists. Therefore, the analysis on the trade-off between the complexity and performance is important. Fig. 1 presents number of operations and performance comparison.

Observation 1: Even for short packet sizes, LDPC codes with early stopping decoding always performs better than Polar code with list size 4 and Turbo code with early stopping based on CRC.

- For fixed decoding complexity, the coding gain of LDPC codes is always larger than those of Polar and Turbo codes.

- For fixed coding performance (BLER), the decoding complexity of LDPC codes is always smaller than those of Polar and Turbo codes.

Figure 1. Performance of FEC Schemes (K=100, N=300)

  - Analysis on BLER for fixed FAR

False alarm rate (FAR) is one of important requirement for the efficient communication system since the retransmission in the IP packet layer, of which packet size is longer than that of PHY layer, is needed if the FAR is high. LDPC codes have the self-error-detection capability based on syndrome-check; LDPC codes usually can provide a certain level of FAR without CRC bits as shown in Fig 2. However, polar and turbo codes do not have self-error-detection capability, therefore, a concatenation with CRC is needed to guarantee a certain level of FAR. In CRC-aided list decoding, some CRC bits are used for improving BLER instead reduce the detection error. To compare LDPC codes and other coding schemes in a fair way, we present the code block error rate performance for each coding scheme when false alarm rates and decoding complexity are almost the same for all codes as in Fig. 3. More results are presented in [3].

Observation 2: When false alarm rates and decoding complexity are the same, the performance of polar codes with list-4 decoding and LDPC codes with offset MS decoding is comparable for short information block size K<1000.

Figure 2. FAR of LDPC Codes with KLDPC=117

Figure 3. CBLER with K = 100 and max FAR = 2-24 (offset MS vs List-4)

  - Flexibility

In [4], IR-HARQ schemes for LDPC codes and polar codes are summarized and some results of performance evaluation on AWGN channels and tapped-delay-line (TDL) channels are given. The rate-compatible quasi-cyclic LDPC codes are introduced in many contributions including [5]. We compare the IR-HARQ performance between LDPC codes and IF-polar codes introduced in [8-9].

Fig. 4 shows the performance comparison on AWGN channels. Fig. 5 shows performance comparison over TDL channels. The performance gap is much larger than AWGN channels. Even in the first transmission over the ETU channels, the performance of polar codes is worse than that of LDPC codes. In fast fading channels, some symbols are fall into a deep fade like puncturing, and it is well-known that the performance of polar codes are sensitive to these unexpected puncturing. In addition, the base assumption of polar coding theory, discrete memoryless channels, does not hold for fast fading channels.

Observation 3: On the ETU channels with Doppler spread 70Hz (relatively fast and selective fading), the performance of polar-IR is worse than that of LDPC-IR by 4dB, 6.6dB, 7.5dB in BLER = 1% in the second, the third, and the fourth retransmission, respectively.

Fig 4. BLER of LDPC-IR and Polar-IR with IF over AWGN channels

Figure 5. BLER of LDPC-IR and polar-IR with IF over ETU-70Hz channels

In [5], we present details of the proposed lifting method and show the coding performance of LDPC codes obtained by the lifting method in terms of code block sizes and rates. Fig. 6 shows performance of proposed QC LDPC and LTE turbo codes base on below conditions.

Figure 6: Performance of proposed QC LDPC and LTE turbo codes (R=8/9, 1/3)

Observation 4: The proposed length-compatible QC LDPC code support a stable performance in terms of code block sizes and code rates.

  - Latency

It is discussed the requirement of latency on channel decoder for data channel based on the special subframe, so-called self-contained frame structure [7]. Considering both the decoding latency and performance, the LDPC decoder can meet the latency requirements for all subcarrier spaces from 15kHz to 120kHz. Here, the LDPC code is decoded by layered decoding with 16 layers for code rate 1/3 as shown in Fig. 7.

Figure 7: Performance vs Latency (R=1/3, K=1000)

Observation 5: Summary of condition to meet the decoding latency requirements for eMBB data channel

|  |  |  | LDPC | Turbo | Polar |
| --- | --- | --- | --- | --- | --- |
| K=1000 | R=1/3 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |

* could be unsatisfied for some cases

  - Complexity

It is well-known that LDPC code can support whole range of NR required code length and rate with lifting, shortening and puncturing. In addition, LDPC code has benefit of more coding gain compare to other candidates given similar complexity [2]. It would also be worth to note that LDPC codes which is agreed to be used for eMBB data channel can be implemented efficiently to support from small code block size to large code block size base on the lifting method. It is very obvious that one code solution is much better than two code solution in terms of implementation complexity. If second code for data channel would be added, it will be more complicated not only from specification but also implementation perspective, compared to LDPC code as a single code solution. First, the additional operation must be defined to support flexible change the encoder and decoder. Second, the decoder area will be increased to add the second decoder. However, one decoder, especially for small block length, will be always in idle state as the relevant minimum eMBB data payload would be larger than 320 bits. Given the above proposition, LDPC code should be adopted as a single solution for whole eMBB data channel.

Proposal 1: Adopt LDPC code for eMBB uplink data channel for whole range of information block size

Conclusion

In this contribution, we present the following observations and proposal for analysis on candidate channel coding schemes for eMBB data channel for short packet service scenarios:

Observation 1: Even for short packet sizes, LDPC codes with early stopping decoding always performs better than Polar code with list size 4 and Turbo code with early stopping based on CRC.

- For fixed decoding complexity, the coding gain of LDPC codes is always larger than those of Polar and Turbo codes.

- For fixed coding performance (BLER), the decoding complexity of LDPC codes is always smaller than those of Polar and Turbo codes.

Observation 2: When false alarm rates and decoding complexity are the same, the performance of polar codes with list-4 decoding and LDPC codes with offset MS decoding is comparable for short information block size K<1000.

Observation 3: On the ETU channels with Doppler spread 70Hz (relatively fast and selective fading), the performance of polar-IR is worse than that of LDPC-IR by 4dB, 6.6dB, 7.5dB in BLER = 1% in the second, the third, and the fourth retransmission, respectively.

Observation 4: The proposed length-compatible QC LDPC code support a stable performance in terms of code block sizes and code rates.

Observation 5: Summary of condition to meet the decoding latency requirements for eMBB data channel

|  |  |  | LDPC | Turbo | Polar |
| --- | --- | --- | --- | --- | --- |
| K=1000 | R=1/3 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |

* could be unsatisfied for some cases

Proposal 1: Adopt LDPC code for eMBB uplink data channel for whole range of information block size

- References

- "Chairman’s note," 3GPP TSG RAN WG1 #87, Reno, USA, Nov 14-18, 2016.

- R1-1612549, Samsung, Performance of Short-Length Channel Codes for NR, 3GPP TSG RAN WG1 #87, Reno, Nevada, US, 14th – 18th Nov., 2016

- R1-1612550, Samsung, Short-length LDPC Codes for NR, 3GPP TSG RAN WG1 #87, Reno, Nevada, US, 14th – 18th Nov., 2016

- R1-1612555, Samsung, HARQ Performance Comparison of LDPC and Polar, 3GPP TSG RAN WG1 #87, Reno, Nevada, US, 14th – 18th Nov., 2016

- R1-1609065, Samsung, "Length-Compatible Quasi-Cyclic LDPC Codes," 3GPP TSG RAN WG1 #86bis, Lisbon, Portugal, 10– 14 Oct. 2016.

- R1-1609064, Samsung, “IR-HARQ performance of rate-compatible quasi-cyclic LDPC codes”, 3GPP TSG RAN WG1 #86bis, Lisbon, Portugal, 10– 14 Oct. 2016.

- R1-1609073, Samsung, "Discussion on latency of channel codes for NR," 3GPP TSG RAN WG1 #86bis, Lisbon, Portugal, 10– 14 Oct. 2016.

- B. Li, D. Tse, K. Chen, and H. Shen, “Capacity-achieving rateless polar codes,” ArXiv preprint arXiv:1508.03112, 2015.

- R1-167210, Huawei, HiSilicon HARQ scheme for polar codes, 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22nd-26th August, 2016.