3GPP TSG RAN WG1#NR																			R1-1701018

Spokane, WA, USA, 16th - 20th January 2017

Agenda item:		5.1.3.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On HARQ feedback determination

Document for:		Discussion and Decision

# 1	Introduction

An objective of the 5G study item [1] is to identify and develop technology components needed for new radio (NR) systems being able to use any spectrum band ranging at least up to 100 GHz. The goal is to achieve a single technical framework addressing all usage scenarios, requirements and deployment scenarios defined in TR38.913 [2].

In RAN1#86bis and RAN1#87, following agreements related to the timing of HARQ feedback were made [3], [4]:

Agreements:

- Timing relationship between DL data reception and corresponding acknowledgement can be (one or more of, FFS which ones)

  - dynamically indicated by L1 signaling (e.g., DCI)

  - semi-statically indicated to a UE via higher layer

  - a combination of indication by higher layers and dynamic L1 signaling (e.g., DCI)

- FFS: minimum interval between DL data reception and corresponding acknowledgement

- FFS: common channels (e.g. random access)

Agreements:

- It should be possible to dynamically indicate (at least in combination with RRC) the timing between data reception and hybrid-ARQ acknowledgement transmission as part of the DCI.

In RAN1#85 and RAN1#87, following agreements related to the multiplexing of different numerologies on a same NR carrier were made [5], [4]:

Agreements:

- Forward compatibility of NR shall ensure smooth introduction of future services and features with no impact on the access of earlier services and UEs

  - Multiplexing different numerologies within a same NR carrier bandwidth (from the network perspective) is supported

    - FDM and/or TDM multiplexing can be considered

Agreements:

- NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion.

  - Potential specification impact includes but is not limited to:

    - FFS: CSI-RS measurement

    - FFS: the time and frequency granularity of dynamic resource allocation

  - FFS: if spectrum confinement (filtering, windowing,…) can be dynamically varied or not

Flexible HARQ ACK timing facilities e.g. highly flexible UL/DL configuration but impacts also determination of HARQ feedback transmitted on UL. Further, if dynamic resource allocation of different numerologies per UE turns out to be agreed by RAN1, it will also impact HARQ feedback determination. These impacts are addressed in this contribution.

# 2	Discussion

In LTE, the timing relationship between DL data reception and corresponding acknowledgement is predetermined in the specification. In LTE TDD, the HARQ ACK timing as well as the DL subframes for which HARQ ACK is reported in a certain UL subframe (aka DL association set) is determined based on the subframe number and used UL/DL configuration. That kind of approach turns out cumbersome when more flexible UL/DL configurations are supported and tabulation of all possible combinations becomes increasingly complex. This can be seen e.g. with LTE LAA, where HARQ feedback for LAA SCell is essentially determined as if LAA SCell were FDD SCell – just for the sake of simplicity.

In NR, highly flexible UL/DL configuration efficiently adapting air interface to the variations on traffic is desired. Correspondingly, determination of HARQ feedback, including determination of HARQ ACK codebook size as well as DL association set (i.e. set of DL slot for which HARQ ACK is reported in a certain UL slot) should be flexible and not limited to a manageable subset of predetermined UL/DL configurations.

The determination of HARQ feedback can be based on a simple set of rules using few parameters such as the indicated timing between DL data and associated HARQ ACK as well as the minimum processing time between the DL data reception and transmission of associated HARQ ACK as inputs to the determination.

Proposal #1: Determination of DL association set and codebook size for HARQ feedback is flexible and based on the indicated timing relationship between DL data reception and corresponding acknowledgement.

In the case that the timing between DL data and associated HARQ ACK is indicated dynamically by L1 signaling, DL association set and HARQ ACK codebook size also need to be determined dynamically. Dynamic determination of HARQ ACK codebook involves inevitable error cases. For example, missing DL grant for the first DL slot to be included in the HARQ ACK codebook may lead, depending on the design, erroneous ordering of HARQ ACK bits or erroneous HARQ codebook size. Obviously, appropriate mechanisms, such as counter DAI in LTE, to mitigate such error cases need to be employed with dynamic DL association set and HARQ ACK codebook size determination.

Proposal #2: DL association set and HARQ ACK codebook size are dynamically determined in the case of dynamically indicated timing relationship between DL data reception and corresponding acknowledgement.

Observation #1: Appropriate mechanisms, such as counter DAI, to mitigate error cases related to the dynamic determination of DL association set and HARQ ACK codebook size need to be studied.

Currently it is unclear whether dynamic resource allocation of different numerologies per UE on same NR carrier will be supported or not. In the case that different numerologies can be dynamically allocated per UE on the same NR carrier, HARQ feedback design is further complicated as different numerologies are associated with e.g. different processing times. HARQ feedback for different numerologies may even occur in the same time unit. To avoid excessive complexity, HARQ feedback for different numerologies even on the same NR carrier can be determined separately. The HARQ feedbacks can be transmitted separately with the corresponding numerologies or can be combined into single transmission.

Observation #2: If dynamic resource allocation of different numerologies per UE on same NR carrier is supported, HARQ feedback may be determined separately for each numerology.

# 3	Conclusions

In this contribution, we have considered the impacts from flexible HARQ ACK timing and from potential dynamic resource allocation of different numerologies per UE to HARQ feedback determination. Based on the discussion, we make the following observations and proposals:

Proposal #1: Determination of DL association set and codebook size for HARQ feedback is flexible and based on the indicated timing relationship between DL data reception and corresponding acknowledgement.

Proposal #2: DL association set and HARQ ACK codebook size are dynamically determined in the case of dynamically indicated timing relationship between DL data reception and corresponding acknowledgement.

Observation #1: Appropriate mechanisms, such as counter DAI, to mitigate error cases related to the dynamic determination of DL association set and HARQ ACK codebook size need to be studied.

Observation #2: If dynamic resource allocation of different numerologies per UE on same NR carrier is supported, HARQ feedback may be determined separately for each numerology.

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #86 bis, 3GPP.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #87, 3GPP.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #85, 3GPP.