3GPP TSG RAN WG1 NR Ad-Hoc Meeting                                                              R1-1701190

Spokane, USA, 16th - 20th January 2017

# Source: 	ZTE, ZTE Microelectronics

Title:	Hybrid CSI feedback for NR MIMO

Agenda Item:	5.1.2.2

Document for: 	Discussion and Decision

- Introduction

In RAN1#87, the following working assumption is made on supported number of ports for Type I codebook [1].

# Working assumption:

- For Type I,  CSI feedback using a PMI codebook for X CSI-RS ports is supported

- Supported values of X are at least 1,2,4,8,12,16,[24],32

  - Note: For X=1, Type I does not have PMI feedback

- Support for other values of X is not precluded

- Note the number of ports in CSI-RS resource configuration may not be the same with the number of ports in the PMI codebook

In this contribution, we discuss the necessity of supporting codebook with 12-24 CSI-RS ports for NR MIMO and share our views on this working assumption based on hybrid CSI feedback.

- Discussion on Hybrid CSI and supported number of CSI-RS ports for Type I codebook

In LTE Rel.13 and Rel.14, two alternative approaches are proposed to enhance the legacy non-precoded CSI-RS based CSI feedback: BF CSI-RS based feedback and hybrid CSI based feedback. We discuss these two alternatives in this section to identify whether it is necessary to support 12-24 CSI-RS ports for Type I codebook.

BF CSI-RS based feedback

For BF CSI-RS based feedback, TXRUs are virtualized into no more than 8 BF CSI-RS ports. Then UE can perform port-level or resource-level beam selection based on the BF CSI-RS. For resource-level beam selection, K>1 resources are configured for each UE. For each resource, only 8 CSI-RS ports are required. This configuration can also be performed in cell-specific manner. For port-level beam selection, K=1 resource is configured for each UE, and UE can feed back the preferred beam with W2-only codebook. In this case, gNB can configure up to 8 ports for each UE. However, for the port-level beam selection, BF CSI-RS should be configured UE specifically, and the accurate UE specific beamforming vectors are difficult to acquire especially for FDD. Hence in Rel.14, hybrid CSI is introduced to solve this problem.

Hybrid CSI based feedback

In the current discussion on hybrid CSI in Rel.14, it has already been agreed that hybrid CSI with both Class A and Class B reports is introduced. Class A feedback only contains long-term CSI feedback, i.e., i1 and RI, and it is used to acquire long-term beam information for channel virtualization. Class B feedback is used to acquire accurate beam information based on the virtualized channel. Moreover, as large number of ports lead to large CSI-RS and CSI overhead, Class A feedback can be configured with large feedback period and associated CSI-RS period. Hence the saved overhead can bring performance gain compared with legacy Class A approach. For NR, hybrid CSI can be applied to the case that long-term FDD reciprocity only gives coarse beam range. Then Class A feedback can be configured with CBSR and large number of CSI-RS ports to obtain long-term channel information, and Class B feedback is used to refine the beam information from Class A feedback.

Figure 1 Unified CSI feedback framework using hybrid CSI

As discussed in [3], we should strive for commonality as well as flexibility considering different implementation methods.  CSI framework using hybrid CSI is a good solution for achieving common CSI framework for hybrid beamforming system and codebook based digital precoding system (which can be regarded as one of the implementation methods for FDD massive MIMO system) as shown in Figure 1.   It is quite common to consider this CSI acquisition flow for hybrid beamforming system. e.g. [4].

In addition, CSI framework using hybrid CSI is a good solution for achieving common CSI framework taking into account different levels of channel reciprocity as discussed in [2].  Regardless whether channel reciprocity is used, the common part of it is beamformed CSI-RS based Class B solution for acquiring short term channel information.  To acquire long term channel information, we can solely rely on channel reciprocity if it is accurate enough.  If the channel reciprocity is not accurate enough, flexible codebook can be used to improve the accuracy of the long term channel information i.e. W1 taking into account different levels of channel reciprocity.

Proposal 1:  Hybrid CSI framework is used for common CSI framework considering different levels of channel reciprocity for both sub-6GHz and above-6GHz massive MIMO systems.

We simulate the case that Class A is used to virtualize 32 ports to 8 ports, and 8-port Class B is used to obtain accurate CSI. Moreover, we evaluate hybrid CSI with different NP CSI-RS periodicities to show the benefit of difference configuration cases. Simulation parameters are shown in Appendix. Simulation results are shown in Table I. It can be seen in Table I that with proper CSI-RS periodicity configuration, hybrid CSI achieves better performance than legacy Class A feedback.

Table I Performance of different NP CSI-RS periodicities for hybrid CSI

|  | T1=5ms Baseline | T1=5ms Hybrid CSI | T1=10ms Hybrid CSI | T1=20ms Hybrid CSI | T1=50ms Hybrid CSI | T1=100ms Hybrid CSI | T1=200ms Hybrid CSI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RU | 0.474 | 0.478 | 0.458 | 0.446 | 0.444 | 0.457 | 0.481 |
| Mean | 28.69 (0%) | 28.64 (-0.17%) | 30.27 (5.51%) | 31.15 (8.57%) | 31.18 (8.68%) | 29.94 (4.36%) | 28.29 (-1.39%) |
| 5% | 7.71 (0%) | 7.48 (-2.98%) | 8.21 (6.49%) | 8.64 (12.06%) | 8.83 (14.53%) | 8.49 (10.12%) | 7.38 (-4.28%) |
| 50% | 26.49 (0%) | 25.64 (-3.21%) | 27.97 (5.59%) | 28.57 (7.85%) | 28.78 (8.64%) | 27.03 (2.04%) | 24.84 (-6.22%) |

Based on the above discussion, in order to avoid unnecessary overhead, we have the following proposal.

Proposal 2: Hybrid CSI with Class A and Class B should be supported for PMI based feedback in NR

- Class A feedback should contain only long-term CSI and support up to 32 CSI-RS ports.

Fig.1 Hybrid CSI with 8-port Class A and 8-port Class B

For the configuration of Class A, gNB can only use partial ports to acquire UE specific beamforming for the CSI-RS associated with Class B feedback. For example, in Fig. 1, 8-port CSI-RS resource is configured to acquire the UE specific beamforming in vertical domain. Then for the 2nd eMIMO-Type, gNB configures 8 beamformed CSI-RS ports by using the beams contained in the W1 feedback of the 1st eMIMO-Type on each polarization. This implies hybrid CSI with 8-port Class A can be scalable to antenna array with large than 8 ports. The performance of this scalability is evaluated by comparing with legacy 32-port Class A. In the simulation, hybrid CSI schemes with different CSI-RS periodicity for the first eMIMO-Type are evaluated. The simulation results are shown in Fig. 2.

Fig.2 Performance comparison among different single CSI and hybrid CSI schemes

It can be seen from the simulation results that the performance of hybrid CSI based feedback with 8 CSI-RS ports is similar to the one of PMI based feedback with 32 CSI-RS ports. With appropriate periodicity for the first eMIMO-Type, hybrid CSI with smaller number of ports even performs better. Hence it can be concluded that hybrid CSI with 8-port Class A can be scalable to antenna array larger than 8 ports.

Observation: Hybrid CSI with 8-port Class A can be scalable to antenna array larger than 8 port with similar performance as legacy Class A larger than 8 ports.

Proposal 2 implies that only long-term CSI requires larger than 8 CSI-RS port. For long-term CSI feedback, we don’t see the necessity of supporting all the cases of 12, 16 and 24 ports. The long-term CSI feedback performance of these ports can be similar after virtualization. Additionally, the above observation implies hybrid CSI with 8-port Class A can be scalable to support 12-, 16- and 24-port antenna array with good performance. Hence we cannot confirm the working assumption in Section 1 currently since it’s not necessary to support all 12, 16 and 24 ports.

Proposal 3: Down-selection is needed for 12, 16 and 24 ports.

- Conclusions

In this contribution, we discuss the necessity of supporting 12-24 CSI-RS ports for Type I codebook in NR.  Based on the above discussion and simulations, we propose NOT to confirm the working assumption in Section 1 now.  Besides, we have the following observation and proposals.

Observation: Hybrid CSI with 8-port Class A can be scalable to antenna array larger than 8 port with similar performance as legacy Class A larger than 8 ports.

Proposal 1:  Hybrid CSI framework is used for common CSI framework considering different levels of channel reciprocity for both sub-6GHz and above-6GHz massive MIMO systems.

Proposal 2: Hybrid CSI with Class A and Class B should be supported for PMI based feedback in NR

- Class A feedback should contain only long-term CSI and support up to 32 CSI-RS ports.

Proposal 3: Down-selection is needed for 12, 16 and 24 ports.

- References

[1] Chairman’s notes, 3GPP RAN1#87

[2] R1-1700143, Channel Reciprocity Based Massive MIMO for FDD, ZTE, ZTE Microelectronics

[3] R1-1701189, On Type I Codebook design for NR MIMO, ZTE, ZTE Microelectronics

[4] R1-1700598, Hybrid Beamforming for NR, NTT DOCOMO

Appendix I

| Parameters | Assumptions |
| --- | --- |
| Cellular Layout | Hexagonal grid, 7 sites, 3 Macro cells per site, geographical based wrap-around |
| Channel Model | 3D UMi ISD 200 |
| Operating bandwidth (BW) | 10 MHz |
| Tx Power | 3D UMI ISD 200: 41 dbm |
| UE Speed | 3km/h |
| Antenna configuration | Transmitter: （M,N,P,Q）=（4,4,2,32） Receiver: 2Rx cross-polarized antenna at UE |
| Antenna element spacing | (dV,dH)=( 0.8λ, 0.5λ,) |
| CQI/PMI reporting interval and frequency granularity | Single CSI-RS: 5ms for Class A CSI,6RB Hybrid CSI :T1=5~20ms for Class A CSI, WB T2=5ms for Class B CSI, 6RB |
| Feedback scheme | PUSCH mode 3-2, PMI feedback |
| Delay for scheduling and AMC | 6ms |
| Scheduler | Proportional Fair |
| Receiver | MMSE-IRC With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | FTP1 model with 0.5Mbyte |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |