3GPP TSG RAN WG1 NR Ad-Hoc#2  	R1-1710751

# Qingdao, P.R. China, 27th – 30th June 2017

Agenda item:	5.1.4.2.2

Source:	Samsung

Title: 				Circular Buffer Management for Polar Code Rate-Matching

Document for:		Discussion and Decision

1. Introduction

In NR meeting, we use following terminology for rate-matching of polar codes.

Terminology for use in RAN1 from now on: - Puncturing refers to non-transmission of coded bits such that the non-transmitted bits are unknown at the receiver and the corresponding LLRs can be set to zero. - Shortening involves setting input bits to a known value, and non-transmission of coded bits corresponding to those input bits, such that the corresponding LLRs can be set to a large value at the receiver.

2. Unified Bit Extraction for Puncturing

First, we define following basic notations for polar codes in this contribution.

- K: number of information bits excluding CRC bits
- J+J': number of assistance CRC bits
- R: desired code rates (CRC bits are classified as parity bits)
- M: number of codeword bits (M=K/R)
- N: mother polar code size
- L: list size of successive-cancellation list (SCL) decoder

In most of puncturing schemes, bit puncturing begins from the top of the polar code graph. For example, quasi-uniform puncturing (QUP) is one that bit puncturing begins from the top of the graph [1]. In a unified rate-matching scheme proposed in [2], it is also considered to puncture low-indexed bit first. Since bit shortening must begin from the bottom of the graph due to the weight-1 column selection criterion [3], there should be additional buffer management to handle both shortening and puncturing if we consider bit puncturing from the 0-th bit.

In [4], however, a unified rate-matching scheme that exploits a single bit extraction method is proposed, and it achieves excellent rate-matching performance. For the bit extraction from the buffer, bit puncturing and bit shortening all begin from the (N-1)-th, and there is no change in the order of incapable bits when puncturing is applied.

We investigate whether this unified bit extraction can be applied for different rate-matching schemes or not by some simulations. We consider a rate-matching interleavers proposed in [2]. Two bit extraction methods depicted in Fig. 1 are evaluated based on the simulation settings described in Table 1. In the unified bit extraction in Fig 1-(a), buffered bits are always extracted from the 0-th bits for puncturing, shortening, and repetition. On the other hand, the two-way bit extractions depicted in Fig. 1-(b) follows in one’s usual way that bits are loaded in a reverse order of the circular buffer when puncturing and repetition occur.

Figure 1  Rate-matching with (a) unified bit extraction; and (b) two-way bit extractions

Table 1  Performance Evaluation Settings

| Code sequence | Optimized CN sequence [5] |
| --- | --- |
| Rate-matching scheme | Unified rate-matching in [2] |
| Channels | AWGN channel |
| Modulation | QPSK |
| Decoding algorithm | CRC-aided SCL decoding with 19-bit CRC |
| List size (L) | 8 |
| Information bits (K) | 16:8:200 |
| Code rate (R) | 2/3, 1/2, 2/5, 1/3, 1/4, 1/5, 1/6 |
| Max. mother code size (N0max) | 512 |
| Rate-matching scheme | Unified interleaving by sub-block permutation on P16 Unified bit extraction from a circular buffer for puncturing/shortening/repetition |
| Rate-matching criteria | β=1.15, Rrepthr=0.4, Rmin=1/6, and Rpsthr=0.35 |

Note that an incapable bit allocation rule does not change by unified bit extraction. Fig. 2 shows an example regarding how subchannels at the encoder input are incapable by puncturing in [2].

Figure 2  Incapable subchannel allocation by puncturing from the bottom of the graph

Fig. 3 shows the performance evaluation results, and there is no difference in performance between unified bit extraction and two-way bit extractions. Therefore, it is reasonable to apply the unified bit extraction for all types of rate-matching scheme.

Figure 3  Performance evaluation of rate-matching with unified and two-way bit extractions

Observation 1: A unified bit extraction method achieve the same performance as two-way bit extractions if the puncturing pattern is well designed.

Proposal 1: Adopt the unified bit extraction for polar code rate-matching.

References

- K. Niu, K. Chen, and J.-R. Lin, “Beyond turbo codes: rate-compatible punctured polar codes,” in Proc. IEEE Int. Conf. Commun. (ICC), June, 2013.

- R1-1707845, MediaTek, “Polar code rate-matching design,” 3GPP TSG RAN WG1 #89, Hangzhou, P.R. China, 15-19, May 2017.

- R. Wang and R. Liu, “A novel puncturing scheme for polar codes,” IEEE Commun. Lett., vol. 18, no. 12, pp. 2081-2084, Dec. 2014.

- R1-1710750, Samsung, “Design of unified rate-matching for polar codes,” 3GPP TSG RAN WG1 NR Ad-hoc #2, Qingdao, P.R. China, 27-30, June 2017.

- R1-1710749, Samsung, “Design of combined-and-nested polar code sequences,” 3GPP TSG RAN WG1 NR Ad-hoc #2, Qingdao, P.R. China, 27–30 June 2017