3GPP TSG RAN WG1 Ad-hoc Meeting 1901                            	    R1-1900103

Taipei, 21st – 25th January, 2019

Title:          	Discussion on configured grant for NR-U

Source: 	ZTE

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

# Introduction

At the RAN #82 meeting, the new WID on NR-U (NR-based Access to Unlicensed Spectrum) [1] aims to identify solutions and techniques for next generation wireless systems operating on unlicensed bands. And in the WID, one of the objectives is to specify configured grant operation design.

In this contribution, we discuss configured grant support aspects for NR-U.

# Discussion on Configured Grant for NR-U

Autonomous uplink access (AUL) with frame structure type 3 has been specified in Rel-15 FeLAA WI. Meanwhile, configured grant Type1 and Type2 were also discussed in Rel-15 for NR UL data transmission. Compared with the grant-based access, both AUL and configured grant scheme have the following advantages: energy efficiency on UE side, very low cost/complexity with UE design, reduced signalling overhead and transmission latency. These advantages are also the goals that NR-U needs to achieve. And in the 92bis meeting, it has reached agreement to support configured grant scheme for NR-U UL transmission, and some enhancements or changes considering unique characteristic of unlicensed band should be studied.

## Resource allocation

For NR-U configured grant UL transmission, both time domain and frequency domain resource allocation should be considered.

Firstly, in time domain, gNB can configure a UE with a periodicity and repetition times by RRC as in licensed band. And repetition times can be considered as pre-configured transmission occasions within the periodicity. UE can transmit PUSCH after a successful LBT before any of the candidate transmission opportunities scheduled by the configured grant, and gNB can perform blind detection on uplink data according to the configuration. Therefore, the resource allocation modes adopted for NR licensed configured grant transmission should be the baseline for NR-U configured grant transmission.

Secondly, in order to increase UL transmission opportunities, multiple frequency domain opportunities can also be considered for NR-U. For example, multiple BWPs can be configured to UE. When UL data arrives, UE can attempt to perform LBT in multiple BWPs according to the resource configuration for configured grant. If any LBT on these BWPs succeeds, UE can transmit on either multiple BWPs or a selected one. More details may be further studied on multiple BWPs transmission opportunities.

Observation: the resource allocation modes adopted for NR licensed configured grant transmission can be the baseline for NR-U. And repetition times can be considered as pre-configured transmission occasions within the periodicity.

Proposal 1: Multiple candidate time domain and/or frequency domain transmission opportunities can be configured for configured grant.

## UCI, DFI and HARQ

For NR configured grant, as HARQ ID is implicit from selected time resource, therefore, only synchronous HARQ is supported for configured grant. However, if the UE fails LBT on the predetermined HARQ locations, it needs to wait till next HARQ instances for uplink transmission, which will cause large delay. Considering the uncertainty of medium access, asynchronous HARQ should be supported for NR-U configured grant, and the details can be further studied. In the #93 meeting, it has been reached the agreement to introduce UCI on PUSCH to carry HARQ process ID, NDI, RVID, therefore, asynchronous HARQ can be achieved.

For UCI, besides HARQ process ID, NDI, RVID, other information, such as COT sharing information can also be included. As the PUSCH starting position and duration can be configured in RRC or indicated by DCI, this information is not necessary to be carried in UCI. And as the gNB can identify the UE ID through the DMRS, UE ID is also no longer necessary to be included in UCI.

Besides, it has also reached the agreement to introduce Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission, and UE may perform non-adaptive retransmission for configured grant according to DFI including NACK feedback. Meanwhile, according to the NR licensed configured grant scheme, if UE does not receive CS-RNTI scrambled DCI to trigger UE to perform configured grant retransmission before the expiration of the timer, then the UE interpret it as ACK. However, both the DFI and the CS-RNTI scrambled DCI may be blocked by LBT, and if the DFI or the CS-RNTI scrambled DCI cannot be transmitted due to gNB LBT failure before the timer expires, then UE interpret it as ACK which might be not correct. Therefore, a method should be proposed to improve the DFI and the CS-RNTI scrambled DCI transmission opportunity. The scheme of the COT sharing may be used as a tool to solve this problem. After PUSCH is received, gNB can transmit DFI or the CS-RNTI scrambled DCI within the same COT. At UL-DL switching point, no LBT or one shot LBT will be performed for gNB, which can decrease the LBT impact on DFI and the CS-RNTI scrambled DCI transmission.

Proposal 2: besides HARQ process ID, NDI, and RVID, COT sharing information can also be included in the UCI.

Proposal 3: a method for decreasing the LBT impact on DFI and the CS-RNTI scrambled DCI transmission can be considered, such as no LBT or one shot LBT at the UL-DL switching point.

## CBG based transmission

Rel-15 NR supports CBG based transmission and HARQ-ACK feedback. gNB or UE can retransmit the failed CBGs instead of the entire TB. This mechanism is extremely useful when some CBGs are suffered from bursty interference in unlicensed band. Therefore, CBG based transmission for NR-U should be supported and inherited. And this has been identified in the SI.

Besides, how to design CBG related control information included in the DFI or UCI should also be considered. Here two options can be considered for configured grant CBG operation. And these two options can be combined used.

- Option1: UE can select the configured grant UL CBG transmission and the transmitted CBGs information, such as the CBGTI can be carried in the UCI.

- Option2: the DFI could provide CBG level feedback indication while UE can retransmit the failed CBGs based on the DFI feedback information. Or gNB use the grant based schedule the CBG retransmission and the CBGTI in the DCI to indicate which CBG is scheduled.

For option2, how to feedback the CBG level ACK/NACK and the TB level ACK/NACK as part of the DFI should be defined. For this problem, R15 DL HARQ-ACK feedback scheme can be the reference.

Proposal 4:

- CBG related control information, such as CBGTI can be included in the UCI.

- CBG level ACK/NACK feedback information can be included in the DFI.

- R15 DL HARQ-ACK feedback scheme can be the reference for CBG level and TB level ACK/NACK feedback design.

## Channel access related

### LBT aspects

For LBT, below aspects should be considered. Channel access priority should be RRC configured. The CW adjustment can reuse the AUL channel access scheme. Besides, the configured grant UE can share the gNB initiated COT as the scheduled UE and use cat2 LBT or no LBT if the gap between the end of the DL transmission and the beginning of the UL burst is not more than 16 sec.

### UE initiated COT sharing

In FeLAA, UE supporting AUL can initiate a COT and eNB can share the COT to transmit DL control information to the UE. But UL-DL-UL sharing is not supported constrained by frame structure 3. However, in NR, as flexible frame structure is allowed, and multiple UL to DL and DL to UL switching within a shared COT obtained by UE can be supported. Besides, in the SI stage it was identified that sharing resources with gNB within COT(s) that is acquired by UE(s) as part of configured grant based transmissions should be supported. Besides, the resource sharing mechanisms and rules should be determined. For this problem, the rules should be the same as gNB initiated COT sharing. For example, if the BWP transmitted for gNB is the same as the UE configured grant UL transmission, then gNB can perform a cat2 LBT or no LBT if the gap between the end of the configured grant UL transmission and the beginning of the DL burst is not more than 16 sec. Besides, gNB can schedule this UE to transmit UL in the remaining of the COT according to the rules defined by EN 301 893.

Proposal 5: the resource sharing mechanisms and rules for UE initiated COT sharing is the same as the gNB initiated COT sharing.

### multiple user access

For NR configured grant, as gNB may configure multiple UEs to share the same resource, UL transmission collisions might occur. There are two cases need to considered.

- Case1: configured grant UE is allocated to occupy the part channel bandwidth

In this case, different UE allocate different interlaces, but LBT is performed on the whole BWP or bandwidth. If one UE successfully performs LBT earlier and transmits data on the configured BWP, then other UEs may not be able to access channel due to LBT failure on the same BWP. Therefore multiple channel access or UL multiple user multiplexing cannot be achieved. Some mechanism should be considered for resolving the contention among multiple UEs attempting to access the same PUSCH resource. For example, blank pattern method as discussed in [2] or same UL data transmission start point mechanism can be used to reduce UL collisions.

- Case2: configured grant UE is allocated to occupy the full channel bandwidth

In this case, a configured grant UE is allocated to occupy the full channel bandwidth and different UE allocate the same resource to avoid the resource waste if the UE perform LBT failure or has no data to transmit. To alleviate the collision problem when multiple UEs attempting to access the same PUSCH resource, FeLAA AUL approach (i.e., UE-selected starting offset and RRC configured starting offset set) can be the baseline. Further enhancement can be considered for NR flexible and more starting point support.

Proposal 6: to support multiple user access for NR-U considering LBT influence, below methods can be considered:

- same UL data transmission start point mechanism

- blank pattern method

- FeLAA AUL approach based

# Conclusion

In this contribution, we discuss the configured grant aspects for NR operation in unlicensed spectrum, and have the following proposals:

Proposal 1: Multiple candidate time domain and/or frequency domain transmission opportunities can be configured for configured grant.

Proposal 2: besides HARQ process ID, NDI, and RVID, COT sharing information can also be included in the UCI.

Proposal 3: a method for decreasing the LBT impact on DFI and the CS-RNTI scrambled DCI transmission can be considered, such as no LBT or one shot LBT at the UL-DL switching point.

Proposal 4:

- CBG related control information, such as CBGTI can be included in the UCI.

- CBG level ACK/NACK feedback information can be included in the DFI.

- R15 DL HARQ-ACK feedback scheme can be the reference for CBG level and TB level ACK/NACK feedback design.

Proposal 5: the resource sharing mechanisms and rules for UE initiated COT sharing is the same as the gNB initiated COT sharing.

Proposal 6: to support multiple user access for NR-U considering LBT influence, below methods can be considered:

- same UL data transmission start point mechanism

- blank pattern method

- FeLAA AUL approach based

# Reference

- 3GPP, RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm, RAN#82, December, 2018.

- 3GPP, R1-1808321, Discussion on Channel access mechanism for NR-U, ZTE, RAN1#94, August,2018