3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1900142

Taipei, 21st – 25th January, 2019

Source:	vivo

Title:	Performance evaluation on multi PDCCH based multi TRP transmission

Agenda Item:	7.2.8.6

Document for:	Discussion and Decision

- Introductions

In RAN #81 meeting, revised Rel-16 NR MIMO working item [1] was approved and the objectives of enhancements on multi-TRP/Panel transmission are as following:

  - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul:

    - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission

    - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission

    - Multi-TRP techniques for URLLC requirements are included in this WI

In RAN1 #94bis [2], the following agreements were achieved on performance evaluation for eMBB.

Agreement

For multi-TRP/panel performance evaluation:

  - For eMBB in FR1, 10MHz BW and 15kHz SCS are baseline.

  - For eMBB in FR1, 20MHz BW and 30kHz SCS are optional.

  - For eMBB in FR2, 80MHz BW and 120kHz SCS are baseline.

Agreement

For eMBB multi-TRP/panel performance evaluation, FTP traffic model 1 with packet size 0.5Mbytes as a baseline, and other traffic model is not precluded. RU=20/40/60% are baseline, and optional low RU (e.g. 5/10) can be considered.

Agreement

For eMBB multi-TRP/panel performance evaluation, MMSE IRC is the baseline, and advanced receiver is not precluded. Practical channel estimation and feedback model are used.

Agreement

- For eMBB multi-TRP performance evaluation, ideal and non-ideal backhaul are considered, the following delay values are assumed:

  - Ideal backhaul: 0ms

  - Non-ideal backhaul: 2ms, 5ms, 50ms(optional)

- Companies to provide the delay values used in their evaluations

Agreement:  SLS assumption for eMBB multi-TRP/panel enhancement

| Parameters | Dense urban (Macro Only) | Indoor hotspot |
| --- | --- | --- |
| Carrier frequency | 2GHz/4GHz is baseline (each company to choose 1 or more) 30GHz is optional | 4GHz is baseline, 30GHz is optional |
| Channel model | TR38.901 |  |
| TP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) 16 ports: (M, N, P, Mg, Ng Mp, Np) = (8,4,2,1,1,2,4) (dH, dV) = (0.5, 0.8)λ for FR1 2 ports (8,8,2,1,1) and 8 ports (4,8,2,2,2) for 30GHz Other antenna configurations is not precluded (such as 32 ports) | 2 ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for 4GHz 2 ports: (4,4,2,1,1) for 30GHz Other antenna configurations is not precluded. |
| UE antenna configuration | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for FR1 For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 | 4Rx Port: (Baseline) (M,N,P,Mg,Ng,Mp,Np) = =(1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for 4GHz For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 |
| Coordination assumptions | Each company to provide details on cluster size, coordination scheme, etc |  |

Agreement

Baseline scheme to evaluate eMBB multi-TRP enhancements is DPS or single TRP

- Each company to provide the details on backhaul delay, CSI reporting, transmission scheme, scheduling, etc.

In this paper we provide our preliminary link-level simulation (LLS) and system-level simulation (SLS) results for multi-TRP transmission.

- LLS evaluation

LLS evaluations are first conducted to see how much performance gains there are if multiple TRPs are jointly used for transmission. The evaluation assumptions in Table 1 are adopted.

Table 1: LLS assumption multi-TRP enhancement

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Subcarrier Spacing for data | 15kHz |
| Data allocation | 8 RBs, PRG4. The first two symbols allocated for PDCCH |
| Channel Model | CDL-A model, refer to Table 7.7.1-1 in 38.901 - delay spread =100ns - UE speed=3km/h. |
| BS antenna configurations for two TRP | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), (dV, dH) = (3, 3)λ, one-to-one mapping: 4 Tx-Ports |
| UE antenna configurations | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), (dV, dH) = (3, 3)λ, one-to-one mapping: 4 Rx-Ports |
| MIMO mode | SU-MIMO, UE capability: Max Rank=2 or 4 |
| UE receiver type | MMSE-IRC |
| Link adaptation | Based on Type I CSI codebooks AMC enabled, RI and PMI adaptation enabled, Max Retransmission=4 |
| Channel estimation | Realistic: mmse-2D-filter |
| DMRS configuration | DMRS: Type I Double-symbol, only front-loaded DMRS |
| Number of DMRS CDM groups without date | 2, power-boosting= -3dB |
| Beam sweeping | Disabled |

A UE receives data from two TRPs in NC-JT manner, in which equivalent channel is given by

He=H1, H2W100W2=H1W1, H2W2

where H1, H2 are channels from the two TRPs, and W1, W2 are the selected precoders of the two TRPs. Then SINR per layer can be calculated assuming MIMO detection of the equivalent channel He.

Figure 1: Throughputs of different DMRS configurations

In Figure 1, the difference between Multi-TRP-0, Multi-TRP-1 and Multi-TRP-2 is whetherthe DMRS resources between different TRPs are orthogonal or not. Multi-TRP-0 scheme has orthogonal DMRS ports between each other. DMRS ports in Multi-TRP-1 scheme and Multi-TRP-2 scheme are both pseudo-orthogonal. For Multi-TRP-1, different antenna port and different scrambling IDs are used. For multi-TRP-2, the same antenna port index are used for the two TRPs together with different scrambling ID.

From the simulation, it could be seen that multi-TRP transmission could bring some performance gains compared to single TRP.  At least from single UE perspective, NC-JT is a valid solution to increase the throughput. Another observation is that DMRS orthogonality is important for multi-TRP demodulation. Unlike MU-MIMO transmission, UE needs to equalize the interference between different TRPs.

Observation 1: From single UE perspective, multi-TRP NC-JT transmission could bring accountable performance gains compared to single TRP transmission.

Observation 2:  DMRS orthogonality should be guarantteed for NC-JT transmission.

- SLS evaluation

3.1 Simulation assumptions

We perform performance evaluation for eMBB in FR1 with 10MHz BW and 15kHz SCS, 4GHz carrier frequency. The baseline scheme is single-TRP while multi-TRP transmission scheme is dynamic switching between DPS and NCJT. SU-MIMO is applied for both single TRP and multi-TRP cases.

For Indoor Hotspot scenario of multi-TRP transmission, four neighboring TRPs forming a cluster connect to one gNB with ideal backhaul. A UE with its associated TRP in the cluster measures the RSRP of all TRPs in the cluster and selects at most one candidate coordinating TRP, with the RSRP gap lower than a predefined threshold compared to the serving TRP.

Subband PMI, subband CQI feedback of Type I codebook is applied for CSI feedback. The UE satisfying the threshold compares the estimated throughput among the two possible DPSs transmitting from its associated TRP and coordinating TRP, and one NC-JT transmitting from the two TRPs. And then, UE reports the recommended transmission scheme and its corresponding CSI. The post-SINR of NC-JT calculation is the same as the description in LLS. Rank 1 or 2 is chosen for each TRP to maximize the NCJT estimated overall throughput.

The scheduler in the cluster schedules the data transmitting from the TRP(s) to UE(s) according to proportional fair algorithm. For SU-MIMO scheduling, if a UE is scheduled from two TRPs in one subband, either TRP is unable to schedule another UE in the subband. One codeword per TRP is transmitted if the UE is scheduled as NCJT.

Other simulation assumptions can be found in the Appendix.

3.2 Performance evaluation results

We provide preliminary results for full-buffer in Figure 2. It can be seen in the very heavy-loaded system, compared to single-TRP DPS can achieve considerable gain in both cell average spectral efficiency and cel-edge spectral efficiency. However, performance degradation is observed for NC-JT. A UE scheduled in NC-JT scheme occupies doubled-resources from two TRPs, which may restrict the flexibility to schedule another properUE. It can be expected to achieve positive gain for NC-JT in a very light-loaded traffic such as FTP1 model with low RU, and by adopting a more sophisticated scheduler. CSI feedback may also need to be enhanced.

Figure 2: Cell average SE and cell edge SE comparison with single-TRP for Indoor Hotspot

.

Observation 3: In heavy-loaded system, NC-JT transmission does not provdide performance gains compared to DPS transmission.

- Conclusion

In this contribution, we have the following observations according to our preliminary evaluation results for multi-TRP transmission.

Observation 1: From single UE perspective, multi-TRP NC-JT transmission could bring accountable performance gains compared to single TRP transmission.

Observation 2:  DMRS orthogonality should be guarantteed for NC-JT transmission.

Observation 3: In heavy-loaded system, NC-JT transmission does not provdide performance gains compared to DPS transmission.

References

- RP-182067, Revised WID: Enhancements on MIMO for NR

- RAN1 #94bis Chairman’s Notes.

Appendix

Table 3: SLS assumption for eMBB multi-TRP enhancement

| Parameters |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Indoor hotspot (InH) |
| Frequency Range |  | FR1 only, 4GHz |
| Inter-BS distance |  | 20m for InH |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at TRP |  | 2 Tx ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) for InH 4 Tx ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) (dH, dV) = (0.5, 0.8)λ for UMa |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 23dBm for InH |
| BS antenna height |  | 3m for InH |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| Configuration for multi-TRP | Cluster | 4 neighboring TRPs for InH |
|  | Maximal number of coordinating TRPs | 2 |
|  | Backhaul assumption | Ideal |
| Transmission scheme |  | Baseline: SU-MIMO, rank 1 or rank 2 per TRP DPS: rank 1 or rank 2 per TRP Multi-TRP: dynamic switching between DPS and NCJT, rank 1 or rank 2 per TRP |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms - Type I codebook - CSI of maximum estimated multi-TRP transmission scheme: DPS, and NCJT |
| Traffic model |  | Full-buffer |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Ideal |