3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                                        	R1-1900144

Taipei, January 21st – 25th, 2019

Source:	vivo

Title:	Evaluation results and remaining issues on evaluation methodology for UE power saving

Agenda Item:	7.2.9.1

Document for:	Discussion and Decision

- Introductions

In RAN1 #94bis and #95 meeting, the power models for UE adaptation to power and traffic, and for RRM enhancement were agreed [1] [2]. It was agreed that applications including FTP, web-browsing, video streaming, instant messaging, VoIP, gaming, background app sync can be considered for traffic modeling for power saving proposal evaluation. Simulation methodology and scenarios were also agreed. Calibrations for FTP and VoIP were completed in RAN1 #95.

In the last two meetings, there are lots of proposed power saving schemes discussed and captured in UE power saving TR. Based on the agreed power models and simulation methodology; it is feasible to evaluate the power saving gain of the proposed power saving schemes.

In this contribution, the UE power saving gain for different power saving schemes are simulated and compared with Rel-15 existing schemes. The remaining evaluation methodology, e.g. the methodology for power scaling for CSI-RS is given.

- Simulation results for PDCCH monitoring and blind decoding reduction

  - Evaluation methodologies and schemes

In RAN1 #95 meeting, it is agreed that:

Agreements: The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows, - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling - Power saving signal triggering PDCCH monitoring - Go-to-sleep signaling to skip PDCCH monitoring - PDCCH skipping - DCI based indication for PDCCH skipping (e.g., indication in DCI content, new SFI state). - L1 signal/signaling (other than DCI) based triggering - Multiple CORESET/search space configurations - Configuration of different PDCCH periodicities with dynamic signaling - Adaptation of CORSET/search space configuration – DCI/timer/HARQ-ACK based indication - Dynamic/semi-persistent CORSET/search space ON/OFF - Adaptation between DRX ONduration timer and inactivitytimer - Separated PDCCH monitoring of DL and UL - L1 signaling triggering to assist UE in reducing the number of PDCCH blind decoding - Reduced PDCCH monitoring on SCell (including cross carrier scheduling) - Network assistance – RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and channel estimations before PDCCH decoding

To evaluate the power saving gain of PDCCH monitoring reduction schemes, two traffic models including gaming and FTP model 3 are used. For gaming, the traffic model is based on R1-070674 [3], shown in Annex 2. The mean inter-packet interval for gaming is about 55ms. Each downlink gaming packet can be transmitted in one slot, since the packet size (hundreds of bits) is relatively small. For FTP model 3, the packet size is 0.5 Mbytes and the mean inter-arrival time is 200 ms. For FTP packet transmission, each packet is transmitted by 5 slots, 7 slots and 26 slots, for UEs with high SINR (32.4dB), UEs with medium SINR (15.2dB) and UEs with low SINR (2.4dB), respectively. The calculation for the number of slots needs to transmit a packet is based on Table A2 in Annex 1 assuming 100MHz bandwidth. These SINR values are based on 5%, 50% and 95% of downlink geometry of dense urban scenario with single layer, as shown in Figure 1 below.

Figure 1. Geometry CDF for dense urban (single layer, 4GHz frequency)

In total four schemes are modeled in the evaluation, where scheme 1 and 2 are Rel-15 enabled schemes, and scheme 3 and 4 are for power saving enhancements. FDD is assumed in the simulation with only DL packet modeled.  30KHz SCS with 0.5ms slot is used. HARQ retransmission is not modeled.

- Scheme 1: Rel-15 one slot PDCCH monitoring periodicity with C-DRX

In this scheme, the UE is configured with one-slot PDCCH monitoring periodicity and C-DRX is enabled. The C-DRX parameters are selected according to the traffic modes, as shown in the following table, in which 30KHz SCS is assumed.  Other C-DRX parameters, including short DRX cycle timer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, or drx-HARQ-RTT-TimerUL are not modelled in the simulation.

Table 1: C-DRX parameter settings for scheme 1 (slot length is 0.5ms)

| C-DRX parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| DRX cycle | 80 slots | 320 slots |
| Ondurationtimer | 8 slots | 16 slots |
| Inactivitytimer | 20 slots | 200 slots |

- Scheme 2: Rel-15 PDCCH monitoring with different periodicities, no C-DRX

In this scheme, UE monitors PDCCH with a RRC configured PDCCH monitoring periodicity. No CDRX is assumed. The PDCCH monitoring periodicity is selected according to the traffic characteristics, e.g. mean interval of packet arrival. In each monitoring periodicity, a monitoring duration is selected. For gaming traffic, since the packet is relatively small, fixed 1 slot duration is used regardless of UE SINR. For FTP 3, different monitoring durations are selected according to the UE SINR conditions.

Table 2: PDCCH monitoring parameter settings for scheme 2

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicities | 1, 80, 120 slots | 1, 100, 320, 400 slots |
| PDCCH monitoring durations | Fixed 1 slot | For 1 slot periodicity: 1 slot For 100 slots monitoring periodicities - High UE SINR: 2 slots For 320 or 400 slots monitoring periodicities - High UE SINR: 5 slots - Medium UE SINR: 7 slots - Low UE SINR: 26 slots |

- Scheme 3: Dynamic switching of PDCCH monitoring periodicities by DCI indication

In this scheme, multiple potential PDCCH monitoring periodicities are configured by RRC signalling, and DCI indicates one of them. No CDRX is assumed. The potential PDCCH monitoring periodicities for dynamic switching are N1 and N2 slots, where N2 is selected according to the traffic characteristics. Since dynamic switching between 1-slot periodicity and N2 slots periodicity is supported, the PDCCH monitoring duration is fixed as 1 slots, regardless of UE SINR conditions.

Table 3: PDCCH monitoring parameter settings for scheme 3

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicity N1 | 1 slot | 1 slot |
| PDCCH monitoring periodicity N2 | 80, 120 or 160 slots | 100, 320 or 400 slots |
| PDCCH monitoring durations | Fixed 1 slot | Fixed 1 slot |

- Scheme 4: DCI explicit indication of PDCCH monitoring occasions within a periodicity

In this scheme, UE is explicitly indicated by DCI the exact PDCCH monitoring occasions for the upcoming monitoring periodicity, where the monitoring periodicity is RRC configured. For example, for a 10 slots monitoring periodicity, there can be a 10bits-bitmap to indicate whether the UE is required to monitor PDCCH in each slot. No CDRX is assumed.

Table 4: PDCCH monitoring parameter settings for scheme 4

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicities N3 | 80 slots | 100, 320 or 400 slots |

  - Evaluation results

Simulation results for gaming

The power and delay simulation results for gaming are shown in Table 5. The latency of each packet can be obtained by:

(1)

The simulation results for gaming traffic are shown in Table 5. The scheme 1 and scheme 2 (with 1 or 80 slots PDCCH monitoring periodicities) are taken as the baseline for comparison. Since the delay requirement for gaming is 60ms [3] (i.e., 120 slots), all the four schemes can meet this requirement. It can be seen that scheme 3 can provide large power saving benefit with similar or longer packet delay. It can also be seen that scheme 4 provides large power saving benefit without sacrificing packet latency.

For fixed PDCCH monitoring periodicity (Scheme 2), it is difficult to determine the accurate PDCCH monitoring periodicity (N) to balance the tradeoff between power consumption and packet delay. However, for dynamic PDCCH monitoring periodicities switching (Scheme 3), power consumption and packet delay are not too sensitive to the periodicity settings. This is also applicable for explicit PDCCH monitoring occasion indication (Scheme 4).

Table 5: Simulation results for gaming traffic

| Power saving schemes | Parameter values (slot) | average power (unit per slot) | average packet delay (in slots) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity (N=1) | Power gain over fixed PDCCH monitoring periodicity (N=80) |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {80,8,20} | 17.59 | 32.65 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | N=1 | 50.86 | 0.00 |  | 0% (baseline#2) |  |
|  | N=80 | 7.60 | 39.60 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | {N1,N2}={1,80} | 7.60 | 39.60 | 57% | 85% | 0% |
|  | {N1,N2}={1,120} | 5.55 | 60.27 | 68% | 89% | 27% |
|  | {N1,N2}={1,160} | 4.59 | 79.79 | 74% | 91% | 40% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | N3=80 | 7.60 | 40.61 | 57% | 85% | 0% |

Simulation results for FTP model 3

The simulation results for FTP model 3 are shown in Table 6~8, for UEs with high SINR (32.4dB), UEs with medium SINR (15.2dB) and UEs with low SINR (2.4dB), respectively. Higher SINR leads to less slots needed to deliver one packet, thus lower power consumption is expected.

Four baseline cases are selected, one is CDRX and the other three cases are fixed PDCCH monitoring periodicities with 1, 100, and 320 slots. Compared with baseline #1 (CDRX with fixed 1-slot PDCCH monitoring periodicity), or baseline#2 (fixed 1-slot PDCCH monitoring periodicity without C-DRX), Scheme 3 and Scheme 4 can achieve large power saving benefit with some tolerable average packet delay loss. Compared with baseline #3 (fixed 100-slot PDCCH monitoring periodicity without C-DRX), or baseline#4 (fixed 320-slot PDCCH monitoring periodicity without C-DRX), power saving benefit from Scheme 3 and Scheme 4 is smaller than the former cases, but the delay improvement is large. For example, Dynamic PDCCH monitoring periodicity switching between 1 slot and 100 slot can achieve 53.51 slots average packet delay.  It can also be seen that in most cases the power saving benefit reduces for low SINR UEs compared to high SINR UEs.

Table 6: Simulation results for FTP model 3 traffic (UE with high SINR: one packet is transmitted in 5 slots).

| Power saving schemes | Parameter values (slot) | average power (unit per slot) | average packet delay (slot) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={1,1} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={100,2} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,5} |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 20.78 | 91.47 | 0% (baseline#1) |  |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =1 duration =1 | 51.25 | 5.00 |  | 0% (baseline#2) |  |  |
|  | periodicity =100 duration =2 | 7.26 | 437.85 |  |  | 0% (baseline#3) |  |
|  | Periodicity = 320 duration = 5 | 3.93 | 724.83 |  |  |  | 0% (baseline#4) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,100} duration =1 | 7.26 | 53.51 | 81% | 92% | 0% | 2% |
|  | Periodicity {N1,N2}={1,320} duration =1 | 3.85 | 161.01 | 81% | 92% | 47% | 2% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 3.54 | 200.75 | 83% | 93% | 51% | 10% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 100 | 7.26 | 55.46 | 81% | 92% | 0% | 2% |
|  | Periodicity = 320 | 3.85 | 165.54 | 81% | 92% | 47% | 2% |
|  | Periodicity = 400 | 3.54 | 206.29 | 83% | 93% | 51% | 10% |

Table 7: Simulation results for FTP model 3 traffic (UE with medium SINR: one packet is transmitted in 7 slots).

| Power saving schemes | Parameter values (slot) | average power (unit per slot) | average packet delay (slot) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={1,1} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,7} |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 21.45 | 93.61 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =1 duration =1 | 51.75 | 7.00 |  | 0% (baseline#2) |  |
|  | Periodicity = 320 Duration = 7 | 4.74 | 726.80 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,320} duration =1 | 4.60 | 162.86 | 79% | 91% | 3% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 4.29 | 202.06 | 80% | 92% | 10% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 320 | 4.59 | 168.35 | 79% | 91% | 3% |
|  | Periodicity = 400 | 4.29 | 209.30 | 80% | 92% | 10% |

Table 8: Simulation results for ftp modle 3 traffic (UE with low SINR: one packet is transmitted in 26 slots).

| Power saving schemes | Parameter values (slot) | average power (unit per slot) | average packet delay (slot) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={1,1} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,26} |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 27.68 | 114.71 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =1 duration =1 | 56.51 | 26.00 |  | 0% (baseline#2) |  |
|  | Periodicity = 320 Duration = 26 | 12.43 | 744.96 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,320} duration =1 | 11.71 | 177.09 | 58% | 79% | 6% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 11.40 | 212.23 | 59% | 80% | 8% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 320 | 11.70 | 195.06 | 58% | 79% | 6% |
|  | Periodicity = 400 | 11.40 | 237.90 | 59% | 80% | 8% |

Based on the above results,

Observation 1: For gaming traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with similar or tolerable packet delay loss.

Observation 2: For FTP traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with tolerable packet delay loss. Larger power saving gain for higher SINR UEs compared to low SINR UEs.

Proposal 1: At least one of the following enhanced PDCCH monitoring adaptation schemes are adopted for Rel-16 UE power saving

- Dynamic switching of PDCCH monitoring periodicities by DCI indication

- DCI explicit indication of PDCCH monitoring occasions within a periodicity

- Simulation results for adaptation of CORSET/search space configuration –HARQ-ACK based indication

In RAN1 #95 meeting, it is agreed that [2]:

Agreements: The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows, - Multiple CORESET/search space configurations - Configuration of different PDCCH periodicities with dynamic signaling - Adaptation of CORSET/search space configuration – DCI/timer/HARQ-ACK based indication

- Adaptation of CORSET/search space configuration –HARQ-ACK/NACK based indication

For services with fixed or almost fixed inter-packet arrival, e.g. VoIP or real time video, the packet arrives with fixed periodicity, e.g. 20ms. Semi-persistent scheduling (SPS) can be configure to schedule PDSCH for the new transmission of packets, to save power of PDCCH monitoring. Assuming a 10% retransmission probability it is difficult to forecast the potential retransmissions. It is better to configure some PDCCH monitoring occasions for potential retransmission next to the semi-persistent scheduling occasions and such additional PDCCH occasions are monitored only when the latest semi-persistent scheduling is not decoded successfully (as in Figure 2). By this method, UE can skip the unnecessary monitoring occasions for retransmissions and therefore save power. However, such “event triggered” PDCCH monitoring occasions are not supported in Rel-15 thus can be enhanced.

Figure 2: PDCCH monitoring triggered by decoding failure of latest SPS PDSCH

We evaluated the above enhanced scheme (Figure 2) compared with Rel-15 baseline (shown in Figure 3), in which all the PDCCH monitoring occasions are activated by RRC configuration without additional triggering

Figure 3: Rel-15 baseline, i.e. PDCCH is always monitored according to RRC configuration

Only voice packet is modelled and SID packet is not modelled. DL only is assumed. SPS is used to transmit new transmission of voice packets. So PDSCH only power model is used for new transmission of voice packets. The RTT (round trip time) between new transmission and the relevant 1st retransmission is 10ms. 10% BLER is assumed for initial SPS PDSCH transmission. 0% BLER is assumed for the 1st retransmission, i.e., no 2nd retransmission. The used power model is listed in Table 9 below, which is the same as the agreed power model in the RAN1 #94bis and #95 meetings.

According to the simulation results, about 20% power is saved by the enhanced PDCCH monitoring scheme, compared with the existing scheme, due to the reduced PDCCH monitoring for potential retransmissions.

Proposal 2: For DL SPS, consider the UE monitoring of PDCCH search space for retransmission triggered by decoding failure of SPS PDSCH.

Table 9. The power model used for simulation

| Power State | Relative Power |
| --- | --- |
| Light Sleep | 20 |
| PDCCH-only | 100 |
| PDCCH + PDSCH | 300 |
| PDSCH only | 280 |
| Additional transition energy for light sleep | 100 X 1ms |

- Remaining issues for evaluation methodology

In RAN1 #95 meeting, the remaining FFS for CSI-RS are as follows,

| CSI-RS | FFS for scaling w.r.t. # of symbols for CSI-RS FFS for neighbor cell measurement FFS for #measured cells |
| --- | --- |

For a slot with CSI-RS only, scaling w.r.t. # of symbols for CSI-RS is possible since the CSI-RS pattern may only have one symbol duration. The power consumption for CSI-RS is impacted by the following aspects,

- The number of symbols in a slot for CSI-RS reception

- The number of CSI-RS ports for UE to process

Current assumption for scaling of SSB is that power for one SSB is 0.75 of power for two SSB, i.e. 75 power units. Similar to that, linear scaling of CSI-RS measurement can be considered for these two aspects.

For example, assuming 100 power units for CSI-RS with >=M symbols and >=N CSI-RS ports, than the scaling m symbols and n CSI-RS ports can be as follows,

Pm, n=α∙β∙Pt

Where, α=0.75, if m<M1, if m≥M, β=0.75, if n<N1, if n≥N and Pt=100

For typical M and N values, it is proposed that M=4 and N=3.

Proposal 3: The following power scaling for CSI-RS is used.

Pm, n=α∙β∙Pt

Where, α=0.75, if m<M1, if m≥M, β=0.75, if n<N1, if n≥N,  Pt=100, M=4, N=3

Pt is the power units for CSI-RS with >=M symbols and >=N CSI-RS ports. Pm, n is the power units for CSI-RS with m symbols and n CSI-RS ports.

- Conclusion

In this contribution, the UE power saving gain for different power saving schemes are simulated and compared with Rel-15 existing schemes. The remaining evaluation methodology, e.g., the methodology for power scaling for CSI-RS are given. The following proposal and observations are given:

Observation 1: For gaming traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with similar or tolerable packet delay loss.

Observation 2: For FTP traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with tolerable packet delay loss. Larger power saving gain for higher SINR UEs compared to low SINR UEs.

Proposal 1: At least one of the following enhanced PDCCH monitoring adaptation schemes are adopted for Rel-16 UE power saving

- Dynamic switching of PDCCH monitoring periodicities by DCI indication

- DCI explicit indication of PDCCH monitoring occasions within a periodicity

Proposal 2: For DL SPS, consider the UE monitoring of PDCCH search space for retransmission triggered by decoding failure of SPS PDSCH.

Proposal 3: The following power scaling for CSI-RS is used.

Pm, n=α∙β∙Pt

Where, α=0.75, if m<M1, if m≥M, β=0.75, if n<N1, if n≥N,  Pt=100, M=4, N=3

Pt is the power units for CSI-RS with >=M symbols and >=N CSI-RS ports. Pm, n is the power units for CSI-RS with m symbols and n CSI-RS ports.

References

- Chairman's Notes, RAN1 #94bis meeting

- Chairman's Notes, RAN1 #95 meeting

- R1-070674, “LTE physical layer framework for performance verification”, Orange, China Mobile, KPN, NTT DoCoMo et al.

Annex 1-Genaral simulation assumptions

Table A1: Reference Configuration and power models for FR1

| Reference Configuration | Power State | Characteristics | Relative Power |
| --- | --- | --- | --- |
| Downlink: TDD, FR1, 30 kHz SCS, 1CC, 100 MHz BW, PDCCH region of 2 symbol at beginning of a slot, k0 = 0, max. #CCE = 56, 36 PDCCH blind decoding, PDSCH of max data rate with 256QAM 4x4 MIMO, #RB for TRS = 52, 4RX, Capability 1 Uplink: TDD, FR1, 30 kHz SCS, 1CC, 100MHz BW, 1TX, 2 power levels 0dBm and 23dBm Power values are averaged over the operations within a slot. | Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
|  | Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
|  | Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
|  | PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
|  | SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
|  | PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 |
|  | UL | Long PUCCH or PUSCH. FFS the power scaling for short PUCCH and SRS. | 250 (0 dBm) 700 (23 dBm) |

Table A2: MCS index, spectral efficiency and SNR for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency | SNR(dB) |
| --- | --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 | -6.3875 |
| 1 | 2 | 193 | 0.3770 | -4.3670 |
| 2 | 2 | 308 | 0.6016 | -1.9720 |
| 3 | 2 | 449 | 0.8770 | 0.0389 |
| 4 | 2 | 602 | 1.1758 | 1.9436 |
| 5 | 4 | 378 | 1.4766 | 4.0346 |
| 6 | 4 | 434 | 1.6953 | 4.9933 |
| 7 | 4 | 490 | 1.9141 | 5.8226 |
| 8 | 4 | 553 | 2.1602 | 6.7614 |
| 9 | 4 | 616 | 2.4063 | 7.7246 |
| 10 | 4 | 658 | 2.5703 | 8.3331 |
| 11 | 6 | 466 | 2.7305 | 9.6872 |
| 12 | 6 | 517 | 3.0293 | 10.5948 |
| 13 | 6 | 567 | 3.3223 | 11.5450 |
| 14 | 6 | 616 | 3.6094 | 12.4509 |
| 15 | 6 | 666 | 3.9023 | 13.3469 |
| 16 | 6 | 719 | 4.2129 | 14.4346 |
| 17 | 6 | 772 | 4.5234 | 15.4004 |
| 18 | 6 | 822 | 4.8164 | 16.3262 |
| 19 | 6 | 873 | 5.1152 | 17.3044 |
| 20 | 8 | 682.5 | 5.3320 | 18.6130 |
| 21 | 8 | 711 | 5.5547 | 19.4207 |
| 22 | 8 | 754 | 5.8906 | 20.2124 |
| 23 | 8 | 797 | 6.2266 | 21.0512 |
| 24 | 8 | 841 | 6.5703 | 22.0117 |
| 25 | 8 | 885 | 6.9141 | 23.0826 |
| 26 | 8 | 916.5 | 7.1602 | 23.9406 |
| 27 | 8 | 948 | 7.4063 | 24.9249 |
| 28 | 2 | reserved |  |  |
| 29 | 4 | reserved |  |  |
| 30 | 6 | reserved |  |  |
| 31 | 8 | reserved |  |  |

Annex 2-Traffic and delay model for gaming

Traffic modeling:

Interactive real-time gaming is a traffic with small and random packet arrivals. 3GPP proposes some guidance for gaming modeling in [3]. In this paper, the gaming model is based on Table A3, where the average packet size (parameter a in Table A3) is updated from 120 Bytes to 220 Bytes.

Table A3: Downlink Gaming Network Traffic Parameters

| Parameter | Statistical Characterization |
| --- | --- |
| Initial packet arrival | Uniform Distribution PDF: , |
| Packet arrival | Largest Extreme Value Distribution (also known as Fisher-Tippett distribution) PDF: , Values for this distribution can be generated by the following procedure: , where y is drawn from a uniform distribution in the range |
| Packet size | Largest Extreme Value Distribution (also known as Fisher-Tippett distribution) a = 120 220 Bytes, b = 36 |
| UDP header | Deterministic (2 Bytes). This is added to the packet size accounting for the UDP header after header compression. |

Delay modeling and requirements:

To model the impact of packet delay, in R1-070674 [3], the following delay modeling and requirements are defined.

“A maximum delay of 160 ms is applied to all uplink packets, i.e. a packet is dropped by the mobile station if any part of the packet has not started physical layer transmission, including HARQ operation, 160 ms after entering the mobile station buffer. The packet delay of a dropped packet is counted as 180 ms. A mobile network gaming user is in outage if the average packet delay is greater than 60 ms. The average delay is the average of the delays of all packets, including the delay of packets delivered and the delay of packets dropped.”

In this paper, the delay for each packet is considered according to the above mode. In Section 2, we compare the power consumption performance of different schemes given that the delay requirement (60ms) of gaming can be met.

Annex 3-Traffic model for VoIP

Table A4. Parameter for VoIP

| Parameter | Characterization |
| --- | --- |
| Codec | RTP AMR 12.2, Source rate 12.2 kbps |
| Encoder frame length | 20 ms |
| Voice activity factor (VAF) | 50% (c=0.01, d=0.99) |
| SID payload | Modelled 15 bytes (5Bytes + header) SID packet every 160ms during silence |
| Total voice payload on air interface | 40bytes (AMR 12.2) |
| One way delay budget | 50ms |

Figure A1: 2-state voice activity model

In the model, as shown in Figure A1, the probability of transitioning from state 1 (the active speech state) to state 0 (the inactive or silent state) while in state 1 is equal to , while the probability of transitioning from state 0 to state 1 while in state 0 is . The model is assumed updated at the speech encoder frame rate , where  is the encoder frame duration (typically, 20ms). Other details of VoIP modeling can be find in R1-070674 [3].

Annex 4-Traffic model for FTP molde 3

Table A5. Parameter for ftp model 3

| Model | FTP model 3 |
| --- | --- |
| Packet size | 0.5 Mbytes |
| Mean inter-arrival time | 200 ms |
| DRX setting | Period = 160 ms Inactivity timer = 100 ms On duration: 8 msec |