3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	Tdoc R1-1900169

Taipei, Taiwan, 21st – 25th January 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Power Distribution Scenario

Document for:	Discussion, Decision

# Introduction

Power distribution is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In this contribution we evaluate reliability and latency performance of URLLC power distribution use case using the ITU evaluation methodology [2]. Assumptions on link and system level simulations are based on the conclusion in [3] with additional adjustment to appropriately capture relevant deployment. The results show that it is possible to achieve the strict requirement of URLLC power distribution use case.

System level evaluation on URLLC capacity for power distribution use case is also given.

# Discussion

According to [3], requirements for power distribution use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for power distribution use case

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Power distribution | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms |
|  | 99.999 | 15(end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs |

In this contribution, we focus on the strict requirement, i.e., 99.9999% reliability within 3ms latency for the packet size of 100 bytes. We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate performance of URLLC for power distribution using the ITU methodology. That is, we first obtain SINR distribution from a system level simulation. Then reliability is computed from link level simulation results at SNR corresponding to 5%-tile value which typically represents the cell-edge user in the macro scenario.

## System Level Simulation

Based on system level simulation assumption in Table A-1 in the appendix, we obtain DL and UL SINR distribution with full buffer assumption as shown in Fig. 1 for different ISD values. We note that the full buffer assumption here can be seen as a worst-case traffic assumption in terms of interference.

Figure 1: DL and UL SINR distribution for different inter-site distance values for full-buffer traffic model.

Based on the results in Fig. 1, for ISD = 500m the 5%-tile SINR are 1.93 dB (DL) and 2.17 dB (UL), while for ISD =150m the 5%-tile SINR are 1.51 dB (DL) and 1.57 dB (UL).

With the system level simulation assumption in Table A-1 for power distribution with full-buffer assumption, the 5%-tile SINR are 1.93 dB (DL) and 2.17 dB (UL) for ISD = 500m, and 1.51 dB (DL) and 1.57 dB (UL) for ISD =150m, respectively.

Based on system level simulation assumptions in Table A-2 in the appendix we obtain percentage of UEs meeting URLLC requirements according to Table 2. For both DL and UL, grant-based and slot-based single-transmission is used.

Table 2: Percentage of UEs fulfilling URLLC requirements

|  | URLLC requirements (Delay, Reliability) |  |  |
| --- | --- | --- | --- |
|  | 3 ms, 99.9999% | 3 ms, 99.99% | 3 ms, 99.9% |
| Downlink | 95.8% | 98.4% | 99.9% |
| Uplink | 90.3% | 97.7% | 99.3% |

With the system level simulation assumption in Table A-2 for power distribution with FTP mode 3:

- 96% of UEs fulfilling (3 ms, 99.9999%) requirement can be reached for DL.

- 90% of UEs fulfilling (3 ms, 99.9999%) requirement can be reached for UL.

- With relaxed reliability requirements (99.99% and 99.9%) more than 95% of UEs fulfilling the requirements for both DL and UL.

## Link Level Simulation

Based on link level simulation assumption in Table A-3 in the appendix, we present BLER performances of PDCCH and PDSCH/PUSCH.

### PDCCH

For PDCCH, we consider DCI size =40 bits excluding CRC, AL4,8,16, and 1os CORESET. PDCCH BLER for different AL are given in Fig. 3.

Figure 3: PDCCH BLER with DCI size 40 for different AL

### PDSCH/PUSCH

For data channels, we consider packet size of 100 bytes, transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. BLER for different MCSs supported within 40 MHz BW (e.g., MCS2 to MCS6 from Table 5.1.3.1-3 [4]) are given in Fig. 4.

Figure 4: BLER of data channels for different MCSs

## Reliability evaluation

According to the definition in [2], reliability is defined as a success probability of a packet transmission within certain latency target. In this contribution, the reliability requirement is 99.9999% within one-way latency of 3 ms.

Based on the latency analysis in [5], we see that it is possible to have up to two DL and up to two DL CG-UL with 7os duration transmissions (including retransmission) within 3 ms latency.

For simplicity, we evaluate the overall reliability of a single DL and single CG-UL transmission (which would satisfy the latency requirement of 3ms anyway) as follows.

(1)  BLER after one DL transmission = Pr(PDCCH failure) + Pr(PDCCH success)*Pr(PDSCH failure)

(2)  BLER after one CG-UL transmission = Pr(PUSCH failure)

Based on results in Figure 1-3, we see that at SNR = 1.51 dB (5%-tile DL SINR for ISD = 150m which is the stricter value compared to that of ISD = 500m), using PDCCH with AL4 or higher and PDSCH with MCS index 6 or lower can achieve the overall BLER for single DL transmission according to (1) at the level below 1E-6.

Similarly, at SNR = 1.57 dB (5%-tile UL SINR for ISD = 150m), using PUSCH with MCS index 6 or lower can achieve the overall BLER for single CG-UL transmission at the level below 1E-6.

With up to two retransmissions, the requirements can be achieved at even higher spectral efficiency.

Thus there exist several scheduling options for DL and CG-UL transmissions with 7-os duration which fulfill the reliability requirement, i.e., achieving overall BLER of 1E-6 within 3ms latency at SNR below the 5%-tile SINR value.

It is possible to have up to 2 DL and CG-UL transmissions with 7os duration in a FDD configuration with 30 kHz SCS within 3 ms one-way latency.

Reliability requirement of 99.9999% within 3 ms one-way latency for power distribution use case can be achieved with several transmission configurations.

# Conclusion

In section 2 we made the following observations:

Observation 1	With the system level simulation assumption in Table A-1 for power distribution with full-buffer assumption, the 5%-tile SINR are 1.93 dB (DL) and 2.17 (UL) for ISD = 500m, and 1.51 dB (DL) and 1.57 (UL) for ISD =150m, respectively.

Observation 2	With the system level simulation assumption in Table A-2 for power distribution with FTP mode 3:

	96% of UEs fulfilling (3 ms, 99.9999%) requirement can be reached for DL.

	90% of UEs fulfilling (3 ms, 99.9999%) requirement can be reached for UL

	With relaxed reliability requirements (99.99% and 99.9%) more than 95% of UEs fulfilling the requirements for both DL and UL.

Observation 3	It is possible to have up to 2 DL and CG-UL transmissions with 7os duration in a FDD configuration with 30 kHz SCS within 3 ms one-way latency.

Observation 4	Reliability requirement of 99.9999% within 3 ms one-way latency for power distribution use case can be achieved with several transmission configurations.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- 3GPP RAN1 #94bis Chairman’s note

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019

# Appendix

Table A-1: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m, 150m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 16Tx/16Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| Traffic | Full buffer |

Table A-2: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, Isotropic |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC: 10 (on average) eMBB: 10 (on average) |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | P0=-70 dBm, α=0.8 |
| HARQ/repetition | Single transmission |
| Channel estimation | No channel estimation error |
| Traffic | FTP model 3, arrival interval 100 ms, TBS=100 |
| URLLC requirements | Latency: 3 ms, Reliability 99.9999% |

Table A-3: Link level simulation assumption (Urban macro scenario)

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for modeling fading channel |
| BS antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |