3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	Tdoc R1-1900170

Taipei, Taiwan, 21st – 25th January 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Factory Automation Scenario at 4 GHz

Document for:	Discussion, Decision

# Introduction

Factory automation is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In this contribution we evaluate DL reliability performance of URLLC using the ITU evaluation methodology [2].

Assumptions on link and system level simulations are based on the conclusion in [3] with additional adjustment to appropriately capture relevant deployment. The results show that it is possible to achieve reliability target with existing NR Rel. 15 design with some enhancement on the UE capability.

# Discussion

According to [3], requirements for factory automation use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for factory automation use case

| Factory automation requirement | 99.9999% PHY reliability | 1 ms PHY one-way latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms |
| --- | --- | --- | --- |

We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate reliability performance of URLLC for factory automation using the ITU methodology. That is, we first obtain SINR distribution from a system level simulation. Then reliability is computed from link level simulation results at SNR corresponding to X%-tile value.

In the macro scenario considered in the ITU evaluation, 5%-tile SINR was considered, typically representing a cell-edge user. For indoor factory automation use case, it is reasonable to strengthen the coverage requirement. Here we consider 1%-tile SINR for evaluating reliability of URLLC indoor factory automation. This translates to 99% coverage, i.e., 99% of the UEs can achieve the URLLC reliability requirement. Note that this level is reasonably high considering practical deployments where critical UEs requiring very high reliability and low latency are less likely to be positioned in poor coverage area.

## System Level Simulation

Based on system level simulation assumption in Table A-1 in the appendix, we obtain DL and UL SINR distributions with full buffer assumption as shown in Fig. 1 below. We note that the full buffer assumption here can be seen as a worst-case traffic assumption in terms of interference.

Figure 1: DL and UL geometry

From Fig. 1, the 1%-tile DL and UL SINR are -1.45 dB and -1.43 dB respectively.

With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile SINR is -1.45 dB (DL) and -14.3 dB (UL).

## Link Level Simulation

Based on link level simulation assumption in Table A-2 in the appendix, we present BLER performances of PDCCH and PDSCH. For PDCCH, we consider DCI size =40 bits excluding CRC, AL 4,8,16, and 1os CORESET duration. PDCCH BLER for different AL are given in Fig. 2.

Figure 2: PDCCH BLER for different AL

For data channels, we consider TBS = 256 bits (=32 bytes), transmission duration of 4 OFDM symbols with 1 DMRS symbol overhead. PDSCH/PUSCH BLER for different MCSs supported within 40 MHz BW (e.g., MCS1 to MCS6 from Table 5.1.3.1-3 [4]) are given in Fig. 3.

Figure 3: PDSCH/PUSCH BLER (single transmission) for different MCSs

## Reliability evaluation

According to the definition in [2], reliability is defined as a success probability of a packet transmission within certain latency target. In this contribution, the reliability requirement is 99.9999% within one-way latency of 1ms.

It is argued that with the enhanced UE capability, i.e., lower processing time than that of UE capability #2 (see [5] for the proposed value of UE capability #3), 2-symbol PDCCH monitoring occasion periodicity, and configured grant UL occasion of every 2 symbols, it is possible to have a single transmission of 4-os PDSCH or single CG UL transmission of 4-os PUSCH within 1 ms. For example, Fig. 4 below illustrates the worst-case latency of a single PDSCH transmission with DL-UL TDD slot format with one symbol guard period. The worst-case latency in this case is equal to 3+19+4+3.5/2 = 27.75 symbols. With 30 kHz SCS, this translate to 0.991 ms which is below 1ms. If other slot formats with DL and UL symbols within a slot are configured, the latency can be lowered even further. Similarly, Fig. 5 illustrates the UL worst-case latency.

Figure 4: Worst-case latency for a single DL transmission of 4os PDSCH in TDD with DUDU slot pattern

Figure 5: Worst-case latency for a single CG UL transmission of 4os PUSCH in TDD with DUDU slot pattern

In the above, delay of TDD deployment is studied since it’s more much difficult to achieve low latency with TDD than with FDD.

We then evaluate the overall reliability of a single DL transmission as follows.

- BLER of single DL transmission = Pr(PDCCH failure) + Pr(PDCCH success)*Pr(PDSCH failure)

- BLER of single CG UL transmission = Pr(PUSCH failure)

At SNR = -1.45 dB (1%-tile SINR), we can see that using PDCCH with AL8 or higher, and 4os-PDSCH with MCS index 6 or lower can achieve the overall BLER for single DL transmission at the level below 1E-6. Similarly, at SNR = -1.43 dB, 4os-PUSCH with MCS index 6 or lower can achieve BLER below 1E-6.

That is, there exist several options for a single DL transmission or CG-UL transmission with 4-os duration which fulfill the reliability requirement, i.e., achieving overall BLER of 1E-6 within 1ms latency at SNR below the 1%-tile SINR.

It is possible to have single DL or single CG-UL transmission with 4os duration in a TDD configuration with 30 kHz SCS within 1 ms one-way latency.

Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation can be achieved with several transmission configurations.

# Conclusion

In section 2 we made the following observations:

Observation 1	With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile SINR is -1.45 dB (DL) and -14.3 (UL).

Observation 2	It is possible to have single DL or single CG-UL transmission with 4os duration in a TDD configuration with 30 kHz SCS within 1 ms one-way latency.

Observation 3	Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation can be achieved with several transmission configurations.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- 3GPP RAN1 #94bis Chairman’s note

- 3GPP TS 38.214 V15.3.0

- R1-1900161 Scheduling/HARQ/CSI Processing Timeline Enhancements for NR URLLC, Ericsson, January 2019.

- R1-1900168 Channel Model for Factory Automation Scenario, Ericsson, January 2019.

# Appendix

Table A-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| Channel model | Modified Indoor Hotspot LOS model with blockers 3GPP TR 38.901 where extra blockage loss is added on top of the InH LoS loss (see e.g., Fig. A-1 and [6]) |
| UE Tx power | 23dBm |
| BS antenna configurations | 16Tx/16Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8), dH = dV = 0.5 λ |
| BS antenna height | 10m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| BS Tx power | 24 dBm per 20 MHz |
| Simulation bandwidth | 40 MHz |
| Duplex mode | TDD (50% DL) |
| SCS | 30 kHz |
| UE distribution | 100% of users are indoor |
| Traffic | Full buffer |

Table A-2: Link level simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-D 30ns |
| Deployment | Modified indoor hot-spot with blockers |
| UE speed | 3 km/h |
| BS TX antenna configuration | 2 Tx ports |
| UE RX antenna configuration | 4 RX ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Figure A-1 Modified indoor layout of factory hall with blockers