3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	Tdoc R1-1900171

Taipei, Taiwan, 21st – 25th January 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Factory Automation Scenario at 30 GHz

Document for:	Discussion, Decision

# Introduction

Factory automation is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In this contribution we evaluate DL reliability performance of URLLC using the ITU evaluation methodology [2].

Assumptions on link and system level simulations are based on the conclusion in [3] with additional adjustment to appropriately capture relevant deployment. The results show that with some enhancement on the UE processing time capability, it is possible to achieve reliability target with existing NR Rel. 15 design.

# Discussion

According to [3], requirements for factory automation use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for factory automation use case

| Factory automation requirement | 99.9999% PHY reliability | 1 ms PHY one-way latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms |
| --- | --- | --- | --- |

We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate reliability performance of URLLC for factory automation using the ITU methodology. That is, we first obtain SINR distribution from a system level simulation. Then reliability is computed from link level simulation results at SNR corresponding to X%-tile value.

In the macro scenario considered in the ITU evaluation, 5%-tile SINR was considered, typically representing a cell-edge user. For indoor factory automation use case, it is reasonable to strengthen the coverage requirement. Here we consider 1%-tile SINR for evaluating reliability of URLLC indoor factory automation. This translates to 99% coverage, i.e., 99% of the UEs can achieve the URLLC reliability requirement. Note that this level is reasonably high considering practical deployments where critical UEs requiring very high reliability and low latency are less likely to be positioned in poor coverage area.

## System Level Simulation

Based on system level simulation assumption in Table A-1 in the appendix, we obtain DL and UL geometry with full buffer assumption as shown in Fig. 1 below. We note that the full buffer assumption here can be seen as a worst-case traffic assumption in terms of interference.

Figure 1: DL and UL SINR

From Fig. 1, the 1%-tile DL SINR and UL SINR are -1.57 dB and -1 dB, respectively.

With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile DL SINR is -1.57 dB and the 1%-tile UL SINR is -1 dB.

## Link Level Simulation

Based on link level simulation assumption in Table A-2 in the appendix, we present BLER performances of DL control channel and data channel. For PDCCH, we consider DCI size =40 bits excluding CRC, AL8,16, and 1os CORESET. PDCCH BLER for different AL are given in Fig. 2.

Figure 2: PDCCH BLER for different AL

For data channel, we consider TBS = 256 bits (=32 bytes), transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. BLER for different MCSs supported within 160 MHz BW (e.g., MCS0 to MCS2 from Table 5.1.3.1-3 [4]) are given in Fig. 3 for both single transmission and one retransmission with soft combining.

Figure 3: BLER of single transmission and one retransmissions for different MCSs

## Reliability evaluation

According to the definition in [2], reliability is defined as a success probability of a packet transmission within certain latency. In this contribution, the considered requirement is 99.9999% reliability within one-way latency of 1ms.

It is shown that with the current Rel-15 UE processing capability for 120 kHz SCS, the worst-case latency for DL retransmission or CG UL retransmission of 7-os PDSCH/PUSCH exceed the 1 ms latency budget (see e.g., [5]).

However, with the enhanced UE capability (see [5] for the proposed value of UE capability #3 for 120 kHz SCS numerology), it is possible to have one DL retransmission of 7-os PDSCH or one CG UL retransmission of 7-os PUSCH within 1 ms. For example, Fig. 4 below illustrates the worst-case latency of PDSCH transmission with one retransmission in a DUDU TDD slot format with two-symbol guard period. The worst-case latency in this case is equal to 101 symbols. With 120 kHz SCS, this translate to 0.9018 ms which is below 1ms. If other slot formats with DL and UL symbols within a slot are configured, the latency can be lowered even further. Similarly, Fig. 5 shows the worst-case latency for CG UL retransmission.

Figure 4: The worst-case latency for one DL retransmission of 7os PDSCH in TDD with DL-UL slot pattern

Figure 5: The worst-case latency for one CG UL retransmission of 7os PUSCH in TDD with DL-UL slot pattern

In the above, delay of TDD deployment is studied since it’s more much difficult to achieve low latency with TDD than with FDD.

In the following we provide an expression for the success probability of DL and CG UL data transmission based on probability terms in Table 2.

Table 2: Success probabilities for calculating total reliability

| Probability | Success probability of |
| --- | --- |
| p1 | PDCCH |
| p2 | PDSCH/PUSCH |
| p3 | PUCCH NACK reception |
| p4 | PUCCH DTX detection |

DL HARQ (re)transmission

In the DL we can describe the total reliability after N transmission as:

pt=n=1Ni=1nn-1n-i(1-p1)p4n-ip1p3i-1∙j=1i-1(1-p2,j)∙p1p2,i

where for any positive integer k,  p2,k is the successful probability of a data being correctly received after k attempts of PDSCH transmissions are soft combined conditioned that previous soft-combined k-1 attempts of PDSCH transmissions fail. In this expression the DL control transmissions are assumed to be uncorrelated with each other and with data. This approximation can be motivated by e.g. interleaved PDCCH mapping where different candidates may be used between attempts. The data transmission attempts are assumed to be correlated with each other where the soft combining effect is captured.

Configured grant UL transmission

With configured grant UL, the first UL transmission is done without the scheduling DCI. The total reliability can be described similarly as:

pt=p2,1+1-p2,1n=1N-1i=1nn-1n-i1-p1n-ip1i-1∙j=1i-1(1-p2,j+1)∙p1p2,i+1

Here the PDCCH reliability only has an effect starting from the first retransmission.

Based on the results in Section 2.2 (assuming PDCCH AL16 and PDSCH/PUSCH MCS index 0), we have link level results at the 1%-tile DL and UL SINR (-1.57 dB and -1 dB respectively) summarized in Table 3 below. For PUCCH, we assume regular PUCCH requirements for p3 and p4. The total reliability after one retransmission for DL and CG UL transmission are given at the end of Table 3.

Table 3: Probabilities terms for total reliability

| Probability | Value |
| --- | --- |
| p1 | 1-1.5*10-5 |
| p2,1 for DL | 1-1.5*10-5 |
| p2,2 for DL | 1-3*10-7 |
| p2,1 for UL | 1-4.5*10-6 |
| p2,2 for UL | 1-2*10-7 |
| p3 | 1-10-2 |
| p4 | 1-2*10-2 |
| DL total reliability after one retransmission | 99.999955% |
| CG UL total reliability after one retransmission | 99.999999993% |

That is, we show that for factory automation scenario at 30 GHz carrier, the requirement of 99.9999% PHY reliability within 1 ms one-way latency for DL and UL transmissions can be achieved.

With enhanced UE processing capability for 120 kHz SCS, it is possible to have one DL or one CG UL retransmission with 7os duration in a TDD configuration with 120 kHz SCS within 1 ms one-way latency.

Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation at 30 GHz carrier can be achieved if enhanced UE processing capability is adopted.

# Conclusion

In section 2 we made the following observations:

Observation 1	With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile DL SINR is -1.57 dB and the 1%-tile UL SINR is -1 dB.

Observation 2	With enhanced UE processing capability for 120 kHz SCS, it is possible to have one DL or one CG UL retransmission with 7os duration in a TDD configuration with 120 kHz SCS within 1 ms one-way latency.

Observation 3	Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation at 30 GHz carrier can be achieved if enhanced UE processing capability is adopted.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- 3GPP RAN1 #94bis Chairman’s note

- 3GPP TS 38.214 V15.3.0

- R1-1900161 Scheduling/HARQ/CSI Processing Timeline Enhancements for NR URLLC, Ericsson, January 2019.

- R1-1900168 Channel Model for Factory Automation Scenario, Ericsson, January 2019.

# Appendix

Table A-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Inter-BS distance | 20m |
| Carrier frequency | 30 GHz |
| Channel model | Modified Indoor Hotspot LOS model with blockers 3GPP TR 38.901 where extra blockage loss is added on top of the InH LoS loss (see e.g., Fig. A-1 and [7]) |
| UE Tx power | 23dBm |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 2, 4), dH = dV = 0.5 λ |
| BS antenna height | 10m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 7 dB |
| UE antenna configuration | 2 Tx/Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 5 dBi |
| UE receiver noise figure | 10 dB |
| BS Tx power | 26 dBm |
| Simulation bandwidth | 160 MHz |
| Duplex mode | TDD (50% DL) |
| SCS | 120 kHz |
| UE distribution | 100% of users are indoor |
| Traffic | Full buffer |

Table A-2: Link level simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 30GHz |
| Channel model | CDL-A 20ns |
| Deployment | Modified indoor hot-spot with blockers |
| UE speed | 3 km/h |
| BS TX antenna configuration | 2 Tx ports |
| UE RX antenna configuration | 2 RX ports |
| System bandwidth | 160 MHz |
| Sub-carrier spacing | 120 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Figure A-1 Modified indoor layout of factory hall with blockers