3GPP TSG RAN WG1 meeting AH1901   				 	R1-1900206

Taipei, Taiwan, January 21-25, 2019

### Agenda Item:	7.2.10.3

### Source:	MediaTek

### Title:	Baseline algorithm for RSTD to UE position calculation

Document for:	Discussion and decision

# Introduction

In OTDOA, the procedure for positioning an UE contains the following steps:

- gNBs transmit PRS for the UE to measure RSTDs (reference signal time differences)

- The UE measures PRSs and reports RSTDs to the location server

- The location server calculates the UE’s position based on the reported RSTDs.

For step (3) above, a baseline algorithm transforming RSTDs to the UE’s position has not been agreed yet.

For studying the accuracy of OTDOA positioning, and for performance evaluation alignment among companies, it is necessary that companies agree on a certain baseline algorithm.

In this contribution, we consider using Taylor series expansion algorithm (TS algorithm) [1] & Chan’s algorithm [2] for transforming RSTDs to UE positions. Roughly speaking, we point out the following observations:

- TS algorithm may not converge to a position close to an UE due to a bad initial guess of UE positions, while Chan’s algorithm doesn’t require such an initial guess.

- Chan’s algorithm outperforms TS algorithm in UMa and UMi scenario

- Under some assumptions, Chan’s algorithm fulfils regulatory and commercial horizontal positioning accuracy requirements in UMa, UMi, and indoor offices.

Base on the above observations, we propose to adopt Chan’s algorithm for transforming RSTDs to UE positions. The details of our analysis, observations and proposals are given in the following sections.

In Section 2, we describe the TS algorithm and Chan’s algorithm. In Section 3, we show our simulation results for 2D and 3D positioning. We conclude this paper in Section 4.

# Algorithms for RSTD to UE position calculation

## The TS algorithm

To describe the TS algorithm [1], we first introduce some notations. Let (x,y) be the coordinates of the UE. Let (xi, yi) be the coordinates of gNB i, where i=1,2…,M. Note that (xi, yi) are known and (x,y) is unknown. Let di=(xi-x)2+(yi-y)2  . Without loss of generality, we assume that gNB 1 is the reference gNB. See Figure 1 for an illustration:

Figure 1: notations for gNB and UE coordinates and the corresponding distances

Let di,1=di-d1. Let RSTDi,1 be the RSTD between gNB i and gNB 1 measured by the UE.

Define hi,1 as

hi,1=x1-xd1-xi-xdi     y1-yd1-yi-ydi.

Let θ=x yT. Since di,1 and hi,1 are functions of θ, we also write di,1=di,1(θ) and hi,1=hi,1(θ).

The TS algorithm can be described as follows:

θ(k+1)=θ(k)+HkT∙H(k)-1∙HkT∙D-dk,

where

θ(k)=xk ykT is the kth estimation of θ, and k=0,1,…,Niter;

d(k)=d2,1k  d3,1k  …  dM,1kT, where di,1k=di,1(θ(k));

D=RSTD2,1   RSTD3,1  …   RSTDM,1T∙c, where c is the speed of light;

H(k)= [h2,1(k)   h3,1(k) …  hM,1(k)]T, where hi,1(k)=hi,1(θ(k)).

Note that this algorithm requires an initial guess [x0 y0] for the UE’s position. Note also that D is given by UE’s RSTD measurement and the value of D does not change during iterations.

## Chan’s algorithm

To describe Chan’s algorithm [2], again we first introduce some notations. Let (x,y), (xi, yi), di, di,1 and RSTDi,1 be the same as those defined in Section 2.1. Note that di,1=RSTDi,1×c if the RSTD measurement is ideal (i.e., error-free). Let Ki=xi2+yi2, xi,1=xi-x1, and yi,1=yi-y1.

Step 1 of Chan’s algorithm is to compute

xyd1 =(GTQ-1G)-1GTQ-1p                                                         (1)

where G=x2,1y2,1d2,1x3,1y3,1d3,1⋮⋮⋮xM,1yM,1dM,1, p=-12d2,12-K2+K1d3,12-K3+K1⋮dM,12-KM+K1 and Q is the covariance matrix of the measured RSTD vector [RSTD2,1   RSTD3,1  …   RSTDM,1]T. Note that in right-hand side of (1), Ki, xi,1, yi,1 are known, and di,1, Q are unknown. The term di,1 is estimated by using di,1=RSTDi,1∙c. The diagonal terms of matrix Q is estimated by using variance of observed samples of RSTDi,1, while the non-diagonal terms of matrix Q are assumed to be zero. Note also that (x,y,d1) is an estimation of (x,y,d1).

Step 2 of Chan’s algorithm is to solve

(x*-x1)2 (y*-y1)2 =(FTΨ-1F)-1FTΨ-1q                                          (2)

where F=100111, Ψ=4L(GTQ-1G)-1L, L=x-x1000y-y1000d1, and q=x-x12y-y12d12. Note that there are two solutions for both x* and y*. Note also that the right-hand side of (2) may be negative. We choose the following solution as the positioning output of Chan’s algorithm:

x*=signx-x1∙Re{ϕ1}+x1

y*=signy-y1∙Re{ϕ2}+y1

where ϕ1 ϕ2T=(FTΨ-1F)-1FTΨ-1q, signt=1 for t≥0, and signt=-1 for t<0.

# Performance evaluation

In all simulation, an UE will measure RSTDi,1 for 60 times, compute the corresponding mean value μi,1 and standard deviation σi,1, and reports μi,1,σi,1 to the location server. The model for one shot RSTDi,1 measurement is RSTDi,1=RSTDi,1ideal+N(0,σrstd).

## 2D positioning for UMa & UMi

We consider the following network topology for UMa and UMi.

Figure 2: Network topology for UMa and UMi

The UEs are dropped in the center hexagon. As shown in the following figure, UEs are dropped on the blues lines, with dUE being the distance from gNB 1 to the blue lines.

Figure 3: Locations of UEs for UMa and UMi

For the TS algorithm, assume there is not prior information of an UE’s coordinates, and the UE knows its serving cell ID. We choose the center of the serving cell as the initial guess of the UE’s coordinates [x0 y0]. Also we set the iteration number Niter=1000 for the TS algorithm.

In Figure 4 we show the simulation results of TS algorithm and Chan’s algorithm in UMa & UMi. For each UE, we use the center 7 TPs for positioning, i.e., only RSTD2,1, RSTD3,1,…, RSTD7,1 are measured. Other simulation settings are given in Table 1.

Figure 4: Comparison of TS & Chan’s algorithm with σrstd=3Ts and dUE=isd2/3.

| Ts | 1/(2048*15000) seconds |
| --- | --- |
| isd | 500 for UMa; 200 for UMi |
| UE height | 1.5m |
| BS height | UMa: 20m, 25m, 30m, 35m, 40m, 45m, 50m, 20m, 25m, 30m, 35m, 40m, 45m, 50m, 20m, 25m, 30m, 35m, 40m for gNB 1,2,…,19, respectively. UMi: 10m for all gNBs |

Table 1: Simulation settings for Figure 4.

From Figure 4, we have the following observations:

Observation 1: Chan’s algorithm outperforms the TS algorithm in UMa and UMi scenarios. In particular, in our simulation setting, we see that Chan’s algorithm achieves that horizontal positioning error < 10m for 100% of UEs.

## 2D positioning for indoor office scenario

We consider the following network topology for indoor office:

Figure 5: Network topology for indoor office. UEs are dropped on the red lines.

Note that gNB 1(the reference gNB) is indicated in Figure 5. For the TS algorithm, the initial guess of an UE’s coordinate [x0 y0] is assumed to be at the center of the red line on which the UE is dropped. Also we set the iteration number Niter=1000.

In Figure 6 we show the simulation results of TS algorithm and Chan’s algorithm. For each UE, all TPs are used for positioning, i.e., all RSTD2,1, RSTD3,1,…, RSTD12,1 are measured. Other simulation settings are given in Table 2.

Figure 6: Comparison of TS & Chan’s algorithm with σrstd=3Ts

| Ts | 1/(4096*120000) seconds |
| --- | --- |
| UE height | 1.5m |
| BS height | 3m for all TPs |

Table 2: Simulation settings for Figure 6.

From Figure 6, we have the following observations:

Observation 2: Chan’s algorithm outperforms the TS algorithm in indoor office scenario. In particular, in our simulation setting, we see that Chan’s algorithm fulfils the requirement that horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios.

Observation 3: Depending on the initial guess an UE’s coordinate, the TS algorithm may fail to converge to a position that is close to the UE’s true position.

## 3D positioning using 3D Chan’s algorithm

It is of interest to see if OTDOA can be used for 3D positioning. Since we already see that Chan’s algorithm outperforms the TS algorithm, in this section we consider Chan’s algorithm only. The extension of Chan’s algorithm from 2D to 3D is straight forward.

Applying the same environment settings as that for Figure 4, we show the simulation results of 3D Chan’s algorithm in UMa scenario in Figure 7. We see that the vertical positioning error is quite large, and the result is far from the requirement that vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios. Furthermore, the horizontal positioning accuracy is worse than that given by using 2D Chan’s algorithm.

The reason for this phenomenon is that the height differences between UE and gNBs are too small in comparison with RSTD error (σrstd=3Ts corresponds to around 30m). In Figure 8, we show the simulation result for 3D Chan’s algorithm in UMa scenario with σrstd=4Tc, where Tc=4096*480*103-1 is the basic time unit of NR. Note that  4Tc corresponds to 0.6m. We see that both horizontal and vertical positioning accuracy are greatly improved. In particular, the vertical result is close to the requirement that vertical positioning error < 3m for 80% of UEs.

Observation 4: Depending on the accuracy requirement, OTDOA can be used for 3D positioning if the standard deviation of RSTD measurements can be reduced to a certain level. In our simulation setting, vertical positioning accuracy is close to the requirement that vertical positioning error < 3m for 80% of UEs if σrstd=4Tc.

Figure 7: Simulation results of 3D Chan’s algorithm in UMa scenario

L: vertical     R: horizontal

Figure 8: Simulation results of 3D Chan’s algorithm in UMa scenario with σrstd=4Tc, where Tc=4096*480*103-1 is the basic time unit of NR

Base on the above observations, we have the following proposals:

Proposal 1: Adopt 2D Chan’s algorithm as that described in Section 2.2 for horizontal positioning.

Proposal 2: Further study the feasibility of using 3D Chan’s algorithm for 3D positioning.

## 3D positioning using 2D Chan’s algorithm with ZOD information

If gNBs are capable of measuring ZODs, then it is possible to measure UE’s height by using 2D Chan’s algorithm with ZOD information. This can be done by using the following equation:

z=z1+cotθ∙(x1-x*)2+(y1-y*)2 ,                                   (3)

where (x1, y1) is the coordinate of reference gNB, θ is an estimation of ZOD,  (x*, y*) is the coordinate of the UE estimated by using 2D Chan’s algorithm, and z is the estimated UE height. See the following figure for an illustration of the above equation:

Figure 9: Illustration of equation (3)

Use the same environment settings as that for Figure 4 with σrstd=4Tc, where Tc=4096*480*103-1.  Assume θ=θ+N(0,π/180) and is generate once for each UE. The positioning error given by using 2D Chan’s algorithm with ZOD is shown below:

Figure 10: Positioning error given by using 2D Chan’s algorithm with ZOD

From Figure 10 we see that even when the horizontal error is very small and the estimated error of ZOD is less than 1 degree, the vertical positioning error is still unsatisfactory.

Observation 5: It is not feasible to estimate an UE’s height by using 2D Chan’s algorithm with ZOD.

# Conclusion

We have the following observations and proposals:

Observation 1: Chan’s algorithm outperforms the TS algorithm in UMa and UMi scenarios. In particular, in our simulation setting, we see that Chan’s algorithm achieves that horizontal positioning error < 10m for 100% of UEs.

Observation 2: Chan’s algorithm outperforms the TS algorithm in indoor office scenario. In particular, in our simulation setting, we see that Chan’s algorithm fulfils the requirement that horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios.

Observation 3: Depending on the initial guess an UE’s coordinate, the TS algorithm may fail to converge to a position that is close to the UE’s true position.

Observation 4: Depending on the accuracy requirement, OTDOA can be used for 3D positioning if the standard deviation of RSTD measurements can be reduced to a certain level. In our simulation setting, vertical positioning accuracy is close to the requirement that vertical positioning error < 3m for 80% of UEs if σrstd=4Tc.

Observation 5: It is not feasible to estimate an UE’s height by using 2D Chan’s algorithm with ZOD.

Proposal 1: Adopt 2D Chan’s algorithm as that described in Section 2.2 for horizontal positioning.

Proposal 2: Further study the feasibility of using 3D Chan’s algorithm for 3D positioning.

# References

- "Position-location solution by Taylor-series estimation", W.H. Foy, IEEE Trans. Aerosp. Electron. Syst., vol. AES-12, no. 2, pp. 187-194, 1976.

- “A Simple and Efficient Estimator for Hyperbolic Location”, Y. T. Chan and K. C. Ho, IEEE transactions on signal processing, vol. 42, no. 8, August 1994.