3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900238

Taipei, January 21st – 25th, 2019

Source: 	ZTE Corporation

Title: 	Performance evaluation for urban macro scenarios at 700 MHz

Agenda item:	7.2.6.4

Document for:	Discussion/Decision

Introduction

During previous meetings, the simulation assumptions for eURLLC in Rel-16 have been finalized [1][2][3]. As depicted in Table 1, 700 MHz carrier frequency is applicable for power distribution and transport industry.

Table 1 Scenarios and corresponding carrier frequencies for SLS

| Scenario | Carrier frequency for SLS |
| --- | --- |
| Power distribution | 700 MHz, 4 GHz |
| Factory automation | 4 GHz, 30 GHz |
| Rel-15 enabled use cases | 4 GHz |
| Transport industry | 7 00MHz, 4 GHz |

In this contribution, we provide our geometry and simulation results of transport industry scenario at 700 MHz for calibration and comparison. The detailed geometry and simulation results of transport industry scenario at 4 GHz could be found in our companion contribution [4].

Geometry calibration

Per the discussion in last meeting, companies are encouraged to provide simulation results with the corresponding geometry. Both with-re-drop and without-re-drop are allowed. In this section, both DL and UL geometry without-re-drop for transport industry are present.

The detailed simulation assumptions for DL/UL geometry are summarized in Table A-1 in the appendix. As depicted in Figure 1, the 5% DL Q-value for this scenario is -0.44 dB and the 5% UL Q-value for this scenario is -1.54 dB.

Observation 1:For transport industry at 700 MHz,

- The 5% Q-value of DL geometry is -0.44 dB.

- The 5% Q-value of UL geometry is -1.54 dB.

Figure 1 Geometry of transport industry at 700 MHz

Simulation results

This section provides the detailed simulation results of transport industry at 700 MHz including both DL and UL. During RAN1#94b [2], two options, i.e., percentage of UEs satisfying eURLLC requirements and maximum offered cell load, are agreed to be applied as the performance metrics. Furthermore, companies converged on the following agrements in RAN1#95 for better calibration and comparison [3].

Agreements: - Companies report the value of X used in the evaluations (either ~5% or 0%). - The value of X should be 0% if re-dropping is used in the evaluations.

In this section, we provide both percentage of UEs satisfying eURLLC requirements and maximum offered cell load. All the simulation results are based on the assumption of no re-drop. The detailed simulation assumptions for eURLLC SLS are summarized in Table A-1 in the appendix.

Downlink

All the downlink transmission is grant-based with a latency-prioritized scheduling algorithm. The DL simulation results of remote driving at 700 MHz are summarized in Table 2 and the DL simulation results of ITS at 700 MHz are summarized in Table 3.

Table 2 DL Simulation results of transport industry (Remote driving) at 700 MHz

| Description: Remote driving Reliability: 99.999% Latency: 5 (end to end latency), Note: 3ms air interface latency Packet size: For UL: 2.5 Mpbs; Packet size 5220 bytes. For DL: 1Mbps; Packet size 2083 bytes. Note: Data arrival rate 60 packets per second for periodic traffic model |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 2 | 1.9081 | 4.8% |

| Description: Intelligent transport system (ITS) Reliability: 99.999% Latency: 10(end to end latency), Note: 7ms air interface latency Packet size: 1.1 Mbps, Packet size 1370 bytes. Note: Data arrival rate 100 packets per second for periodic traffic model |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 2 | 2.0920 | 0% |

Table 3 DL Simulation results of transport industry (ITS) at 700 MHz

- Observation 2: For transportation industry at 700MHz with 2 UEs per cell, the outage percentage is smaller than 5% for downlink.

Uplink

All the uplink transmission is grant-based with a latency-prioritized scheduling algorithm. The UL simulation results of ITS at 700 MHz are summarized in Table 4. As we can see, only 4.8% of UEs can’t satisfy the corresponding reliability and latency requirements.

| Description: Intelligent transport system (ITS) Reliability: 99.999% Latency: 10(end to end latency), Note: 7ms air interface latency Packet size: 1.1 Mbps, Packet size 1370 bytes. Note: Data arrival rate 100 packets per second for periodic traffic model |  |  |
| --- | --- | --- |
| Scenario configurations | Offered cell load (Mbps) | UEs in outage |
| UE number per cell: 2 | 2.0918 | 4.8 % |

Table 4 Grant-based UL Simulation results of transport industry (ITS) at 700 MHz

- Observation 3: For transportation industry at 700MHz with 2 UEs per cell, the outage percentage is smaller than 5% for grant based uplink.

Conclusion

In this contribution, the geometry and simulation results of transport industry at 700 MHz are provided for calibration and comparison. To sum up, the following observations are made.

Observation 1:For transport industry at 700 MHz,

- The 5% Q-value of DL geometry is -0.44 dB.

- The 5% Q-value of UL geometry is -1.54 dB.

- .Observation 2: For transportation industry at 700MHz with 2 UEs per cell, the outage percentage is smaller than 5% for downlink.

Observation 3: For transportation industry at 700MHz with 2 UEs per cell, the outage percentage is smaller than 5% for grant based uplink.

Reference

- R1-1810051, “Final Report of 3GPP TSG RAN WG1 #94 v1.0.0”, Gothenburg, Sweden, August 20– 24, 2018.

- R1-1812101, “Final Report of 3GPP TSG RAN WG1 #94bis v1.0.0”, Chengdu, China, October 8–12, 2018.

- Draft Report of 3GPP TSG RAN WG1 #95 v0.2.0, Spokane, USA, 12–16 November 2018.

- 3GPP RAN1 #Ad-Hoc1901, R1-1900080, “Performance evaluation for transport industry at 4 GHz”, Taipei, January 21–25, 2019.

Appendix

Table A-1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Road configuration in Figure 6.1.9-1 in 38.913 and BS placement as depicted in Figure A.1.3-1 in 36.885. |
| Inter-BS distance | 500m |
| Carrier frequency | 700MHz |
| Channel model | UMa in TR 38.901 |
| BS antenna configurations | 2 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 1, 2, 1, 1; 1, 1) |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | - For 2 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UE antenna height | 1.5m |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | 2 |
| Simulation bandwidth | 20 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 60km/h for modeling fading channel |
| UE power control | P0= -90; alpha = 1 |
| HARQ/repetition | Synchronous HARQ Max number of transmission =4 |
| Channel estimation | Ideal |
| Scheduling algorithm | Latency-prioritized scheduling algorithm Each slot has three PDCCH occasions. |