3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                                                       R1-1900277

Taipei, 21st – 25th January, 2019

Source:	OPPO

Title:	Discussion on SS rasters for NR-U

Agenda Item:	7.2.2.3

Document for:	Discussion and Decision

Introduction

Several topics related to the initial access procedure in NR-U have been discussed in the SI stage, e.g. channel access, DRS design, enhancements on PRACH and paging. Besides these topics, as SA deployment is supported in NR-U, the initial cell search procedure and the SS raster design for NR-U need to be discussed as well.

Discussion

In LTE and NR systems[1-3], two kinds of raster have been defined. One is the channel raster, which can be used to indicate the center frequency of a carrier. Another is the SS raster, which indicates the frequency position of synchronization signal and can be used by a UE for system acquisition.

In LTE[1], the SS raster is same as the channel raster which is 100 kHz for all bands given by the following equation,

FDL = FDL_low + 0.1(NDL – NOffs-DL)

where FDL_low and NOffs-DL are given in Table 1 and NDL is the downlink ARFCN.

Table 1   SS raster and channel raster in LTE

| E-UTRA Operating Band | Downlink |  |  |
| --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL |
| … |  |  |  |
| 26 | 859 | 8690 | 8690 – 9039 |
| 27 | 852 | 9040 | 9040 – 9209 |
| 28 | 758 | 9210 | 9210 – 9659 |
| 29 | 717 | 9660 | 9660 – 9769 |
| 30 | 2350 | 9770 | 9770 – 9869 |
| 31 | 462.5 | 9870 | 9870 – 9919 |
| 32 | 1452 | 9920 | 9920 – 10359 |
| 33 | 1900 | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 –41589 |
| … |  |  |  |

In NR, considering the band is wider than that of LTE, if 100 kHz SS raster is reused for cell search, the complexity will be extremely high. So the SS raster needs to be set sparser than that before. However, if the channel raster is set sparser accordingly, the flexibility of cell deployment will be confined. After a long discussion in both RAN1 and RAN4 in Rel-15, finally the channel raster and the SS raster are redesigned separately in NR considering the different requirements and new characteristics of the NR-U system.

In Rel-15[2-3], different channel rasters are used in different licensed bands, e.g. 100kHz for n38 and 15kHz for n41. More detailed designs are shown in Table 2, the channel rasters in each NR operating band are given by the applicable NR-ARFCN. In NR, the SS raster for licensed bands has also been defined as shown in Table 3. Granularities of the SS raster are 1.2MHz and 1.44MHz for 0-3GHz and 3-24.25GHz respectively. This kind of design with sparser SS raster and denser Channel raster has considered the tradeoff between the complexity of cell search and the flexibility of cell deployment.

Table 2   Applicable NR-ARFCN per operating band

| NR Operating Band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
| n77 n78 | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
|  | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
| n78 n79 | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
|  | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
| n79 n80 | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
|  | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n86 | 100 | 342000 – <20> – 356000 | N/A |

Table 3   SS raster for different frequency range

| Frequency range | SS Block frequency position SSREF |
| --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} |
| 3000-24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 |

In Rel-16, SA is supported in NR-U, so the SS raster for unlicensed bands shall be designed accordingly. In a NR-U system, nominal channels with 20MHz bandwidth are predefined and only these nominal channels can be used to set up cells, i.e. the flexibility of cell deployment is restricted due to the granularity of available channels. Based on the restriction, unlicensed bands do not need too much SS rasters and channel rasters within a nominal channel. Actually, setting one or a few SS rasters within each nominal channel could ensure the needs for cell deployment, meanwhile it also benefits the cell search procedure in NR-U SA cases. Thus, the design of SS raster for unlicensed bands shall be reconsidered.

Proposal 1: 	The design of SS raster for unlicensed bands shall be reconsidered in NR-U.

Besides, the design of SS raster may also affect other issues in NR-U, such as the DRS design within an initial BWP, different designs for the signal/channel within a DRS and the method to fulfill OCB requirements in SSB symbols. So it is better to conduct related works as soon as possible. Considering the detailed work for NR-U SS raster design shall be conducted by RAN 4, as a start point, RAN 1 can give some background information and requirements to RAN4, e.g. a LS on this issue may be a good choice if necessary.

Proposal 2: 	The design of SS raster for NR-U shall be conducted as soon as possible and RAN1 could give some background information and requirements to RAN4.

Conclusion

In this contribution, the SS raster in NR-U has been discussed. The following are proposed.

Proposal 1: 	The design of SS raster for unlicensed bands shall be reconsidered in NR-U.

Proposal 2: 	The design of SS raster for NR-U shall be conducted as soon as possible and RAN1 could give some background information and requirements to RAN4.

References

- 3GPP TS 36.101

- 3GPP TS 38.101-1

- 3GPP TS 38.101-2