3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                                                    R1-1900281

Taipei, 21st – 25th January, 2019

Source:	OPPO

Title:	PDCCH enhancement for URLLC

Agenda Item:	7.2.6.1.1

Document for:	Discussion and Decision

Introduction

In RAN1 95, some agreements on PDCCH enhancement for NR URLLC were achieved in the following [1]:

Agreements:

For link-level PDCCH evaluation, the target operating BLER of DCI(s) scheduling HARQ-less PDSCH/PUSCH should be smaller than 1e-x in Rel-16 NR URLLC, at the 5%-tile SINR geometry.

- x is the reliability requirement given in the table of representative use case for evaluation agreed in the RAN1#94bis meeting.

- The 5%-tile SINR geometry is obtained by system-level simulation assuming full buffer for a given evaluation scenario.

- This target assumes no HARQ re-transmission

Agreement:

- No change of DCI format 0_0/1_0 in CSS from Rel-16 URLLC study item perspective

Agreements:

- To further study DCI for URLLC with a size potentially smaller than that of Rel-15 fallback DCI

  - Consider using Rel-15 fallback DCI as a starting point for Rel-16 URLLC DCI

  - Target a reduction of at least 10-16 bits compared to Rel-15 fallback DCI

    - Companies report how to achieve the DCI size reduction

  - The link level performance gain from PDCCH reliability perspective

    - Check at least AL=16

  - PDCCH resource utilization considering all UEs in the cell

    - Check AL=1/2/4/8/16

    - If retransmission is feasible with the latency bound, different BLER target can be used

  - The PDCCH blocking probability when applicable

  - The performance impact from compact DCI including impact to PDSCH/PUSCH capacity when applicable

  - The impact on PDCCH blind decoding/DCI size budget

  - The impact on PDSCH/PUSCH scheduling flexibility

  - At least Rel-15 enabled use cases should be evaluated for the above study

In this contribution, we shall focus on PDCCH enhancement based on PDCCH evaluation.

Discussion

  - PDCCH reliability

As agreed in the RAN1 #95 meeting, evaluation assumption for PDCCH enhancement were discussed. In this contribution, link level simulation is performed to get required SINR to achieve target reliability and system level simulation is performed to get 5%-tile SINR geometry.

The reliability requirement is given in the table of representative use case for evaluation agreed in the RAN1#94bis meeting. For power distribution and factory automation, stricter requirement on reliability, 99.9999%, than Rel-15 is requested.  For other cases, similar or lower requirement on reliability than Rel-15 is requested.

According to 38.212, the fallback DCI payload size is about 40 bit for bandwidth with 20/40MHz and SCS with 30 kHz. The other simulation assumptions are provided in Appendix 6.2. From Figure 1a and Figure 1b, the required smallest SNR to achieve 99.999% and 99.9999% is -8.4dB and -7.8 dB for 4GHz with 4Rx antenna and -5.5 and -5 dB for 700MHz with 2Rx antenna.

Figure 1: Performance of NR-PDCCH for carrier frequency 700MHz and 4GHz.

According to agreed system simulation assumption for Power distribution, Factory automation and Rel-15 AR/VR, the DL geometry is provided in our accompany contribution [2]. The 5%-tile SINR geometry is summarized in Table 1.

Table 1 5%-tile SINR geometry for some URLLC use cases

|  | Power distribution | Factory automation | Rel-15 AR-VR (UMA) | Rel-15 AR-VR (Indoor) |
| --- | --- | --- | --- | --- |
| 5%-tile DL geometry | -2.6/-2.7 | -2.47 | -2.55/-3.35 | -3.42 |

According to link level simulation and system simulation, we could observe:

Observation 1: For carrier frequency 4GHz with 4Rx and 700MHz with 2Rx, Rel-15 NR PDCCH can meet reliability requirement of 99.999% and 99.9999%.

  - Schedule flexibility

In Rel 15, fallback PDCCH reliability has been discussed but flexibility is lost to some extent. For example, MIMO transmission configuration, CSI feedback configuration, BWP switching and CA switching are not supported. However, these features are necessary for URLLC.

Antenna port bitfied

In Rel16, AR/VR is one scenario for physical layer enhancement for URLLC, which means high data rate will be supported in URLLC. Multiple layer transmission is one effective method to improve data rate. In addition, higher availability is one requirement for Rel16 URLLC, which means spectrum efficiency needs to be considered in Rel-16 URLLC. Therefore, multiple layers transmission should be considered in Rel 16, corresponding, antenna port bitfied needs to be added in compact DCI.

CSI request bitfield

MCS does not always adapt to channel due to channel variation from measurement to transmission, however, aperiodic CSI report can provide more accurate channel information due to smaller gap between measurement and transmission, which is benefit for system efficiency and can reduce system congestion. So it is suggested to add CSI request bitfield for compact DCI, including PDSCH and PUSCH scheduling. Considering that aperiodic CSI is used for instantaneous feedback, smaller ReportTriggerSize is enough and is also benefit for reliability of PDCCH.

BWP indicator bitfield

To reduce transmission latency, URLLC is scheduled in BWP with larger SCS than eMBB. Moreover, due to URLLC applies smaller granularity of time resource allocation, Transmission bandwidth for URLLC is wider than eMBB. To reduce transmission resource congestion, BWP with wider bandwidth is more benefit for URLLC than eMBB. So flexible BWP configuration is necessary to schedule URLLC and eMBB dynamically. One scheme is to add BWP indicator bitfield in DCI.

Carrier indicator bitfield

In TDD system, Downlink-uplink configuration may lead schedule latency. But if downlink-uplink configuration is configured to meet latency requirement, it will introduce many downlink-uplink switching point. TDD-FDD CA is one solution to reduce latency from TDD carrier. If that, dynamic carrier switching is necessary.

Proposal 1: To enhance schedule flexibility and efficiency, Antenna port, CSI request, BWP indicator and Carrier indicator need to be added in fallback DCI or new DCI.

According to PDCCH reliability evaluation, the payload size of fallback DCI just meets reliability requirement. If some bit fields are added to support flexible schedule, then some bit fields need to be compressed or removed.

To target URLLC transmission feature, such as larger transmission bandwidth with short time duration, fast schedule and feedback, short process timeline, some fields can be compressed and removed, as shown in the following table 2:

Table 2 Compressed/removed field in fallback DCI targeting new scenario

| Bit fields | Reason | Scheme |
| --- | --- | --- |
| Frequency-domain RA | Larger transmission bandwidth with short time duration is benefit for low latency | - Larger RBG size - Type 1 RA only |
| Time-domain RA | - Different characteristics of time domain resource between eMBB and URLLC - Low latency requirement needs immediate transmission and flexible starting symbol - High reliability requirements needs enough repetition | - URLLC-specific pdsch-symbolAllocation - Redefined pdsch-symbolAllocation - Flexible PDSCH duration,1-14 symbols - PDSCH/PUSCH starting symbol indication relative to PDCCH - PDSCH/PUSCH duration across slot boundary |
| HARQ process number | Low latency requirement makes HARQ round trip time faster | - Less HARQ process number |
| PUCCH resource indicator | HARQ-ACK is not always meaningful. - HARQ-ACK feedback occasion exceeds latency requirement in some SFI configurations. - UE with large queuing delays does not have enough time to feedback HARQ-ACK or retransmission | - PUCCH resource indicator can be removed - Specific value is included in PUCCH resource indicator to indicate no HARQ-ACK transmission. |
| PDSCH-to-HARQ feedback timing indicator | Considering low latency requirement, PDSCH-to-HARQ feeback timing should be small value, such as 0 and 1 slot. | - PDSCH-to HARQ feedback timing indicator bitfield can be reduced. - According to PUCCH resource indicator and UE capability, PDSCH-to-HARQ feedback timing can be determined implicitly. |

Proposal 2: To enhance reliability of DCI, Frequency/Time-domain RA, HARQ process number, PUCCH resource indicator and PDSCH-to-HARQ feedback timing indicator bitfield, can be compressed or removed

  - PDCCH blockage

One scheme to reduce DCI overhead is that multiple transmission resources are configured by RRC or MAC CE and one of configured resources is triggered by DCI when traffic occurs.

Typically, multiple transmission resources are preconfigured and one transmission resource is triggered to adapt to channel condition and traffic requirement. For typical URLLC traffic, preconfigured transmission resources can be triggered by DCI and for non-typical traffic, dynamic DCI can be used.

Generally, there are two schemes on DCI-triggered configured resource in general.

Scheme2.1: All transmission parameters are configured by RRC or MAC CE. Only configured resource index is triggered by DCI.

Scheme2.2: Some transmission parameters, such as Time-Frequency resource and DMRS resource, are configured by RRC or MAC CE. The other transmission parameters, such as HARQ process number and RV are still configured by DCI or indicated implicitly by slot index or other parameters.

Proposal 3: To reduce DCI overhead, multiple transmission resources can be preconfigured by RRC or MAC CE and one of configured resources is triggered by DCI when traffic occurs.

Comparing with scheme1, scheme2 provides significant overhead reduction but flexible configuration combination. Take 2-bit DCI as an example, for scheme 1, if one bit is used for frequency resource and the other bit is used for time resource, then only 4 time-frequency resource configuration can be indicated.  However, for scheme 2, 4 status can indicates 4 flexible combination, such as 1) 1st time frequency resource, 1st HARQ ID and 1st feedback timing, 2) 1st time frequency resource, 1st HARQ ID and 2nd feedback timing 3) 2nd time frequency resource, 1st HARQ ID and 1st feedback timing 4) 1st time frequency resource, 2nd HARQ ID and 1st feedback timing.

Observation 2: Comparing with scheme1, scheme2 provides significant overhead reduction but flexible configuration combination.

  - PDCCH monitoring capability

Higher PDCCH monitoring capability

To meet low latency, frequent PDCCH monitoring is necessary. However, Schedule flexibility and available aggregation level will be restricted by PDCCH monitoring capability. Take SCS = 15kHz as an example, PDSCH scheduling occasion is per 2-symbol. According to defined PDCCH monitoring capability in Rel 15, each PDCCH monitoring occasion can have up to 6 PDCCH candidates and 8 non-overlapped CCEs. Assuming the UE receives DL assignment and UL grant at one time of PDCCH monitoring occasion, each PDCCH (i.e., DL assignment or UL grant) can have up to 4 CCEs, which may not meet reliability requirement for UEs with low SINR.

Proposal 4: Higher PDCCH monitoring capability needs to be considered.

Downlink SPS transmission enhancement

Though higher PDCCH monitoring capability is supported, low latency and high reliability means less PDCCH symbol per PDCCH occasion and higher aggregation per PDCCH, therefore, PDCCH congestion is still a bottleneck for DL URLLC transmission. For example, the 10 MHz system bandwidth consists of 50 PRB and PDCCH REGs/CCE is 6, then if a UL grant or DL assignment uses 8 CCEs no more UL grants or DL assignments can be scheduled on PDCCH.

To resolve PDCCH blockage, downlink SPS transmission is an effective way. On the one hand, SPS with shorter periodicity is considered to support short periodicity traffic. On the other hand, Periodical SPS resource can be applied for non-periodical traffic, similar as grant free in uplink. UE can detect traffic presence or not by DMRS.

Proposal 5: Downlink SPS transmission is an effective way to resolve PDCCH blockage. And downlink SPS is not only configured for periodical traffic but also for non-periodical traffic.

Finer PDCCH monitoring capability reporting

In NR Rel-15, the maximum number of PDCCH blind decoding is limited to 44 per slot, no matter the actual configuration of COREST. Such design has following problems:

- UE capability of PDCCH blind decoding is not fully used, and the loss of scheduling flexibility can be expected.

- UE cannot achieve 44 blind decodings when the configuration of COREST is uneven, i.e. more blind decodings are configured to the last COREST in one slot.

To satisfy the latency requirement of Rel-16 URLLC, PDCCH monitoring capability should be improved, i.e. the configuration of COREST should be more accurate based on the UE capability:

  - Approach 1: UE reports the number of symbols to achieve X blind decodings. The value of X should be further studied.

  - Approach 2: UE reports the maximum number of blind decodings during Y symbols. The value of Y should be further studied.

Approach 1 and approach 2 are essentially same. One example is shown in Figure 1, once UE reports up to 20 blind decodings can be achieved during 2 symbols, up to 140 blind decodings can be configured in one slot for URLLC.

Figure 1: More accurate configuration of COREST

Proposal 6: Finer PDCCH monitoring capability reporting should be reported.

Conclusions

In this contribution, we show our views on PDCCH enhancement for URLLC with following proposals:

Observation 1: For carrier frequency 4GHz with 4Rx and 700MHz with 2Rx, Rel-15 NR PDCCH can meet reliability requirement of 99.999% and 99.9999%.

Proposal 1: To enhance schedule flexibility and efficiency, Antenna port, CSI request, BWP indicator and Carrier indicator need to be added in fallback DCI or new DCI.

Proposal 2: To enhance reliability of DCI, Frequency/Time-domain RA, HARQ process number, PUCCH resource indicator and PDSCH-to-HARQ feedback timing indicator bitfield, can be compressed or removed.

Proposal 3: To reduce DCI overhead, multiple transmission resources can be preconfigured by RRC or MAC CE and one of configured resources is triggered by DCI when traffic occurs.

Observation 1: Comparing with scheme in proposal 2, scheme in proposal 3 provides significant overhead reduction but flexible configuration combination.

Proposal 4: Higher PDCCH monitoring capability needs to be considered.

Proposal 5: Downlink SPS transmission is an effective way to resolve PDCCH blockage. And downlink SPS is not only configured for periodical traffic but also for non-periodical traffic.

Proposal 6: Finer PDCCH monitoring capability reporting should be reported.

References

- RAN1 95 Chairman notes

- R1-1900288 Geometry Results for URLLC evaluation OPPO

- R1-1812221 PDCCH enhancements for URLLC Huawei, HiSilicon

Appendix

  - Requirement for representative use cases

Table A.5.1-1: Representative use cases for Rel-16 NR URLLC evaluation

| Use case | Reliability (%) | Latency | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Power distribution | 99.9999 | 5 ms (end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms | Power distribution grid fault and outage management (TR 22.804:5.6.4) |
|  | 99.999 | 15 ms (end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs | Differential protection (TR 22.804:5.6.6) |
| Factory automation | 99.9999 | 2 ms (end to end latency) Note: 1 ms air interface latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms | Motion control |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1 ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |
|  | 99.9 | 7 ms (air interface delay) | DL & UL: 4096 and 10 K bytes FTP model 3 or periodic with different arrival rates |  |
| Transport Industry | 99.999 | 5 ms (end to end latency) Note: 3 ms air interface latency | UL: 2.5 Mpbs; Packet size 5220 bytes DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model | Remote driving (TS 22.186: 5.5) |
|  | 99.999 | 10 ms (end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps; Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model | Intelligent transport system (ITS) (TS 23.501, TS 22.261) |

  - Link level simulation assumption

Table A.5.2-1: Link-level simulation assumptions at 4 GHz for all cases with urban macro

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for power distribution and Rel-15 enabled use case; |
| BS antenna configuration | 4 Tx antenna ports |
| UE antenna configuration | 4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Q value (i.e. SINR range) | Companies report the 5% Q value |

- Evaluation of 700 MHz and 2 GHz carrier frequency are not precluded.

Table A.5.2-2: Link-level simulation assumptions at 700 MHz for all cases with urban macro

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| BS antenna configuration | 2 Tx/2 Rx antenna ports |
| UE antenna configuration | 2 Tx/2 Rx antenna ports and 2 Tx/4 Rx antenna ports |
| System bandwidth | 20 MHz |
| Other parameters | As shown in Table A.3-1 for 4 GHz |