3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                                                    R1-1900288

Taipei, 21st – 25th January, 2019

Source:	OPPO

Title:	Geometry Results for URLLC evaluation

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Introduction

In RAN1 94 and 94bis, simulation assumptions for URLLC were discussed. In this contribution, geometry CDF in both downlink and uplink are presented.

Simulation

Based on system simulation assumption in the appendix, we obtain DL and UL SINR geometry for power distribution, factory automation and Rel-15 use case AR/VR.

Figure 1 DL and UL geometry for power distribution

Figure 2 DL and UL geometry for factory automation

Figure 3 DL and UL geometry for Rel-15 use case AR/VR

From Figure 1-3, we summarize 5%-tile DL and UL SINR in Table 1.

Table 1 5%-tile DL and UL SINR

| (dB) | Power distribution | Factory automation | Rel-15 use case (AR/VR) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Urban Macro | Indoor hotspot |
| DL SINR geometry | -2.6/-2.7 | -2.47 | -2.55/-3.35 | -3.42 |
| UL SINR geometry | -2.67/-2.86 | -0.88 | -2.5/-3.09 | -1.67 |

Conclusion

In this contribution, geometry CDF in both downlink and uplink are presented for power distribution, factory automation and Rel-15 use case AR/VR and 5%-tile DL and UL SINR is summarized in Table 1.

Appendix

A.4.1	Simulation assumption for electrical power distribution

This subclause describes the simulation assumptions for evaluating electrical power distribution. Table A.4.1-1 shows the evaluation assumptions at 4 GHz for some basic parameters for urban macro. Table A.4.1-2 shows the evaluation assumptions at 700 MHz for power distribution with urban macro scenario, assumptions for the remaining parameters are the same as that at 4 GHz as shown in Table A.4.1-1 Note that this does not imply that Rel-16 NR URLLC is necessarily restricted to urban macro scenario for electrical power distribution. Rural scenario is also applicable.

Table A.4.1-1: System-level simulation assumptions at 4 GHz for urban macro for power distribution

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m Note: Other value (e.g. 150 m) is not precluded |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) for 4 Tx/4 Rx antenna ports; (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4) for 8 Tx/8 Rx antenna ports; dH = 0.5λ, dV = 0.8λ; Antenna tilt =95 |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi as starting point |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC as the baseline receiver |
| Number of UEs per cell | 10 |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3 km/h for modeling fading channel |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |

Table A.4.1-2: System-level simulation assumptions at 700 MHz for urban macro for power distribution

| Parameters | Value |
| --- | --- |
| BS antenna configurations | 2 Tx/2 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8,1,2,1,1;1,1) (dH, dV) = (N/A, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | 2Tx/2 Rx antenna ports (M, N, P, Mg, Ng) = (1,1,2,1,1) 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; 0°, 90° polarization |
| Simulation bandwidth | 20 MHz |
| Other parameters | As shown in Table A.2.1-1 for 4 GHz |

A.4.2	Simulation assumption for factory automation

This subclause describes the simulation assumptions for evaluating factory automation. Table A.4.2-1 shows the evaluation assumptions at 4 GHz for factory automation.

Table A.4.2-1: System-level simulation assumptions at 4 GHz for factory automation

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| UE Tx power | 23dBm |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| BS antenna configurations | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Tx/4 Rx antenna ports; (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2) for 8 Tx/8 Rx antenna ports; dH = dV = 0.5 λ |
| BS antenna height | 10 m |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi as starting point |
| BS Tx power | 24 dBm per 20 MHz |
| BS receiver | MMSE-IRC as the baseline receiver |
| UE receiver noise figure | 9 dB |
| SCS | 30 kHz |
| Simulation bandwidth | 40 MHzFor TDD, 40 MHz for DL/UL. |
| Layout | Single layer as defined in 38.802 Indoor floor:12 BSs per 120 m x 50 m |
| Channel model | ITU InH for 4 GHz |
| Number of UEs per cell | 40 |
| UE distribution | 100% of users are indoor: 3 km/h and/or 30 km/h UE-speed |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |

Table A.4.2-2 shows the evaluation assumptions at 30 GHz for factory automation. Assumptions for the remaining parameters are the same as that at 4 GHz as shown in Table A.4.2-1.

Table A.4.2-2: System-level simulation assumptions at 30 GHz for factory automation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 30 GHz |
| BS receiver noise figure | 7dB as defined in TR 38.802 |
| BS antenna configurations | 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) dH = dV = 0.5 λ |
| UE antenna configuration | 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1) (dH, dV) = (0.5, 0.5) λ Static panel selection |
| UE antenna gain | 5dBi |
| BS Tx power | 23 dBm for 80 MHz bandwidth |
| UE receiver noise figure | 10 dB |
| SCS | 120 kHz |
| Simulation bandwidth | 160 MHz |
| Channel model | 5GCM office for 30 GHz |

1 ms air interface latency is assumed for evaluation for factory automation, with the assumption of 1 ms CN delay in 2 ms end-to-end latency. Other values for evaluation can also be considered.

A.4.3	Simulation assumption for transport industry

This subclause describes the simulation assumptions for evaluating transport industry, including remote driving and intelligent transport system (ITS). For evaluating urban macro scenario for transport industry, simulation assumptions at 4 GHz are provided in Table A.4.3-1 and simulation assumptions at 700 MHz are provided in Table A.4.3-2.

Table A.4.3-1: System-level simulation assumptions at 4 GHz for urban macro for transport industry

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Road configuration in Figure 6.1.9-1 in 38.913 and BS placement as depicted in Figure A.1.3-1 in 36.885. |
| UE antenna height | 3.0 m |
| Number of UEs per cell | 10 |
| UE distribution | 100% of users are outdoors UE speed of 60 km/h |
| Other parameters | As shown in Table A.2.1-1 for power distribution |

Table A.4.3-2: System-level simulation assumptions at 700 MHz for urban macro for transport industry

| Parameters | Value |
| --- | --- |
| BS antenna configurations | 2 Tx/2 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8,1,2,1,1;1,1) (dH, dV) = (N/A, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | 2Tx/2 Rx antenna ports (M, N, P, Mg, Ng) = (1,1,2,1,1) 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; 0°, 90° polarization |
| Simulation bandwidth | 10 MHz, 20 MHz |
| Other parameters | As shown in Table A.2.3-1 for 4 GHz |

A.4.4	Simulation assumption for Rel-15 enabled use case

This subclause describes the simulation assumptions for evaluating Rel-15 enabled use case (e.g. AR/VR). For evaluating Rel-15 enabled use case with urban macro (applicable data packet size 32 bytes and 200 bytes), simulation assumptions are provided in Table A.4.4-1.

Table A.4.4-1: System-level simulation assumptions at 4 GHz for Rel-15 enabled use case with urban macro (applicable data packet size 32 bytes and 200 bytes)

| Parameters | Value |
| --- | --- |
| Number of UEs per cell | 20 |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modelled according to low loss model |
| Other parameters | As shown in Table A.2.1-1 for power distribution |

For evaluating Rel-15 enabled use case with indoor hot-spot, simulation assumptions are provided in Table A.4.4-2.

Table A.4.4-2: System-level simulation assumptions at 4 GHz for Rel-15 enabled use case with indoor hot-spot

| Parameters | Value |
| --- | --- |
| Number of UEs per cell | 20 |
| UE distribution | 100% of users are indoors: 3 km/h UE-speed |
| BS antenna height | 3 m |
| Channel model | ITU InH for 4 GHz |
| Other parameters | As shown in Table A.4.2-1 for factory automation |