3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900362

Taipei, Taiwan, January 21st – 25th, 2019

Agenda Item:	7.2.2.1.2

Source:	Sony

Title:	Considerations on DL Signals and Channels for NR unlicensed operation

Document for:	Discussion / Decision

Introduction

In the RAN#82 meeting, the work item for NR-U was approved [1]. The WID includes the following objectives related to DL signals and channels.

-	For DL data channel, support of multiple PDSCH starting positions.

-	Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption.

-	DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

In this contribution, we discuss the design of DL signals and channels for unlicensed NR.

Discussion

Dynamic PDCCH monitoring

A Load-Based Equipment (LBE) device has to transmit any signal immediately after LBT has succeeded, while NR is a slot-based system. Considering channel utilization efficiency on the unlicensed band, it would be desirable to minimize the use of reservation signals used only for occupying the channel. In principle, NR-U should be designed such that PDSCH is efficiently transmitted on any initial partial slot. Non-slot-based PDSCH transmission was supported in Rel-15 NR, which should be reused for NR-U. However, requiring that the UE always monitors PDCCH for non-slot-based PDSCH scheduling causes high power consumption for UE due to frequent monitoring. Therefore, dynamic switching of PDCCH monitoring periodicity could be considered in NR-U. Only for an initial slot on COT, the UE attempts to monitor PDCCH for non-slot-based PDSCH scheduling, while for other slots the UE attempts to monitor PDCCH for slot-based scheduling

Figure 1: Dynamic switching of PDCCH monitoring periodicity depending on slot type

Proposal 1: Dynamic switching of PDCCH monitoring periodicity should be supported in NR-U.

- For an initial slot on COT, PDCCH monitoring of non-slot-based scheduling should be applied.

- For other slots, PDCCH monitoring of slot-based scheduling should be applied.

Mechanism to detect COT start for UE power saving

We proposed in section 2.1 that dynamic PDCCH monitoring should be considered. Next, the mechanism for how to switch PDCCH monitoring periodicity needs to be considered. One of the solutions is switching based on the detection of an initial partial slot. By detecting the COT starting point, the UE can assume that the duration from the COT starting point to the next slot boundary is an initial partial slot.

Observation 1: Detection of COT starting point is useful for dynamic switching of PDCCH monitoring periodicity.

During the study item phase, the concept of a signal transmitted prior to a NR-U DL transmission burst, which is used to detect transmissions from a serving cell for the purpose of power saving, has been discussed [2]. By using this signal, gNB could indicate to the UE where COT starts. For the UE, by detecting this signal, the UE could be aware of the initial partial slot.

Proposal 2: A signal prior to NR-U DL transmission burst for power saving purpose should be introduced.

Since signal reception with high complexity impacts on UE power consumption, for the purpose of UE power saving, a key design requirement for the signal prior to the NR-DL transmission burst is detectability with low complexity. Highly reliable detection for the signal is also an important requirement because false-detection of serving cell transmissions causes performance degradation due to wrong channel measurement and increasing UE power consumption from unnecessary PDCCH monitoring.

Observation 2: To facilitate DL transmission burst detection, it is desirable that a signal for power saving purposes has low complexity and highly reliable detectability.

However, during the study item phase, the concept of a signal transmitted prior to NR-U DL transmission burst, which could potentially be used to detect transmissions from other networks for the coexistence purpose, has been also discussed [2]. It has not been concluded in RAN1, but it is under discussion in RAN plenary whether the signal should be discussed or not [1].

From the perspective of signal design complexity, a signal for power saving purposes and for coexistence purposes should be shared. On the other hand, these should be separated from the perspective of requirements, because a signal for coexistence purposes requires detectability among all RATs.

Observation 3: It should be carefully considered whether a signal for power saving purpose and for coexistence purposes are shared or separated.

In [3], we proposed that “If it would be agreed on upcoming RAN#83 that extensions of coexistence mechanism is considered for NR-U, RAN1 should consider developing single solution for coexistence with other RATs” and “One of the possible solutions is to utilize a signal which all RATs would be aware of”. If it is concluded that the signal for power saving purposes and for coexistence purposes are shared and if the signal design for power saving purposes is determined before the discussion of channel access extensions is concluded, developing a harmonized design of the signal would be difficult. In order not to spend wasted discussion time, it is better to wait for the detailed decision on the coexistence signal for the signal for the power saving purpose.

Proposal 3: If it is concluded that the signal for power saving purposes and for coexistence purposes are shared, RAN1 needs to postpone the decision on the detailed design of the signal for power saving purpose until after the discussion of channel access extensions is concluded.

COT structure indication

During the study item phase, it was been concluded in [2] that “In addition to the functionalities provided by DCI format 2_0 in Rel-15 NR, indication of the COT structure in the time domain has been identified as being beneficial.”. Like LTE-LAA, information of time-domain COT indication including direction (DL, UL or flexible) and COT length are useful for many purposes (e.g. relaxing PDCCH monitoring, improving accuracy of channel measurement, reducing Tx/Rx process complexity, skipping channel sensing, and so on) and should be delivered via GC-PDCCH. In addition, if wideband operation is allowed, frequency domain COT structure indication is also useful, which is discussed in [4]. Furthermore, the signal transmitted prior to the DL transmission burst discussed in the above section could be used for DL indication at least for initial partial slot. The UE can assume that when the signal is detected, the channel will be occupied as DL during a duration from the detection point of the signal to at least before the next occasion for GC-PDCCH.

Figure 2: Time domain COT structure indication.

Proposal 4: COT structure should be indicated via a signal transmitted prior to DL transmission burst and GC-PDCCH.

- A signal transmitted prior to DL transmission burst indicates that the channel will be occupied as DL during a duration from the detection point of the signal to at least the next occasion for GC-PDCCH.

- GC-PDCCH indicates direction (DL, UL, or flexible), COT length, and potentially information related to occupied bandwidth.

Conclusions

In this contribution, based on the above discussion we have the following proposals and observation:

Proposal 1: Dynamic switching of PDCCH monitoring periodicity should be supported in NR-U.

- For an initial slot on COT, PDCCH monitoring of non-slot-based scheduling should be applied.

- For other slots, PDCCH monitoring of slot-based scheduling should be applied.

Observation 1: Detection of COT starting point is useful for dynamic switching of PDCCH monitoring periodicity.

Proposal 2: A signal prior to NR-U DL transmission burst for power saving purpose should be introduced.

Observation 2: To facilitate DL transmission burst detection, it is desirable that a signal for power saving purposes has low complexity and highly reliable detectability.

Observation 3: It should be carefully considered whether a signal for power saving purpose and for coexistence purposes are shared or separated.

Proposal 3: If it is concluded that the signal for power saving purposes and for coexistence purposes are shared, RAN1 needs to postpone the decision on the detailed design of the signal for power saving purpose until after the discussion of channel access extensions is concluded.

Proposal 4: COT structure should be indicated via a signal transmitted prior to DL transmission burst and GC-PDCCH.

- A signal transmitted prior to DL transmission burst indicates that the channel will be occupied as DL during a duration from the detection point of the signal to at least the next occasion for GC-PDCCH.

- GC-PDCCH indicates direction (DL, UL, or flexible), COT length, and potentially information related to occupied bandwidth.

References

- RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” RAN plenary #82 meeting, December 2018.

- TR 38.889, v16.0.0 ,” Study on NR-based access to unlicensed spectrum,” December 2018.

- R1-1900363, Sony, “Channel access for NR unlicensed operations,” RAN1 Ad-Hoc Meeting 1901, January 2019.

- R1-1900367, Sony, “Wideband operations for NR Unlicensed,” RAN1 Ad-Hoc Meeting 1901, January 2019.