3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1900402

Taipei, 21st – 25th January, 2019

Source:	CATT

Title:	Update of evaluation results

Agenda Item:	7.2.5.5

Document for:	Discussion and Decision

Introduction

Study item on remote interference management for NR was approved in RAN#80 meeting [1]. The study was concluded in RAN#82 meeting and TR 38.866 was approved [2]. Some evaluation results from CATT were not correctly captured.  In this contribution we provide draft TPs to update evaluation results in TR 38.866.

Draft TP

== Unchanged part is omitted ==

- 7.3.1	Case 1: Single sequence and single RS

Table 7.3.1-1 Minimum SNR (in dB) required for one-shot detection with [90]% detection probability under [1]% false alarm rate.

| Source | Alt 1: 1OS CSI-RS, comb factor = 1 | Alt 1: 1OS CSI-RS, comb factor = 2 | Alt 1: 1OS CSI-RS, comb factor = 4 | Alt 2/Alt 3Note 1: 2OS RS, comb factor = 1 | Alt 2: 2OS RS, comb factor = 2 | Alt 2: 2OS RS, comb factor = 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Source 1 (R1-1812218) | - | - | - | -17.0 | - | - |
| Source 2 Note 2 (R1-1812439) | -10.6 | -7.8 | -4.6 | -14.5 | -11.5 | -8.5 |
| Source 3 (R1-1812498) | - | - | - | -13.6 | - | - |
| Source 4 (R1-1813990) |  | -10.56 | -10.38 | -14.6 |  |  |
| Source 5 (R1-1814358) | - | -8.6 | - | -15.0 | - | - |
| Source 6 (R1-1812882) | - | - | -12 | -14.8 | - | - |
| Source 7 (R1-1813889) | -10.6 | -10.6 | -10.6 | -14.3 | - | - |
| Source 8 (R1-1814348) | - | - | - | -14 / -14 | - | - |
| Source 9 (R1-1813429) | - | -12.6 | -12.6 | -15 | - | - |
| Source 10 Note 2 (R1-1813465) | -13.3 | -9.8 | -6.5 | -14.6 | -11.3 | -8.0 |

Note 1: Alt 2 and Alt 3 are identical in terms of performance in all cases.

Note 2: For Case-1 evaluation results of source 2 and source 10, no power boosting is considered for 1OS CSI-RS with comb factor 2 and 4, which is the main reason for the differentiation of the detection SNR. However, this does not follow the definition of reference SNR in Table 7.1.1-2.

== Unchanged part is omitted ==

- 7.3.2	Case 2-1: Single sequence and multiple RSs

Table 7.3.2-1 Minimum SNR (in dB) required for one-shot detection with [90]% detection probability under [1]% false alarm rate. For the case when [90]% detection probability cannot be attained, the detection probability at [-10] dB is provided.

| Source | Number of copies (S) | Alt 1: 1OS CSI-RS, comb factor = 1 | Alt 1: 1OS CSI-RS, comb factor = 2 | Alt 1: 1OS CSI-RS, comb factor = 4 | Alt 2/Alt 3: 2OS RS, comb factor = 1 | Alt 2: 2OS RS, comb factor = 2 | Alt 2: 2OS RS, comb factor = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 (R1-1812218) | 10 | - | - | - | -23.5 | - | - |
| Source 2 Note 3 (R1-1812439) | 10 | -15.0 | -11.9 | -8.5 | -17.2 | -13.8 | -9.6 |
|  | [20] | -16.0 | -12.6 | -8.5 | -18.0 | -14.2 | -9.4 |
|  | [50] | -16.9 | -13.6 | -7.8 | -19.5 | -16.0 | -8.9 |
| Source 3 (R1-1812498) | 10 | - | - | - | -17.1 | - | - |
|  | [20] | - | - | - | -18.3 | - | - |
|  | [50] | - | - | - | -20.1 | - | - |
|  | [100] | - | - | - | -21.6 | - | - |
| Source 4 (R1-1813990) | 10 |  | -15.12 | -15.08 | -17.01 |  |  |
| Source 5 (R1-1814358) | 10 | - | -11.5 | - | -17.4 | - | - |
|  | [20] | - | -11.3 | - | -18.0 | - | - |
|  | [50] | - | NA, Pd=66.3% | - | -18.2 | - | - |
|  | [100] | - | NA, Pd=44.2% | - | -18.8 | - | - |
| Source 6 (R1-1812882) | 10 | - | - | -14.5 | -17.0 | - | - |
|  | [20] | - | - | - | -18.0 | - | - |
|  | [50] | - | - | - | -18.0 | - | - |
|  | [100] | - | - | - | -16.5 | - | - |
| Source 7 (R1-1813889) | 10 | - | - | - | -17.2 | - | - |
|  | [20] | - | - | - | -17.6 | - | - |
|  | [50] | - | - | - | -19.0 | - | - |
|  | [100] | - | - | - | -20.3 | - | - |
| Source 8 (R1-1814348) | 10 | - | - | - | -20 | - | - |
| Source 9 (R1-1813429) | 10 | - | - | - | -17.0 | - | - |
|  | [20] | - | - | - | -17.7 | - | - |
|  | [50] | - | - | - | -18.2 | - | - |
|  | [100] | - | - | - | -19.2 | - | - |

Note 3: For Case 2-1 evaluation results of resource 2, no power boosting is considered for 1OS CSI-RS with comb factor 2 and 4, which is the main reason for the differentiation of the detection SNR. However, this does not follow the definition of reference SNR in Table 7.1.1-2.

== Unchanged part is omitted ==

### 7.3.3	Case 2-2A: Multiple sequences and single RS

Table 7.3.3-1 Minimum SNR required for one-shot detection with [90]% detection probability under [1]% false alarm probability and [1]% error detection probability. For the case when [90]% detection probability cannot be attained, the detection probability at [-10] dB is provided. Note 4

| Source | Number of sequences arriving within the window (n) | Alt 1: 1OS CSI-RS, comb factor = 1 | Alt 1: 1OS CSI-RS, comb factor = 2 | Alt 1: 1OS CSI-RS, comb factor = 4 | Alt 2/Alt 3: 2OS RS, comb factor = 1 | Alt 2: 2OS RS, comb factor = 2 | Alt 2: 2OS RS, comb factor = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 (R1-1812218) | 1 | - | - | - | -15.2 | - | - |
|  | 2 | - | - | - | -15.0 | - | - |
|  | 4 | - | - | - | -14.7 | - | - |
|  | 8 | - | - | - | -14.1 | - | - |
| Source 2Note 5 (R1-1812439) | 1 | -10.6 | -7.8 | -4.6 | -14.5 | -11.5 | -8.5 |
|  | 2 | -10.5 | -7.6 | -4.2 | -14.4 | -11.2 | NA, Pd=58% |
|  | 4 | -10.3 | -6.8 | -3.2 | -14.2 | -10.9 | NA, Pd=48% |
|  | 8 | -9.4 | -5.7 | NA, Pd=14% | -13.8 | -9.7 | NA, Pd=30% |
| Source 3 (R1-1812498) | 1 | - | - | - | -12.8 | - | - |
|  | 2 | - | - | - | 0 | - | - |
|  | 4 | - | - | - | NA, Pd=72.9% | - | - |
|  | 8 | - | - | - | - NA, Pd=69.7% | - | - |
| Source 4 (R1-1813990) | 1 |  | -7.8 | -4.6 | -14.01 |  |  |
|  | 2 |  | -7.6 | -4.18 | -13.95 |  |  |
|  | 4 |  | -6.8 | -2.92 | -13.84 |  |  |
| Source 5 (R1-1814358) | 1 | - | -7.8 | - | -14.5 | - | - |
|  | 2 | - | -7.2 | - | -14.3 | - | - |
|  | 4 | - | -5.6 | - | -14.1 | - | - |
|  | 8 | - | NA, Pd=42.0% | - | -13.5 | - | - |
| Source 6 (R1-1812882) | 1 | - | - | -4.8 | -14.2 | - | - |
|  | 2 | - | - | -4.2 | -14.1 | - | - |
|  | 4 | - | - | -2.8 | -14.0 | - | - |
|  | 8 | - | - | 3.8 | -13.6 | - | - |
| Source 7 (R1-1813889) | 1 | - | - | - | -14.3 | - | - |
|  | 2 | - | - | - | -14.2 | - | - |
|  | 4 | - | - | - | -14.0 | - | - |
|  | 8 | - | - | - | -13.3 | - | - |
| Source 8 (R1-1814348) | 1 |  |  |  | -14 |  |  |
|  | 2 | - | - | - | -13.4 | - | - |
|  | 4 | - | - | - | -12 | - | - |
|  | 8 | - | - | - | -11 | - | - |

Note 4: Without otherwise noted, the average detection probability is summarized in Table 7.3.3-1 and Table 7.3.4-1, which shows little difference with the worst case detection probability.

Note 5: Resource 2 provides the worst case detection probability in Table 7.3.3-1 and Table 7.3.4-1.

Note that for Case 2-2A evaluation results, no power boosting is considered for 1OS CSI-RS with comb factor 2 and 4, which is the main reason for the differentiation of the detection SNR. However, this does not follow the definition of reference SNR in Table 7.1.1-2.

== Unchanged part is omitted ==

### 7.3.4	Case 2-2B: Multiple sequences and multiple RSs

Table 7.3.4-1 Minimum SNR required for one-shot detection with [90]% detection probability under [1]% false alarm probability and [1]% error detection probability when the number of copies per sequence is 10. For the case when [90]% detection probability cannot be attained, the detection probability at [-10] dB is provided.

| Source | Number of sequences arriving within the window (n) | Number of copies (S) | Alt 1: 1OS CSI-RS, comb factor = 1 | Alt 1: 1OS CSI-RS, comb factor = 2 | Alt 1: 1OS CSI-RS, comb factor = 4 | Alt 2/Alt 3: 2OS RS, comb factor = 1 | Alt 2: 2OS RS, comb factor = 2 | Alt 2: 2OS RS, comb factor = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 (R1-1812218) | 1 | 10 | - | - | - | -18.8 | - | - |
|  | 2 | 10 | - | - | - | -18.1 | - | - |
|  | 4 | 10 | - | - | - | -16.6 | - | - |
|  | 8 | 10 | - | - | - | NA, Pd=87% | - | - |
| Source 2 (R1-1812439) | 1 | 10 | -15.0 | -11.9 | -8.5 | -17.2 | -13.8 | -9.6 |
|  |  | [20] | -16.0 | -12.6 | -8.5 | -18.0 | -14.17 | -9.4 |
|  |  | [50] | -16.9 | -13.6 | -7.8 | -19.5 | -16.0 | -8.9 |
|  | 2 | 10 | -14.66 | -11.2 | -6.9 | -16.8 | -12.6 | NA, Pd=64% |
|  |  | [20] | -15 | -11.5 | -5.3 | -17 | -12.3 | NA, Pd=58% |
|  |  | [50] | -15.1 | -10 | NA, Pd=42% | -18 | -11.2 | NA, Pd=45.9% |
|  | 4 | 10 | -13.5 | -8.9 | NA, Pd=28% | -15.66 | -9.6 | NA, Pd=32% |
|  |  | [20] | -13.2 | -5.5 | NA, Pd=22% | -14.5 | -9.0 | NA, Pd=24.8% |
|  |  | [50] | -8.5 | NA, Pd=31% | NA, Pd=14% | -9.0 | NA, Pd=48% | NA, Pd=14.2% |
|  | 8 | 10 | -10.6 | NA, Pd=42% | NA, Pd=12% | -11.54 | NA, Pd=49% | NA, Pd=11% |
|  |  | [20] | NA, Pd=72% | NA, Pd=20% | NA, Pd=8% | NA, Pd=74% | NA, Pd=27% | NA, Pd=6% |
|  |  | [50] | NA, Pd=34% | NA, Pd=6% | NA, Pd=3% | NA, Pd=32.7% | NA, Pd=11.6% | NA, Pd=2.8% |
| Source 3 (R1-1812498) | 1 | 10 | - | - | - | -15.5 | - | - |
|  |  | [20] | - | - | - | -15.7 | - | - |
|  |  | [50] | - | - | - | -14.0 | - | - |
|  |  | [100] | - | - | - | NA, Pd=86.5% | - | - |
|  | 2 | 10 | - | - | - | -14.2 | - | - |
|  |  | [20] | - | - | - | -13.7 | - | - |
|  |  | [50] | - | - | - | NA, Pd=69% | - | - |
|  |  | [100] | - | - | - | NA, Pd=34.5% | - | - |
|  | 4 | 10 | - | - | - | -10.3 | - | - |
|  |  | [20] | - | - | - | NA, Pd=56.9% | - | - |
|  |  | [50] | - | - | - | NA, Pd=13.7% | - | - |
|  |  | [100] | - | - | - | NA, Pd=4.4% | - | - |
|  | 8 | 10 | - | - | - | NA, Pd=39.4% | - | - |
|  |  | [20] | - | - | - | NA, Pd=18.3% | - | - |
|  |  | [50] | - | - | - | NA, Pd=4.4% | - | - |
|  |  | [100] | - | - | - | NA, Pd=1.5% | - | - |
| Source 4 (R1-1813990) | 1 | 10 |  | -11.4 | -7.91 | -16.93 |  |  |
|  | 2 | 10 |  | -11.12 | -3.32 | -16.45 |  |  |
|  | 4 | 10 |  | -8.76 | @-10dB,Pd=0.23 | -15.26 |  |  |
| Source 5 (R1-1814358) | 1 | 10 | - | -10.2 | - | -16.5 | - | - |
|  |  | [20] | - | -9.0 | - | -16.9 | - | - |
|  |  | [50] | - | NA; Pd=40.6% | - | -16.2 | - | - |
|  |  | [100] | - | NA; Pd=24.7% | - | -16.3 | - | - |
|  | 2 | 10 | - | -3.94 | - | -15.8 | - | - |
|  |  | [20] | - | NA; Pd=31.1% | - | -15.3 | - | - |
|  |  | [50] | - | NA; Pd=10.4% | - | -5.3 | - | - |
|  |  | [100] | - | NA; Pd=4.1% | - | NA, Pd=71.9% | - | - |
|  | 4 | 10 | - | NA; Pd=17.1% | - | -13.4 | - | - |
|  |  | [20] | - | NA; Pd=6.3% | - | -4.1 | - | - |
|  |  | [50] | - | NA; Pd=1.6% | - | NA, Pd=32.6% | - | - |
|  |  | [100] | - | NA; Pd=0.6% | - | NA, Pd=15.8% | - | - |
|  | 8 | 10 | - | NA; Pd=3.1% | - | NA, Pd=56.4% | - | - |
|  |  | [20] | - | NA; Pd=1.0% | - | NA, Pd=19.0% | - | - |
|  |  | [50] | - | NA; Pd=0.27% | - | NA, Pd=5.6% | - | - |
|  |  | [100] | - | NA; Pd=0.16% | - | NA, Pd=1.7% | - | - |
| Source 6 (R1-1812882) | 1 | 10 | - | - | -7.5 | -16.4 | - | - |
|  |  | [20] | - | - | - | -16.4 | - | - |
|  |  | [50] | - | - | - | -16.0 | - | - |
|  |  | [100] | - | - | - | -12.6 | - | - |
|  | 2 | 10 | - | - | -3.0 | -15.5 | - | - |
|  |  | [20] | - | - | - | -15.0 | - | - |
|  |  | [50] | - | - | - | 0.0 | - | - |
|  |  | [100] | - | - | - | NA, Pd=44.8% | - | - |
|  | 4 | 10 | - | - | NA, Pd=10.6% | -13.2 | - | - |
|  |  | [20] | - | - | - | -4.0 dB | - | - |
|  |  | [50] | - | - | - | NA, Pd=20.9% | - | - |
|  |  | [100] | - | - | - | NA, Pd=6.4% | - | - |
|  | 8 | 10 | - | - | NA, Pd=2.4% | NA, Pd=56.1% | - | - |
|  |  | [20] | - | - | - | NA, Pd=15.4% | - | - |
|  |  | [50] | - | - | - | NA, Pd=2.6% | - | - |
|  |  | [100] | - | - | - | NA, Pd=0.8% | - | - |
| Source 7 (R1-1813889) | 1 | 10 | - | - | - | -17.2 | - | - |
|  |  | [20] | - | - | - | -17.6 | - | - |
|  |  | [50] | - | - | - | -19.1 | - | - |
|  |  | [100] | - | - | - | -20.2 | - | - |
|  | 2 | 10 | - | - | - | -16.4 | - | - |
|  |  | [20] | - | - | - | -16.2 | - | - |
|  |  | [50] | - | - | - | -15.5 | - | - |
|  |  | [100] | - | - | - | -12.0 | - | - |
|  | 4 | 10 | - | - | - | -14.2 | - | - |
|  |  | [20] | - | - | - | -4.5 | - | - |
|  |  | [50] | - | - | - | NA, Pd=45% | - | - |
|  |  | [100] | - | - | - | NA, Pd=17% | - | - |
|  | 8 | 10 | - | - | - | NA, Pd=60% | - | - |
|  |  | [20] | - | - | - | NA, Pd=23% | - | - |
|  |  | [50] | - | - | - | NA, Pd=5% | - | - |
|  |  | [100] | - | - | - | NA, Pd=1% | - | - |
| Source 8 (R1-1814348) | 1 | 10 |  |  |  | -20 / -20 |  |  |
|  | 2 | 10 | - | - | - | -18 / -18 | - | - |
|  | 4 | 10 | - | - | - | -17 / -17 | - | - |
|  | 8 | 10 | - | - | - | -12 / -12 | - | - |

Note that for Case 2-2B evaluation results, no power boosting is considered for 1OS CSI-RS with comb factor 2 and 4, which is the main reason for the differentiation of the detection SNR. However, this does not follow the definition of reference SNR in Table 7.1.1-2.

== Unchanged part is omitted ==

Conclusion

In this contribution draft TPs to update evaluation results in TR 38.866 are provided. It is proposed to adopt the TPs in section 2.

References

  - RP-181420, “New SI proposal: Study on remote interference management for NR”, RAN#80

  - TR 38.866, “Study on remote interference management for NR”, v16.0.0, Dec. 2018.