3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1900447

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.4.3.1

Source:	AT&T

Title:	Enhancements of Uu to control sidelink

Document for:	Discussion/Approval

# Introduction

In RAN1#95, big progress has been made on UU controlled sidelink transmission.

Agreements:

The following NR sidelink resource allocation techniques by NR Uu for mode-1 are supported:

- Dynamic resource allocation

- Configured grant.

  - FFS whether type-1 and/or type-2

# NR UU enhancement to control NR and LTE sidelink

Dual connectivity between LTE and NR has been supported in access link, e.g. EN-DC, where LTE is considered as MCG and NR as SCG. Similar dual connectivity can be considered for NR UU to control LTE sidelink. In particular, LTE or NR sidelink can be considered as a SCG while NR uu interface is the MCG. In such way, NR UU can control either LTE or NR sidelink just like controlling an access link in dual connectivity framework.

Proposal-1: Dual connectivity framework can support NR UU interface as MCG to control NR and LTE sidelink as SCG.

With the present of local manager, sidelink communication is in hierarchical structure. In another words, vehicle UEs need to associate with local manager somehow. However, there is no need to have full association just like UE-BS in cellular network since local manager essentially is another UE. It simply helps to coordinate the resource utilization in a certain area.  In our opinion, UE can have “diet” connection to local manager, which means UE synchronizes to local manager and blindly decode control channel following the search space configured for it. Also, when UE switch to a different local manager, no need to have normal handover procedure, UE simply just send an SR to another local manager to ask for new resources.

Proposal-2: The sidelink (between UEs or between local manager and UE) is considered as a “diet” connection in dual connectivity framework.

# Conclusion

In this contribution, we proposed to use dual-connectivity framework to let NR uu interface to control both NR and LTE sidelink, more specifically, we have the following proposals:

Proposal-1: Dual connectivity framework can support NR UU interface as MCG to control NR and LTE sidelink as SCG.

Proposal-2: The sidelink (between UEs or between local manager and UE) is considered as a “diet” connection in dual connectivity framework.

# Reference

[1]  	RP-181480   	New SID: Study on NR V2X

[2]	R1-1806986	Autonomous topology management, AT&T