3GPP TSG RAN WG1 AH-1901	R1-1900573

Taipei, Taiwan, 21st – 25th January 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: subset selection for FD compression

Document for:	Discussion and Decision

# Introduction

In RAN1#95, the DFT-based frequency domain (FD) compression was agreed as the supported mechanism for Type II overhead reduction for rank 1-2 [1]. The following agreement was made about basis subset selection.

Agreement In RAN1 NR-AH 1901, select one of the following alternatives for basis subset selection scheme for each layer - Alt1A. Common selection for all the 2L beams, wherein M coefficients are reported for each beam - Wf=fk0 fk1…fkM-1 - W2 is composed of K=2LM linear combination coefficients - The value of M (applied to all 2L beams) is higher-layer configured - Alt1B. Common selection for all the 2L beams, but only a size- K0<2LM subset of coefficients are reported (not reported coefficients are treated as zero) - Wf=fk0 fk1…fkM-1 - W2 is composed of K=2LM linear combination coefficients, but (K-K0) of its coefficients are zero - The value of M (applied to all 2L beams) is higher-layer configured - For evaluation, companies should state their assumption on size-K0 subset selection (applied to all 2L beams), e.g. - The value of K0 is fixed or higher-layer configured, and the subset is dynamically selected by the UE (hence reported with CSI), or - The size-K0 subset and its size are dynamically selected by the UE (hence reported with CSI) - Alt2. Independent selection for all the 2L beams, wherein Mi coefficients are reported for the i-th beam (i=0, 1, …, 2L-1) - Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per beam) are selected - W2 is composed of K=i=02L-1Mi linear combination coefficients - The value of K (applied to all 2L beams) is higher-layer configured - For evaluation, companies should state their assumption on size-Mi subset selection (applied to the i-th beam), e.g. for i=0, 1, …, 2L-1 - The size-Mi subset and the value of Mi are dynamically selected by the UE (hence reported with CSI) - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is determined by a predefined rule in specification - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is higher-layer configured - The size-Mi subset can be chosen either from the fixed basis set or from a beam-common UE-selected intermediate subset of the fixed basis set Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes.

This contribution provides simulation results to compare these alternatives for basis subset selection and makes conclusions in support of our proposal in [2].

# Simulation results for basis subset selection

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix.

Evaluation 1: Alt1A vs. Alt2

We first provide simulation results for Alt1A (common basis subset for all SD beams) and Alt2 (independent basis subset for each SD beam). The results are provided in Figure 1 for the following parameters.

- Spatial compression: L = {3, 4}

- Frequency compression: M = {2, 4, 6, 8}

- Coefficient quantization: (Amp, phase) = (3,3) bits

As reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered. We can observe the following.

Observation 1: For a given (L, M) value, the independent basis (Alt2) shows performance gain over common basis (Alt1A) but the overhead is also higher, which implies that independent basis (Alt2) has no noticeable benefit over common basis (Alt1A) in terms of performance-overhead tradeoff.

Figure 1: Performance-overhead trade-off for Alt1A (common basis) and Alt2 (independent basis)

Evaluation 2: Alt1A vs. Alt1B

We next provide simulation results for Alt1A (common FD basis and all 2LM coefficients are reported) and Alt1B (common FD basis and subset K0 < 2LM coefficients are reported). The results are provided in Figure 2 for the following parameters.

- Spatial compression: L = 4

- Frequency compression:

  - Alt1A: M = {2, 4, 6, 8}

  - Alt1B: M = 8, and K0 = {20, 28, 36}, and the following three alternatives to select K0 coefficients

    - Alt 1B-1: Free selection

    - Alt 1B-2: Constrained selection based on ordered SD/FD beams as shown in Figure 2

      - Size-K0 subset = upper triangular (sorted) coefficients

    - Alt 1B-3: Constrained selection based on SD/FD beam power

      - Size-K0 subset = coefficients with largest beam power

We can observe the following.

Observation 2: Alt 1B with free selection of K0 coefficients (Alt1B-1) achieves the best performance-overhead tradeoff.

Figure 2: ordered SD/FD beams for Alt1B-2

Figure 3: Performance-overhead tradeoff for Alt1A and Alt1B; left: rank 1 and right: dynamic rank 1-2

# Conclusions

In this contribution, simulation results are provided for basis subset selection. The observations made are summarized as follows.

Observation:

- For a given (L, M) value, the independent basis (Alt2) shows performance gain over common basis (Alt1A) but the overhead is also higher, which implies that independent basis (Alt2) has no noticeable benefit over common basis (Alt1A) in terms of performance-overhead tradeoff.

- Alt 1B with free selection of K0 coefficients achieves the best performance-overhead tradeoff

# References

- Chairman’s notes, RAN1#95

- R1-1900566, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |