3GPP TSG RAN WG1 Ad-Hoc Meeting 1901								  R1-1900673

Taipei, China, 21st – 25th January, 2019

Agenda item:	7.2.8.5

Source:	NEC

Title:               Discussion on low PAPR RS

Document for:	Discussion and Decision

- Introduction

In RAN1#95 meeting [1], agreements on low PAPR DMRS design were achieved as

Agreement

Metrics to consider for new sequence design for the pi/2 BPSK DMRS are the gNB receiver complexity, PAPR relative to data and link level throughput/BLER performance considering frequency domain flatness and autocorrelation properties, interference considering cross correlation properties and when applicable (e.g. PUCCH), orthogonality of sequences

Agreement

- For sequences with length 30 or larger, DMRS for ππ/2 BPSK modulation for PUSCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding resulting in a DMRS Type 1 comb structure.

- For sequences with length 30 or larger, DMRS for π/2 BPSK modulation for PUCCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding.

- For sequences with allocation length 6,12,18 and 24 CGS is used for DMRS for π/2 BPSK modulation in case of PUSCH and PUCCH

Agreement

Support PUCCH DMRS enhancements for PUCCH format 3 and 4

In this contribution, we provided our views on the sequence design of low PAPR DMRS for pi/2 BPSK.

- Discussion

- Low PAPR DMRS sequence

As agreed in RAN1#95, for pi/2 BPSK DMRS sequence with length 6,12,18 and 24, CGS sequences should be applied, and the sequences should have not only low PAPR but also good auto/cross correlation properties.

Based on this, we provide the sequences for different length in following tables:

Table 1: Length-6 Pi/2 BPSK DMRS sequences

| Index u | b0,b1,…,b5 | PAPR (dB) |
| --- | --- | --- |
| 0 | 0 0 1 1 1 0 | 0.8630 |
| 1 | 1 1 1 0 1 0 | 0.7534 |
| 2 | 1 0 0 1 0 0 | 0.9821 |
| 3 | 0 0 0 0 1 0 | 1.0299 |
| 4 | 1 1 1 0 0 0 | 0.9821 |
| 5 | 1 0 1 0 0 0 | 0.7791 |
| 6 | 0 1 0 0 1 0 | 1.0299 |
| 7 | 1 0 1 1 1 0 | 0.7534 |
| 8 | 0 1 1 1 0 0 | 0.7534 |
| 9 | 0 0 1 0 1 0 | 0.7791 |
| 10 | 1 1 0 1 1 0 | 1.3019 |
| 11 | 0 0 0 1 0 0 | 1.1643 |
| 12 | 0 1 1 1 1 0 | 1.1933 |
| 13 | 1 1 0 0 1 0 | 1.3019 |
| 14 | 0 1 1 0 0 0 | 1.1239 |
| 15 | 0 1 0 0 0 0 | 1.3079 |
| 16 | 0 1 0 1 1 0 | 1.3874 |
| 17 | 1 0 0 0 0 0 | 1.1917 |
| 18 | 1 0 0 1 1 0 | 1.3249 |
| 19 | 1 1 0 1 0 0 | 1.3079 |
| 20 | 0 1 1 0 1 0 | 1.1566 |
| 21 | 0 1 0 1 0 0 | 1.3874 |
| 22 | 0 0 0 1 1 0 | 1.1643 |
| 23 | 0 0 1 0 0 0 | 1.1566 |
| 24 | 1 1 0 0 0 0 | 1.1917 |
| 25 | 1 0 0 0 1 0 | 1.1240 |
| 26 | 1 0 1 1 0 0 | 0.7534 |
| 27 | 1 1 1 1 1 0 | 1.3249 |
| 28 | 0 0 1 1 0 0 | 1.3184 |
| 29 | 1 1 1 1 0 0 | 1.2500 |

Table 2: Length-12 Pi/2 BPSK DMRS sequences

| Index u | b0,b1,…,b11 | PAPR (dB) |
| --- | --- | --- |
| 0 | 0 1 1 1 1 1 0 0 0 0 1 0 | 0.7531 |
| 1 | 1 1 1 0 0 1 0 1 0 0 1 0 | 0.7531 |
| 2 | 1 1 0 1 0 1 1 0 0 0 1 0 | 0.7532 |
| 3 | 1 0 1 1 0 0 0 1 0 1 1 0 | 0.7532 |
| 4 | 0 0 0 1 0 0 0 0 1 0 1 1 | 1.4685 |
| 5 | 0 1 1 1 0 1 1 1 0 1 1 1 | 1.3169 |
| 6 | 1 1 0 1 0 1 1 1 1 0 0 0 | 0.7794 |
| 7 | 0 0 1 0 0 1 0 0 1 0 1 0 | 1.1549 |
| 8 | 1 0 0 1 0 0 1 0 0 1 0 0 | 0.7838 |
| 9 | 1 0 1 1 0 1 0 0 1 1 0 1 | 1.1568 |
| 10 | 0 1 1 1 0 0 1 0 1 0 0 0 | 0.7793 |
| 11 | 0 0 1 1 1 1 1 1 1 1 0 0 | 1.0159 |
| 12 | 0 0 1 0 1 1 0 1 0 1 1 0 | 0.7531 |
| 13 | 1 1 1 1 1 0 0 0 1 1 1 0 | 1.1917 |
| 14 | 0 0 0 0 0 1 1 0 0 0 1 1 | 1.2060 |
| 15 | 0 0 0 0 1 1 0 1 1 0 1 0 | 0.7794 |
| 16 | 1 1 0 1 0 0 0 1 1 0 1 0 | 0.7532 |
| 17 | 1 0 1 0 0 1 0 1 1 1 0 0 | 0.7532 |
| 18 | 1 0 1 1 0 1 1 0 1 0 0 0 | 1.3230 |
| 19 | 1 1 0 1 1 0 0 0 0 0 0 0 | 1.0308 |
| 20 | 0 0 1 0 0 1 1 0 1 1 0 0 | 1.1047 |
| 21 | 1 1 1 0 0 0 0 0 1 0 0 0 | 1.1917 |
| 22 | 0 1 1 1 0 0 1 1 0 1 0 0 | 1.3426 |
| 23 | 1 0 1 0 1 1 0 0 1 0 0 1 | 1.2060 |
| 24 | 0 0 1 1 1 0 0 0 1 0 0 0 | 1.1547 |
| 25 | 0 1 1 0 1 0 1 0 0 0 1 1 | 0.9819 |
| 26 | 1 0 1 1 0 0 1 0 0 1 1 1 | 0.8100 |
| 27 | 0 1 1 1 0 0 0 0 0 1 0 0 | 1.1549 |
| 28 | 0 0 0 0 1 1 1 0 1 1 1 0 | 1.1583 |
| 29 | 0 1 0 0 0 1 1 0 0 1 0 0 | 1.3278 |

Table 3: Length-18 Pi/2 BPSK DMRS sequences

| Index u | b0,b1,…,b11 | PAPR (dB) |
| --- | --- | --- |
| 0 | 1 0 0 0 0 1 1 1 0 1 1 0 1 0 1 0 0 0 | 0.9947 |
| 1 | 1 1 1 0 0 1 0 0 1 1 1 0 0 1 1 1 0 0 | 0.9344 |
| 2 | 0 0 0 1 1 1 0 1 1 0 0 0 0 0 1 1 0 0 | 1.0099 |
| 3 | 0 1 0 1 1 0 0 1 0 0 1 1 0 0 1 0 1 0 | 0.8499 |
| 4 | 1 1 0 1 1 0 1 1 0 0 0 0 1 1 0 1 1 0 | 0.8453 |
| 5 | 1 0 0 0 0 0 0 1 1 1 0 1 0 0 1 1 0 0 | 0.9406 |
| 6 | 0 1 0 0 1 0 0 1 1 1 0 0 0 0 0 0 0 0 | 1.3496 |
| 7 | 1 0 0 1 0 0 1 0 1 1 0 0 0 1 1 0 1 0 | 1.0459 |
| 8 | 0 1 0 0 1 0 1 1 0 1 0 1 0 1 1 1 1 0 | 0.8727 |
| 9 | 1 0 0 1 1 1 1 1 0 0 0 0 0 1 1 0 0 0 | 0.8446 |
| 10 | 1 1 0 0 1 1 1 0 0 1 1 1 0 0 1 0 0 0 | 0.9234 |
| 11 | 1 0 1 1 1 0 0 0 1 1 1 0 1 1 0 1 0 1 | 1.0584 |
| 12 | 0 1 1 1 1 1 0 0 1 0 0 1 0 0 1 0 1 0 | 0.9480 |
| 13 | 0 0 1 0 1 1 0 0 1 1 0 1 0 0 0 1 1 0 | 1.0731 |
| 14 | 1 1 1 1 0 1 0 1 1 1 0 0 0 0 0 0 1 0 | 0.9940 |
| 15 | 1 1 0 0 1 0 1 1 1 0 1 0 0 1 1 1 0 1 | 1.1083 |
| 16 | 0 0 1 0 1 1 0 0 0 0 1 1 0 1 1 0 1 0 | 0.9545 |
| 17 | 0 1 1 1 0 1 0 1 1 0 0 1 1 1 0 0 0 0 | 1.1226 |
| 18 | 0 1 1 0 1 0 0 0 1 0 0 0 1 1 1 1 0 0 | 0.9477 |
| 19 | 0 1 1 1 0 0 0 1 0 0 0 1 1 0 1 1 1 0 | 0.9502 |
| 20 | 1 1 0 0 1 1 1 1 1 0 0 1 0 0 0 1 1 0 | 0.9459 |
| 21 | 1 0 0 1 1 1 0 0 0 0 1 1 1 0 1 0 0 0 | 1.1820 |
| 22 | 1 0 1 1 0 0 1 1 1 0 0 1 0 0 0 1 1 0 | 0.9459 |
| 23 | 0 0 0 0 1 1 1 1 0 1 1 0 1 1 1 0 1 0 | 1.1743 |
| 24 | 0 0 1 0 0 1 1 1 0 0 0 0 1 1 0 1 1 0 | 0.9308 |
| 25 | 1 1 0 0 1 0 0 0 0 0 1 1 0 1 0 1 0 0 | 0.8747 |
| 26 | 1 1 0 1 1 0 1 0 0 1 0 0 0 0 0 0 0 0 | 1.1454 |
| 27 | 0 0 0 0 0 0 0 1 1 0 0 0 1 0 1 1 1 0 | 1.3100 |
| 28 | 1 1 0 1 1 1 0 0 1 0 1 0 0 0 1 1 1 0 | 0.9750 |
| 29 | 1 1 1 0 0 0 1 1 0 1 1 0 0 0 1 1 1 0 | 0.9485 |

Table 4: Length-24 Pi/2 BPSK DMRS sequences

| Index u | b0,b1,…,b11 | PAPR (dB) |
| --- | --- | --- |
| 0 | 0 1 0 1 1 0 1 0 1 0 1 0 1 1 0 1 1 0 0 1 0 0 1 1 | 0.9885 |
| 1 | 0 1 0 0 0 1 1 0 1 0 1 0 1 1 1 0 1 0 0 1 0 0 1 1 | 0.9121 |
| 2 | 0 1 0 0 1 0 0 1 1 1 1 1 1 1 1 1 1 0 0 1 0 0 1 1 | 0.9939 |
| 3 | 0 1 1 0 1 0 1 1 0 1 1 0 0 1 0 1 0 1 0 1 0 0 1 1 | 1.0320 |
| 4 | 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 0 1 0 0 1 0 0 1 1 | 1.0320 |
| 5 | 0 0 0 0 1 0 1 0 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 1 | 1.1256 |
| 6 | 1 1 0 0 1 0 1 0 0 0 0 1 1 0 0 0 1 0 0 0 1 1 1 1 | 1.3127 |
| 7 | 0 1 0 0 1 1 1 1 1 1 0 0 0 1 0 0 0 1 1 0 0 0 0 1 | 1.3129 |
| 8 | 0 1 1 0 0 1 0 1 1 1 0 0 0 0 0 0 1 1 0 1 0 1 1 1 | 1.3016 |
| 9 | 0 1 1 1 0 0 0 1 0 1 0 0 1 0 1 1 1 0 0 1 0 0 0 1 | 1.0115 |
| 10 | 1 1 0 0 1 0 0 0 1 0 1 1 1 0 1 0 0 1 0 0 0 0 0 1 | 1.1880 |
| 11 | 1 1 1 0 1 1 0 0 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 | 1.2167 |
| 12 | 0 1 0 0 0 1 1 0 1 1 0 0 1 0 1 1 0 0 0 1 1 0 1 0 | 0.8144 |
| 13 | 1 0 0 1 0 1 0 0 1 1 0 0 0 0 1 1 1 1 1 1 1 0 0 1 | 0.8066 |
| 14 | 1 0 0 1 0 1 0 0 1 1 0 1 1 0 0 1 1 0 0 0 0 0 1 1 | 0.8260 |
| 15 | 0 0 0 1 1 1 1 0 0 1 0 1 0 0 1 1 1 0 1 1 1 0 0 1 | 0.8013 |
| 16 | 1 1 0 1 0 1 1 1 0 0 1 1 1 0 0 0 0 0 0 1 1 0 1 0 | 0.8087 |
| 17 | 1 0 1 1 0 1 1 0 0 0 1 0 1 1 0 1 0 0 1 0 0 0 0 1 | 0.8162 |
| 18 | 1 1 1 1 1 0 0 1 0 0 0 1 1 1 1 0 1 1 1 1 1 1 0 0 | 1.3147 |
| 19 | 1 1 1 1 0 1 0 0 1 1 0 0 0 1 1 0 0 0 0 0 0 0 0 1 | 0.9148 |
| 20 | 1 0 1 0 0 0 1 1 1 0 1 1 0 1 0 1 1 0 1 1 1 0 0 0 | 1.0202 |
| 21 | 1 0 1 0 1 1 1 0 0 0 1 0 0 1 0 1 0 0 1 0 0 0 1 1 | 0.8237 |
| 22 | 1 1 1 1 0 0 1 0 0 0 1 1 1 1 1 0 0 1 1 1 0 1 0 1 | 1.2698 |
| 23 | 1 0 1 1 0 1 1 0 0 0 1 0 0 0 0 1 0 0 1 0 1 1 1 0 | 0.8912 |
| 24 | 0 0 0 0 0 0 0 1 1 0 0 0 1 1 1 1 1 1 1 0 0 0 1 1 | 0.9810 |
| 25 | 1 1 1 0 0 0 1 1 0 0 0 0 0 0 0 1 1 0 0 0 1 1 1 1 | 0.9801 |
| 26 | 0 0 0 0 0 0 1 1 1 0 0 1 1 1 1 1 1 1 0 0 1 1 1 0 | 0.9801 |
| 27 | 1 1 0 0 1 1 1 0 0 0 0 0 0 0 1 1 1 0 0 1 1 1 1 1 | 0.9810 |
| 28 | 0 0 0 0 1 0 0 0 0 1 0 0 1 1 1 1 0 1 1 1 1 0 0 1 | 1.2392 |
| 29 | 0 1 1 1 1 0 0 1 0 0 0 0 1 0 0 0 0 1 0 0 1 1 1 1 | 1.2400 |

As lack of binary sequences for length-6, in [2], 8PSK sequence before DFT spread is proposed. We also think it’s a good way to maintain good properties of DMRS sequences. Following Table 5 shows the sequence based on 8PSK.

Table 5: Length-6 8PSK DMRS sequences

| Index u | b0,b1,…,b5 | PAPR (dB) |
| --- | --- | --- |
| 0 | -7 -1 -3 -1 7 3 | 2.0215 |
| 1 | -5 -1 -5 5 1 5 | 1.5395 |
| 2 | -5 -1 3 7 3 7 | 1.1766 |
| 3 | 1 -5 -1 -5 -1 3 | 1.4624 |
| 4 | -7 -7 -3 1 -3 7 | 1.9680 |
| 5 | -7 3 -1 3 -5 -3 | 2.0897 |
| 6 | -7 5 -1 1 -3 1 | 1.8772 |
| 7 | 1 5 -1 3 -5 -3 | 1.5485 |
| 8 | -7 5 1 5 1 -3 | 0.9858 |
| 9 | -5 -1 -7 7 -5 7 | 1.6342 |
| 10 | -3 1 -3 -7 5 1 | 0.9858 |
| 11 | -1 3 7 -7 1 5 | 1.5485 |
| 12 | -7 5 7 -1 -3 3 | 2.0315 |
| 13 | -7 -5 -3 -7 -5 -3 | 1.4695 |
| 14 | 3 -5 -1 -7 3 1 | 1.9298 |
| 15 | -7 7 7 -5 3 -1 | 2.1946 |
| 16 | -7 -1 -3 5 7 3 | 2.0315 |
| 17 | -3 -7 5 1 -3 1 | 0.9858 |
| 18 | -7 1 -3 1 7 5 | 1.9572 |
| 19 | -7 3 -3 -5 -3 3 | 1.9919 |
| 20 | 5 -1 -3 1 -3 1 | 1.6342 |
| 21 | 3 7 3 7 -5 5 | 1.6342 |
| 22 | -7 -1 3 7 3 -1 | 2.0434 |
| 23 | 3 7 -5 7 -5 5 | 1.4624 |
| 24 | -7 3 3 7 -5 7 | 2.0481 |
| 25 | -7 5 -3 1 1 -1 | 2.1947 |
| 26 | -7 -3 3 -1 -7 -5 | 1.9943 |
| 27 | 1 -5 -1 3 -1 3 | 1.6342 |
| 28 | 3 7 5 -1 3 7 | 1.6342 |
| 29 | -5 7 -5 -7 3 7 | 1.6342 |

Proposal 1: Support above tables for low PAPR DMRS sequence with length 6, 12, 18, 24.

For the length of 6, 12, 18, 24, the DMRS sequence before DFT spread is

rn=run,  n=0, 1,…MscPUSCH2-1,

where run=ejπ/2(n mod 2)2[1-2bn+j1-2bn].

And for the length of 6, 12, 18, 24, only group hopping is supported.

And for DMRS sequence length >=30, Gold-sequence was agreed in RAN1#95. In this case, the sequence generator should be initialized with a seed.

The DMRS sequence before DFT spread is rn=ejπ/2(n mod 2)2[1-2cn+j1-2cn]

Similar scheme as CP-OFDM can be reused, in addition, considering there is no scrambling ID for DFT-s-OFDM, and the value of higher layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE is (0,1, … 1007), then the coefficient for the initial seed can be modified as:

cinit=211Nsymbslotns,fμ+l+1nIDRS+1+nIDRSmod 231

where nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and the PUSCH is not a msg3 PUSCH, and nIDRS=NIDcell otherwise

And for the DMRS with length >=30, there is no group hopping and sequence hopping supported.

Proposal 2: For DMRS with length 6, 12, 18, 24, there is no sequence hopping supported. And for DMRS with length >=30, sequence generator is initialized with cinit=211Nsymbslotns,fμ+l+1nIDRS+1+nIDRSmod 231，and there is no sequence and group hopping.

- Orthogonal DMRS port

As the new low PAPR DMRS sequences are introduced in time domain, existing orthogonal DMRS ports multiplexed based on FD-OCC for PUSCH and cyclic shift for PUCCH format 4 can not work. In order to maintain same capability of multiplexing, new schemes for orthogonal DMRS ports should be introduced.

Following pre-DFT OCC which is similar as PTRS for DFT-s-OFDM can be reused.

The DMRS sequence for PUSCH before DFT spread is rpn=wpn*rn, where

wpn=1, when p=0,2,4,6-1n, when p=1,3,5,7

And the DMRS sequence for PUCCH format 4 is

rlm=wpm∙rum,  m=0, 1,…MscPUCCH-1,

where rum=ejπ/2(m mod 2)2[1-2bm+j1-2bm].

And the orthogonal sequences wpm is shown as

Table 6: Orthogonal sequences wpm for PUCCH format 4

|  |  | wp0, wp1, … wp11 |
| --- | --- | --- |
| p=0 | p=0 | [+1, +1, +1, +1, +1, +1, +1, +1, +1, +1, +1, +1] |
| p=1 | p=1 | [+1, -1, +1, -1, +1, -1, +1, -1, +1, -1, +1, -1] |
| - | p=2 | [+1, +1, -1, -1, +1, +1, -1, -1, +1, +1, -1, -1] |
| - | p=3 | [+1, -1, -1, +1, +1, -1, -1,+1, +1, -1, -1, +1] |

Proposal 3: For PUSCH and PUCCH format 4, pre-DFT orthogonal sequences are introduced to maintain same number of orthogonal DMRS ports, and above orthogonal sequences should be supported.

- Conclusion

In this contribution, we provided our views on the design of low PAPR RS, and we proposed that:

Proposal 1: Support above tables for low PAPR DMRS sequence with length 6, 12, 18, 24.

Proposal 2: For DMRS with length 6, 12, 18, 24, there is no sequence hopping supported. And for DMRS with length >=30, sequence generator is initialized with cinit=211Nsymbslotns,fμ+l+1nIDRS+1+nIDRSmod 231，and there is no sequence and group hopping.

Proposal 3: For PUSCH and PUCCH format 4, pre-DFT orthogonal sequences are introduced to maintain same number of orthogonal DMRS ports, and above orthogonal sequences should be supported.

- References

- Chairman’s note, 3GPP TSG RAN1 Meeting #95, Spokane, USA, Nov 12 – 16, 2018

- R1-1813445, Lower PAPR reference signals, Qualcomm, 3GPP TSG RAN1 Meeting #95, Spokane, USA, Nov 12 – 16, 2018