3GPP TSG-RAN WG1 Meeting Ad-hoc 1901	R1- 1900752

# Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.9.1

Source:	Apple Inc.

Title:	Evaluation of UE Power Saving Techniques

Document for:	Discussion, Decision

# Introduction

Throughout RAN1 #94b and #95 meeting, UE power consumption model for UE Power Saving SI[1] was agreed capturing most of aspect of model[2].

- Power states and transition

- Power/energy consumption numbers with a reference configuration

- Power scaling factor for scaling model in bandwidth, # of carriers, # of antennas, tx power

- Traffic models

In this contribution, we provide evaluation results of various UE power saving techniques discussed in our companion papers [4][5] where motivations and detailed proposal/procedures are discussed. The evaluation is based on the agreed evaluation methodology and assumptions [6].

Additionally, for simple evaluation of proposals, we discuss interpolation method for determining new power state and  statistical grant arrival model which captures statistical traffic arrival of FTP3 model in [6].

# Evaluation Results of UE Power Saving Techniques

## UE-preferred Bandwidth Part Configuration and Change Request

The detailed proposals and procedures for UE-preferred BWP configuration and change request can be found in the section 2.1 of our companion paper [4]. In this section, we evaluate UE power consumption in various scenarios with different traffic load and different bandwidth. We show that there exist an optimal BWP size minimizing UE power consumption to serve given traffic load.

### Assumptions

A traffic model of FTP3 is considered with an average inter-arrival time of 200ms. File sizes of 0.1, 0.5, 2, and 4 MB are assumed. For UL tx power of 0dBm case, each slot contains 2 PDCCH symbols, 10 PDSCH symbols, and 1 PUCCH symbol, i.e., self-contained slot is considered. For UL tx power of 23dBm case, 2 PDCCH symbols, 12 PDSCH symbols (1 DMRS symbols included) for DL slot and 14 symbol long PUCCH for ACK is considered. FR1 power model from [7] is used. DL BWP and UL BWP are assumed to be the same.

### Performance Evaluation

Case 1 : Continuous PDCCH Monitoring without Sleep

In this case we consider a scenario with continuous PDCCH monitoring with random packet arrival with a given traffic load. Depending on the arrived packet size and bandwidth, network may require one or more number of PDSCH transmissions and each PDSCH transmission requires ACK transmission in PUCCH. Note that no sleep operation is considered in this case. Figure 1 shows our evaluation results. We make following observations.

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption. A larger bandwidth consumes more RF power, which increases power consumption. A smaller bandwidth leads to longer DL PDSCH reception, which also increases power consumption. For example, Figure 1 (left) shows that 20MHz BWP is optimal with 2 and 4MB packet sizes.

- The optimal BWP size depends on traffic load. When traffic load is low (0.1MB) in Figure 1 (left), narrow bandwidth tends to give lower power consumption since narrow bandwidth is already wide enough to serve the traffic with a single PDSCH transmission for each packet arrival. In this case, further increasing BWP size is unnecessary because it increases bandwidth to more than enough size and accordingly increases unnecessary power consumption of both RF and BB. While in high traffic load case (4MB), using narrow BWP requires multiple times PDSCH transmissions to serve a single packet, which increasing power consumption. Using too large BWP could also increases power consumption since bandwidth is larger than enough. Further increasing BWP does not reduce UE processing in time (slot) but increases power consumption in RF with increased sampling rate, ADC, etc. So, for 4MB case, BWP of 20MHz minimizes UE power consumption.

- The optimal BWP is dependent on UL power level. With higher UL power levels, the optimal BWP tends to increase because it reduces not only number of PDSCH receptions but also number of ACK transmissions in UL. For example, with 4MB packet size, 20MHz and 80MHz BWP are optimal size at 0dBm and 23dBm UL tx power, respectively.

Figure 1 UE power consumption with tx power 0dBm (left) and 23dBm (right). Continuous PDCCH monitoring is assumed.

Observation 1

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption.

- The optimal BWP size depends on traffic load.

- The optimal BWP is dependent on UL transmission power level.

Case 2 : Grant arrival with Sleep Operation

In this case, we consider random traffic arrival with sleep operation between DL grant arrivals. We assume that network can send sleep indication to UE when there is no traffic arrival until next packet arrival time. When sleep operation is considered, the optimal BWP size for a given traffic size could be different from that in Case 1. Figure 2 show that the optimal BWP size is 100MHz for most packet sizes. It turns out that a larger BWP allows UE to complete DL PDSCH reception as soon as possible and then switch to deep sleep state to save power. This is true for most cases. In this case, increasing bandwidth

Figure 2 UE power consumption with tx power 0dBm (left) and 23dBm (right). Sleep operation assumed between two consecutive packet arrivals.

Figure 3 shows power saving gain of optimal BWP with sleep operation w.r.t smallest BWP of 10MHz for Case 2.

Figure 3 UE power saving gain compared to the case of bandwidth of 10MHz with tx power 0dBm (left) and 23dBm (right). Sleep operation is assumed between two consecutive packet arrivals.

Observation 2

- A larger BWP allows UE to complete DL PDSCH reception quickly and then to switch to sleep mode to save power.

- The optimal BWP size depends on the existence of sleep operation.

- With sleep operation, power saving gain of optimal BWP could be as large as 80%.

## UE-preferred Timeline Adaptation for Power Saving

The detailed motivation, proposals and procedures can be found in the section 2.2 of our companion paper [4]. In this section, we evaluate UE power consumption for different choices of K0(or K2), K1 values for a given traffic arrival scenario. To evaluate the performance gain, we consider a case where UE monitors PDCCH every D slots and UE receives a grant every D slots. This is static model where traffic arrives regularly. The scheduling of PDSCH and ACK is up to network decision taking into account other UEs resource allocation. In real scenario, the period D could also understood as random grant inter-arrival time. But, in this analysis, we assumed the fixed value for simplicity. In this analysis, we evaluate the average power consumption during D slots for various K0 and K1 values.

### Assumptions

We make following assumptions for the evaluation.

- Packet arrives once every D=100 slots (50ms in SCS=30kHz)

- Network schedules a packet (one PDSCH) every D slots.

- The packet could be served with a single PDSCH transmission with a given bandwidth.

- The number of OFDM symbols for PDCCH/PDSCH/PUCCH(ACK) is 2/12/14 respectively.

- If K1>0, UE receives PDSCH only for 12 symbols without PDCCH decoding in the slot where PDSCH is transmitted.

- For PDSCH-only slot, we assume that the power consumption of power state MS(2)PDSCH(12) is equivalent to that of PDCCH(2)PDSCH(12) which is 300, where the number denote the length of OFDM symbols for PDCCH and PDSCH respectively.

- Network puts the UE into sleep state by sleep indication for the slots where UE does not monitor PDCCH, receive PDSCH, or transmit ACK, where the sleep state is chosen by UE such that UE consume lowest power consumption during the sleep period.

### Performance Evaluation

We evaluate the average power consumption for duration D for different combination of K0 and K1 values. Figure 4 shows the typical timeline for PDCCH/PDSCH/ACK transmission and sleep operation between active slots where transmission/reception happens. For D slots, one PDCCH, one PDSCH and one PUCCH(ACK) is received. If there is any time duration for potential sleep for K0>0, K1>0, then UE enters one of the three sleep states which gives the lowest power consumption.

Figure 4 A typical time line for PDCCH/PDSCH/ACK transmission (K0=3, K1=3)

Figure 5 shows average power consumption for five different cases (K0,K1)=(0,1), (1,1),(4,4), (8,8), and (16,8). The case (0,1) has 41% power saving gain compared to (16.8) case. The general trend is that the shorter values for K0/K1, the higher power saving gain. This is due to the fact that UE can enter deeper sleep mode when sleep time is longer. Considering that there is significant inter-grant arrival duration between grants as shown in [5], it is expected that controlling processing time line of tx/rx activities such that they happen close in time can provide a significant potential for UE power saving.

Figure 5 Average power consumption for PDCCH#=2, PDSCH#=12, PUCCH#=14

Observation 3

- Using smaller values of K0 and K1 together with sleep operation could provide power saving gain of ~ 41% for a given reference configuration.

Observation 4

- Scheduling Tx and Rx activities close-in-time could greatly increase power saving by removing unnecessary transitions and ensuring longer sleep duration with lower power consumption.

## UE-preferred Time Domain Resource Allocation Table Configuration

The motivation, proposal, procedures of UE-preferred time domain resource allocation table (TDRA) configuration can be found in our companion paper [4]. In this section, we provide the performance evaluation results only.

### Assumptions

We make following assumptions for evaluation.

- PDCCH occupies the first 2 symbols of the slot.

- RRC configures the TDRA table with multiple entries, which may cover a wide range of TDRA patterns including both same-slot and cross-slot scheduling patterns. Examples of such table can be 38.214 Table 5.1.2.1.1.-2, Table 5.1.2.1.1.-4, and Table 5.1.2.1.1.-5.

- To project power consumption of the new power model, we use the interpolation method described in section 3.1.

- We assume Y=90, i.e., power consumption of slot configuration PDCCH(2)MS(10)PDSCH(2).

- We assume the power penalty of unnecessary PDSCH buffering is 30 based on difference of PDCCH-only with same-slot and PDCCH-only with cross-slot.

### Performance Evaluation

In this section, we provide evaluation results for both cross-slot and same slot scheduling cases.

Case-1: baseline PDCCH-only with same slot scheduling (with PDSCH buffering). For the baseline case, there is no chance for UE power saving. The power consumption for UE with and without grant, as agreed in the last meeting, is projected to be 300 and 100, respectively.

Power saving with cross-slot scheduling : Case-2

Case-2: cross-slot scheduling with LTE-like PDCCH/PDSCH allocation.

Figure 6 (a) plots a cross-slot scheduling pattern, which was also discussed in the last meeting. With cross-slot scheduling, if UE knows that a given TDRA could be potentially scheduled by DCI, UE can shut-off RF after 2 symbols in case there is no grant from previous slot. The power consumption for UE, as agreed in last meeting, can be reduced by 30% from 100 to 70 if there is no grant. Figure 6 (b) shows the power consumption of Case-1 (blue) and Case 2 (red) as a function of grant arrival probability. When grant arrival probability is low (close to 0), the difference of 30 (=100-70) comes mostly from the difference of PDCCH-only with same slot scheduling and PDCCH-only with cross slot scheduling. We can see the gain of such TDRA diminishes when the grant arrival probability becomes higher. When grant arrival probability is high (close to 1), every slot has PDCCH and PDSCH. Thus there is no difference in terms of power consumption for both cross and same slot scheduling.

(a) TDRA pattern                          (b) Power consumption

Figure 6 Case-2: cross-slot scheduling with LTE-like time-domain resource allocation.

Power saving with same-slot scheduling : Case-3 and Case 4

Case-3: same-slot scheduling with scattered PDCCH/PDSCH allocation. Figure 7 (a) plots a new power model, which is same-slot scheduling with scattered PDCCH/PDSCH. If UE knows that given TDRAs are to be potentially scheduled by DCI, UE can blindly shut-off RF after 2 symbols. Depending on outcome of PDCCH (whether grant is present or not), UE can decide whether to turn back RF on to receive PDSCH or not. In such case, whether grant is present or not, UE can achieve power saving. We can show that the UE achieves a constant of 30 power saving across the board which is plotted in Figure 8(a) as well.

Case-4: same-slot scheduling with overlapped PDCCH/PDSCH allocation. Figure 7 (b) plots same-slot scheduling with overlapped PDCCH/PDSCH allocation. Similar to Case 3, if UE knowns a given TDRA is to be potentially scheduled by DCI, UE can achieve constant power saving no matter there is grant or not.

(a) case-3                                           (b) case-4

Figure 7 same-slot scheduling power saving cases.

In both Figure 8(a) and (b), increasing overhead m makes overall power consumption increases due to increased overhead.

(a) case-3                                    (b) case-4

Figure 8 power consumption comparison b/w knowing TDRA vs. unknown TDRA before DCI decoding

Figure 9(a) and (b) show the power saving gains of different cases for overhead m=2 and 3. Case 3 and Case 4 provide significant power saving across the board up to 30% when load is low, and up to 15% when load is very high.

Figure 9 Power saving comparison between different cases.

Based on above discussion, we make following observation.

Observation 5

- UE can avoid unnecessary PDSCH buffering if and only if PDSCH is transmitted after PDCCH decoding finishes.

- This applies to cross-slot and same-slot scheduling.

Observation 6

- Power saving gain of controlling TDRA (including K0 and SLIV ) approach is as high as 30% and gradually decreases to 0% for cross slot scheduling and 15% for same slot scheduling case.

## Fast SCell Activation/Deactivation

The motivation, proposal and procedure of fast SCell activation/deactivation mechanism can be found in the section 2.4 of our companion paper [4].

### Performance Evaluation

In this section, we show the experiment results in real networks under given test scenarios.

#### Power Efficiency of UE-initiated Fast SCell Activation/Deactivation

The first scenario we have consider is file transfer. Since, in this scenario, the activation / deactivation of SCell kicks in only in the beginning and end of file transfer respectively, the file size has impact on overall power consumption. We have following setup for this experiment.

- Consider two file sizes of 1MBs (most typical download size) and 25MBs (average size).

- Total three carriers (1 PCell and 2 SCells) are activated for downloading a file.

- The power efficiency is measured for the following two schemes.

  - S1) Network-initiated SCell activation/deactivation mechanism : SCells are activated/deactivated based on indication from network

  - S2) UE-initiated Fast SCell activation/deactivation mechanism : UE sends a message to network to request activation/deactivation of SCells.

- Power saving gain is used as performance measure, which is defined as follows.

  - Power saving gain = (the total energy consumed by S1 - the total energy consumed by S2 ) / the total energy consumed by S1

- The throughput status of each carrier is denoted as either G(Good), M(Medium), B(Bad) depending on the achievable throughput (instead of channel condition or SNR). This means that if a carrier supports many UEs simultaneously, then it’s throughput status could be B(Bad) even with a very high SNR.

| Condition | Throughput (Mbps) |
| --- | --- |
| Good (G) | 25 |
| Medium (M) | 15 |
| Bad (B) | 5 |

Figure 10 shows the power efficiency of UE-initiated Fast SCell activation/deactivation mechanism. For small file size (1MB), we see a significant power saving gain up to 43%. The gain is essentially coming in from cutting down the PDCCH monitoring time in a multi-carrier scenario. For 25MB file size, the gain is limited to 3% since it takes much longer time to download due to the PDCCH monitoring duration being insignificant. The gain gradually reduces as the total aggregated throughput decreases. This is because the reduced aggregated throughput increases the total file transfer time.

Figure 10 Power saving gain of UE-initiated fast SCell activation/deactivation mechanism compared to network-initiated mechanism. G/M/B denotes Good/Medium/Bad throughput for each component carrier.

Observation 8

- UE-initiated SCell activation/deactivation scheme always provide power saving gain compared to default network-initiated CA activation/deactivation mechanism.

- The power saving gain for most typical size file (1MB) is around 43%.

#### Power Efficiency of UE-initiated Approach vs Single Component Carrier (PCell only)

In the second scenario, we consider the same file transfer with following setup.

- Consider two file sizes of 1MBs (most typical download size) and 25MBs (average size).

- Following two schemes are considered.

  - S3) PCell only : Only PCell is used for file transfer.

  - S2) UE-initiated Fast SCell activation/deactivation mechanism

- For S2, total three carriers (1 PCell and 2 SCells) are activated for downloading a file.

- Same metric is used as in previous section.

In this scenario, it has been observed that, especially for cases with Bad component carriers activated, the power saving gain is negative as shown in Figure 11. Note that in those cases with negative gains (GMB, GBB, MBB), the additional component carriers (2nd and 3rd) have Medium or Bad throughput (except the BBB case). The throughput contribution of carriers with Bad/Medium throughput is low compared to amount of additional energy consumed. The power cost for activating additional carrier (×2.5 for 3CA) does justify the marginal throughput improvement from UE power saving perspective. Thus, the power efficiency is negative. In such case, it is better to use Good PCell only.

In our field test, we often see cases like GMB, GBB, or MBB. We guess such configuration happens since some networks do not consider the achievable throughput of additional component carrier. The UE-initiated activation/deactivation request is very effective measure to handle such case. If there is no UE-initiated activation/deactivation mechanism, then UE’s power consumption would increase significantly without throughput increase, which is the situation that UE should avoid.

In the last case (BBB), primary carrier is also in Bad status. Thus, by activating additional carriers, throughput is tripled anyway. This is exactly the same situation as GGG, thus we see high power saving gain in the case.

Figure 11 UE-initiated fast SCell activation/deactivation power saving gain w.r.t PCell only case. G/M/B denotes Good/Medium/Bad throughput.

Observation 7

- In case network activates additional carriers which supports low throughput, UE-initiated SCell deactivation mechanism could be effectively used to request the deactivation of the carriers with low power efficiency (bit/Joules).

## MIMO Layer Restriction

### Performance evaluation

In this section, we do simple back-of-the-envelope type calculation based on power model agreed.

#### Assumptions

We make following assumptions.

- Assume K0=0, K1=0

- In each slot, grant arrival occurs i.i.d fashion with probability p.

- We assume channel rank is limited to 2.

- We consider following two cases.

  - C1) Configured max number of MIMO layer is 4.

  - C2) Configured max number of MIMO layer is 2.

- Let PPDCCH-only,4Rx be the power for PDCCH-only state with 4 Rx.

- Let PPDCCH-only,2Rx be the power for PDCCH-only state with 2 Rx.

  - The power scaling factor for 2Rx is 0.7 according to [7].

  - Thus, we have PPDCCH-only,2Rx = 0.7PPDCCH-only,4Rx

- Let PPDCCH+PDSCH,4Rx be the power for PDCCH decoding, PDSCH decoding with 4 Rx.

- Let PPDCCH+PDSCH,2Rx be the power for PDCCH decoding, PDSCH decoding with 2 Rx.

  - The power scaling factor for 2Rx is 0.7 according to [7].

  - Thus, we have PPDCCH+PDSCH,2Rx = 0.7PPDCCH+PDSCH,4Rx

#### Analysis

Case 1 : Continuous PDCCH monitoring with no sleep

In the Case 1, we make an additional assumption that UE continuously monitors PDCCH every slot.

For slots without grant arrival, UE performs PDCCH decoding only with configured number of antennas in both cases C1 and C2. Thus there is 30% of power saving for 2Rx.

For slots with grant arrival, UE performs PDCCH and PDSCH decoding with configured number of antennas in both cases C1 and C2. Thus there is 30% of power saving for 2Rx.

Considering both cases, we have following average power.

- Average Power of C1 = PPDCCH+PDSCH,4Rx × p + PPDCCH-only,4Rx × (1-p)

- Average Power of C2 = PPDCCH+PDSCH,2Rx × p + PPDCCH-only,2Rx × (1-p) = 0.7 × Average Power of C1

Observation 10

- This there is 30% of power saving gain when UE is configured with max MIMO layer of 2 compared to the case with max MIMO layer of 4.

Case 2 : PDCCH monitoring with sleep between grant arrival

In the Case 2, we assume that there exist a Genie sleep indication which makes UE to enter sleep state between two consecutive grants. Using the statistical model in our companion paper [6][8], we can get following results. For evaluation of 4Rx, we used P0. For evaluation of 4Rx, we used 0.7P0.

Figure 12 shows the average power consumption of UE configured with max 4Rx and that of UE configured with max 2Rx. Unlike the Case 1, when sleep operation is introduced, the power saving gain is reduced because there is no more power saving between 4Rx and 2Rx during sleep operation.

Figure 12 Average Power consumption for 4Rx and 2Rx with Genie sleep indication between grants (left). Power saving gain of 2Rx compared to 4Rx is shown as a function of grant arrival probability p.(right)

Observation 8

- The sleep operation could reduce the power saving gain of MIMO layer restriction.

# Remaining Issues in Evaluation Methodology

## Power Model based on Interpolation Method

During RAN1#95 meeting, there was a discussion on additional power states. One of remaining issues is power state for mini slot transmission for slot configuration, shown in Figure 7(a) with PDCCH(2)MS(10)PDSCH(2). To estimate power number for this slot, we discussed interpolation method based on following two end points. The basic idea is to use two power numbers as end points, first point with 0 (or 1) PDSCH symbols and the second point with 12 PDSCH symbols as two end points, to estimate power number in between them, as depicted in Figure 13. The open issue is the choice of power number of first point Y and overhead m in symbols for transition involved with micro sleep.

Figure 13 Interpolation method for determining power consumption for slot configuration with PDCCH(2)MS(10-N)PDSCH(N)

### Choice of Y

It is better to use a PDSCH configuration with at least one PDSCH symbol, however, there is no agreed number for that. So, instead we consider using PDCCH-only in cross-slot of 70 as first point. Another option is to use PDCCH-only in same-slot of 100. As a reasonable choice, we can use Y=90 for evaluation.

### Choice of overhead m

Overhead involved in transition to/from microsleep within slot is modeled as an extra OFDM symbols for PDSCH just for simplicity. Basically, agreed PDCCH-only powers (both with same and cross-slot) in RAN1 already captures  involved transition energy in those numbers 70 and 100 for simple modeling. Thus, what needs to be considered additionally is the additional processing power for PDSCH symbols. Considering overhead by m is increasing already considered overhead or double counting.

However, there was some concern on micro sleep operation and associated overhead for other slot configurations other than already agreed PDCCH-only cases. So, just for evaluation purpose, we can keep m as an additional potential overhead.

### Power Estimation

For Case 3, based on the discussion, it is straight to get following equations for power number for a given PDSCH symbols number N and overhead m.

P3N,m=Y+min12,N+m*300-Y12=1-xY+x∙300,

where x=min⁡(12,(N+m))12 is the ratio of PDSCH plus overhead symbols.

For Case 4, the power number Z for slot configuration in Figure 7(b) with PDCCH(2)PDSCH(2)MS(12) needs to be determined. Since we know that Z should be larger than Y and less than P3N=2,m=0=125, we assumes Z=120. For the additional overhead m, we could use following interpolation formula, which is based on the same principle.

P4N,m=Z+min12,N-2+m*300-Z14-2=1-wZ+w∙300,

Where Z=120 and  w=min⁡(12,(N-2+m))12 is the ratio of additional PDSCH symbols plus overhead symbols out of 12.

Proposal 1

- For evaluation, use interpolation method with Y=90 and m=[1~3].

Note that alternative methodology based on power breakdown was discussed in [3]. This approach also gives similar results.

# FTP3 and Grant Arrival Model

In RAN1 we have agreed to use FTP3 traffic model, which is basically continuous time random process modeling traffic arrival with Poisson distribution. Packet inter-arrival time and packet size are two important parameters of the FTP3 model. Packet inter-arrival time has an exponential distribution and packet size is fixed.

In this section, we discuss Grant Arrival Model (GAM) as a simple discrete version of FTP3 model. GAM is a simple mathematical model for modeling dynamic grant arrivals. In this model, we consider a slotted system, where, in each slot, grant arrives with a probability p. Then, the grant inter-arrival time becomes a random variable with geometric distribution. This model may be useful to model UE operation with especially self-contained type slot operation, where PDCCH, PDSCH, and ACK/NACK are transmitted in the same slot or back-to-back. During inter-grant arrival time, UE can either continuously monitor PDCCH or sleep. Sleep power during inter-grant arrival time can be also modeled as a random variable as a function of inter-grant time D. The traffic load can be controlled by changing probability p.

Figure 14 Grant arrival model as a discrete version of FTP3 model with back-to-back scheduling assumption

The agreed power consumption number in RAN1 evaluation methodology can be used to compute power consumption during grant processing. Note that ACK could be easily handled as either short or long PUCCH.  Note that all the power scaling factors could be also easily applied. Our companion paper [4][5] use this model to show potential gain of sleep indication mechanism.

Proposal 2

- For UE power saving SI evaluation, consider grant arrival model.

Connection between FTP3 and grant arrival model

# Conclusions

In this contribution, we have provided evaluation results for proposals discussed in [4][5]. In addition to that, we also discussed the remaining issues of estimating power consumption of slot configurations with in-slot micro sleep operation.

Observation 1

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption.

- The optimal BWP size depends on traffic load.

- The optimal BWP is dependent on UL transmission power level.

Observation 2

- A larger BWP allows UE to complete DL PDSCH reception quickly and then to switch to sleep mode to save power.

- The optimal BWP size depends on the existence of sleep operation.

- With sleep operation, power saving gain of optimal BWP could be as large as 60%.

Observation 3

- Using smaller values of K0 and K1 together with sleep operation could provide power saving gain of ~ 41% for a given reference configuration.

Observation 4

- Scheduling Tx and Rx activities close-in-time could greatly increase power saving by removing unnecessary transitions and ensuring longer sleep duration with lower power consumption.

Observation 5

- UE can avoid unnecessary PDSCH buffering if and only if PDSCH is transmitted after PDCCH decoding finishes.

- This applies to cross-slot and same-slot scheduling.

Observation 6

- Power saving gain of controlling TDRA (including K0 and SLIV ) approach is as high as 30% and gradually decreases to 0% for cross slot scheduling and 15% for same slot scheduling case.

Observation 7

- In case network activates additional carriers which supports low throughput, UE-initiated SCell deactivation mechanism could be effectively used to request the deactivation of the carriers with low power efficiency (bit/Joules).

Observation 8

- The sleep operation could reduce the power saving gain of MIMO layer restriction.

Proposal 1

For evaluation, use interpolation method with Y=90 and m=[1~3].

Proposal 2

- For UE power saving SI evaluation, consider grant arrival model.

# References

- RP-181432 Study on UE Power Saving in NR, CATT

- R1-xxxxxx Chairman’s Note RAN1 #95, Spokane, Nov, 2018

- R1-1812924 Evaluation Methodology for UE Power Saving, Apple, Nov, 2018

- R1-1900753 Power Saving Techniques based on UE Adaptation, Apple, Jan, 2019

- R1-1900754 Power Saving Techniques based on Explicit Network Indication, Apple, Jan, 2019

- TR38.840 v0.1.0 Study on UE Power Saving, Nov, 2018

Optimal in the sense that it minimizes power consumption among given set of BWP sizes