3GPP TSG-RAN WG1 Meeting Ad-hoc 1901	R1- 1900754

# Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.9.2.2

Source:	Apple Inc.

Title:	Power Saving Techniques based on Explicit Network Indication

Document for:	Discussion, Decision

# Introduction

In NR, we have new features to support higher data rates applications (eMBB) entailing higher power consumption. To overcome increased power consumption, techniques such as BWP (in frequency domain) and periodic PDCCH monitoring (in time domain) were introduced in R15. In this paper, we propose enhancements based on explicit indication from network to UE. Our companion paper [6] includes techniques for UE-assisted techniques for UE power saving.

# Potential Enhancements for UE Power Saving

## Explicit Signal based Sleep Indication

### Motivation

In R15 MTC and NB-IoT, it was agreed to introduce power efficient wake up type signal for UE power saving. The focus was on IDLE mode power saving enhancement since IoT devices stay mostly in IDLE state for extending battery life. In this sense, the context is different from NR UE power saving, where we think eMBB is most popular use case. Accordingly, the approach for power saving signal in NR could be very different from that in MTC/NB-IoT.

To achieve meaningful power saving gain, NR needs power saving techniques for CONNECTED mode. Considering that total power contribution from Idle mode is relatively small compared to that in CONNECTED mode [1][2][3], it becomes clear that power saving techniques for CONNECTED mode could be much more effective than that for IDLE mode. So, it should be the focus of NR UE power saving study.

When a UE in RRC CONNECTED mode is scheduled by gNB to receive DL/UL data, the grant transmission time to the UE of interest could be random. This is because that traffic arrival for the UE from application server is random and the gNB should serve multiple UEs in the cell. Figure 1 shows an exemplary CDF of grant inter arrival time of video and web-browsing in LTE. It is observed the mean grant-inter arrival time is 5ms for video traffic and 6.2ms for web-browsing. Note that the tail of distribution reaches to ~ 20ms for video and ~40ms for web-browsing. This indicates that there could be time gap of order of ms to tens of ms during which UE can enter sleep state for power saving. The further analysis could found in [4].

Figure 1 CDF of grant inter-arrival time for video (left) and web-browsing (right)

Another dimension that network can control is BWP size. If BWP size is large, then larger amount of data could be transmitted in one PDSCH and this could potentially increase scheduling interval until next PDSCH schedule time, which could make UE sleep longer time in deeper power state. Thus, we briefly reiterate BWP scheduling problem which was discussed in the BWP discussion of our companion paper[6]. The question is when network is to transfer a given amount of data D to UE using bandwidth B every time period T, how to choose bandwidth B. The choice of B has impact on UE processing/awake time and sleep time. Thus, it has significant impact on overall UE power consumption for time period T.

Figure 2 BWP scheduling problem with two strategies : i) large BWP and long deep sleep, ii) narrow BWP and short shallow sleep

This is an optimization problem and there exist an bandwidth size B* which minimum UE power consumption. In our experience, extending sleep duration with quick processing (with increased processing capability) tends to lower overall power consumption, see section 2.1 of [5]. Note that in this section, the discussion focus is on sleep indication only with fixed BWP size.

### Proposals

To take advantage of such time gap between grants, we could think of explicit sleep indication based UE power saving mechanism as shown in Figure 2. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing power saving gain.

Figure 3 Expected UE power consumption with explicit signal based sleep indication

Observation 1

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 1

- NR supports explicit-signal based sleep indication for UE power saving.

### Procedure

In this section, we discuss the channel structure, signal content, and UE behavior aspect for supporting network-initiated sleep indication.

Channel/Signal Structure

There could be multiple options for sleep indication signaling.

- O1) New signal/channel

- O2) Existing channel

There could be some benefit of having new signal or channel such as low power operation. However, considering that sleep indication is most useful when there are dynamic arrival of traffic (when active data transfer happening) or CDRX ON duration with potentially with long inactivity timer values, it is better to use existing channel such as PDCCH because UE is already in the state where it can monitor PDCCH. So, having new signal/channel could just increase complexity without much gain.

Proposal 2

- NR supports sleep indication using existing channel structure (e.g., PDCCH)

Signal Content

The most simplest approach would be to include a single bit indicating sleep indication for a certain time interval. Multiple bits could be signaled to signal more information. However, this would require more complex processing / operation at network side. For example, if length of sleep is indicated, then it network need to more accurately predict sleep duration until next grant arrival. Thus as a first step toward dynamic sleep operation, we think including single bit indicator should be good enough, which can also minimize any necessary DCI size change and its negative impact on coverage. The sleep duration could be configured by RRC or MACCE.

Proposal 3

- NR supports single bit sleep indication to indicate sleep for a specified time duration.

- The sleep duration is configured by RRC or MACCE.

UE behavior

Currently there is no clear definition of “sleep”. One possible definition is skipping of configured PDCCH monitoring. In this case, UE is still supposed to handle RRC configured activities such CSI-RS monitoring, SRS transmission, CSI reporting, etc during sleep duration. This approach leaves power saving as an UE implementation while minimizing the impact on network operation.

Another possible definition is literal sleep allowing UE to ignore RRC configured activities. This approach allows UE to enter sleep with higher chance but requires additional handling of RRC configured signals. For example, UE should take into account missing samples of RSRP (if any) in its filtering operation. Skipping CSI-RS or SSB could have negative impact on beam management process, which could potentially delay/slow beam failure recovery as well. Network should also take into account the missing SRS transmission (if any) in its UL channel sounding procedure. The missing CSI reporting, if any, could be also taken into account for network for DL channel status tracking. At this point in time, we see that many of such issues arising by ignoring RRC scheduled activities. For these reason, it may be good choice to define sleep as PDCCH monitoring skipping.

Proposal 4

- Sleep signal from network indicates that UE is not supposed to monitor PDCCHs during sleep duration.

### Performance Evaluation

For evaluation, we consider a statistical model with random grant arrivals. The model is simple yet good enough to capture the interplay between sleep and traffic load. For simple analysis, we make following assumptions.

#### Assumptions

- Traffic pattern: We model a grant arrival event with a Bernoulli random variable with parameter p, i.e., the grant arrival for any given slot occurs with probability p in i.i.d fashion. Accordingly, inter-grant arrival time can be modeled as a random variable D with geometric distribution as shown in Figure 3.

Figure 4 Illustration of grant arrivals and inter-grant arrival time (ACK or PUCCH power could be different from PDCCH/PDSCH processing power)

- Sleep indication : We assume that gNB can send Genie sleep indication to UE to indicate the for sleep duration D. This indicates that grant arrives with probability 1 after sleep period ends. Due to this assumption, the results we generate below should be understood as an upper bound of performance gain.

- BLER and ACK/NACK : For simplicity, we assume BLER=0 and ACK/NACK can be sent in the same slot as grant received (i.e., self-contained slot) or following slot without an error.

#### Analysis

Average Power Consumption with Sleep indication

Figure 4 shows an example of power state transitions in a single time duration of length 1+D including one grant slot and one following sleep duration D. Note that the inter-grant time D could be equal to or larger than 0. P0 denotes power consumption for normal PDCCH monitoring and PDSCH decoding (P0=300). PDi is sleep power for i-th duration Di.  Note that PDi is a function of Di, i.e., different length of sleep duration has different sleep power. The random variable ΔDi accounts for the energy cost for state transition.

Figure 5 Illustration of power consumption with sleep between two consecutive grants

Under this mode, one can compute the average power consumption Pavg over N such periods. The numerator is the total energy consumed for processing N grants and N sleep oepartion. The denominator corresponds to the total time for N periods. Note that each period includes one slot for a grant and one sleep duration of length D.

If we send N infinity, then by law of large numbers Pavg converges to the ratio of mean energy consumed and mean duration of D+1.

Now we calculate Pavg with followings. D is a geometric random variable with support {0,1,2,…} and pmf

.

Then, the mean of D is given by

.

The expected value of the product PDD is given as

,

where Pd the sleep power is given as

.

The expected value of transitional power cost is given by

,

where

.

Average Power with No Sleep

In case there is no sleep operation, the average power is determined based on the portion of PDCCH-only states and that of PDSCH+PDCCH decoding states. Since a grant arrives with probability p, the portion of slots with PDSCH is given as p. Accordingly, PDCCH-only monitoring slots take the portion of 1-p. Then, the average power is given as

Extension for long PUCCH with 14 symbols

This model could be easily extended to case supporting long PUCCH. We assume that PDCCH and corresponding PDSCH are transmitted in the same slot as LTE. Then, corresponding long PUCCH are transmitted in the next slot. It is assumed that K1=1 always for ease of analysis. This allows us to scale power consumption of DL part (first slot) and UL (second slot) independently, which allow to scale up PUCCH power to 23dBm which fixed DL processing power.

Figure 6 Extended grant arrival model with ACK/PUCCH in separate slot

The average power with sleep is updated as follows. First 1 in denominator is changed to 2 to capture PUCCH in a separate slot. In the numerator, PUL is added to capture UL transmission power.

Since p fraction of slots are used for PDCCH+PDSCH and another p fraction of slots are used for PUCCH transmission, the remaining fraction of slots is 1-2p.

The average power without sleep can be updated as follows.

#### Results

Figure 5 and Figure 6 show the power saving performance of sleep indication based on the statistical model described in the previous section. From the results, one can see that by dynamically indicating the sleep duration, significant gain can be attained especially when packet arrival rate is low (e.g., ~55% at p=0.1). Power saving gain depends on the grant arrival probability p. When p is close 0, a UE with no-sleep keep monitors PDCCHs which consumes on average power of 100. Whereas, a UE with sleep indication mechanism can sleep for a long time so the average power consumption approaches to 0. When p is close 1, both UEs receives grants almost every slot and there is no time to enter sleep. So, the power consumption converges to 300.

Figure 7 Average power consumption vs. grant arrival rate (p)

Figure 8 Power saving percentage of sleep indication vs. grant arrival rate (p)

Observation 2

- When the grant arrival probability is 0.05, the power saving gain of the explicit signal based sleep indication mechanism is 71%.

- When the grant arrival probability is 0.1, the power saving gain of the explicit signal based sleep indication mechanism is 55%.

- When the grant arrival probability is 0.2, the power saving gain of the explicit signal based sleep indication mechanism is 37%.

# Conclusions

In this contribution, we have analyzed the performance of the explicit signal based sleep indication mechanism. We made following observations and proposals.

Observation 1

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 1

- NR supports explicit-signal based sleep indication for UE power saving.

Proposal 2

- NR supports sleep indication using existing channel structure (e.g., PDCCH).

Proposal 3

- NR supports single bit sleep indication to indicate sleep for a specified time duration.

- The sleep duration is configured by RRC or MACCE.

Proposal 4

- Sleep signal from network indicates that UE is not supposed to monitor PDCCHs during sleep duration.

Observation 2

- When the grant arrival probability is 0.05, the power saving gain of the explicit signal based sleep indication mechanism is 71%.

- When the grant arrival probability is 0.1, the power saving gain of the explicit signal based sleep indication mechanism is 55%.

- When the grant arrival probability is 0.2, the power saving gain of the explicit signal based sleep indication mechanism is 37%.

# References

- R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

- R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

- RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

- R1-1714092 Control Information for UE Power Saving, Apple, Aug, 2017

- R1-1900752 Evaluation of UE power saving techniques, Apple, Jan, 2019

- R1-1900753 Power Saving Techniques based on UE Adaptation, Apple, Jan, 2019