3GPP TSG RAN WG1 Ad-Hoc Meeting 1901						      R1-1900797

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item: 	7.2.4.1.5

Source:	InterDigital Inc.

Title:	Link Level Evaluations on Waveform for NR V2X

Document for:	Discussion and Decision

# Introduction

In the WG1 #95 meeting, the following were agreed on for the physical layer structure [1].

Agreement:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

It is well known that DFT-s-OFDM has lower PAPR compared to CP-OFDM, resulting in smaller back-off and less non-linear distortion when a power amplifier (PA) is used. In this contribution, we present link level simulation results to compare CP-OFDM and DFT-s-OFDM waveforms with a polynomial PA model [2] incorporated into the simulation.

# Evaluation Results

The BLER performance of CP-OFDM and DFT-s-OFDM are compared using the simulation assumptions given in the Appendix. Specifically, the BLER is evaluated when there is no PA and when the target output power of the PA is set to one of 25 dBm, 26 dBm, or 27 dBm. Two different allocation sizes of 5 and 15 RBs are used, and the relative speed is set to 30 km/h or 280 km/h.

Figure 2-1 and Figure 2-2 illustrate the BLER for 5 and 15 RBs with 30km/h UE speed. In addition, Table 1 provides the gain of DFT-s-OFDM over CP-OFDM at 10-1 BLER level. The results show that CP-OFDM outperforms DFT-s-OFDM without PA. But with PA, the gain of DFT-s-OFDM increases with increasing output power, reaching to 3.5 dB when the target output power is set to 27 dBm. We can also see that the gain is slightly less with a larger allocation size because CP-OFDM provides higher coding gain in a more selective channel.

Figure 2-1 BLER comparison with 30 km/h and 5 RBs

Figure 2-2 BLER comparison with 30 km/h and 15 RBs

Table 1 Gain (dB) of DFT-s-OFDM over CP-OFDM at 10-1 BLER

| Allocation size | Target output power |  |  |  |
| --- | --- | --- | --- | --- |
|  | No PA | 25 dBm | 26 dBm | 27 dBm |
| 5 RB | -0.9 | 0.7 | 2.1 | 3.5 |
| 15 RB | -1.4 | 0.34 | 1.8 | 3.5 |

Figure 2-3 and Figure 2-4 illustrate the BLER for 5 and 15 RBs with UE speed 280km/h while Table 2 provides the gain of DFT-s-OFDM over CP-OFDM at 10-1 BLER level. The results show that CP-OFDM outperforms DFT-s-OFDM without PA. As the output power is increased, however, the gain of DFT-s-OFDM increases, reaching to about 3 dB when the target output power is set to 27 dBm. It is noticeable that the gain of DFT-s-OFDM is reduced slightly with the increased Doppler.

Figure 2-3 BLER comparison with 280 km/h and 5 RBs

Figure 2-4 BLER comparison 280 km/h and 15 RBs

Table 2 Gain (dB) of DFT-s-OFDM over CP-OFDM at 10-1 BLER (280 km/h)

| Allocation size | Target output power |  |  |  |
| --- | --- | --- | --- | --- |
|  | No PA | 25 dBm | 26 dBm | 27 dBm |
| 5 RB | -1.5 | 0.1 | 1.4 | 2.9 |
| 15 RB | -1.6 | -0.6 | 1.4 | 3.3 |

The results presented above show that the performance of DFT-s-OFDM can offer much better tolerance to nonlinear PA and, therefore, it can provide significantly larger coverage by using higher Tx power than CP-OFDM due to smaller PAPR.

# Conclusion

In this contribution, we provided an evaluation of CP-OFDM and DFT-s-OFDM waveforms using link level BLER results that incorporated the impact of a nonlinear PA. Based on the results, we propose the following:

Proposal 1: DFT-s-OFDM is supported in V2X sidelink.

# References

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #95 Meeting, Spoken, USA, Nov. 2018

- R1-165006 “On the evaluation of PA model,” 3GPP TSG RAN WG1 #85 Nanjing, China, 23th – 27th May 2016

# Appendix

Table 3 Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Bandwidth | 5 RB, 15 RB |
| Channel | CDL 100A |
| MCS | QPSK, ½ code rate |
| Waveform | CP-OFDM, DFT-s-OFDM |
| Subcarrier Spacing | 30 kHz |
| Slot format | DMRS are mapped to every other subcarrier |
| CP length | Normal CP |
| Frequency synchronization error | Not modeled |
| Time synchronization error | Not modeled |
| UE relative speed | 30 km/h, 280 km/h |
| Channel estimation | Realistic |
| UE receiver algorithm | MMSE |
| PA MODEL | Polynomial [2], target output power set to 25, 26, 27 dBm |
| Number of Tx/Rx antennas | 1 Tx, 2Rx |