3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		                      R1-1900805

Taipei, Taiwan, 21st – 25th, January 2019

Agenda Item:	7.2.6.1.5

Source:	InterDigital Inc.

Title:	Preliminary performance evaluations for AR/VR

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #94bis [1], there were agreements related to simulation assumptions for URLLC, including AR/VR applications and the following parameters were agreed for the evaluation of AR/VR use case:

Agreements:

- Take the following table as the table of representative use cases for selection for Rel-16 NR URLLC evaluation.

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |
|  | 99.9 | 7ms (air interface delay) | DL & UL: 4096, 10 K FTP model 3 or periodic with different arrival rates |  |

  - Note: The above packet size already includes header overhead.

In this document, we present simulation results for the performance of NR URLLC for AR/VR applications.

# Discussion

In TS 22.261 “Service requirements for the 5G system”, SA has defined the high-level requirements for AR/VR applications. According to this document, to support VR environments with low motion-to-photon capabilities, the 5G system shall support:

- motion-to-photon latency in the range of 7-15ms while maintaining the required user data rate of [1Gbps] and

- motion-to-sound delay of [<20ms].

Noting that the motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset and the motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears. In RAN1 meeting #94bis, it was agreed that, for evaluation of AR/VR, large packet sizes in the range of 4k & 10k bytes to be considered. To transmit a 10k packet in one TB assuming a robust MCS (e.g., QPSK modulation with the ½ rate channel code) while trying to meet 99.9% reliability requires roughly 512 RBs for baseline numerology of 15kHz SCS. However, according to TS 38.104, the maximum number of RBs supported for the baseline numerology of 15kHz SCS is 270 which is corresponding to 50MHz BW. Since the current NR design cannot support 10k TB size with the above assumptions, the 10k packet should be split to multiple TBs using higher layer segmentation and transmitted on different carriers in the case of carrier aggregation or consecutively in time over multiple slots within the 7 msec air interface latency budget. Accordingly, in this contribution, we just evaluate the 4k packet size.

Based on the above discussion, assuming that a 10k packet can reliably be transmitted over 100MHz BW in FR1 over 1 msec, the maximum data rate will be around 80 Mbps which is much lower than the required 1 Gbps user data rate defined by SA and also needed to deliver high quality video experiences with the VR360 (See Table 1). Nevertheless, assuming two 100MHz BW carriers, two 10k packets can be supported which will be sufficient to support 8K UHD in FR1. In conclusion, a high quality VR360 experience with 1 Gbps user data rate can only be supported with very large bandwidths in FR2 but the real challenge would be meeting the 99.9% reliability in the higher bands.

Table 1: Typical data rate requirements for VR360 using various video formats

| Video Format | Spatial Resolution (Pixels/Frame) | Frame rate | Typical Bitrates (Mbps) |
| --- | --- | --- | --- |
| HD | 1920x1080 (~2MP) | 60 | 7-10 |
| 4K UHD | 3840x2160 (~8MP) | 60 | 12-25 |
| 8K UHD | 7680x4320 (~32MP) | 60 | 112-160 |
| VR360 | 21600x10800 (~233MP) | 60 | 800-1100 |

# System-level simulations results

The assumptions for system-level simulations are shown in the Appendix. Based on these assumptions (which are based on the agreements in RAN1 #94bis), the system-level simulation results for Indoor open-office channel model are shown in Figure 1.

Figure 1. System-level simulation results for Indoor

Based on these simulation results, the 5-percentile SINR is shown in Table 2.

Table 2. System-level simulation results for 5-percentile SINR

| Test Case | 5-percentile SINR (dB) |
| --- | --- |
| 40 MHz DL | -4.167 |
| 80 MHz DL | -4.167 |
| 40 MHz UL | -10.74 |
| 80 MHz UL | -12.94 |

- Link-level simulations results

The performance of NR URLLC for AR/VR with packet size of 4096, with 1 to 4 transmissions (0 to 3 retransmissions) is presented in Figure 2 and Figure 3, for the cases of 30kHz subcarrier spacing (with 80 MHz bandwidth) and 15kHz subcarrier spacing (with 40 MHz bandwidth), respectively. As can be seen from the simulation results for both these two cases, one-shot transmission achieves the required reliability of 99.9% at around -1 dB with precoder cycling. Also in case precoder cycling is used, one retransmission is enough to lower the required SNR to less than -4.167 dB, which satisfies the 5-percentile coverage for the Indoor open-office Downlink channel model. It should be noted that for this case, based on the agreed simulation assumptions, the air interface latency is supposed to be limited by 7ms, hence, 1-3 retransmissions still satisfy the latency limits.

However, for uplink, even 3 retransmissions cannot satisfy the 5-percentile coverage (which correspond to -10.74 dB and -12.94 dB for the cases of 40 MHz and 80 MHz bandwidth) with fixed precoder or precoder cycling. This shows that for the case of large AR/VR packet sizes, meeting the reliability requirement in uplink is challenging. One solution might be using more bandwidth or having narrower beams (which is possible when using millimeter waves).

Observation 1: For the case of DL AR/VR with 4096-bye packet sizes, NR is able to satisfy the air interface latency limit of 7ms and reliability of 99.9%, assuming 40MHz bandwidth.

Observation 2: For the case of UL AR/VR with 4096-bye packet sizes, meeting the URLLC reliability requirement is challenging (it requires more than 4 transmissions).

Figure 2. Performance of PDSCH/PUSCH for AR/VR with packet size of 4096 bytes, over 80MHz bandwidth and 30kHz subcarrier spacing.

Figure 3. Performance of PDSCH/PUSCH for AR/VR with packet size of 4096 bytes, over 40MHz bandwidth and 15kHz subcarrier spacing.

# Summary

In this contribution, simulations results for performance of URLLC for AR/VR applications was presented. The following observations and proposals were made:

Observation 1: For the case of DL AR/VR with 4096-bye packet sizes, NR is able to satisfy the air interface latency limit of 7ms and reliability of 99.9%, assuming 40MHz bandwidth.

Observation 2: For the case of UL AR/VR with 4096-bye packet sizes, meeting the URLLC reliability requirement is challenging (it requires more than 4 transmissions).

# References

[1] RAN1#94bis Chairman’s Notes, Chengdu, China, Oct. 2018

# Appendix: Simulation assumptions

Table A1. Indoor system-level evaluation parameters

| Parameters | Values |
| --- | --- |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 40 MHz, 80 MHz |
| SCS | 30 kHz, 15 kHz |
| BS antenna configurations | 4 Tx/4 Rx antenna ports dH = dV = 0.5 λ (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2Tx antenna ports and 4 Rx antenna ports Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 For 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UE antenna gain | 0dBi as starting point |
| UE receiver noise figure | 9dB |
| BS Tx power | 24 dBm for 20 MHz bandwidth (i.e. 27dBm for the total bandwidth of 40 MHz) |
| Channel model | 3gpp “indoor office” channel for 4 GHz |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Number of UEs per cell | 5 |
| Inter-BS distance | 20m |
| BS antenna height | 3m |
| UE Tx power | 23 dBm |
| BS antenna element gain + connector loss | 5dBi |
| UE antenna height | Follow the modelling of TR 38.901 (1.5m) |
| UL power control | Alpha = 1, P0 = -116.44 dBm |
| UE distribution | 100% of users are indoor: 3 km/h UE-speed |

Table A2. Assumptions for the link-level simulations.

| Configuration Parameters | URLLC configuration A |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C with 100ns delay spread |
| Bandwidth | 40 MHz, 80 MHz |
| Subcarrier spacing | 15 kHz, 30kHz |
| Antenna setting | PDSCH 4TX, 4RX |
| Tx diversity | fixed precoding, precoder cycling |
| Speed | 3km/h |
| Channel estimation | Practical: 1os DMRS type 1 |
| Frequency allocation | Full BW allocation |
| Time allocation | type A |
| PDCCH | Polar codes, 40b payload excl. CRC. AL16, 1 OFDM Sym, 20MHz |
| Data | LDPC, QPSK, 4096 BYTES Repetition (RV Seq = [0 2 3 1]) MCS 13 (QPSK, CR ~ 0.5) |