3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		                      R1-1900807

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.6.3

Source:	InterDigital Inc.

Title:	Potential enhancements for UL configured grant

Document for:	Discussion and Decision

# Introduction

In RAN1#94, the discussion on the enhanced UL grant-free transmission was kicked off and the following agreements were made [1]:

Agreements:

- Study further whether/how multiple active configured grants for a BWP of a serving cell.

  - Identify potential specification impacts and options for both type 1 and type 2

    - At least Activation/deactivation mechanism for Type2

      - E.g., whether each configuration is activated/deactivated or multiple configurations are activated/deactivated

    - Study how to support repetitions with multiple configurations for a BWP of a serving cell

    - FFS HARQ process ID determination for both type 1 and type 2

    - FFS other specification impacts for both type 1 and type 2

  - Study the performance impacts

Agreements:

- Study further whether/how on ensuring K repetitions.

- Study further on PUSCH repetitions within a slot for configured grant.

In RAN1#94b, the enhanced UL grant-free transmission was further discussed and the following agreements were achieved [2]:

Agreements:

- To study further from at least the following:

  - Option 1: multiple active configured grant configurations for a BWP of a serving cell

  - Option 2: repetition(s) across the boundary of a period P

  - Option 3: one transmission cross boundary of a period P

  - FFS the UE behavior when repetitions are collided with the resource which are not available for UL transmissions

  - Note: Switch grant free to grant based retransmission which is available in Rel.15

In RAN1#95, the following was agreed for the UL configured grant [3]:

Agreements:

- Multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency

  - FFS details

  - Note: it is understood that the above may be related to RAN2-led work on intra-UE multiplexing

- Send LS to RAN2 – R1-1814112, which is endorsed by updating “kindly asks”, and final LS is in R1-1814116

  - RAN1 kindly ask RAN2 to take RAN1 agreements into account in their future work

Agreements:

- One PUSCH transmission instance is not allowed to cross the slot boundary for UL configured grant

Agreements:

- For whether to support explicit HARQ-ACK for configured grant for UL, at least study further gNB’s missed detection performance of the PUSCH under configured grant

  - Study how to resolve gNB’s missed detection if it is an issue

  - Study should take at least following into account:

    - Companies report the false alarm target

    - Companies report the DMRS configuration assumptions

    - The number of UEs sharing the time/frequency-domain grant free resource: 1 is the baseline, larger than 1 can also be considered

In this contribution, we provide our views on the enhanced UL configured grants transmissions for URLLC.

# Explicit HARQ-ACK

As agreed in RAN1#95, whether to support explicit HARQ-ACK for configured grant for UL depends on the results of the study on gNB’s missed detection performance of the PUSCH under configured grant.

In case of agreement on supporting explicit HARQ-ACK for configured grant for UL, three different options may be considered for transmitting the explicit HARQ-ACK feedback (as proposed in RAN1#94 [4]):

- Option 1: UE-specific DCI

- Option 2: group-common DCI

- Option 3: Sequence-based ACK

Among these three options, we believe the group common DCI for signaling of the explicit HARQ-ACK feedback provides a good trade-off between the PDCCH overhead associated with the UE-specific DCI and the relatively high specification impact associated with the sequence-based ACK. Option 2 relies on the joint coding of the ACK/NACK feedbacks for multiple users which are then transmitted using a group common DCI on PDCCH. A similar approach has been already adopted in NR for signaling a group of TPC commands for PUCCH or PUSCH transmission by a group of UEs on DCI format 2_2 or for the transmission of a group of TPC commands for SRS transmissions by one or more UEs on DCI format 2_3. The main advantage of this option compared to Option 1 is its capability to avoid the undesirable overhead on control channel by allowing ACK/NACK feedback for multiple users to be transmitted on a single PDCCH resource.

Proposal 1: A mechanism based on group-common DCI should be introduced for explicit indication of the HARQ-ACK feedback for configured UL grant.

# Ensuring K repetitions

# Another aspect of configured grant in NR is K-repetition feature, where the UE repeats the TB across the K consecutive slots applying the same symbol allocation in each slot with redundancy versions taken from a pre-configured redundancy version pattern. The K-repetition transmission helps to enhance reliability, and if the gNB decodes the TB in the middle of the K transmissions it can inform the UE to stop transmitting the remaining of the K-repetition sequence by transmitting an UL grant.

# One of the limitations of the NR K repetitions in NR is that for any RV sequence, the repetitions shall be terminated at the last transmission occasion among the K repetitions within the period P regardless of the starting occasion of the initial transmission (P is an allowed periodicity configured by higher layer and depends on the configured subcarrier spacing for UL transmission without UL grant). The reason for this restriction is that each period P could potentially have a different HARQ process ID and accordingly if the K repetitions crosses the two different periods then there will be an ambiguity between the gNB and the UE for determination of the HARQ process ID.

# Given that Rel-16 has envisioned higher reliability requirements (up to 1E-6 level), it is essential to make sure that NR adopts a mechanism to ensure K repetitions can be completed by the URLLC UE for UL transmission without grant by allowing the repetitions across multiple periods. One simple solution could be to introduce a similar mechanism as in FeLAA where the UCI associated with the UL data (e.g., HARQ ID, RV, etc.) is transmitted along with the UL-SCH in the UL configured grant. The Rel-15 NR already supports the UCI reporting in PUSCH where when a UE has a PUSCH transmission that overlaps with a PUCCH transmission that includes HARQ-ACK information and/or semi-persistent/periodic CSI, the UE multiplexes the HARQ-ACK information and/or the semi-persistent/periodic CSI in the PUSCH. In Rel-16, NR may extend this approach to support the transmission of the UCI associated with the UL-SCH together with the UL-SCH in the UL configured grant for PUSCH transmission.

Proposal 2: NR should adopt a mechanism to ensure K repetitions can be completed by the URLLC UE for UL transmission without grant in Rel-16 by allowing the repetitions across multiple periods.

Proposal 3: NR should support the transmission of at least HARQ ID as a UCI multiplexed on the PUSCH for the UL configured grant in Rel-16.

# Mini-slot repetition

As was discussed in our companion contribution [5], the mini-slot based repetition is beneficial for UL configured grant, as the overloading factor increases by randomizing the interference for UL data transmission.

Proposal 4: Mini-slot repetition with frequency hopping should be supported for UL Configured grant.

# Summary

In this contribution, we discussed our views on the possible enhancements for UL configured grants transmissions for URLLC. We have the following proposals:

Proposal 1: A mechanism based on group-common DCI should be introduced for explicit indication of the HARQ-ACK feedback for configured UL grant.

Proposal 2: NR should adopt a mechanism to ensure K repetitions can be completed by the URLLC UE for UL transmission without grant in Rel-16 by allowing the repetitions across multiple periods.

Proposal 3: NR should support the transmission of at least HARQ ID as a UCI multiplexed on the PUSCH for the UL configured grant in Rel-16.

Proposal 4: Mini-slot repetition with frequency hopping should be supported for UL Configured grant.

# References

RAN1#94 Chairman’s Notes, Gothenburg, Sweden, August 2018.

RAN1#94b Chairman’s Notes, Chengdu, China, Oct. 2018.

RAN1#95 Chairman’s Notes, Spokane, USA, Nov. 2018.

R1-1809979 Summary of 7.2.6.3 Enhanced UL grant-free transmissions, NTTDCM, RAN1#94, Gothenburg, Sweden, August 2018.

R1-1900804 “Potential PUSCH enhancements for URLLC”, InterDigital Inc., Taipei, Taiwan, Jan. 2019