3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900886

Taipei, Taiwan, 21st – 25th January 2019

Agenda item:	7.2.4.1.3

Source: 	Qualcomm Incorporated

Title: 	Synchronization design for NR V2X

Document for:	Discussion/Decision

# Introduction

In RAN1 #94 , #94bis, and #95, several agreements were reached on synchronization for NR V2X as summarized in Appendix B. In this contribution, we further discuss the synchronization mechanisms and signal design for NR V2X.

# Synchronization signal design

## Sidelink SSB Design

When GNSS/gNB signals are not available, synchronization signals transmitted by UEs may be helpful for UEs to maintain time/frequency synchronization. In this section, we provide some preliminary considerations on sidelink SSB design. Similar to NR Uu, sidelink SSB may consist of PSS, SSS and MIB, but the signal details and SSB structure for sidelink may be different considering the potentially different requirements.

## SSB Bandwidth

NR SSB has a bandwidth of 20 RBs, this well fits the larger available bandwidths in NR deployments. For sidelink, however, 20 RBs may not be suitable. For example, given a deployment scenario of 10 MHz sidelink bandwidth with 60 kHz subcarrier space, only 12 RBs are available. In this case, although PSS and SSS can still be accommodated (assume they follow the same sequence length as NR Uu), 20 RBs bandwidth makes the transmission of S-SSB infeasible. Therefore, to facilitate flexible NR V2X deployments, S-SSB bandwidth needs to be reduced comparing to NR Uu. Considering that PSS and SSS occupy 12 RBs, A bandwidth of 12 RBs seems good option for S-SSB. Note that, more symbols may be needed for PSBCH to achieve similar link budget as Uu PBCH. The exact number of symbols needed for PSBCH can be further studied, and it will be determined by sidelink MIB payload.

Proposal 1: S-SSB has a bandwidth of 12 RBs; number of symbols needed for S-SSB will be determined by the sidelink MIB payload.

## Sidelink PSS

Different from LTE, NR Uu uses m-sequences as primary synchronization signal. For NR V2X, it is straightforward to follow NR Uu design, i.e., use m-sequence for S-PSS. However, to avoid confusing cellular UEs, S-PSS should be orthogonal to NR PSS sequences. Details can be studied further.

NR Uu PSS may have 3 dB EPRE boost thanks to the blanked REs on PSS symbol. To achieve similar synchronization performance on sidelink, S-PSS can be repeated, e.g., on two consecutive symbols. RAN1 should study the performance of PSS detection with repetition.

Another feature of NR Uu PSS is, it conveys part of the cell ID. There is no such requirement on sidelink, the number of S-PSS sequences can thus be reduced. For example, like in LTE sidelink, which has two S-PSS sequences; or further reduce the S-PSS sequences number to one. This will ease the detection of S-PSS since UEs detects only one PSS; also, because there are more hypotheses for NR SSS than in LTE, it is possible that synchronization source type or other information conveyed during synchronization can be carried by S-SSS.

Proposal 2: S-PSS is transmitted on two consecutive symbols; RAN1 studies the performance of S-SSB with the assumption that only one S-PSS sequence is specified.

## Sidelink SSS

In synchronizations, detection of PSS always comes first; which means that when S-PSS is orthogonal to NR Uu PSS, there would be no confusion detecting Uu SSB and S-SSB, and thus NR Uu SSS sequences can be reused for S-SSS. Note that, when there is only one sequence for S-PSS, the number of available hypotheses for S-SSS would be 336 according to NR SSS definition. The actual number of S-SSS sequences would be determined by the ID information carried by S-SSS.

Since S-PSS transmitted by multiple UEs may be combined in SFN manner, S-PSS may not be a good choice for CFO estimation. To improve the CFO estimation performance, S-SSS can be repeated on two symbols, and the receiver estimates CFO using S-SSS. Note that, for a better estimation performance, the two S-SSS symbols may be separated.

Proposal 3: Reuse NR Uu SSS sequences for S-SSS; S-SSS is transmitted on two separated symbols.

# Synchronization procedure

# Background on LTE V2X synchronization procedure

LTE V2X synchronization mechanism relies on deriving time and frequency synchronization from one of the three potential synchronization source types: (i) GNSS, (ii) eNB, and (iii) SyncRef UE. SyncRef UEs are the UEs which may themselves be directly synchronized to GNSS or eNB (such that it’s a hop-0 SyncRef UE), or maybe synchronized to a hop-0 SyncRef UE (such that it’s a hop-1 SyncRef UE), and so on.

Sidelink synchronization signals (SLSS) and channel: Consists of PSSS, SSSS, and PSBCH. PSSS/SSSS resemble Rel-8 PSS/SSS, while PSBCH resembles PSSCH channel and contains information such as in-coverage indicator, tdd-config, etc. The sidelink synchronization signals and channel are transmitted every 160ms (by a SyncRef UE) for V2X communications and over a fixed transmission BW of 6 PRBs.

Sidelink synchronization resources: If no resources are configured for a given sidelink carrier, then only GNSS/eNB can be used as synchronization sources. If SLSS based synchronization is desired (i.e. use SyncRef UE to derive time/frequency synchronization, if needed), then the resource(s) to use for transmission of SLSS/PSBCH.

To enable SLSS based synchronization, one resource is configured for in-coverage UE by the eNB (and network is required to ensure it’s the same as the first resource that is (pre)configured for out-of-coverage operation). For out-of-coverage operation, ether two or three resources can be configured. The synchronization procedure and priority of synchronization sources differ depending whether two or three resources are (pre)configured.

Sidelink synchronization procedure: The priority between the synchronization source types are specified and depend on the following configurations and coverage scenario:

- In-coverage and GNSS is configured to be higher priority than eNB

- In-coverage and eNB is configured to be higher priority than GNSS

- Out-of-coverage and GNSS is pre-configured to be higher priority than eNB

- Out-of-coverage and eNB is pre-configured to be higher priority than GNSS

UE capability: Due to the complexity in UE implementation, support of sidelink synchronization signal transmission and reception is a UE capability. This motivates studying potential enhancements with reduced complexity for NR V2X synchronization.

# NR V2X synchronization procedure

## Discussion

As highlighted above, the current LTE V2X synchronization procedure and requirements present considerable UE implementation complexity. Moreover, in our view, such complexity may not be needed in all operation scenarios. This motivates the need to seek reduced complexity enhancements to the LTE V2X synchronization procedure.

As motivation, let’s consider the example of out-of-coverage operation with GNSS pre-configured as the highest priority. In such case, the priority of the synchronization source types is given as follows:

Priority of synchronization source types with out-of-coverage with GNSS (pre)configured as highest priority:

- GNSS

- SyncRef UEs directly synchronized to eNB 
(SLSSID in slss_net, INC = 1)

- SyncRef UEs directly synchronized to GNSS 
(SLSSID = 0, INC = 1 /OR/ SLSSID = 0, resource = 3)

- SyncRef UEs in-directly synchronized to eNB 
(SLSSID in slss_net, INC = 0)

- SyncRef UEs in-directly synchronized to GNSS
(SLSSID = 0, INC = FALSE & resource 1 or 2)

- SyncRef UEs in-directly synchronized to GNSS via OOC UE
(SLSS ID = 169, INC = False; with three resources configured)

- Independent SyncRef UEs

In this scenario, unless the UE is directly synchronized to GNSS (highest priority) is required to constantly perform a full (synchronous and asynchronous) search for other SyncRef UEs and reselect if a higher priority source is found. This requires the UE to implement a searcher for sidelink synchronization that is constantly running asynchronous to the data reception operation (i.e. UE needs to be able to handle two asynchronous timings – one for data transmission and reception, and an asynchronous timing for searcher operation). While some allowance (0.5% for a sidelink carrier) of dropping V2X communication data reception are allowed by RAN4 specification [36.133], it may not be desired to drop V2X data reception unless really needed.

However, if we consider the scenario of out-of-coverage operation for V2X in an ITS channel (no eNB coverage), we can potentially limit the attention to GNSS and SyncRef UEs directly or in-directly synchronized to GNSS. In such a case, if we further remove the consideration of an independent synchronization source, then the synchronization mechanism reduces to a synchronization search only. In other words, much of the complexity in synchronization arises from mixing of eNB and GNSS synchronization sources, and handling of independent synchronization source.

In realistic scenarios like: (a) urban driving where GNSS coverage is spotty, and (b) tunnel scenario with GNSS coverage outside the tunnels: the vehicular UE will have GNSS coverage certain times and may lose it for some time. In such scenario(s), even when UE has lost GNSS, an asynchronous search for SyncRef UEs is not needed in most cases as the UE’s oscillator would not have drifted significantly. From a UE complexity and system performance viewpoint, two enhancements can then help in such realistic scenario:

- Relaxed time/frequency synchronization error requirements for NR V2X sidelink transmission and reception, as the current requirements of +-12Ts are overly stringent and can be optimized for V2X use case.

- Reduced complexity synchronization procedure to allow for synchronous-only synchronization source search and/or time/frequency synchronization adjustment.

For (2) above, we can potentially study both SLSS based and non-SLSS based synchronization procedures. The motivation is to consider the case when the UE was synchronized to GNSS/eNB/gNB/SyncRef UE at a prior time and has not drifted significantly (such that it can still receive packets from other synchronized UEs). For SLSS-based, this will entail studying if we can perform synchronized search only for SyncRef UEs. For non-SLSS based, this will entail studying if we can derive timing from other UEs from which we can receive packets.

Proposal 4: Revaluate the time/frequency error requirements for NR V2X data/control and synchronization signal transmissions.

Proposal 5:  Introduce reduced complexity synchronization procedure for NR V2X to address the scenario wherein the UE was synchronized to GNSS/eNB/gNB/SyncRef UE at a prior time and has not drifted significantly (e.g. can still receive packets from other synchronized UEs).

## Reduced complexity synchronization procedure for NR V2X

In this section, we evaluate two potential reduced complexity synchronization procedures for NR V2X to address the scenario wherein the UE was synchronized to GNSS/eNB/gNB/SyncRef UE at a prior time and has not drifted significantly. Simulation results are provided to motivate the support of such procedures for NR V2X.

## Non-SLSS based synchronization

In non-SLSS based synchronization procedure, once the UE loses GNSS synchronization, but it can still receive packets from other GNSS-synchronised UEs (since its oscillator has not drifted significantly), then it can derive the time/frequency synchronization from the DMRS of the received data.

In terms of signalling, the only indication needed is to include the GNSS coverage state (yes/no) in the UEs transmission (e.g. MAC header). The current synchronization procedure specified for SLSS can simply be extended, for example, add specification changes of the likes of the following:

- If GNSS coverage is not available

  - If the UE can receive decode packet from another UE (potential SynchRef-noSLSS UE) and determines the GNSSCoverage indicator in the MAC header of the packet to be set to true; and

  - If the RSRP measured on the DMRS of the PSSCH exceeds a configured threshold; and

  - If the RSRP measured on the DMRS of the PSSCH exceeds the last measured RSRP of the previously selected SynchRef-noSLSS UE;

  - then

    - Select the potential SynchRef-noSLSS UE as the synchronization reference for time/frequency synchronization

Note that such a procedure will be supplement to SLSS-based synchronization.

Proposal 6: Introduce non-SLSS based synchronization enhancements.

Proposal 7: Support UE transmitting its GNSS coverage state in (e.g. in  MAC header) of its transmission to enable non-SLSS based synchronization.

## Synchronous-SLSS only based synchronization

For SLSS-based synchronization (as standardized in LTE V2X), once the UE loses GNSS synchronization, it needs to perform a complete asynchronous search for SLSS transmission to derive it time/frequency synchronization. However, in case the UE’s oscillators have not drifted significantly, it is entirely possible to perform the SLSS search only on a restricted window (being termed as synchronous search for SLSS in this contribution). Furthermore, note that is repeated PSS symbols are used in NR Sidelink SSB design (similar to LTE SLSS), then the window for the synchronous SLSS search is +- half of PSS symbol length (as opposed to +- half of CP). E.g., for 30kHz that corresponds to a window of ~ +-15us. For the oscillator to drift by 15us, it’ll take over 50seconds (assuming 10ppb/sec oscillator drift) and can thus provide the UE significant time to drift and still be able to find a SLSS UE with reduced complexity search.

Furthermore, the specification impact is expected to be quite low as it only uses a subset of the synchronization procedure as would be specified for the baseline SLSS procedure (fully asynchronous search) that can be used based on UE capability.

Proposal 8: Introduce reduced complexity synchronous-SLSS based synchronization enhancements (with reduced search window for SyncRef UEs).

Observation 1: Specification impact to support reduced-complexity synchronization procedures (non-SLSS based synchronization and synchronous-SLSS based synchronization) is expected to be quite low.

Observation 2: The reduced-complexity synchronization procedures would supplement the SLSS-based synchronization and their support is expected be up to UE capability.

## Simulation results

In this section, we compare the synchronization performance of the following synchronization mechanisms:

- Case 1: GNSS only

- Case 2: GNSS only + (sync+async) SLSS

- Case 3: GNSS only + non-SLSS based synchronization

- Case 4: GNSS only + sync-SLSS based synchronization

Case 1 corresponds to Release 14 procedure with GNSS only synchronization. When the UE goes out of GNSS coverage, the UE can still keep transmitting as long as the oscillator has not drifted enough and can guarantee to still meet the time/frequency error requirements (as required by Rel-14 specifications).

Case 2 corresponds to Release 14 procedure with GNSS and SLSS based synchronization. When the UE goes out of GNSS coverage, the UE starts to look for SLSS transmissions from other UEs and selects/reselects the highest priority SyncRef UE to derive its time/frequency synchronization. The transmission of SLSS follows the rules for Release 14 with 2 synchronization resources being configured and appropriate thresholds for SyncRef UE detection, SLSS transmission, minimum SyncRef UE S-RSRP etc.

Case 3 corresponds to the proposed reduced-complexity non-SLSS based synchronization as described in Section 3.4.1. When the UE goes out of GNSS coverage, but can still receive packets from another UE within GNSS coverage, then it derives its time/frequency synchronization based on DMRS of the received packet.

Case 4 corresponds to the subset of the Release 14 SLSS based procedure where the UE is able to search for SyncRef UEs only within a small (synchronous) window of +- CP. Note that +-CP is used here as an pessimistic case, and results can significantly be improved with higher window sizes for the same initial conditions as used in this contribution. E.g., with repeated PSS symbols, the window is +- half-of-the PSS symbol length.

## Simulation modelling

GNSS coverage drop procedure

The simulations are for urban drop. In the drop, we additionally model GNSS coverage areas as hotspots in the drop where we assume the GNSS coverage is available. Each GNSS hotspot is modelled as circular geographical areas with random radius of 200m and centre is dropped uniformly with the geographsical area of the drop (as depicted in Figure 1). The number of hotspots is varied as a parameter to model different densities of UEs that are in GNSS coverage.

If the UE is within the GNSS coverage area, then UE is declared to synchronized to GNSS. If UE is outside the GNSS coverage area, then the UE is assumed to not have GNSS coverage and the oscillator (XO) will be drifting as per the XO drift model. Actual mobility of the UEs is simulated, and thus the coverage state of a UE will (may) change over the simulation time such that it may be in GNSS coverage area at certain point in time and may loss the GNSS coverage and have its oscillator drifting.

Figure 1: Modelling of GNSS coverage areas within the Urban drop

UE oscillator modelling (frequency drift modelling at the UE)

When the UE is within GNSS coverage, we assume that the oscillator can be perfectly disciplined and results in zero time/frequency error for communications. This is an idealistic assumption but does not affect message of this simulation. Practically there will still be some residual time/frequency error but will be quite small and can be modelled as such as well without changing the results presented in this contribution.

When the UE looses GNSS coverage, the XO drift is modelled as:

Funcertainity=10ppb/sec×∆t

where ∆t is the time elapsed since the UE was in GNSS coverage (i.e. start of drift) and Funcertainity is the frequency uncertainty of the oscillator. The corresponding timing uncertainty is then the area under the triangle as:

Tuncertainity=0.5 ×10ppb/sec ×(∆t)2

If the UE is synchronized to another UE, the time and frequency uncertainty in the synchronization source is accounted for as an offset in the above equations, i.e. Tunc = Tunc (source) + Tunc (XO); Func = Func(source) + Func(XO). The oscillator still drifts from the time got synchronized to the UE.

At the initial time of drop, if the UE is not inside GNSS coverage, the timing and frequency uncertainty is chosen uniformly in [-3.5us 3.5us] and [-100ppb, 100ppb], respectively.

UE time / frequency error requirements for transmission

We assume the minimum requirements from R-14 on time/frequency error requirements for transmission, i.e. maximum timing error is within 391ns and the maximum frequency error is 0.1ppm.

Based on the model above, if the UE loses GNSS synchronization, the UE can still guarantee that it can meet the time and frequency error requirements for the following times, respectively:

For Funcertainity<0.1ppm, ∆t<10sec

For Tuncertainity<391ns, ∆t<8.8sec

Modelling of time/frequency error in reception

Given the timing/frequency errors at the transmitter and the receiver, we use the following model to assume if the Rx UE can receive the transmission from the Tx UE.

For timing difference, we assume the receiver assumes that the packet should arrive within [CP-, CP+] of its own reference timing (that could offset from the true timing by the timing uncertainty at the receiver), where CP- + CP+ = CP (length of cyclic prefix) that is assumed to be 2.3us in this simulation. We model ISI-free reception, such that is the transmission can be received only if there is no-ISI affect. Clearly this is a pessimistic assumption (particularly for lower MCS), nonetheless, can still help us to motivate the non-SLSS based synchronization. Under realistic assumptions, the argument / benefits of non-SLSS based synchronization become even more compelling.

For ISI-free reception, we want to have

-1×CP-≤Tuncertainity, TX+Tpropagation-Tuncertainity, RX≤CP+

Figure 2: Modelling assumption for ISI-free reception at the receiver

For frequency error between transmitter and receiver, we assume a CFO pulling range of 1kHz (that can be achieved by DMRS symbols that are 0.5ms apart (slot length with 30kHz SCS).

For the simulation results presented in this contribution, CP- is assumed to be 25% of the entire CP duration, and CP+ is assumed to be 75% of the CP duration.

## Results

In this section, we present the simulation results based on the modelling described in the previous subsection, and summarized in Appendix A.

Figure 3: System level performance comparison of different synchronization procedures

Figure 3 demonstrates the advantage of reduced-complexity synchronization procedures (i.e., Case 3: non-SLSS based synchronization mechanism, and Case 4: synchronous-SLSS based synchronization mechanism) as compared to Rel-14 based procedures (Case 1: GNSS-only and Case 2: GNSS + asynchronous SLSS).

For GNSS-only synchronization, a large fraction of the packets cannot be transmitted since the UE may lose it GNSS coverage, particularly with low number of hostpots indicating spotty GNSS coverage in an urban area.

For asynchronous-SLSS based synchronization, UEs that loose GNSS coverage, will find a SyncRef UE and get synchronized to the same (or become independent synchronization sources). The complexity is high as full asynchronous search is needed.

For the proposed non-SLSS and synchronous-SLSS based synchronization on the other hand provide huge improvement (as compared to GNSS-only) in the system outage performance at low implementation complexity at the UE. In other words, reduced-complexity synchronization procedures promises to yield most of the synchronization benefits of fully asynchronous SLSS-assisted synchronization, while at significantly lower UE complexity of implementation.

As a note, the initial conditions on used in this contribution have been made more severe as compared our prior contribution (R1-1813423) to see the performance in more pessimistic scenarios.

Observation 3: Simulation results indicate significant advantage of reduced-complexity synchronization procedures (non-SLSS based and synchronous-SLSS based synchronization) in terms of system synchronization performance (average fraction of packets dropped) as compared GNSS-only synchronization.

Observation 4: Reduced-complexity synchronization procedures promises to yield most of the synchronization benefits of fully asynchronous SLSS-assisted synchronization, while at significantly lower UE complexity of implementation.

# Conclusion

In this contribution, we propose the following aspects for NR V2X synchronization study:

(Synchronization signal design)

Proposal 1: S-SSB has a bandwidth of 12 RBs; number of symbols needed for S-SSB will be determined by the sidelink MIB payload.

Proposal 2: S-PSS is transmitted on two consecutive symbols; RAN1 studies the performance of S-SSB with the assumption that only one S-PSS sequence is specified.

Proposal 3: Reuse NR Uu SSS sequences for S-SSS; S-SSS is transmitted on two separated symbols.

(Synchronization requirements and procedure)

Proposal 4: Revaluate the time/frequency error requirements for NR V2X data/control and synchronization signal transmissions.

Proposal 5:  Introduce reduced complexity synchronization procedure for NR V2X to address the scenario wherein the UE was synchronized to GNSS/eNB/gNB/SyncRef UE at a prior time and has not drifted significantly (e.g. can still receive packets from other synchronized UEs

Proposal 6: Introduce non-SLSS based synchronization enhancements.

Proposal 7: Support UE transmitting its GNSS coverage state in the MAC header of its transmission to enable non-SLSS based synchronization.

Proposal 8: Introduce reduced complexity synchronous-SLSS based synchronization enhancements (with reduced search window for SyncRef UEs).

Observation 1: Specification impact to support reduced-complexity synchronization procedures (non-SLSS based synchronization and synchronous-SLSS based synchronization) is expected to be quite low.

Observation 2: The reduced-complexity synchronization procedures would supplement the SLSS-based synchronization and their support is expected be up to UE capability.

(Simulation results)

Observation 3: Simulation results indicate significant advantage of reduced-complexity synchronization procedures (non-SLSS based and synchronous-SLSS based synchronization) in terms of system synchronization performance (average fraction of packets dropped) as compared GNSS-only synchronization.

Observation 4: Reduced-complexity synchronization procedures promises to yield most of the synchronization benefits of fully asynchronous SLSS-assisted synchronization, while at significantly lower UE complexity of implementation.

# References

- RP-181429, “New SID: Study on NR V2X”, Vodafone

# Appendix A

Summary of simulation and modelling assumption for the results presented in this paper.

| Parameter | Value / Comment |
| --- | --- |
| Drop | Urban Drop |
| GNSS coverage drop | N GNSS hotspots are dropped uniformly in the simulation area (N is varied in the simulation) Each hotspot is 200m radius. UE is assumed to be in GNSS coverage if it lies within the GNSS hotspot, and out of GNSS-coverage otherwise. |
| XO time/frequency drift model | Funcertainity=10ppb/sec×∆t Tuncertainity=0.5 ×10ppb/sec ×(∆t)2 |
| Tx accuracy requirements | Maximum timing error ±12Ts (391ns) Maximum frequency error ±0.1ppm |
| Rx modelling to incorporate time/frequency errors | Declare decode failure if not ISI-free reception -1×CP-≤Tuncertainity, TX+Tpropagation-Tuncertainity, RX≤CP+ CP- + CP+ = 2.3us; CP- = 0.25 * 2.3us; CP+ = 0.75 * 2.3us Declare decode failure if frequency error not within CFO pull range of 1 kHz If ISI-free and CFO within pull range, decoding is attempted as normal based on the link level performance curves. |
| Synchronization mechanism | Case 1: GNSS only synchronization Case 2: GNSS + non-SLSS based synchronization |

# Appendix B

Agreements:

- NR V2X Sidelink Synchronization includes at least the following

  - Sidelink synchronization signal(s)

  - PSBCH

  - Sidelink synchronization sources and procedure(s)

    - Study potential synchronization sources –GNSS, gNB, eNB, UE, LTE UE

      - Note: this doesn’t mean all of them are to be supported

Agreements:

At least GNSS, gNB, NR UE, and eNB are supported as the synchronization source for NR V2X.

eNB as a synchronization source for NR V2X UEs supporting LTE Uu/PC5 or Uu only (no change to the eNB behaviour)

Whether a source is supported is for further NR V2X UE capability consideration

Agreements:

NR V2X sidelink operation includes the following cases:

NR V2X sidelink is synchronized with LTE V2X sidelink

NR V2X sidelink synchronization procedure operates independently to the LTE V2X sidelink synchronization procedure

Working assumption:

- For the purpose of evaluation, the initial frequency error should be within ±[5] ppm with the assumption of uniform distribution [-5, 5] for NR V2X sidelink synchronization.

  - Note: This is the error of the local oscillator for the Tx and Rx with respect to the absolute carrier frequency.

Agreements:

- The design of NR V2X sidelink synchronization signals and PSBCH uses NR SSB structure as the starting point with the following properties,

  - NR V2X synchronization signals include sidelink PSS (S-PSS) and sidelink SSS (S-SSS) and are structured with PSBCH in a block format (S-SSB)

Agreements:

- Periodic transmission of S-SSB in NR V2X  is supported

  - FFS:  whether one/more S-SSB is transmitted in a period

Agreements:

- Confirm the working assumption that initial frequency error before synchronized to any synchronization source should be within ±5 ppm for the purpose of evaluation.

Agreements:

- S-SSB has the same numerology, which includes SCS and CP length, as that of control and data channels for a given carrier

Agreements:

- The transmission bandwidth for S-SSB is within the BW of the (pre)-configured SL-BWP.

  - FFS:  The actual transmission BW for S-SSB and sync raster

Agreements:

- For evaluation of V2X S-SSB, the transmission bandwidth of S-SSB is in proportion to the SCS for the design of V2X S-SSB.

  - Alt1: 24 PRBs

  - Alt2: 20 PRBs

  - Other values are not precluded

Agreements:

- For the evaluation of S-PSS/S-SSS, the sequences and/or polynomials used in NR Uu PSS/SSS are used as the starting point of the NR V2X S-PSS/S-SSS design.

  - Others are not precluded.

Agreements:

- The aspects of synchronization sequence for NR V2X to be considered for the evaluation include,

- The length of S-PSS and S-SSS sequences

- If and how to distinguish from NR Uu PSS and SSS sequences

- The number of NR SL-SSID targeted in the design of NR V2X S-PSS/S-SSS

  - Use cases of NR SL-SSID should be addressed

Agreements:

- Using the below table as a starting point for evaluation assumptions for sidelink synchronization LLS.

  - Detection probability of S-PSS/S-SSS

  - Decoding BLER of PSBCH

  - Check further offline regarding UE speeds (absolute vs. relative, including current channel model assumptions in the TR)  on Friday, confirmed to be relative speed and thus, the speeds in the table below need to be doubled

  - Discuss further offline payload size of PSBCH  to revisit in the next RAN1 meeting. Companies to report the assumed payload size of PSBCH in their evaluations

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 6 GHz | 30 GHz |
| Channel Model | CDL channel models |  |
| Subcarrier Spacing(s) | 15, 30, 60 kHz | 60, 120 kHz |
| SNR Range | > -6 dB | > -6 dB |
| UE Speed | 3 km/h, 120 km/h (mandatory) 30km/h, 250 km/h (optional) | 3 km/hr, 120 km/h (mandatory) |
| Interference model | Scenario 1: no interference Scenario 2: effect of interference includes in the model | Scenario 1: no interference |
| Initial Frequency Offset | TX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency RX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency |  |

Agreements:

- The study of NR V2X synchronization includes synchronization based on S-SSB

- The study also includes use of other sidelink signals/channels (e.g., other RSs in the SL, using PSSCH, using PSCCH, etc.) for the sidelink synchronization