3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900894

Taipei, 21st – 25th January 2019

Agenda item:	7.2.5.3 RIM RS resource and configurations

Source: 	Qualcomm Incorporated

Title: 	RIM RS design and configurations

Document for:	Discussion and Decision

# Introduction

In RAN-Plenary 82, CLI and RIM WI was approved with following scope on RIM. In this paper, we will discuss the detail design of RIM RS according to the scope.

The detailed objectives for remote-interference management are:

- Specify RIM RS resource and configurations, including [RAN1]

  - A basic RIM-RS resource

  - Configuration of RIM-RS and distinguishable RIM RS-1/2 resources, including sequence type, time and frequency transmission pattern

  - Determine gNB set identification information through detection of RIM-RS(s) by implicit or explicit indication. Determine further information that can be carried by the RIM-RS, such as “Ducting phenomenon exists”, “Enough mitigation” & “Not enough mitigation”,   [RAN1, SA5]

- Specify the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared [RAN3]

- Identify corresponding OAM functions to support RIM operation [RAN1, RAN3].

# Basic RIM RS

During the study item, RAN1 decided to reuse the Gold sequence defined in TS 38.211 for RIM-RS to avoid introducing new sequence.

On the RS structures, the alt 2 in [2] is recommended to be specified in WI phase as the basic RIM-RS.

-	Alt 2: 2-symbol RS, where two copies of the RS sequence are concatenated and one CP is attached at the beginning the concatenated sequences;

The RIM RS (also include RS-1 and RS-2 in FW-1) is expected to be differentiated by frequency, time and sequence.

There are two proposals on the total available RIM RS:

- The RS pool equals to the whole gNBs which means one unique ID per gNB

- Reuse the RS in a cluster manner and cluster size is derived by the remote interference propagation

We prefer the 2nd option as it provides more flexibility for the network.

On the frequency domain, we propose to specify a RIM RS with a fixed bandwidth and the Tx side follows a RIM raster manner. For example, the RIM RS is a 5MHz signal and the transmission always starts from a multiple of 5MHz from the frequency boundary. In this sense, the detector only performs blind detection starting from certain frequency position and the detection complexity will be reduced.

On the bandwidth of RIM RS, we propose to use the minimum value (e.g. 5MHz) among candidates as more RS is allowed for certain bandwidth. During offline discussion, infra vendors also mentioned the detection of TD-LTE RIM RS will cost tens of minutes as the RS periodicity is quite long. With more available RS in frequency domain, the time domain RS periodicity will be reduced, and the smaller detection time is expected at the gNB side.

On sequence side, the simulation results reveal too many sequences will lead to degradation of detection performance. We will prefer to keep the available sequence to less than 10, e.g. 8 which is similar as TD-LTE.

Proposal 1: Reuse the RS in a cluster manner and cluster size is derived by the remote interference propagation

Proposal 2:  specify basic RIM RS with a fixed bandwidth and the transmission always starts from a multiple of the basic bandwidth.

Proposal 3: RIM RS, we propose to use the minimum value (e.g. 5MHz) among candidates as more RS is allowed for certain bandwidth.

Proposal 4: sequence for RIM RS use the available sequence to less than 10, e.g. 8 which is similar as TD-LTE.

# RIM RS resource configuration

To support coverage enhancement and mis-detection of short UL, multiple configuration is supported including contiguous and discrete repetition. To further reduce the detection complexity, the group agreed that the multiple configurations share the same sequence and frequency and the only difference is the time position. For both cases, the gNB should follow the network configuration to perform multiple copies of the RIM-RS as the gNB may not have the whole picture of the network status.

Figure 1 multiple (two) configurations of RIM-RS

To compensate the short UL configuration, the transmitter needs to transmit multiple (e.g. 2) discrete RS to make sure the receiver at least can receive one RS. On the receiver side, the detector is better be aware the reference time when the RS is sent as it will be used to estimate the number of backup symbols. As the multiple use the same frequency and sequence, using different sequence or frequency to implicitly carry the reference time is not implementable. Unless explicitly carry the reference time as the payload, there is no way to notify the detector the reference time. In this case, the receiver has to use network implementation way to estimate the propagation for Tx back off – e.g. 1st assume the received RS is transmitted at the DL boundary, if the interreference remains, plus the gap between two RS configurations which is notified by network.

Proposal 5: except explicitly sending the RS order, the receiver have to use network implementation way to estimate the propagation for Tx back off.

Observed by CMCC, the ducting phenomenon will last for hours. The RS is expected to carry some information more than just to aid the aggressor to calculate the back off symbols.  In the scope the following information is suggested to be carried by RS

- Ducting phenomenon exists, Enough mitigation, Not enough mitigation

Carrying extra payload for the two messages (enough or not sufficient mitigation) is not cost efficient. We prefer to use RS to differentiate them in terms of sequences, frequencies, or time. Take frequencies as an example, the information “ducting phenomenon exists and enough mitigation” could be transmitted in a cell agnostic manner, while cell ID is more preferable for “Not enough mitigation”.

Proposal 6: the information “Ducting phenomenon exists, Enough mitigation, Not enough mitigation” can be carried by frequency, sequence, or time without extra payload.

# Set ID configuration

During the study phase, the gNB set is identified to be useful to reduce the RS ID size and improve the detection probability.

According to the RAN3 consensus, either or both of victim and aggressor gNB would be configured as the gNB set.  Each set is assigned a set ID, and is configured with a RIM Reference Signal (RIM-RS) and the radio resources to send and receive the RIM-RS. The grouping of gNBs into sets, the set ID, the RIM-RS configuration and the associated RIM-RS radio resources for sending and/or receiving the RIM-RS are performed by the OAM system, and these resources may be assigned in static or a non-static manner.

If the gNB has been pre-configured a set ID, the OAM can further re-configure the set ID.

The set size is not discussed during the SI phase due to lack of time. Our understanding is the gNBs in a set should be relatively small – e.g. less than 10. One reason for this is if a set have more gNBs, propagation between gNBs in one set might be larger than a symbol at 30kHz SCS and the detector may have some difficulty to converge the receive signals.

A gNB set ID may be reused inside a PLMN and the OAM should have capability to differentiate the gNBs in different geometries to use the same set ID.

Proposal 7: Set ID should be relatively small – e.g. less than 10 to avoid large propagation in one set.

# Conclusions

In this contribution, we have made the following observations:

Proposal 1: Reuse the RS in a cluster manner and cluster size is derived by the remote interference propagation

Proposal 2:  specify basic RIM RS with a fixed bandwidth and the transmission always starts from a multiple of the basic bandwidth.

Proposal 3: RIM RS, we propose to use the minimum value (e.g. 5MHz) among candidates as more RS is allowed for certain bandwidth.

Proposal 4: sequence for RIM RS use the available sequence to less than 10, e.g. 8 which is similar as TD-LTE.

Proposal 5: except explicitly sending the RS order, the receiver have to use network implementation way to estimate the propagation for Tx back off.

Proposal 6: the information “Ducting phenomenon exists, Enough mitigation, Not enough mitigation” can be carried by frequency, sequence, or time without extra payload.

Proposal 7: Set ID should be relatively small – e.g. less than 10 to avoid large propagation in one set.

# Reference

- RP-182864, Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, RAN Plenary #82, Dec. 2018

- TR 38.866, Study on remote interference management for NR, version 16.0.0, Dec. 2018