3GPP TSG-RAN WG1 Ad-Hoc Meeting 1901	R1-1900906

Taipei, Taiwan, January 21st – 25th, 2019

Agenda item:	7.2.8.3

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Multi-beam Operation

Document for:	Discussion/Decision

# Introduction

The work item on Rel-16 MIMO enhancements has been specified [1].

MIMO WI from RP-182067

The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows. - Extend specification support in the following areas [RAN1] - (A) Enhancements on multi-beam operation, primarily targeting FR2 operation: - (B) Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - (C) Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - (D) Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR

Agreement In Rel-16, an identifier (ID) that can be used at least for indicating panel-specific UL transmission is supported, where detailed usages for the panel-specific UL transmission are FFS - The ID should be defined considering the possibility to reuse/modification of Rel-15 specification support or introducing new ID - Note: RAN1 to avoid unnecessary specification support requiring UE to explicitly disclose its UL antenna panel implementation - FFS: Whether UE capability signalling is introduced for panel-specific UL transmission Agreement RAN1 has identified the following scenarios to be important for SCell BFR - Scenario 1: SCell with both uplink and downlink - Scenario 2: SCell with downlink only - PCell can be in FR1 or FR2 for scenarios above Agreement - Support L1-SINR measured from - For signal part, SSB and/or NZP CSI-RS - FFS: For interference part - Companies are encouraged to provide simulation results on how to measure/define L1-SINR, e.g. whether interference is measured from dedicated IMR - For example, take Rel-15 L1-RSRP and/or SINR specified in 38.215 as a comparative reference for evaluation purposes Agreement: Decide (agree on) either one of the followings in RAN1 NR-AH 1901: - Alt.1: Support sub-time unit for beam management RS shorter than 1 OFDM symbol in a reference numerology. - No new RS for beam management is introduced in Rel-16. - FFS: details including IFDMA-based, DFT-based, larger subcarrier spacing based, etc, or limited to only for P-3. - Alt.2: No support of sub-time unit for beam management RS shorter than 1 OFDM symbol. Agreement For interference part, down-select at least one from the following alternative: - Alt 1: Dedicated resource(s) for interference measurement - FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214 - FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both - FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement - Alt 2: The same reference signal as signal part as specified in 38.215 - Alt 3: Alt1 when SSB is used for signal part, Alt2 when CSI-RS is used for signal part - Companies are encouraged to provide simulation results for down-selection #RAN1 94: Companies are encouraged to evaluate and study further mechanism(s) to support multiple UL Tx panel/beam indication for PUSCH - FFS on how to support the indication, e.g., by multiple SRI fields, an extension of the existing SRI field, an indication of selected panel(s), single SRS resource transmitted associated with multiple panels etc. - FFS on relation to TPMI/TRI fields, PC/TA mechanisms - Companies to consider specification implications of multiple UL Tx panel/beam support Agreement - L1-SINR is supported. L1-RSRQ is not supported. - Companies to study and provide definition of L1-SINR - Study the reporting content, e.g. - Whether CRI/SSBRI is reported - Whether differential group/non-group reporting is applied - Whether L1-RSRP is reported - Study the interference measurement mechanism

The WI for Rel-16 covers several key features aimed at enhancing multibeam operation which we have categorized into features (A), (B), (C) and (D) as shown above. In this contribution we discuss some potential enhancements for beam management for Rel-16. Section 2 discusses feature (D) on beam failure recovery for SCell. Section 3 discusses feature (C) covering UL Tx panel indication. Sections 5, 7, 8 and 9 focus on feature (B), and Section 6 discusses feature (A) focussing on robustness improvement for multi-beam operation.

# SCell Recovery

In this section we discuss feature (D) of the approved work item, for specifying SCell BFR procedure. For intra-band CA operation where PCell and SCell may share the same QCL, beam failure recovery for SCell can be derived from the PCell BFR in Rel-15. For FR1+FR2 operation where for example PCell is FR1 and SCell is FR2, it may not be possible to determine the quality of SCell link from the PCell beams. This serves as a good use case for supporting and specifying SCell beam failure recovery. In last meeting, two scenarios have been identified as important for SCell BFR, Scenario 1 ( SCell with both UL and DL) and Scenario 2 (SCell with DL only).

SCell beam failure detection and recovery could be derived from existing Rel-15 and performed in implementation specific manners for inter-band CA operations. SCell beam failure detection (BFD) could be based either on the absence of ACK/NACK feedback for the scheduled DL transmission in SCell or depending on CQI report in SCell. These, however, incur latency and overhead. Once beam failure is detected, BFR could be attempted directly on SCell recovery resources as in Rel-15.

As a baseline option, a standalone SCell-based beam failure recovery may be defined in Rel-16, like the BFR procedure in Rel-15 for SpCell. This does not consider the advantage of having an active connection to the gNB and control channel via the PCell. Additionally, recovery resources need to be explicitly configured for the SCell and results in latencies incurred in typical beam recovery procedure. The overheads involved include dedicated RACH resources for SCell BFR, identification of candidate beam by the UE and RACH procedure-based recovery attempts on SCell without any PCell assistance.

We prefer to use PCell resources to indicate SCell beam failure to the gNB, thereby reducing overhead and efficiently utilizing channel resources. Natural choices of UL resources are SR (PUCCH)/PUSCH/RACH. PUSCH or MAC-CE requires an UL grant, which may not be available all the time. RACH is probably expensive resource given that PCell is operational. Hence our preference is to use SR (PUCCH) which has very low resource utilization.

The proposal includes first detecting SCell failure by monitoring the configured (virtual) CORESETs in the SCell. Once the beam failure is detected, the UE sends uplink signal such a usual SR or a dedicated SR to the gNB in the PCell. In response, the gNB may request for beam report in the PCell. Once the UE sends the corresponding L1 report regarding the SCell beams in the PCell, gNB activates/pairs the (new) CORESETs. The high-level call flow is summarized in Figure 1.

Detection of SCell Failure

R16 SCell failure report via PCell

SCell beam activation/pairing by gNB

Figure 1: Rel-16 SCell Beam Failure Recovery

For SCell failure report via PCell, two key proposals for Rel-16 are

- Dedicated SR in PCell for SCell failure indication, in response to which gNB may request L1 report and UE reports the preferred candidate beam of SCell

- Reusing existing SR in PCell for SCell failure indication. This requires defining additional triggering conditions for transmitting SR and new MAC-CE definition along with PUSCH that contains SCell recovery information.

Proposal  UE signalling of SCell beam failure can be based on dedicated SR in PCell or reusing an existing PCell SR mechanism to additionally report SCell beam failure.

Further procedure for SCell beam activation/pairing depends on whether beam correspondence is assumed or not.

When beam correspondence is available

- Based on the L1 report/MAC-CE information from the UE about the SCell, the gNB performs SCell CORESET TCI state activation via the PCell.

Figure 2 shows the proposed solution for Rel-16 that leverages PCell connection for performing SCell recovery under beam correspondence.

SCell

Beam Failure Detection

PCell activates SCell CORESET TCI states

DL/UL

Recovered

SCell

RS

gNB

UE

Sending Recovery to PCell via Dedicated SR

PCell Requests

Beam Index report

UE Sends

L1 Report

DL/UL

Recovered

Figure 2: PCell-aided SCell Recovery for Rel-16 under beam correspondence

In the absence of beam correspondence, we need RACH procedure to enable UL beam pairing. Leveraging the active PCell connection, the proposal is as follows:

- PCell triggers dynamic RACH resources associated with SSB index derived from UE SCell recovery report.

A high-level solution/proposal is shown in Figure 3.

SCell

Beam Failure Detection

PCell Triggers dynamic RACH for SCell Recovery

Beam Pairing

DL/UL

Recovered

SCell

RS

gNB

UE

Sending Recovery to PCell via Dedicated SR

PCell Requests

Beam Index report

UE Sends

L1 Report

DL/UL

Recovered

Figure 3: SCell Recovery with dynamic RACH for Rel-16 in the absence of beam correspondence

# Latency and Overhead comparison

For different parameter combinations, the following table looks at the latency numbers for supporting the baseline, standalone SCell-based beam failure recovery and the PCell-aided SCell recovery proposal.

Table 1: Latency comparison between baseline scheme and the proposed SCell recovery mechanism

| Parameter Combinations (Value in Slots for 120Khz numerology) |  |  |  |  |  |  |  |  |  |  | Average Latency (#slots) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # RS occasions for averaging | RS periodicity | SR/PUCCH periodicity | SR to PDCCH triggering L1 Report | RACH occasion periodicity | Dynamic RACH periodicity | # RACH ReTx | RAR Delay | Delay to Activate dynamic RACH | MAC-CE Delay after CORESET activation | Baseline option |  | PCell aided SCell recovery |
| 1 | 16 | 1 | 1 | 80 | 40 | 1 | 8 | 8 | 24 | 80 |  | 70 |
| 1 | 16 | 1 | 1 | 80 | 80 | 1 | 8 | 8 | 24 | 80 |  | 90 |
| 2 | 40 | 40 | 1 | 40 | 40 | 4 | 16 | 16 | 24 | 240 |  | 258 |
| 1 | 160 | 80 | 1 | 160 | 80 | 8 | 80 | 8 | 24 | 1384 |  | 794 |
| 2 | 80 | 160 | 1 | 320 | 160 | 2 | 8 | 8 | 24 | 632 |  | 402 |
| 4 | 8 | 1 | 1 | 80 | 40 | 4 | 4 | 8 | 24 | 336 |  | 206 |
| 2 | 160 | 640 | 1 | 40 | 40 | 1 | 2 | 8 | 24 | 366 |  | 376 |

Observation 1: Considerable latency benefits when the dynamic RACH periodicity is smaller than regular RACH occasion periodicity.

Even when dynamic RACH periodicity is assumed to be same as the regular RACH periodicity, the increase in latency is marginal using the proposed PCell-aided SCell recovery.

The amount of channel resources needed for the baseline scheme and the proposed PCell-aided SCell recovery is shown below:

Table 2: Resource overhead between baseline scheme and the proposed SCell recovery mechanism

| Averaging duration (ms) | RS periodicity (#slots) | SCell failure probability | #RACH resources per 5ms | # SR resources | Average resource overhead per 5 ms |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Baseline option | PCell aided SCell recovery |
| 100 | 16 | 1% | 8 | 1 | 8 | 0.1 |
| 1000 | 16 | 2% | 16 | 1 | 16 | 2 |
| 100 | 8 | 2% | 8 | 1 | 8 | 0.4 |
| 100 | 8 | 5% | 16 | 1 | 16 | 1 |
| 1000 | 16 | 5% | 16 | 1 | 16 | 5 |

Observation 2: Significant reduction in overhead and efficient utilization of resources can be obtained with the proposed PCell aided SCell recovery since the RACH resources are activated only after SCell BFD is indicated to the gNB via the PCell.

When existing SR resource is utilized for indicating SCell beam failure on the PCell, there is no extra resource overhead. If dedicated SR is used for this purpose then additional overhead is required for the proposed scheme, which however, is marginal compared to the gain from having dynamic RACH resources.

A summary comparison between extending the existing Rel-15 BFR for SCell recovery in Rel-16 v/s using the proposed procedure is shown below.

|  | Duplicating PCell Recovery in SCell | SCell Recovery with PCell assistance |
| --- | --- | --- |
| Spec impact | Minimal change | Change to enable dynamic RACH resources |
| Resources | Dedicated RACH resource needed per candidate beam per UE | Lower resource usage due to dynamic RACH |
| Latency | High; function of RACH periodicity | Lower, since the RACH periodicity could be smaller due to the on-demand nature |

Based on the latency and overhead benefits we have the following proposal.

Proposal  Support PCell triggered dynamic RACH resources on SCell for SCell beam failure recovery.

The FR1+FR2 proposals mostly extend to FR2+FR2 case as well. Specifically, for robustness, the dedicated SR indicating SCell failure to PCell (in FR2) may be sent via multiple beams. Further, the recovery signal may be linked to SRs in multiple PUCCH resource IDs.

SCell recovery should also support varied scenarios such as when there is no PDCCH in the SCell and is cross-carrier scheduled from another Carrier. Additionally, further study is needed for handling the case of multiple SCells in a resource efficient manner

Proposal  Study SCell Recovery for cross-carrier scheduled carriers. Study resource efficient recovery mechanism for supporting SCell Recovery for multiple SCells.

# Multiple UL Tx-Panel Transmission & UL Beam Selection

In this section we discuss feature (C) of the work item. Multiple UL Tx-Panel indication was also discussed in the last meeting. A key motivation for conveying panel information is for improved robustness. In R15 it is not explicit if the reference signals are received in the same panel or different panels. Different panels based on panel location direction, and separation may provide robustness to blocking effects, such as finger blocking or human body blocking. Grouping of RS based on panels, can provide better information to the network to identify robust beams for transmission and reception. Another aspect is power saving and latency. UE may have only one panel active at a time, to save power.  Panel switching may lead to increased latency, and hence for instance switching from PDSCH beam received on one panel, to a PDSCH beam that can be received on a different panel may require additional time to activate the new panel. Panel information can enable efficient power and resource utilization. Simultaneous PUSCH transmission/selection across multiple panels can also improve robustness of PUSCH transmission.

In current R15 for non-codebook-based transmission gNB can signal UE to transmit across SRS resources associated with different DL reference signals. If the reference signals were received from different panels, then UE can transmit using multi-panels. However, for the code-book based transmission there is only limited support for multi-panel transmission, as the SRI indicates only one SRS resource, that has only one spatial QCL source.

Below, we highlight some further issues related to enabling both codebook and non-codebook based multi-panel PUSCH transmission.

- Antenna port mapping:   In R15, there is no clear distinction between antenna ports mapped to a single panel vs antenna ports mapped to multiple panels.  To enable transmission of multiple antenna ports out of multiple panels, a distinction in mapping can be signalled. One approach is to associate a panel index with an SRS resource set.

- spatialRelationInfo:  SRS-Resource currently specifies only one reference signal in spatialRelationInfo.  Different panels in the UE maybe QCLed with different DL RSs, depending on the panel location and channel conditions.  For example, transmissions from UE panels 1 & 2 based on panel location in the device, may be QCLed with SSB-Index i and SSB-Index j, respectively. Multiple spatialRelationInfo is needed to enable transmission over multi-panel transmissions, which is not possible for Codebook based PUSCH transmission. A natural way is to map a SRS resource set to a panel index, and UE can transmit using SRS resources from two separate SRS resource sets, wherein each SRS resource in the set is mapped to a different panel.

- Tx power control/MCS/TA:  Different UE panels based on panel location, and channel conditions may have different pathloss/delay to the gNb. Power control enhancements and separate MCS for each link may be necessary to efficiently utilize asymmetric links.

Based on our observations above, we make the following proposals:

Proposal  NR shall enable mechanism to associate each SRS resource set with a panel for both ‘codebook’ and ‘noncodebook’ based PUSCH transmission.

Proposal  SRI field in the DCI can be expanded to select multiple SRS resources belonging to multiple SRS resource sets, where each set is associated with a panel.  SRI to SRS resource table mapping shall be extended to include SRS resources across SRS resource sets.

Proposal  Enhancements to power control shall be specified for multi-panel to support asymmetric links.

In addition, we make the following proposal in regard to the UE assisted UL beam selection.  The motivations for UE aided UL beam selection are:

- Based on UE architecture and power consumption considerations, certain UL beam may be preferable for UE.

- To meet the maximum possible emission (MPE) requirement and still use the strongest beam for DL, the UE may prefer a different UL beam (not necessarily associated with same DL beam) to steer transmission away from human body.

Proposal  UL Beam Selection: SRI field in the DCI can be used to indicate multiple SRS resources associated with different SRS resource sets for UL transmission, and the UE may select a subset of the indicated SRS resources for UL transmission.

This allows the UE to select a preferential UL beam among many feasible beams indicated by gNb.

# L1-SINR

Beam management can be improved by utilizing L1-SINR compared to just L1-RSRP. In the last meeting there has been agreement on the RS signals that can be used for signal measurement. The determination of resources for interference measurement is under discussion, and the following 3 alternatives have been specified.

Agreement

For interference part, down-select at least one from the following alternative:

- Alt 1: Dedicated resource(s) for interference measurement

  - FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214

  - FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both

  - FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement

- Alt 2: The same reference signal as signal part as specified in 38.215

- Alt 3: Alt1 when SSB is used for signal part, Alt2 when CSI-RS is used for signal part

- Companies are encouraged to provide simulation results for down-selection

Alt-2 is clearly a drawback for the case of SSB based SINR measurement. Interference measurement from SSB is not an accurate representation of the actual interference the UE will observe during data/control transmission. For instance, the interference measured in SSB will be reflective of the interference due to colliding SSBs from other cells, which may or may not be present when a UE is scheduled data transmission. Based on the above reasons Alt 2 is not preferred.

For Alt-3 for the case of SSB based SINR dedicated resources will be configured, which will avoid the mismatch in SINR calculation based on SSB alone. However for CSI-RS the interference is also measured on the same CSI-RS resource in Alt-3. This implies that the interfering gNBs will have to model interference during the case when serving gNB is transmitting CSI-RS. This can reduce scheduling flexibility at the gNBs.

Dedicated IMR resources as in Alt-1 can provide improved scheduling flexibility. Moreover Alt-3 can be treated as a special case of Alt-1, and gNB can choose to reuse NZP CSI-RS for IMR measurement to reduce resource overhead. Based on the above discussions we have the following proposal.

Proposal  For interference measurement for L1-SINR support Alt-1: Dedicated resource(s) for interference measurement. Additionally,

- UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214

- Resource(s) for interference measurement can be NZP based or ZP based or both

- NZP CSI-RS resource(s) configured for channel measurement can be reused for interference measurement.

# Considerations on Sub-time Unit for Beam Management RS

In the last meeting support of sub-time unit for beam management RS was discussed and the following agreement was made

Agreement:

Decide (agree on) either one of the followings in RAN1 NR-AH 1901:

- Alt.1: Support sub-time unit for beam management RS shorter than 1 OFDM symbol in a reference numerology.

  - No new RS for beam management is introduced in Rel-16.

  - FFS: details including IFDMA-based, DFT-based, larger subcarrier spacing based, etc, or limited to only for P-3.

- Alt.2: No support of sub-time unit for beam management RS shorter than 1 OFDM symbol.

The benefit of Alt.1 is reduced latency. However, Alt 1 comes with a few disadvantages. Alt -1 requires UE to switch beam multiple times within a slot. Need rationale for this.

Ideally, the waveform of Alt. 1 may accommodate multiple beam sweeps in an OFDM symbol and reduce latency. Given the restriction of UE beam switching capability, e.g. the minimum time gap between consecutive beam switches, however, the gain in latency reduction may not be at least fully achieved in practice. Moreover, besides an increasing complexity in RF design, Alt. 1 also comes with certain disadvantages that may result in a degradation of measurement accuracy, especially in low SINR due to partitioning of resources for  . For the larger subcarrier spacing based waveform, inter-symbol interference due to channel delay and beam switching time delay may add a non-negligible error floor in channel measurement, due to the shorter CP length. Additionally at low SINRs, there will be a degradation in measurement accuracy using Alt. 1.

Another issue with Alt-2 is any FDM transmission of another signal with IFDMA CSI-RS, will lead to destroying the repetition in time property of the IFDMA signal. This will significantly impact SINR measurement accuracy using sub-time unit beam management. Avoiding FDM transmission during IFDMA causes significant scheduling restriction at the gNB and will increase overhead.  Based on the above reasons, we propose to agree on Alt. 2 in Rel. 16.

Proposal   NR shall adopt Alt2: No support for sub-time unit for beam management RS shorter than 1 OFDM symbol.

# Signalling Reduction Considerations

Signalling overhead reduction is one of the aspects under study in R16. R15 beam management offers considerable flexibility but at the expense of higher signalling overhead. In cases where the UE has limited compatibility such as single beam operation, the UE capability information can be utilized to minimize signalling overhead. Another aspect is DL and UL beam management can be unified. In cases where for example DL CORESET beam and UL PUCCH beam needs to be switched, by current R15 beam management, a separate MAC-CE must be provided for each beam switch. If the UL beam switching and DL beam switching, are linked together a single MAC-CE command can be used to switch both the UL and DL beam. Multiple PUCCH resources can be linked to a single CORESET, thereby a MAC-CE command to switch the CORESET will also switch the spatial relation info of the PUCCH resources linked to the CORESET, providing overhead reduction benefits. Based on the above discussion we have the following proposals.

Proposal  For deployments comprising a common beam for both DL and UL, a link can be configured between CORESETS and a set of PUCCH resources, such that TCI update of a CORESET beam, also updates the spatial relation info of the PUCCH resources linked to the CORESET.

# Enhancements to multi-beam operation for robustness

In this section we consider feature (A) of the approved WI looking at enhancements to multi-beam operation. Signals in NR systems may suffer from a sudden dramatic RSRP loss due to blockage effects, e.g. from a passing-by human body or a moving vehicle near a UE device. One effective approach to overcome such blockage effects, is to leverage macro-diversity, by receiving from multiple beams of different angular directions. In this section, we discuss enhancements to multi-beam receptions of control and data channels.

## Control Channel Robustness

In Figure 4, we illustrate the diversity gain in PDCCH reception from multiple beams using CDL-B channel model and Blockage Model A defined in 3GPP TR 38.901. The simulation assumptions are described in the Appendix. Numerical results indicate that in the presence of blockage effects, it is essential to exploit angular diversity from beams in different directions, which significantly outperforms using a higher aggregation level on a single beam in the PER curves. In the simulation, the multiple beams for a PDCCH are assumed to come from a single TRP. Of course, in the case where the channel from a single TRP does not offer sufficient angular diversity, PDCCH reception from multiple TRPs becomes a natural choice of solution. It, however, can be made transparent to UE whether the multiple beams are from a single or multiple TRPs. Therefore, we have the following proposal.

Proposal   For enhanced reliability and robustness, support single DCI transmission over multiple TCI states

Figure 4: Comparison of PER performance between single beam and multi-beam PDCCH

In general, soft-combining of the received PDCCHs from multiple beam is not required to achieve the angular diversity gain. For instance, as a more direct method, gNB can simply send multiple PDCCHs scheduling the same PDSCH assignment without informing UE the existence of the repetition; at UE side, it suffices to decode at least one PDCCH. Compared with the method that requires soft-combining, the direct method saves the overhead to convey the PDCCH repetition to UE.

Improving DL control channel robustness/reliability alone is not sufficient, since for closed loop communication, both the UL as well as the DL control channel should be reliable. Hence enhancements to UL control channel for robustness/reliability should be specified for Rel-16. Similar to PDCCH transmission via multiple beams, PUCCH can also be repeated across multiple beams for improved UL reliability. Another UL power efficient approach is to allow PUCCH resource selection at the UE. The UE can be configured with multiple PUCCH resources, each associated with different TCI states and the UE may select one or more of the resources to transmit PUCCH. The resource selection can be up to UE implementation.

Proposal  Study and specify PUCCH repetition/ selection across multiple beams for enhanced reliability and robustness

## Data Channel Robustness

Rel-16 should also study enhancements to date channel robustness, especially when retransmission is not possible, e.g. Message 2, and when the maximum number of the retransmissions becomes limited, e.g. in a URLLC use case. Like the control channel case, data channel can leverage diversity from multiple TCI beams to improve robustness to both fading and blockage effects. For example, in Rel-15, slot aggregation has been supported in data channel to improve coverage, where all the aggregated data slots are sent on the same TCI beam. In Figure 5, we compare the BLER performance between using a single beam and 2 beams to transmit 2 aggregated slots. The simulation setup is provided in the Appendix. The results show an approximate 2 dB gain at a targeting BLER of 10^-2 when using 2 beams. In Figure 5, we assume no blockage effects for simplicity. Therefore, the gain of using multiple beams solely comes from a lack of beam diversity in a single beamformed channel. A larger performance gain, however, is expected in the presence of blockages; as multiple beams can provide additional diversity in the angular domain.

Proposal  Study and specify multi-TCI transmission for data channels.

Figure 5: Comparison of BLER performance between single beam and multi-beam PDSCH

# Fast Beam Selection via L1- Event Trigger

In this section we consider feature (B) of the approved WI looking at beam selection enhancements for lower latency and overhead reduction. One approach for beam selection in R15 baseline is enabling periodic beam report. Periodic beam report can increase overhead, and power consumption of the UE.  To reduce overhead and power consumption, UE triggered beam report based on L1-Event can be supported. In this way, UE reports for instance the top N beams only, when the current set of top N beams change. L1 events can be specified similar L3 events, and triggering conditions and thresholds can be configured.  A low overhead UL signal such as an SR can be sent in response to an L1-Event trigger.

Another example of L1-Event trigger is partial BFR. To maintain beams proactively and to support L1-event based beam switching ahead of full beam failure, Rel-15 implementation may be leveraged to switch away from failing beams. UE triggered L1-reporting for switching beams and proactive beam failure monitoring can be specified, thereby reducing latency.

An example of L1-event trigger applied to enhancing BFR procedure is shown below. The main motivation in this proposal is that instead of waiting till all the beams in the failure detection set fall below a threshold, we look at early indication to the gNB based on L1 events in the monitored beams. This can enable the gNB to proactively switch to better TCI states ahead of a full beam failure. Additionally, this can be event triggered from the UE side rather than from the gNB side which might ensue higher latency. Using L1-event triggered beam switching, the issue of limited link utilization during the time-consuming Rel-15 BFR procedure can be mitigated, as illustrated in Figure 6.

Beams 1 failed

RS

RS

RS

RS

Both beams failed

RS

RS

Set q1

RS

RS

RS

RS

Beam Failure detected

(BFI maxCount = 2)

Delay to indicate reports to higher layer

RACH

PDCCH

RACH for Recovery

CORESET activation latency

Set q0

RS

RS

RS

RS

Send UE triggered L1 report to switch beam

Proactively avoid the scenario where all beams fail

R-15 flow

R-16 Enhancement

Figure 6: L1-event triggered fast beam selection for enhancing BFR procedure

We studied the impact of blockages on the top four beams over time to see if L1 event triggered reporting arises in practical using Blockage Model A in TS38.901, with blocker elevation width of 20 degrees. We considered UMi scenario with spatial correlation distance dcorr = 10 m for the four non-self-blockers. Figures 7 and 8, show the non-uniform variations in the indices of the top four beams over the simulation interval due to blockers. The non-uniform variations can be exploited to reduce overhead/power consumption by triggering L1 report only when needed. The latency can also be improved at low overhead, by configuring an smaller periodicity SR to indicate the L1-Event trigger to the gNB.

Proposal  Study and specify L1 event trigger-based report for fast beam selection.

Figure 7: Index of the top 4 beams

Figure 8: SNR of the initial best four beams

Figure 9Blocker centres’ evolution over time

# Enhancements to Beam Selection in CDRX

In this section we consider feature (B) of the approved WI looking at beam selection enhancements for lower latency and overhead reduction. In CDRX operation fast beam selection is crucial for power efficient operation in FR2. An example R-15 beam selection during the CDRX ON duration is shown in the figure below. In the example operation gNB configures two CORESETS in each slot to monitor PDCCH. Each CORESET maybe associated with a different TCI state for robustness.

When the gNB has some data to transmit in the ON duration, it may trigger AP CSI-RS via DCI. The UE may report its preferred beam, and further communication can continue based on the indicated report.  The latency for beam selection via this procedure for a few CSF combinations is provided below.

Table 4: Processing time in OFDM symbols (120Khz Numerology) for an example UE implementation

|  | RSRP (CRI/SSBIdx) | Wideband CRI-RI-CQI | Sub-band CRI-RI-CQI |
| --- | --- | --- | --- |
| TDCI-CSIRS:Time from DCI to AP-CSI-RS Tx | 28 | 28 | 28 |
| Num CSIRS Symbols | 2 | 2 | 2 |
| TCSIRS-UL: Time from end of AP-CSI-RS to First UL symbol CSI-Report | 28 | 36 | 140 |
| Total Latency of AP CSI-RS Based | 58 | 66 | 170 |

A component for the latency is the delay between the DCI trigger and AP-CSI-RS transmission. From a robustness aspect the DCI triggering the AP-CSI-RS might not be received due to potential blocking and may have to be retransmitted via a different beam. Hence schemes have to be studied and specified that have lower latency, at the same time good robustness for beam selection during CDRX operation.

Proposal  Mechanisms to reduce latency in beam selection for CDRX operation shall be studied and specified.

# Conclusion

In this contribution we discussed enhancements for Rel-16 NR MIMO beam management. Below are the observations and proposals

Observation 1: Considerable latency benefits when the dynamic RACH periodicity is smaller than regular RACH occasion periodicity.

Observation 2: Significant reduction in overhead and efficient utilization of resources can be obtained with the proposed PCell aided SCell recovery since the RACH resources are activated only after SCell BFD is indicated to the gNB via the PCell.

Proposal 1: UE signalling of SCell beam failure can be based on dedicated SR in PCell or reusing an existing PCell SR mechanism to additionally report SCell beam failure.

Proposal 2: Support PCell triggered dynamic RACH resources on SCell for SCell beam failure recovery.

Proposal 3: Study SCell Recovery for cross-carrier scheduled carriers. Study resource efficient recovery mechanism for supporting SCell Recovery for multiple SCells.

Proposal 4: NR shall enable mechanism to associate each SRS resource set with a panel for both ‘codebook’ and ‘noncodebook’ based PUSCH transmission.

Proposal 5: SRI field in the DCI can be expanded to select multiple SRS resources belonging to multiple SRS resource sets, where each set is associated with a panel.  SRI to SRS resource table mapping shall be extended to include SRS resources across SRS resource sets.

Proposal 6: Enhancements to power control shall be specified for multi-panel to support asymmetric links.

Proposal 7: UL Beam Selection: SRI field in the DCI can be used to indicate multiple SRS resources associated with different SRS resource sets for UL transmission, and the UE may select a subset of the indicated SRS resources for UL transmission.

Proposal 8: For interference measurement for L1-SINR support Alt-1: Dedicated resource(s) for interference measurement. Additionally,

- UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214

- Resource(s) for interference measurement can be NZP based or ZP based or both

- NZP CSI-RS resource(s) configured for channel measurement can be reused for interference measurement.

Proposal 9:  NR shall adopt Alt2: No support for sub-time unit for beam management RS shorter than 1 OFDM symbol.

Proposal 10: For deployments comprising a common beam for both DL and UL, a link can be configured between CORESETS and a set of PUCCH resources, such that TCI update of a CORESET beam, also updates the spatial relation info of the PUCCH resources linked to the CORESET.

Proposal 11:  For enhanced reliability and robustness, support single DCI transmission over multiple TCI states

Proposal 12: Study and specify PUCCH repetition/ selection across multiple beams for enhanced reliability and robustness

Proposal 13: Study and specify multi-TCI transmission for data channels

Proposal 14: Study and specify L1 event trigger-based report for fast beam selection.

Proposal 15: Mechanisms to reduce latency in beam selection for CDRX operation shall be studied and specified.

# Appendix

# 11.1 Additional results for multi-TCI PDCCH

Table 3. Simulation Assumptions in Section 6.1

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Subcarrier Spacing for data | For 30 GHz: 120kHz |
| Data allocation | Agg level 4,8,16 |
| Channel Model | CDL-B model - delay spread =100ns , UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in subclause 7.7.5.1 in TR 38.901 accordingly. Companies to report phase noise modelling and PTRS considerations if used. |
| Criteria for beam selection | Best CPO beam pointing towards the average strongest cluster |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 2, 2; 1 , 1). ((dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS antenna element radiation pattern | For 30 GHz: According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 1; 1, 1); * Θmg,ng=90°; Ω0,1=Ω0,0+180°; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0, 360] degree |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| UE mobility feature | Blockage model added as Blockage A model in 3GPP 38.901 |
| Transmission scheme | Single TCI beam, single transmission; Single TCI beam, two transmissions (TDM, soft combine); 2 TCI beams, 2 transmissions (soft-combine) |
| MIMO mode | SU-MIMO |
| UE receiver type | MMSE-IRC as baseline |

# 11.2 Additional results for multi-TCI PDSCH

Table 4. Simulation Assumptions in Section 6.2

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Subcarrier Spacing for data | For 30 GHz: 120kHz |
| Data allocation | Bandwidth 65 RB PDSCH Idx: Symbol 4-14 DMRS idx: Symbol 3 |
| Channel Model | CDL-B model - delay spread =30,100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in subclause 7.7.5.1 in TR 38.901 accordingly. Companies to report phase noise modelling and PTRS considerations if used. |
| Criteria for beam selection | Best CPO beam pointing towards the average strongest cluster |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 2, 2; 1 , 1). ((dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS antenna element radiation pattern | For 30 GHz: According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); baseline for UL panel-specific beam selection (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 1; 1, 1); also can be used except for evaluating UL panel-specific beam selection * Θmg,ng=90°; Ω0,1=Ω0,0+180°; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0, 360] degree |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| UE mobility feature | Blockage model added as Blockage A model in 3GPP 38.901 |
| Transmission scheme | Single TCI beam, single transmission; Single TCI beam, two transmissions (soft combine); 2 TCI beams, 2 transmissions (soft-combine) |
| MIMO mode | SU-MIMO |
| UE receiver type | MMSE-IRC as baseline |

# References

- RP-182067 Rel-16 MIMO Work Item

- Chairman’s Notes, 3GPP TSG-RAN WG1 Meeting #94b

- Chairman’s Notes, 3GPP TSG-RAN WG1 Meeting #95