3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1900908

Taipei, Taiwan, 21st – 25th January, 2019

Agenda item:	7.2.8.5

Source: 	Qualcomm Incorporated

Title: 	Lower PAPR reference signals

Document for:	Discussion/Decision

# Introduction

In RAN1 #95 meeting, the following agreements were made on the low PAPR reference signal design in Rel-16:

Agreement

- For sequences with length 30 or larger, DMRS for ππ/2 BPSK modulation for PUSCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding resulting in a DMRS Type 1 comb structure.

- For sequences with length 30 or larger, DMRS for π/2 BPSK modulation for PUCCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding.

- For sequences with allocation length 6,12,18 and 24 CGS is used for DMRS for π/2 BPSK modulation in case of PUSCH and PUCCH

Agreement

Support PUCCH DMRS enhancements for PUCCH format 3 and 4

In this contribution, we provide our views on the design of the remaining problems related to the pi/2 BPSK DMRS design, including:

- DMRS design to support MU-MIMO with pi/2 BPSK for PUSCH and PUCCH format 4

- CGS design for length 6/12/18/24 DMRS sequence

# π/2 BPSK based DMRS design for DFT-S-OFDM waveform

## DMRS base sequence generation

For the design of the Gold sequence for DMRS sequence of length 30 or larger, the NR pseudo-random sequence generator defined in Section 5.2.1 of [2, TS 38.211] can be used. Also, the DMRS initialization method for CP-OFDM could be reused. In this section, we describe the detailed procedures for DMRS sequence generation.

In particular, the base sequence before the transform precoding shall be generated according to

sn=ejπ2n mod 22[1-2⋅cn+j(1-2⋅c(n)))],  n=0,…,MscPUSCH/2-1                            (1)

where the pseudo-random sequence ci is defined in Section 5.2.1 of [2, TS 38.211] if MscPUSCH≥60. The pseudo-random sequence generator shall be initialized with (same as in NR Rel-15)

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCID mod 231

where nSCID=0 is used in the equation, and NID0 is given by higher-layer parameters scramblingID0 in the DMRS-UplinkConfig same as in NR Rel-15. In this way, no new RRC parameter or DCI signaling is needed for the DMRS sequence initialization.

Proposal 1: For length 30 or larger DMRS for π/2 BPSK modulation for PUSCH/PUCCH, NR Rel-16 reuse Rel-15 Gold sequence generator and DMRS initialization method for CP-OFDM. In particular, Rel-16 Gold sequence generator for length 30 or larger DMRS for π/2 BPSK modulation for PUSCH/PUCCH shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCID mod 231

where  nSCID=0 is used.

## Orthogonal DMRS design for PUSCH

During RAN1#95 meeting, it was agreed that the new DMRS design should support comb-2 structure in the frequency domain for PUSCH. To generate comb-2 in the frequency domain, it is equivalent to repeat the same time-domain sequence two times and then apply an OCC [+1, +1] and [+1, -1] on each repetition followed by a DFT whose length is equal to N=MscPUSCH. Here,  MscPUSCH is the bandwidth of the PUSCH in terms of resource elements. In other words, block-wise spreading and pre-DFT-OCC can be used.

More specifically, to generate a sequence of length N with comb 2 structure in the frequency domain, we can take a (time-domain) sequence sN/2 and perform a repetition as follows:

s(0)=[sN/2, sN/2]  and s(1)=[sN/2, -sN/2]  .

After performing an N-point DFT on s(0), the length-N sequence S(0)=DFT(s(0)) will only occupy even subcarriers, and the length-N sequence S(1)=DFT(s(1)) will only occupy odd subcarriers. This way we can achieve a comb-2 structure in the frequency domain.

In NR release 15, four orthogonal DMRS ports can be supported on one OFDM symbol for DMRS configuration type 1. Indeed, with ZC based DMRS, two orthogonal DM-RS ports can be supported on the same comb by using time-domain cyclic shift (or equivalently, a frequency domain OCC). In NR Rel-16, the same DMRS multiplexing capacity need to be supported for the new DMRS sequence design for π/2  BPSK modulation. In other words, two DMRS ports must be supported in the same comb. However, for the time-domain π/2 BPSK based DMRS, time-domain cyclic shift does not work, since the time-domain π/2 BPSK based DMRS is not necessarily flat in frequency. As a result, time-domain cyclic shift does not result in orthogonal DMRS sequences.

Observation 1: For time-domain pi/2 BPSK based DMRS sequence, time-domain cyclic shift does not result in orthogonal sequences for DMRS ports in the same comb.

To support orthogonal DMRS sequences on the same comb, intra-symbol time-domain OCC [+1, +1] and [+1, -1] can be used between the different portions of the sequence s. Assume that a length-N/2 base sequence s contains two equal-length portions s1 and s2, each of length N/4. Then, by combining frequency domain comb (implemented by time domain repetition) and time-domain OCC, we can generate four length-N orthogonal DMRS sequences. As an example, Fig. 1 shows the flow chart to generate the DMRS sequence for DMRS port 1 from a DMRS base sequence s of length N/2.

Figure 1. Flow diagram for DMRS generation for DMRS port 1

More specifically, to generate the DMRS sequence on antenna port p from a base sequence s(n), n=0,1,…,MscPUSCH/2-1, we first map the base sequence to the intermediate quantity  s(p)(k) according to

s(p)(k)=wt(k')s(n)                                                                              (2)

k=0,1,…,MscPUSCH-1

k'=4kMscPUSCH

n=k modMscPUSCH2

where wtk' is given by Table 1 and corresponds to the DMRS port p.

The intermediate vector [s(p)(0),…, s(p)(MscPUSCH-1)] shall be transform-precoded according to

ak(p)=1MscPUSCH i=0MscPUSCH-1s(p)ie-j2πikMscPUSCH-1                                                         (3)

k=0,1,…,MscPUSCH-1

resulting in a block of complex-valued symbols a0(p),…, aMscPUSCH-1(p).

The block of complex-valued symbols a0(p),…, aMscPUSCH-1(p) shall then be mapped to the allocated subcarriers for the PUSCH in a continuous manner.

It can be readily checked that, the DMRS sequences generated from the same base sequence by the forgoing procedure are orthogonal to each other. Furthermore, the resulted time-domain sequences are still π/2 BPSK modulated sequences (i.e., the phase change between adjacent time-domain samples are kept constant at π/2  or -π/2).

Table 1: Parameter  wt(k')

| DMRS port p | CDM group | wt(k') |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | k’=0 | k’=1 | k’=2 | k’=3 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 1 |  | 1 | -1 | 1 | -1 |
| 2 | 1 | 1 | 1 | -1 | -1 |
| 3 |  | 1 | -1 | -1 | 1 |

In Fig. 2 and 3, we show the LLS results for the MU-MIMO performance for the proposed DMRS multiplexing scheme. As shown in the LLS results, in case of 2 UE multiplexing, TD-OCC may lead to slight performance degradation compared with comb due to multi-path. However, the loss is negligible.

(a)

(b)

Figure 2. LLS performance for MU-MIMO with pi/2 BPSK and DMRS multiplexing; a) 4 Rx, 10 RB allocation with 1/2/4 UEs; b) 8 Rx, 10 RB allocation with 1/2/4 UEs.

Proposal 2: For PUSCH with pi/2 BPSK modulation, NR Rel-16 supports the same capacity of orthogonal DMRS multiplexing as in NR Rel-15.

- Support four orthogonal DMRS ports on the same OFDM symbol via preDFT intra-symbol repetition and preDFT intra-symbol TD-OCC as shown in Equations (2) and (3). The transform-precoded complex symbols [a0p,…, aMscPUSCH-1p] shall be mapped to the allocated PUSCH subcarriers in a continuous fashion.

## Orthogonal DMRS design for PUCCH Format 4

In NR Rel-15, the PUCCH format 4 can support 2 or 4 users on the same resource block. The same capacity may need to be supported in NR Rel-16 when pi/2 BPSK is used. Towards, this end, we need to specify the DMRS structure for multi-user multiplexing. For PUCCH format 4, the base sequences are length-12 π/2 BPSK sequences followed by DFT, and is not flat in the frequency domain. Therefore, cyclic shift-based method as in NR Rel-15 does not work.

Instead, we propose to use pre-DFT OCC to orthogonalize the DMRS. In particular, given a base sequence s=[s0,…,s(11)] before transform precoding, UE shall generate the time-domain DMRS sequence skn according to

spn=wtn⋅sn,                                                                           (4)

n=0,…,11

where p denotes the orthogonal sequence index, and wt(n) corresponds to the orthogonal sequence index, and is shown in Table 2.

The block of complex symbols sk0,…, sk11 shall then be transform precoded according to

z(k)=112 m=011s(p)me-j2πmk12                                                                      (5)

k=0,1,…,MscPUSCH-1

resulting in a block of complex-valued symbols z0,…,z11.

Table 2: Orthogonal sequences wtn for PUCCH Format 4

| Orthogonal sequence index p | wp |
| --- | --- |
| 0 | [+1, +1, +1, +1, +1, +1, +1, +1, +1, +1, +1, +1] |
| 1 | [+1, +1, +1, +1, +1, +1, -1, -1, -1, -1, -1, -1] |
| 2 | [+1, +1, +1, -1, -1, -1, +1, +1, +1, -1, -1, -1] |
| 3 | [+1, +1, +1, -1, -1, -1, -1, -1, -1, +1, +1, +1] |

An LLS performance comparison for PUCCH format 4 with varying number of UEs can be found in Fig. 3. As the figure indicates, the 4 UE MU performance is almost identical to the single-user performance. This is mainly because the data of PUCCH format 4 are orthogonal between the multiple users (i.e., CDM-ed). For comparison, in PUSCH, there is a performance gap between 1UE and 2 UE case, and between the 2 UE and 4 UE case, since the data for PUSCH are not spatially multiplexed, and is not orthogonal.

Figure 3. LLS performance for PUCCH format 4 with pi/2 BPSK modulation and DMRS multiplexing; payload size =11 bits

Proposal 3: For PUCCH format 4 with pi/2 BPSK modulation, NR Rel-16 supports the same MU capacity as in NR Rel-15.

- Support four orthogonal DMRS sequences on the same DMRS symbol via preDFT intra-symbol TD-OCC as shown in Equations (4) and (5).

## CGS-based π/2 BPSK sequences for small length

For  sequences of length less than 30, it was agreed in RAN 1 # 95 to use computer generated sequences as DMRS. Since time-domain pi/2 BPSK sequence is used as DMRS for length 30 or larger, it is natural to use computer-generated time-domain π/2 BPSK sequences for length less than 30.

### CGS design for length-12, 18, and 24

The criteria used for searching the π/2 BPSK CGS are listed below:

- Low auto-correlation;

- Low PAPR

- Low cross-correlation.

In particular, all  π/2 BPSK CGS for length 12 shall satisfy the following auto-correlation property to avoid self-interference due to filtering:

0≤n≤11xnx*(n+dmod 12)=0,      d=-2,-1,1,2.

This property guarantees good channel estimation performance regardless of what FDSS is used (as long as the number of taps of the FDSS in time domain satisfies the corresponding RAN 4 requirement).

Similarly, all length-18 π/2 BPSK CGS shall satisfy the following auto-correlation property:

0≤n≤11xnx*(n+dmod 12)=0,      d=-3,-1,1,3

and

0≤n≤11xnx*(n+dmod 12)=constant ,      d=-2, 2.

All length-24 π/2 BPSK CGS shall satisfy the following auto-correlation property:

0≤n≤23xnx*(n+dmod 12)=0,      d=-5,-4,-3,-2,-1,1,2,3,4,5.

For π/2 BPSK sequences that satisfy the auto-correlation properties that listed above, 30 sequences with low PAPR and low cross-correlation properties are further selected.

A list of selected CGS sequences for length 12, 18, 24 are shown in Tables 3-5 below. The binary sequences in the tables are modulated to π/2 BPSK symbols according to (1), followed by DFT-s-OFDM with FDSS.

Table 3: A set of length-12 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 0 1 1 1 1 0 1 0 0 0 1 1 |
| 1 | 0 1 0 1 1 0 1 0 0 0 1 1 |
| 2 | 0 1 1 0 1 0 1 0 0 0 1 1 |
| 3 | 0 1 0 1 0 1 1 0 0 0 1 1 |
| 4 | 0 1 1 1 1 1 1 0 0 0 1 1 |
| 5 | 0 1 0 1 1 1 1 0 0 0 1 1 |
| 6 | 0 1 1 0 0 0 0 0 0 0 1 1 |
| 7 | 0 1 1 1 1 0 0 0 0 0 1 1 |
| 8 | 0 1 0 1 1 0 0 0 0 0 1 1 |
| 9 | 0 1 1 0 1 0 0 0 0 0 1 1 |
| 10 | 0 1 1 1 1 0 1 1 1 0 1 1 |
| 11 | 0 1 0 1 1 0 1 1 1 0 1 1 |
| 12 | 0 1 1 0 1 0 1 1 1 0 1 1 |
| 13 | 0 1 1 1 0 1 1 1 1 0 1 1 |
| 14 | 0 1 0 0 0 1 1 1 1 0 1 1 |
| 15 | 0 1 1 0 1 1 1 1 1 0 1 1 |
| 16 | 0 1 0 1 0 0 0 1 1 0 1 1 |
| 17 | 0 1 0 0 0 0 0 1 1 0 1 1 |
| 18 | 0 1 0 0 0 1 0 1 1 0 1 1 |
| 19 | 0 1 1 1 1 1 0 1 1 0 1 1 |
| 20 | 0 1 0 1 1 1 0 1 1 0 1 1 |
| 21 | 0 1 1 0 0 0 1 0 1 0 1 1 |
| 22 | 0 1 1 1 0 1 1 0 1 0 1 1 |
| 23 | 0 1 1 0 1 1 1 0 1 0 1 1 |
| 24 | 0 1 1 0 0 0 0 0 1 0 1 1 |
| 25 | 0 1 1 1 1 0 0 0 1 0 1 1 |
| 26 | 0 1 0 1 1 0 0 0 1 0 1 1 |
| 27 | 0 1 1 0 1 0 0 0 1 0 1 1 |
| 28 | 0 1 1 1 0 1 1 1 0 1 1 1 |
| 29 | 0 1 0 0 0 0 1 0 0 1 1 1 |

Table 4: A set of length-18 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 0 1 1 0 1 1 1 0 0 0 0 0 0 0 1 1 0 0 |
| 1 | 0 1 1 0 1 1 1 1 1 0 0 0 0 0 1 1 0 0 |
| 2 | 0 0 0 1 1 1 1 1 0 1 1 0 0 0 1 1 0 0 |
| 3 | 0 0 1 0 0 1 0 1 0 1 1 0 0 0 1 1 0 0 |
| 4 | 0 0 0 1 1 1 0 1 1 0 0 0 0 0 1 1 0 0 |
| 5 | 0 1 1 1 1 1 1 0 0 1 0 0 0 0 1 1 0 0 |
| 6 | 0 0 1 0 0 1 1 1 1 1 1 0 0 0 1 1 0 0 |
| 7 | 0 0 0 1 1 0 1 1 1 1 1 0 0 0 1 1 0 0 |
| 8 | 0 1 1 1 0 1 1 0 1 0 1 1 1 0 1 1 0 0 |
| 9 | 0 0 0 1 1 0 0 0 0 0 1 1 1 0 1 1 0 0 |
| 10 | 0 0 1 0 0 1 1 1 1 0 1 1 1 0 1 1 0 0 |
| 11 | 0 1 1 0 0 0 0 0 0 0 1 1 1 0 1 1 0 0 |
| 12 | 0 1 1 1 0 1 0 1 1 0 0 0 0 0 1 1 0 0 |
| 13 | 0 0 0 0 0 1 1 0 1 0 1 1 1 0 1 1 0 0 |
| 14 | 0 0 0 1 1 0 0 1 0 1 0 1 0 0 1 1 0 0 |
| 15 | 0 1 1 0 1 0 1 1 1 1 1 0 0 0 1 1 0 0 |
| 16 | 0 1 1 1 1 1 0 1 0 1 1 0 0 0 1 1 0 0 |
| 17 | 0 0 0 0 1 0 0 1 0 1 1 0 0 0 1 1 0 0 |
| 18 | 0 1 1 0 1 0 0 1 0 0 0 0 0 0 1 1 0 0 |
| 19 | 0 1 0 1 0 1 1 0 0 1 0 0 0 0 1 1 0 0 |
| 20 | 0 0 1 0 0 1 1 0 1 0 0 0 0 0 1 1 0 0 |
| 21 | 0 0 1 0 1 0 0 1 1 0 1 0 0 0 1 1 0 0 |
| 22 | 0 1 1 0 0 0 0 0 0 1 0 0 0 0 1 1 0 0 |
| 23 | 0 0 0 1 1 0 0 0 0 0 0 0 0 0 1 1 0 0 |
| 24 | 0 1 1 0 0 0 1 0 1 0 0 0 0 0 1 1 0 0 |
| 25 | 0 0 0 1 1 0 0 0 1 0 1 0 0 0 1 1 0 0 |
| 26 | 0 1 1 0 0 0 0 0 1 0 1 0 0 0 1 1 0 0 |
| 27 | 0 1 0 1 0 0 0 0 0 1 1 0 0 0 1 1 0 0 |
| 28 | 0 0 1 0 1 0 0 0 0 1 1 0 0 0 1 1 0 0 |
| 29 | 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 1 0 0 |

Table 5: A set of length-24 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 0 1 0 1 1 0 1 0 1 0 1 0 1 1 0 1 1 0 0 1 0 0 1 1 |
| 1 | 0 1 0 0 0 1 1 0 1 0 1 0 1 1 1 0 1 0 0 1 0 0 1 1 |
| 2 | 0 1 0 0 1 0 0 1 1 1 1 1 1 1 1 1 1 0 0 1 0 0 1 1 |
| 3 | 0 1 1 0 1 0 1 1 0 1 1 0 0 1 0 1 0 1 0 1 0 0 1 1 |
| 4 | 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 0 1 0 0 1 0 0 1 1 |
| 5 | 0 0 0 0 1 0 1 0 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 1 |
| 6 | 0 1 1 0 0 0 0 1 0 0 0 0 0 0 0 1 1 1 0 1 0 0 1 1 |
| 7 | 0 1 0 0 0 1 1 1 1 0 1 1 1 1 1 1 1 0 0 1 0 0 1 1 |
| 8 | 0 1 0 1 1 0 0 0 1 0 0 0 1 0 1 0 0 1 0 1 0 0 1 1 |
| 9 | 0 1 0 1 1 0 1 0 1 1 1 0 1 1 1 0 0 1 0 1 0 0 1 1 |
| 10 | 0 1 0 0 0 1 1 0 1 0 1 0 0 1 0 1 0 0 0 1 0 0 1 1 |
| 11 | 0 1 0 1 0 1 0 1 1 0 1 0 0 1 1 0 1 1 1 1 0 0 1 1 |
| 12 | 0 1 0 1 0 1 1 0 1 1 1 1 1 0 0 1 1 1 1 1 0 0 1 1 |
| 13 | 0 1 0 1 1 0 0 0 0 0 0 1 1 1 0 1 0 0 0 1 0 0 1 1 |
| 14 | 0 1 0 1 0 0 0 1 0 1 1 0 1 1 1 0 0 1 0 1 0 0 1 1 |
| 15 | 0 1 1 0 0 1 0 1 0 1 1 0 1 0 1 0 1 1 1 1 0 0 1 1 |
| 16 | 0 1 0 1 0 1 1 0 1 1 0 1 1 1 1 1 0 1 1 0 0 0 1 1 |
| 17 | 0 1 1 1 1 0 0 1 0 0 0 0 1 0 0 1 0 1 0 1 0 0 1 1 |
| 18 | 0 1 0 1 0 1 1 0 1 1 1 1 0 1 1 0 0 0 0 1 0 0 1 1 |
| 19 | 0 1 1 1 1 1 1 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 1 1 |
| 20 | 0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 1 1 1 0 0 1 1 |
| 21 | 0 1 0 1 0 0 0 1 1 0 1 0 0 0 1 0 0 1 0 1 0 0 1 1 |
| 22 | 0 1 0 1 0 1 1 0 1 1 0 1 0 1 0 1 1 0 0 1 0 0 1 1 |
| 23 | 0 1 0 1 1 0 1 1 1 0 1 1 1 1 1 0 0 1 0 1 0 0 1 1 |
| 24 | 0 1 0 0 0 0 0 1 0 1 1 0 0 1 0 0 0 0 0 1 0 0 1 1 |
| 25 | 0 1 0 0 1 0 0 0 0 1 1 0 0 1 0 1 0 1 0 1 0 0 1 1 |
| 26 | 0 1 0 1 0 1 0 0 1 0 1 0 0 0 0 1 1 0 0 1 0 0 1 1 |
| 27 | 0 1 0 1 0 0 1 0 1 0 0 1 1 0 0 0 0 0 0 1 0 0 1 1 |
| 28 | 0 1 1 0 1 1 0 1 0 1 0 1 0 1 1 0 0 1 0 1 0 0 1 1 |
| 29 | 0 1 0 0 0 0 0 1 0 1 1 0 0 1 1 1 1 1 1 1 0 0 1 1 |

In the RAN1 #95 meeting, a joint proposal on length-12, length-18, and length-24 CG sequences for pi/2 BPSK as listed in Table 6-8 was made in [1]. In the joint proposal, CGSs are proposed based on slightly different metrics.  Since the joint proposal [1] yields similar link-layer simulation performance as in Table 3/4/5, it could be supported as an alternative option for the length-12, length-18, and length-24 CGS design in NR Rel-16.

Table 6: A set of length-12 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 1 1 0 0 0 0 0 1 1 1 1 0 |
| 1 | 0 1 0 0 0 1 1 0 1 0 1 1 |
| 2 | 1 1 0 1 0 1 1 0 0 0 1 0 |
| 3 | 1 0 1 0 1 1 0 1 1 0 1 0 |
| 4 | 1 0 1 0 0 1 0 0 1 0 1 0 |
| 5 | 1 0 1 1 0 0 0 1 0 0 0 1 |
| 6 | 1 0 1 0 1 1 0 0 1 0 0 1 |
| 7 | 1 0 1 0 0 0 1 0 0 0 0 0 |
| 8 | 1 1 1 1 0 1 1 1 0 1 0 1 |
| 9 | 0 0 0 1 0 0 0 0 1 0 1 1 |
| 10 | 0 1 1 0 0 0 1 0 0 0 0 1 |
| 11 | 0 0 0 0 1 0 0 0 1 1 0 1 |
| 12 | 1 0 1 0 0 0 0 1 0 0 0 1 |
| 13 | 0 0 0 1 0 0 0 1 1 1 1 0 |
| 14 | 1 1 0 0 0 0 1 0 0 0 1 1 |
| 15 | 1 1 0 0 0 0 0 1 1 1 1 0 |
| 16 | 0 1 0 0 0 1 1 0 1 0 1 1 |
| 17 | 1 1 0 1 0 1 1 0 0 0 1 0 |
| 18 | 1 0 1 0 1 1 0 1 1 0 1 0 |
| 19 | 1 0 1 0 0 1 0 0 1 0 1 0 |
| 20 | 1 0 1 1 0 0 0 1 0 0 0 1 |
| 21 | 1 0 1 0 1 1 0 0 1 0 0 1 |
| 22 | 1 0 1 0 0 0 1 0 0 0 0 0 |
| 23 | 1 1 1 1 0 1 1 1 0 1 0 1 |
| 24 | 0 0 0 1 0 0 0 0 1 0 1 1 |
| 25 | 0 1 1 0 0 0 1 0 0 0 0 1 |
| 26 | 0 0 0 0 1 0 0 0 1 1 0 1 |
| 27 | 1 0 1 0 0 0 0 1 0 0 0 1 |
| 28 | 0 0 0 1 0 0 0 1 1 1 1 0 |
| 29 | 1 1 0 0 0 0 1 0 0 0 1 1 |

Table 7: A set of length-18 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 0 1 1 0 1 1 1 0 0 0 0 0 0 0 1 1 0 0 |
| 1 | 0 0 0 1 1 1 1 1 0 1 1 0 0 0 1 1 0 0 |
| 2 | 0 0 0 1 1 1 0 1 1 0 0 0 0 0 1 1 0 0 |
| 3 | 0 1 1 0 0 0 0 0 0 0 1 1 1 0 1 1 0 0 |
| 4 | 0 1 1 1 0 1 0 1 1 0 0 0 0 0 1 1 0 0 |
| 5 | 0 0 0 1 1 0 0 1 0 1 0 1 0 0 1 1 0 0 |
| 6 | 0 0 0 1 0 0 1 0 0 1 0 0 0 1 1 1 1 1 |
| 7 | 0 0 1 0 1 0 1 0 0 1 1 0 0 0 0 0 1 1 |
| 8 | 1 1 0 0 1 0 1 1 1 0 1 0 0 1 1 1 0 1 |
| 9 | 1 1 1 1 0 0 0 0 1 0 0 0 1 0 0 0 0 1 |
| 10 | 1 1 1 0 0 0 0 1 0 0 0 1 0 0 0 0 1 1 |
| 11 | 1 0 1 1 1 0 0 0 1 1 1 0 1 1 0 1 0 1 |
| 12 | 1 0 1 1 0 1 0 1 1 1 0 0 0 0 0 1 1 0 |
| 13 | 1 1 0 0 1 0 1 1 0 1 1 0 1 0 0 1 0 1 |
| 14 | 0 0 0 0 0 1 1 1 0 1 1 0 1 0 1 1 0 0 |
| 15 | 0 0 1 1 1 0 1 1 0 1 0 0 0 1 1 0 1 0 |
| 16 | 1 1 1 0 1 0 1 1 0 1 0 0 0 0 0 1 1 0 |
| 17 | 0 1 0 0 1 0 0 0 1 1 1 0 1 0 0 1 1 1 |
| 18 | 1 0 0 0 1 0 1 0 1 0 0 0 1 1 0 1 0 1 |
| 19 | 1 0 1 1 1 0 0 1 0 1 0 0 1 1 1 0 1 0 |
| 20 | 1 1 1 1 0 0 0 1 1 0 1 1 1 0 1 0 0 0 |
| 21 | 1 0 0 1 1 1 0 0 0 0 1 1 1 0 1 0 0 0 |
| 22 | 1 0 0 0 0 1 1 1 0 0 1 0 0 0 1 0 1 1 |
| 23 | 1 0 0 0 0 1 1 1 0 1 0 0 0 1 0 0 1 1 |
| 24 | 0 0 0 0 0 0 0 0 0 1 0 0 1 0 0 1 1 1 |
| 25 | 0 1 0 0 1 0 0 1 1 1 0 0 0 0 0 0 0 0 |
| 26 | 1 1 0 1 1 0 1 0 0 1 0 0 0 0 0 0 0 0 |
| 27 | 0 0 0 0 0 0 0 1 1 0 0 0 1 0 1 1 1 0 |
| 28 | 1 0 0 0 1 0 1 1 1 0 0 0 0 0 0 0 0 1 |
| 29 | 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 1 0 |

Table 8: A set of length-24 CGSs for Pi/2 BPSK DMRS

| Index | Sequence |
| --- | --- |
| 0 | 0 1 0 1 1 0 1 0 1 0 1 0 1 1 0 1 1 0 0 1 0 0 1 1 |
| 1 | 0 1 0 0 0 1 1 0 1 0 1 0 1 1 1 0 1 0 0 1 0 0 1 1 |
| 2 | 0 1 0 0 1 0 0 1 1 1 1 1 1 1 1 1 1 0 0 1 0 0 1 1 |
| 3 | 0 1 1 0 1 0 1 1 0 1 1 0 0 1 0 1 0 1 0 1 0 0 1 1 |
| 4 | 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 0 1 0 0 1 0 0 1 1 |
| 5 | 0 0 0 0 1 0 1 0 0 1 0 1 0 1 0 1 1 0 0 1 0 0 1 1 |
| 6 | 1 1 0 0 1 0 1 0 0 0 0 1 1 0 0 0 1 0 0 0 1 1 1 1 |
| 7 | 0 1 0 0 1 1 1 1 1 1 0 0 0 1 0 0 0 1 1 0 0 0 0 1 |
| 8 | 0 1 1 0 0 1 0 1 1 1 0 0 0 0 0 0 1 1 0 1 0 1 1 1 |
| 9 | 0 1 1 1 0 0 0 1 0 1 0 0 1 0 1 1 1 0 0 1 0 0 0 1 |
| 10 | 1 1 0 0 1 0 0 0 1 0 1 1 1 0 1 0 0 1 0 0 0 0 0 1 |
| 11 | 1 1 1 0 1 1 0 0 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 |
| 12 | 0 1 0 0 0 1 1 0 1 1 0 0 1 0 1 1 0 0 0 1 1 0 1 0 |
| 13 | 1 0 0 1 0 1 0 0 1 1 0 0 0 0 1 1 1 1 1 1 1 0 0 1 |
| 14 | 1 0 0 1 0 1 0 0 1 1 0 1 1 0 0 1 1 0 0 0 0 0 1 1 |
| 15 | 0 0 0 1 1 1 1 0 0 1 0 1 0 0 1 1 1 0 1 1 1 0 0 1 |
| 16 | 1 1 0 1 0 1 1 1 0 0 1 1 1 0 0 0 0 0 0 1 1 0 1 0 |
| 17 | 1 0 1 1 0 1 1 0 0 0 1 0 1 1 0 1 0 0 1 0 0 0 0 1 |
| 18 | 1 1 1 1 1 0 0 1 0 0 0 1 1 1 1 0 1 1 1 1 1 1 0 0 |
| 19 | 1 1 1 1 0 1 0 0 1 1 0 0 0 1 1 0 0 0 0 0 0 0 0 1 |
| 20 | 1 0 1 0 0 0 1 1 1 0 1 1 0 1 0 1 1 0 1 1 1 0 0 0 |
| 21 | 1 0 1 0 1 1 1 0 0 0 1 0 0 1 0 1 0 0 1 0 0 0 1 1 |
| 22 | 1 1 1 1 0 0 1 0 0 0 1 1 1 1 1 0 0 1 1 1 0 1 0 1 |
| 23 | 1 0 1 1 0 1 1 0 0 0 1 0 0 0 0 1 0 0 1 0 1 1 1 0 |
| 24 | 0 0 0 0 0 0 0 1 1 0 0 0 1 1 1 1 1 1 1 0 0 0 1 1 |
| 25 | 1 1 1 0 0 0 1 1 0 0 0 0 0 0 0 1 1 0 0 0 1 1 1 1 |
| 26 | 0 0 0 0 0 0 1 1 1 0 0 1 1 1 1 1 1 1 0 0 1 1 1 0 |
| 27 | 1 1 0 0 1 1 1 0 0 0 0 0 0 0 1 1 1 0 0 1 1 1 1 1 |
| 28 | 0 0 0 0 1 0 0 0 0 1 0 0 1 1 1 1 0 1 1 1 1 0 0 1 |
| 29 | 0 1 1 1 1 0 0 1 0 0 0 0 1 0 0 0 0 1 0 0 1 1 1 1 |

Proposal 4: For length 12, 18, and 24, NR Rel-16 supports one of the following options as DMRS design for both PUSCH and PUCCH Format 3 and 4.

- Option 1: NR Rel-16 supports the binary CGS in Table 3, 4, 5 followed by pi/2 BPSK modulation followed by transform precoding as DMRS.

- Option 2: NR Rel-16 supports the binary CGS in Table 6, 7, 8 followed by pi/2 BPSK modulation followed by transform precoding as DMRS.

### CGS design for length-6

For length-6, there are not enough binary sequences to form 30 DMRS base sequences. Indeed, if we fix the phase of the first symbol, there are 25=32 length-6 π/2  BPSK sequences in total. And only 6 of them are distinct from each other if we identify sequences that can coincide with each other after cyclic-shift as one sequence. In other words, all length-6 pi/2 BPSK sequences can be generated by performing a cyclic shift over one of these 6 sequences in the time domain, modulo a constant phase rotation. As an example, it can be readily checked that the sequence [0 0 0 1 0 1] and [0 0 1 0 0 0] from [6, Table A1] are the same sequence under cyclic-shift and constant phase rotation operation. Even if we only take into account the +1 and -1 cyclic shifts of a binary sequence, there’re only 10 distinct length-6 π/2  BPSK sequences.

Instead, we shall search for sequences with a larger constellation in the time-domain. In particular, the DMRS sequence before DFT spread is given by

sun=ejϕnπ/8,   0≤n≤5

where the value of ϕ(n) is given by Table 4, and u indicates is the base sequence group number as in NR Rel-15.

Table 9: A set of length-6 CGSs

| u | ϕ0,…, ϕ(5) | PAPR (dB) |
| --- | --- | --- |
| 0 | -7 5 -7 -3 -5 5 | 1.4610 |
| 1 | -7 -3 -7 -3 7 5 | 1.4610 |
| 2 | -7 -3 3 7 3 -3 | 1.5421 |
| 3 | -7 5 -7 -3 7 5 | 1.6373 |
| 4 | -7 -3 -7 -3 -5 5 | 1.6373 |
| 5 | -7 1 -1 5 -7 5 | 1.6492 |
| 6 | -7 5 -1 1 -3 1 | 1.8773 |
| 7 | -7 -3 -7 -5 5 1 | 1.8773 |
| 8 | -7 -5 3 7 5 -1 | 1.9518 |
| 9 | -7 3 -3 -5 -1 7 | 1.9518 |
| 10 | -7 1 -3 1 7 5 | 1.9574 |
| 11 | -7 -3 -3 -1 -7 5 | 1.9661 |
| 12 | -7 -7 -3 1 -3 7 | 1.9661 |
| 13 | -7 5 -5 -1 -3 5 | 1.9682 |
| 14 | -7 -1 5 7 5 -1 | 1.9911 |
| 15 | -7 3 -3 -5 -3 3 | 1.9911 |
| 16 | -7 -3 3 -1 -7 -5 | 1.9939 |
| 17 | -7 -3 -5 -3 7 3 | 1.9939 |
| 18 | -7 -1 -3 -1 7 3 | 2.0232 |
| 19 | -7 5 7 -1 -3 3 | 2.0314 |
| 20 | -7 -1 -3 5 7 3 | 2.0314 |
| 21 | -7 -1 3 7 3 -1 | 2.0425 |
| 22 | -7 3 -1 -5 -1 3 | 2.0425 |
| 23 | -7 3 3 7 -5 7 | 2.0490 |
| 24 | -7 5 -7 -3 -3 7 | 2.0491 |
| 25 | -7 -5 3 7 3 -3 | 2.0927 |
| 26 | -7 3 -1 3 -5 -3 | 2.0928 |
| 27 | -7 1 -3 5 7 5 | 2.1111 |
| 28 | -7 5 -3 1 1 -1 | 2.1966 |
| 29 | -7 7 7 -5 3 -1 | 2.1966 |

Proposal 5: For length 6, NR Rel-16 supports the pre-DFT sequences sun=ejϕnπ/8, n=0,…,5  with ϕ(n) defined in Table 9 followed by transform precoding as DMRS for π/2 BPSK modulation for PUSCH.

## Backward compatibility issues

For an NR Rel-16 UE, it may need to support both the NR Rel-15 DMRS and the Rel-16 DMRS. For example, this may be needed for the UE in order to be able to communicate with gNBs that implement only one of the NR releases. In this case, the gNB may need to indicate the UE what DMRS scheme to use for uplink transmission with π/2 BPSK modulation. To differentiate the NR Rel-15 and the NR Rel-16 DMRS schemes, we make the following proposal.

Proposal 6: NR Rel-16 supports RRC signaling to differentiate the use of Rel-15 or Rel-16 DMRS sequence for uplink transmission with π/2 BPSK modulation.

# PAPR reduction for DMRS with CP-OFDM waveform

Significant progress was made regarding the CP-OFDM low-PAPR for DMRS in the previous meeting, with the following agreement being made:

Agreement

For CP-OFDM and for both DMRS type 1 and 2, the following cinitcinit for CDM group λ is used for Rel-16 DMRS sequence generation

and where nSCIDnSCID is provided by DCI. Lambda is absolute CDM group index.

It was also concluded to perform further study on the signaling aspects between the Rel-15 and Rel-16 sequences of CP-OFDM:

For further study:

When 2nd or 3rd CDM group is used, there are two behaviors for a Rel.16 UE: Rel.15 or Rel.16 sequences

- Alt.1 RRC signaling to configure the use of R.15 or R.16 sequence for 2nd and 3rd CDM group

- Alt.2 RRC + DCI signaling to switch between R.15 or R.16 sequence for 2nd and 3rd CDM group

  - DCI code points or explicit (new) bit can be used

More analysis of potential benefits needed, downselect next meeting between Alt. 1 and 2

We now present our views regarding the above two alternatives. Alt. 2 suggests that there will be DCI signaling to pick between the Rel-15 and Rel-16 sequences for the 2nd or 3rd CDM group. Such a feature would only make sense if it is obviously clear that there are many cases during which a Rel-15 port and a Rel-16 port would not be possible to be orthogonally MU-MIMO paired in a dynamic fashion across different UEs. We argue that this is not a problem for the following reasons. 2 UEs from different Release could still be paired across 2 CDM groups (FDM), or even within the 1st or 2nd CDM group since the sequences used for these groups are the same. A reasonable scheduler for example, could first pair some Rel-15 and Rel-16 ports inside the first CDM group, pair some others inside the 2nd CDM group, and then FDM in the 3rd CDM group either all Rel-15 ports or all Rel-16 ports. Recall that for 2-symbol DMRS a scheduler can already pair 4 UEs in the 1st or 2nd CDM group (e.g., X Rel-15 ports + Y Rel-16 ports such that X+Y<=4), and also add additional Rel-16-only ports or Rel-15-only ports in the 3rd CDM group to achieve orthogonal MU-MIMO port pairing. Finally, it should be noted that non-orthogonal MU-MIMO pairing is always an option and can still be used by a scheduler as needed to transmit data to both Rel-15 and Rel-16 UEs. Based on the above, adding the additional complexity at the UE to support DCI-switching between the sequences, or using the precious PDCCH capacity for such a reason is not needed.

Proposal 7: NR Rel-16 supports Alt. 1 to differentiate the use of Rel-15 or Rel-16 DMRS sequences for CP-OFDM.

# Conclusions

We have the following observation based on the study and simulation results for PAPR reduction for DMRS in Rel-16.

Observation 1: For time-domain pi/2 BPSK based DMRS sequence, time-domain cyclic shift does not result in orthogonal sequences for DMRS ports in the same comb.

We have made the following proposals for PAPR reduction for DMRS in NR Rel-16.

Proposal 1: For length 30 or larger DMRS for π/2 BPSK modulation for PUSCH/PUCCH, NR Rel-16 reuse Rel-15 Gold sequence generator and DMRS initialization method for CP-OFDM. In particular, Rel-16 Gold sequence generator for length 30 or larger DMRS for π/2 BPSK modulation for PUSCH/PUCCH shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCID mod 231

where  nSCID=0 is used.

Proposal 2: For PUSCH with pi/2 BPSK modulation, NR Rel-16 supports the same capacity of orthogonal DMRS multiplexing as in NR Rel-15.

- Support four orthogonal DMRS ports on the same OFDM symbol via preDFT intra-symbol repetition and preDFT intra-symbol TD-OCC as shown in Equations (2) and (3). The transform-precoded complex symbols [a0p,…, aMscPUSCH-1p] shall be mapped to the allocated PUSCH subcarriers in a continuous fashion.

Proposal 3: For PUCCH format 4 with pi/2 BPSK modulation, NR Rel-16 supports the same MU capacity as in NR Rel-15.

- Support four orthogonal DMRS sequences on the same DMRS symbol via preDFT intra-symbol TD-OCC as shown in Equations (4) and (5).

Proposal 4: For length 12, 18, and 24, NR Rel-16 supports one of the following options as DMRS design for both PUSCH and PUCCH Format 3 and 4.

- Option 1: NR Rel-16 supports the binary CGS in Table 3, 4, 5 followed by pi/2 BPSK modulation followed by transform precoding as DMRS.

- Option 2: NR Rel-16 supports the binary CGS in Table 6, 7, 8 followed by pi/2 BPSK modulation followed by transform precoding as DMRS.

Proposal 5: For length 6, NR Rel-16 supports the pre-DFT sequences sun=ejϕnπ/8, n=0,…,5  with ϕ(n) defined in Table 9 followed by transform precoding as DMRS for π/2 BPSK modulation for PUSCH.

Proposal 6: NR Rel-16 supports RRC signaling to differentiate the use of Rel-15 or Rel-16 DMRS sequence for uplink transmission with π/2 BPSK modulation.

Proposal 7: NR Rel-16 supports Alt. 1 to differentiate the use of Rel-15 or Rel-16 DMRS sequences for CP-OFDM.

# Appendix 1: Simulation Assumptions

Table 10. Link-level simulation assumptions

| System bandwidth | 20 Mhz |
| --- | --- |
| Numerology | 30 KHz SCS |
| Channel | TDL-C 300ns/1000ns |
| Number of Antennas | UE Tx=1, gNB Rx = 4, 8 |
| PUSCH duration | 13 OFDM symbols, with one front-loaded DMRS symbols symbol |
| Number of UEs | 1 UE |
| # RBs for PUSCH | 10 RBs |
| Carrier Frequency | 4 GHz |
| Coding | NR LDPC + CRC for PUSCH; Reed-Muller code for PUCCH with 11 bits payload |
| UE Speed | 12 Kmh |
| Channel Estimation | Practical Channel Estimation and Ideal Noise Estimation |
| MU-MIMO receiver | MMSE receiver |
| PUCCH duration | Length 14, with 2 DMRS symbols |

# References

- R1-1814205, “Joint proposal on Length-12, length-18, and length-24 CG sequences for pi/2 BPSK”, 3GPP RAN1 #95,  Spokane, USA, 12th-16th Nov, 2018.