3GPP TSG-RAN WG1 Ad-Hoc Meeting 1901	R1-1900920

Taipei, 21st – 25th January, 2019

Agenda item:	7.2.13.2

Source: 	Qualcomm Incorporated

Title: 	Cross-Carrier Scheduling with Different Numerologies

Document for:	Discussion/Decision

# Introduction

For carrier aggregation, the remaining issues for cross-carrier scheduling for different numerology will be discussed in this contribution.

# Discussion

## Background

In RAN-P #81, the work item on multi-RAT dual-connectivity and carrier aggregation enhancements was approved. One of the objectives of this work item is to devise schemes for allowing cross-carrier scheduling with different numerologies as follows:

- Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers [RAN1, RAN2, RAN4]

  - This objective applies to CA only.

  - Target completion by RAN#84.

While most of the details for cross-carrier scheduling for same numerology had been resolved in RAN1 #95 there remain some issues for cross-carrier scheduling for different numerologies, and they will be discussed in this contribution.

## Summary of Identified Issues

Cross-carrier scheduling issues for different numerologies are summarized in the table below. “Yes” means there are potential issues. The cases to consider are: (i) Larger SCS scheduling smaller SCS, (ii) Smaller SCS scheduling larger SCS, and in general, cross-band scheduling either within or across FR1 and FR2.

| Identified Issues | (i) Larger SCS Smaller SCS | (ii) Smaller SCS Larger SCS |
| --- | --- | --- |
| Extra buffering requirement for DL | No (for a reasonable implementation) | Yes (typically) |
| Challenging non-casual processing | Depends (Need to clarify which slot can schedule and any other constraints; Less issue if a simple scheme is adopted) | Yes (typically DCI is available later on smaller SCS than on larger SCS) |
| QCL assumption | Three scenarios need to be considered: - FR1 scheduling FR2 - FR2 scheduling FR1 - Between different bands within the same FR. |  |
| CCE/BD limits & overbooking handling | Depends (No issue if a simple scheme is adopted) | Yes |
| Number of valid DCI capability | Depends (No issue if a simple scheme is adopted) | May have impact to UE capability |

There is a strong use case for FR1 scheduling FR2. This is because FR1 (i.e. sub6) tends to have better coverage and it is more reliable to deliver DL control information on FR1. Cross-carrier scheduling can be an effective way for delivering DL control information for FR2 on FR1.

On the other hand, for cross-carrier scheduling from FR2 to FR1, there does not seem to be a strong use case. Considering the specification effort and the limited time available, we recommend the following prioritization:

- Small SCS -> Large SCS is supported in Rel-16

- Large SCS -> Small SCS is deferred to Rel-17

Proposal 1: Support for cross-carrier scheduling with different numerologies can be prioritized as follows: The feature for carriers with small SCS cross-scheduling large SCS is supported in Rel-16. The feature for carriers with large SCS cross-scheduling small SCS can be deferred to Rel-17.

## Issues of Extra Buffering and Causality

### Extra buffering due to k0

Compared to the self-scheduling baseline, the buffering requirement for cross-carrier scheduling can be higher due to worse PDCCH timeline. This is evident for the scenario where the carrier with larger SCS schedules the one with smaller SCS.

Self-scheduling: CC2 is scheduled by itself, k0=0

Cross-carrier scheduling: CC2 is scheduled by CC1, k0=0 is allowed

The extra buffering requirement is due to the extra latency for PDCCH scheduling CC2 being decoded based on CC1 timeline, relative to the self-scheduling case for which PDCCH can be decoded based on a tighter timeline.

On the first order, the extra buffering requirement is proportional to the following (parameters are for the scheduled carrier):

N_RB * ceil( ((X_CC1+Y_CC1) – (X_CC2+Y_CC2)) / T_sym )

Where,

N_RB - Number of RB in active DL BWP of the scheduled CC

T_sym – symbol duration for the scheduled CC

X_CC1 - End of last symbol of PDCCH for the scheduling CC

Y_CC1 - PDCCH processing delay for the scheduling CC

X_CC2 - End of last symbol of PDCCH for the scheduled CC for the case of self-scheduling

Y_CC2 - PDCCH processing delay for the scheduled CC for the case of self-scheduling

Another scenario is when the SCS of the scheduling carrier is much smaller than that of the scheduled carrier, slot level non-casuality may result. For example, the scheduling carrier’s SCS is 15kHz and the scheduled carrier’s SCS is 120kHz, the disparity is by a factor of eight. It is obvious that an additional constraint of “minimum k0 threshold” would alleviate or eliminate the non-casuality condition.

It is evident that if k0 is constraint to be greater than or equal to some non-zero threshold, the extra buffering requirement can be reduced or eliminated.

In the following, the constraint of “minimum k0 threshold = 1” is imposed.

Cross-carrier scheduling: CC2 is scheduled by CC1 with k0>=1:

### Extra buffering due to A-CSI triggering offset

Cross-carrier A-CSI request should also be considered. In RAN1#92, the following was agreed:

For CSI acquisition, aperiodic CSI-RS triggering offset can be 0, 1, 2, 3, 4 slots.

- If all the associated trigger states do not contain QCL Type D information, aperiodic CSI-RS triggering offset is fixed to zero

Unless one of the associated trigger states contain QCL Type D, the CSI-RS triggered by A-CSI request would be sent to the UE in the same slot as the UL grant. If this is the case, the minimum k0 threshold discussed above would not help in reducing buffering requirement, as UE would always have to buffer the Rx samples (in case of presence of A-CSI-RS) of the cross-carrier scheduled carrier, until at least all the grants are decoded in the scheduling carrier.

The issue of additional buffering due to cross-carrier A-CSI triggering with different numerologies has been raised and discussed during RAN plenary #82 as a potential CR for Rel-15 [2]. The proposed solution is to define some non-zero offset in the spec for the scenario.

We propose that the minimum k0 threshold discussed previously can be generalized as the “minimum DL scheduling offset” which also applies to A-CSI trigger for cross-carrier A-CSI triggering with different numerologies. For aperiodic CSI triggering, the actual slot timing of the CSI-RS relative to the triggering grant can be a function of the minimum DL scheduling offset and the configured A-CSI triggering offset. For example, one possible option is to take the max of minimum DL scheduling offset and the A-CSI-RS triggering offset.

### Solution: minimum DL scheduling offset

Cross-carrier scheduling support in Rel-15 should not result in significantly higher complexity in implementation (i.e. in terms of buffering and non-casual processing). Therefore, a minimum DL scheduling offset for cross-carrier scheduling should be specified, and proposed as follows:

Proposal 2: A minimum DL scheduling offset should be introduced for Rel-16 to address the buffering and causality issues with cross-carrier scheduling with different numerologies.

Proposal 3: Timing determination for A-CSI-RS relative to the triggering grant can be a function of the configured A-CSI triggering offset and the minimum DL scheduling offset.

For UL, there is already a hard requirement for casuality. K2 should accommodate N2 which is defined from the end of the PDCCH on the scheduling carrier, to the start of the PUSCH on the scheduled carrier; It should also accommodate the maximum timing difference between the scheduling and scheduled carriers. Causality is always enforced.

There are other benefits for minimum DL scheduling offset and minimum UL scheduling offset, not limited to cross-carrier scheduling with different numerologies. For example, for UE power saving. This aspect is discussed in our other contribution [3].

Because the minimum DL scheduling offset that UE can support for cross-carrier scheduling is related to UE capability, we may introduce this as a new parameter in UE capability. The following constraint on minimum DL scheduling offset should be defined:

Proposal 4: For cross-carrier scheduling with CIF, the minimum DL scheduling offset should be defined such that the end of the PDCCH symbol on the scheduling carrier is before the start of the corresponding PDSCH (or A-CSI-RS) on the scheduled carrier.

The following is an example of minimum DL scheduling offset values for different SCS settings. The values in parentheses are the time margin from end of the PDCCH symbols on the scheduling carrier to the start of the corresponding PDSCH on the scheduled carrier.

| Table 1. Example minimum values X |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scheduling CC Scheduled CC | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| 15 kHz | 0 |  |  |  |
| 30 kHz | 1 (285.7us) | 0 |  |  |
| 60 kHz | 2 (285.7us) | 1 (142.9us) | 0 |  |
| 120 kHz | 3 (160.7us) | 2 (142.9us) | 2 (178.6us) | 0 |

Note that PDCCH processing delay is not considered in above constraint. This is because PDCCH processing delay is implementation specific and it is expected that UE implementation can at least accommodate the extra buffering resulting from the additional PDCCH processing delay due to cross-carrier scheduling. Alternatively, if minimum DL scheduling offset can be reported as UE capability, PDCCH processing delay for the UE can be accommodated.

Proposal 5: UE shall report the minimum DL scheduling that it can support in UE capability.

The minimum DL scheduling offset is used as follows. If the indicated k0 is smaller than the threshold, it can be handled as an error condition and the DCI can be considered invalid. Alternatively, the threshold can be used as a lower bound on the indicated k0. If the indicated k0 is smaller than the threshold, use the threshold value as k0. The former approach is preferred.

Proposal 6: If the indicated k0 for cross-carrier scheduling is smaller than the minimum DL scheduling offset, the DCI is considered invalid.

## QCL Assumptions under Cross-Carrier Scheduling

In R15 for cross carrier scheduling of FR2 via FR1, it has been agreed in the last meeting that the TCI state should be indicated in the DCI and the offset should be larger than a threshold. This leads to scheduling restrictions at gNB and increased latency, as for this case the PDSCH in FR2 should be always scheduled with an offset larger than a threshold. In R16 this operation can be enhanced by allowing for cross-carrier scheduling with offset less than a threshold for the FR1-FR2 case. This requires specification of a rule for determining the default beam for PDSCH reception. The default rule for same carrier scheduling is a function of the TCI states used to monitor CORESETs. For cross-carrier scheduled case there may not be a CORESET monitored in the secondary cell for default rule determination. Additionally, there should a mechanism to update the default beam dynamically, via for instance MAC-CE to be robust to fast beam changes. Considering the above requirements our proposal is to define a dummy CORESET in the secondary cell. The default beam can be based on the TCI state associated with the dummy CORESET whenever the offset is less than a threshold. A mechanism to realize a dummy CORESET is to associate a search space to the CORESET, with the total number of PDCCH candidates to be monitored is set to 0.

Proposal 7: For the case where SCC in FR2 does not have a configured CORESET, and is cross carrier scheduled via DCI from a different carrier, a dummy CORESET can be configured in the SCC. If the offset between the reception of the DL DCI corresponding to the cross-carrier PDSCH is less than a threshold, the UE may assume the default beam for PDSCH reception is associated with the TCI state associated with the dummy CORESET.

- A dummy CORESET can be realized by associating a search space to a CORESET wherein the total number of PDCCH candidates that are monitored in the CORESET is set to zero.

## CCE/BD Limits & Overbooking Handling

PDCCH CCE/BD limits and overbooking rule are not defined for cross-carrier scheduling CA with mixed numerology. The main difficulty for this case is that a slot of the scheduling cell and a slot of the scheduled cell are not fully overlapping with each other if the two cells have different numerologies. Therefore, it is not so straightforward to define a “per slot” limit for cells with different numerologies when they are scheduled by the same cell. One way to resolve this problem is to convert the different numerology cross-carrier scheduling case into a same numerology cross-carrier scheduling case. In particular, a common reference numerology can be defined for cells (note: a scheduling cell is its own scheduled cell) scheduled by the same cell. Based on the reference numerology, CCE/BD limit can be defined as if all cells scheduled by the same cell have a numerology equal to the reference numerology. Since PDCCH is received by the UE in the scheduling cell, it is convenient to use the scheduling cell’s numerology as the reference numerology.

Previously, when overbooking is studied for cross-carrier scheduling with same numerology and self-scheduling with same or different numerologies, we have made the following conclusions:

- Overbooking is allowed only for PCell and PSCell.

- CCEs/BDs are separately counted for scheduled cells without considering the overlap between two PDCCH candidates that are associated with different scheduled cells.

- The non-CA CCE/BD limit is an upper bound of the per scheduled cell number of CCE/BD that the UE needs to process.

- The BD capability is split among cell groups of same numerology in proportion to the number of cells in the numerology group. The per numerology group BD/CCE limit is determined accordingly which is the second upper bound of the per scheduled cell CCE/BD number that the UE needs to handle.

Based on agreements and conclusions as well as the reference numerology, we have the following proposal for the limit of total CCE/BD number for each numerology group.

Proposal 8: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, calculate numbers X0, X1, X2, X3 and the corresponding T= X0 + X1 + X2 + X3 where Xi denotes the total number of CCs that are scheduled by CCs that have numerology i. For numerology i, the maximum total number of (BDs or CCEs) for these Xi CCs that the UE needs to process is given by

- Floor{Xi / T * (Mi or Ni) * y} per slot of numerology i, if T is > 4 and the UE reports a BD capability y < T

- Xi*(Mi or Ni) per slot of numerology i,  if T is <= 4 or the UE reports a BD capability y >= T

  - Where Mi and Ni represent the number of BDs and CCEs per slot of numerology i specified for non-CA case, respectively

  - Note: some of the values of Xi may be zero depending on the CA configuration.

The following proposal extends previous agreements that BDs/CCEs are separately counted and only PCell and PScell are allowed to be overbooked to mixed numerology cross-carrier scheduling CA.

Proposal 9: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, the number of BDs/CCEs of PDCCH candidates associated with each CC is counted separately for CCs without considering overlap between PDCCH candidates that are associated with different CCs.

- The number of BDs/CCEs of PDCCH candidates associated with PCell or PSCell does not exceed the limit for non-CA case.

- The number of BDs/CCEs of PDCCH candidates associated with PCell or PSCell does not exceed the limit for the numerology group associated with the numerology of the PCell or PSCell.

The following proposal extends previous agreements that SCells are not allowed to be overbooked to mixed numerology cross-carrier scheduling CA.

Proposal 10: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, network ensures that

- The number of separately counted BDs or CCEs of configured PDCCH candidates associated with each SCell does not exceed the respective non-CA case limit.

- Summation of the number of separately counted BDs or CCEs of mapped PDCCH candidates associated with PCell (PSCell) and the number of separately counted BDs or CCEs of configured PDCCH candidates associated with SCells in the set of Xi CCs does not exceed the per numerology group limit for numerology i.

# Conclusion

The following observation and proposals have been made on issues in CA and BWP.

Proposal 1: Support for cross-carrier scheduling with different numerologies can be prioritized as follows: The feature for carriers with small SCS cross-scheduling large SCS is supported in Rel-16. The feature for carriers with large SCS cross-scheduling small SCS can be deferred to Rel-17.

Proposal 2: A minimum DL scheduling offset should be introduced for Rel-16 to address the buffering and causality issues with cross-carrier scheduling with different numerologies.

Proposal 3: Timing determination for A-CSI-RS relative to the triggering grant can be a function of the configured A-CSI triggering offset and the minimum DL scheduling offset.

Proposal 4: For cross-carrier scheduling with CIF, the minimum DL scheduling offset should be defined such that the end of the PDCCH symbol on the scheduling carrier is before the start of the corresponding PDSCH (or A-CSI-RS) on the scheduled carrier.

Proposal 5: UE shall report the minimum DL scheduling that it can support in UE capability.

Proposal 6: If the indicated k0 for cross-carrier scheduling is smaller than the minimum DL scheduling offset, the DCI is considered invalid.

Proposal 7: For the case where SCC in FR2 does not have a configured CORESET, and is cross carrier scheduled via DCI from a different carrier, a dummy CORESET can be configured in the SCC. If the offset between the reception of the DL DCI corresponding to the cross-carrier PDSCH is less than a threshold, the UE may assume the default beam for PDSCH reception is associated with the TCI state associated with the dummy CORESET.

- A dummy CORESET can be realized by associating a search space to a CORESET wherein the total number of PDCCH candidates that are monitored in the CORESET is set to zero.

Proposal 8: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, calculate numbers X0, X1, X2, X3 and the corresponding T= X0 + X1 + X2 + X3 where Xi denotes the total number of CCs that are scheduled by CCs that have numerology i. For numerology i, the maximum total number of (BDs or CCEs) for these Xi CCs that the UE needs to process is given by

- Floor{Xi / T * (Mi or Ni) * y} per slot of numerology i, if T is > 4 and the UE reports a BD capability y < T

- Xi*(Mi or Ni) per slot of numerology i,  if T is <= 4 or the UE reports a BD capability y >= T

  - Where Mi and Ni represent the number of BDs and CCEs per slot of numerology i specified for non-CA case, respectively

  - Note: some of the values of Xi may be zero depending on the CA configuration.

Proposal 9: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, the number of BDs/CCEs of PDCCH candidates associated with each CC is counted separately for CCs without considering overlap between PDCCH candidates that are associated with different CCs.

- The number of BDs/CCEs of PDCCH candidates associated with PCell or PSCell does not exceed the limit for non-CA case.

- The number of BDs/CCEs of PDCCH candidates associated with PCell or PSCell does not exceed the limit for the numerology group associated with the numerology of the PCell or PSCell.

Proposal 10: For CA with cross-carrier scheduled CCs where at least two CCs have different numerologies, network ensures that

- The number of separately counted BDs or CCEs of configured PDCCH candidates associated with each SCell does not exceed the respective non-CA case limit.

- Summation of the number of separately counted BDs or CCEs of mapped PDCCH candidates associated with PCell (PSCell) and the number of separately counted BDs or CCEs of configured PDCCH candidates associated with SCells in the set of Xi CCs does not exceed the per numerology group limit for numerology i.

# References

- R1-1805200, “On remaining issues of carrier aggregation”, RAN1 #92bis, Ericsson

- RP-182574, “Correction to aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS”, RAN plenary #82, Ericsson

- R1-1900911, “UE Adaptation to the Traffic and UE Power Consumption Characteristics”, RAN1 AH 1901, Qualcomm