3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900933

Taipei, Taiwan, January 21-25, 2019

Agenda item:		7.2.6.4

Source:	Nokia, Nokia Shanghai Bell

Title:	URLLC downlink system-level performance evaluation for Factory Automation

Document for:		Discussion and Decision

# Introduction

The following use cases have been agreed for URLLC system-level performance evaluation [1]:

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Power distribution (22.804:5.6.4 &5.6.6) | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms | Power distribution grid fault and outage management |
|  | 99.999 | 15(end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs | Differential protection |
| Factory automation | 99.9999 | 2(end to end latency) Note: 1 ms air interface latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms | Motion control |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |
|  | 99.9 | 7ms (air interface delay) | DL & UL: 4096, 10 K FTP model 3 or periodic with different arrival rates |  |
| Transport Industry (22.186: 5.5) | 99.999 | 5 (end to end latency) Note: 3ms air interface latency | For UL: 2.5 Mpbs; Packet size 5220 bytes For DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model | Remote driving |
| Transport Industry (23.501, 22.261) | 99.999 | 10(end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps, Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model | Intelligent transport system (ITS) |

Details on the simulation assumptions were discussed and completed in RAN1#94bis meeting as captured in [1]. For factory automation use cases, our previous contribution [2] proposed parameter revisions of the InH channel model to better reflect the propagation conditions in industrial premises. Companies were encouraged to check the details in [2] and use the proposed configuration if possible and appropriate.

In this contribution, we present simulation results for the factory automation use case for the downlink direction and considering both the standard 3GPP InH propagation model [3] and the proposed revision in [2]. The applied simulation assumptions are inline with the agreements in [1] and can be found in Appendix A. Simulation results for the Power distribution use case can be found in our previous contribution [4].

This is a re-submission of our earlier RAN1#95 TDoc R1-1813119.

# Evaluated Propagation Models

Table 1 summarizes the InH channel model as defined in TR 38.901, and the proposed modification in [2]. The main differences are the following:

- LOS propagation: the modified InH assumes free space pathloss exponent (slope n=2) which is slightly larger than what is used for the standard InH model (slope n=1.73).

- NLOS propagation: the modified InH has significantly lower pathloss exponent (2.47 vs 3.83) and lower shadowing standard deviation (5.17 vs 8.03) than the standard InH model.

- LOS probability is also generally lower for the modified InH model. For instance, for a 2D distance of 20 meters between UE and cell, the LOS probability equals 0.81 for the InH model and 0.73 for the modified InH model. For a 2D distance of 40 meters, the LOS probability reduces to 0.61 and 0.39 for the InH model and the modified InH model, respectively.

Figure 1 (left) shows the RSRP distribution towards the serving cell and strongest interfering cell for the Factory automation use case. From a desired-signal perspective, lower received power is experienced for the modified InH model due to the larger LOS pathloss exponent. On the other hand, the experienced interference is also generally lower due to lower LOS probability towards the interfering cells, which compensates for the lower NLOS pathloss exponent of the modified InH model. As a result, the experienced UE channel quality is generally better for the modified InH model, as illustrated in the UE geometry distribution in Figure 1 (right).

Table 1: Comparison of InH and Modified InH propagation models

|  | InH |  | Modified InH ( Open production space - elevated gNB variant) |  |
| --- | --- | --- | --- | --- |
| LOS | PLInH-LOS=32.4+20log10(fc)+17.3log10(d3D) | σSF=3 dB | PLIndustrial-LOS=32.45+20log10(fc)+20log10(d3D) | σSF=3 dB |
| NLOS | PLInH-NLOS= max⁡(PLInH-LOS,PLInH-NLOS') PLInH-NLOS' =17.3+24.9log10(fc)+38.3log10(d3D) | σSF=8.03 dB | PLIndustrial- NLOS=32.45+20log10fc+24.7log10d3D | σSF=5.17 dB |
| LOS pr. | PrLOS=1exp-d2D-in-570.8exp-d2D-in-49211.7,d2D-in≤5 m,5 m<d2D-in≤49 m,49 m<d2D-in |  | PrLOS=1,d2D-in≤10 mexp-d2D-in-1031.6,d2D-in>10 m |  |

Figure 1: RSRP and UE geometry distribution for Factory Automation scenario.

# Downlink Simulation Results for Factory Automation

Within the factory automation umbrella of use cases, motion control has been selected as it is among the most challenging and demanding closed-loop control applications in industry [5]. The end-to-end (E2E) latency requirement is 2 ms, with the assumption of 1 ms RAN delay and 1 ms CN delay. Such E2E latency needs to be guaranteed with 99.9999% reliability.

Detailed simulation assumptions are described in Appendix A. The adopted network layout corresponds to the indoor open office scenario, where 12 cells are deployed with 20 meter inter-site distance (ISD) in a 120 m x 50 m area. The physical layer configuration consists of 30 kHz subcarrier spacing and 2 OFDM symbol mini-slot (duration 71.4 µs) transmissions. The carrier bandwidth is 40 MHz at the 4 GHz frequency band. The reported latency performance includes UE and gNB processing delays for both initial transmission and HARQ retransmissions as described in [6]. Simulation results are presented for 20 and 40 UEs per cell, assuming periodic downlink traffic with 1 or 2 ms periodicity and 32 Byte payload size. The offset of data arrival for each UE is assumed to be uniformly (random) distributed in the [0, traffic_period] interval. Three drops with randomized UE position and data arrival offsets are simulated.

Figure 2 shows the CCDF (complementary cumulative distribution function) of the one-way RAN latency for the packets of all UEs for 20 and 40 UEs per cell, and 2 ms (left) and 1 ms (right) arrival interval. At the 10-6-th percentile, the experienced performance depends on the number of experienced HARQ retransmissions. For 2 ms traffic periodicity, a single retransmission occurs and thus the 1 ms latency requirement is fulfilled; whereas cases with 1 ms periodicity and 40 UEs per cell experience up to two HARQ retransmissions at the 10-6-th percentile thus exceeding the latency target. Table 2 summarizes the UE outage ratio and corresponding resource utilization for the different simulated settings.

Observation 1: For 2 ms traffic periodicity, the 1 ms latency requirement can be fulfilled for 40 UEs per cell in the downlink direction. Reducing the traffic periodicity to 1 ms results in a UE outage rate between 2% and 7% depending on the propagation model.

Figure 2: Empirical latency distribution for 4 UEs per cell and different traffic loads. The CCDF comprises latency samples obtained from all the simulated UEs in the network.

Table 2: Summary of performance for the Factory Automation scenario. 4 UEs per cell. FTP3 traffic.

| Traffic setting (offered load per cell) |  | PRB utilization [%] |  | [%] UEs in outage* |
| --- | --- | --- | --- | --- |
| 20 UEs – 2 ms periodicity (2.56 Mbps) | InH | 5.07% | 0% |  |
|  | Modified InH | 5.03% | 0% |  |
| 40 UEs – 2 ms periodicity (5.12 Mbps) | InH | 10.2% | 0% |  |
|  | Modified InH | 10.1% | 0% |  |
| 20 UEs – 1 ms periodicity (5.12 Mbps) | InH | 10.2% | 0% |  |
|  | Modified InH | 10.1% | 0% |  |
| 40 UEs – 1 ms periodicity (10.24 Mbps) | InH | 20.7% | 7% |  |
|  | Modified InH | 20.3% | 2% |  |
| *Percentage of UEs that do not fulfil the requirements for factory automation. Reliability per UE is measured at the 99.99%-th percentile due to limited number of samples. |  |  |  |  |

Looking at the impact of the propagation model, the Modified InH proposal generally offers better latency and reliability performance than the standard InH model. As discussed in Section 2, the Modified InH proposal has lower LOS probability towards the interfering cells which generally results in lower inter-cell interference and higher SINR.

Observation 2: The Modified InH model generally offers better latency and reliability performance than the standard InH model. This is due to the lower LOS probability towards the interfering cells which generally results in lower inter-cell interference and higher SINR.

# Conclusion

In this contribution we have presented system-level simulation results of the downlink and reliability performance for the Factory Automation scenario for both the standard 3GPP InH propagation model and the proposed revision in [2]. The main findings are summarized with the following observations:

Observation 1: For 2 ms traffic periodicity, the 1 ms latency requirement can be fulfilled for 40 UEs per cell in the downlink direction. Reducing the traffic periodicity to 1 ms results in a UE outage rate between 2% and 7% depending on the propagation model.

Observation 2: The Modified InH model generally offers better latency and reliability performance than the standard InH model. This is due to the lower LOS probability towards the interfering cells which generally results in lower inter-cell interference and higher SINR.

# References

- RAN1#94bis Chairman’s notes, October 2018.

- R1-1810659, Field Measurement Results from an Operational Factory Floor at 3.5 GHz and 28 GHz, October 2018.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 14), v.14.3.0 (2017-12).

- R1-1810663, URLLC downlink system-level performance evaluation, October 2018.

- 3GPP TR 22.804 v16.0.0, Study on Communication for Automation in Vertical domains (CAV), July 2017.

- R1-1808449, IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 2 (URLLC), August 2018.

# Appendix A – Simulation assumptions for factory automation scenario

Table 3: System-level simulation assumptions for factory automation scenario.

| Parameter | Value |
| --- | --- |
| Network layout | 3GPP Indoor Open Office scenario; Indoor Hotspot (InH) propagation; 12 cells in 50 m x 120 m area and 20 m inter-site distance |
| Carrier bandwidth | 40 MHz @4 GHz; FDD duplexing |
| Total transmit power | 27 dBm |
| BS Antenna configuration | 4 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = 0.5λ, dV = 0.5λ; |
| BS antenna height | 10 m |
| BS antenna gain | 0 dBi |
| UE antenna configuration | 4 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = 0.5λ |
| UE antenna height | 1.5m |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| UE Receiver | MMSE-IRC; Single-stream transmission |
| Physical layer config. | 30 kHz subcarrier spacing. 12 subcarriers (360 kHz) per PRB. |
| TTI size | 2 OFDM symbols (71.4 µs) |
| MCS | QPSK to 64QAM |
| Link adaptation | Dynamic link adaptation for both control and data channels. Outer-loop link adaptation targeting 0.1% (10-3) BLER. Average control channel overhead of 20% - PDCCH not explicitly simulated |
| HARQ | Asynchronous HARQ with 14 OFDM symbols RTT and Chase-combining |
| CSI | CQI and PMI, reported every 5 ms; 2 ms processing delay at gNB. Subband size of 8 PRBs |
| UE deployment | 100% indoor randomly and uniformly distributed over the area; 3 km/h semi-static mobility |
| Traffic model | [20, 40] UEs per cell uniformly distributed in the indoor area. Periodic traffic with 32 Byte payload and 1 and 2 ms traffic periodicity; Random offset between UE’s packet transmission interval |
| TB Processing times: | According to [6]: BS processing delay: 2.75 OFDM symbols UE processing delay: 4.5 OFDM symbols HARQ RTT: 14 OFDM symbols Frame alignment: uniform distribution between [0, 2] OFDM symbols |
| Other assumptions: | Three drops with randomized UE position and data arrival offsets. No discarding of packets. |