3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	  	   R1-1900972

Taipei, 21st – 25th January, 2019

Source:	NTT DOCOMO, INC.

Title:	Enhancements to Scheduling/HARQ/CSI Processing timeline for URLLC

Agenda Item:	7.2.6.1.4

Document for: 	Discussion and Decision

- Introduction

In the RAN1 #95 meeting, following agreements were achieved related to the scheduling/HARQ/CSI processing timeline enhancement for URLLC [1]:

Agreements: - In order to evaluate the necessity to introduce a new N1/N2 timing capability in Rel. 16 eURLLC, the following aspects should be considered: - Perform latency analysis to identify the set of scheduling configuration parameters for which the eURLLC latency requirement(s) can/cannot be satisfied under the NR Rel. 15 timing capabilities. - To do this, the worst-case achievable latency should be considered. - Perform system-level and/or link-level evaluations to investigate the gains brought by reducing N1/N2 and allowing for more (re-)transmissions within the eURLLC latency budget. - For system-level evaluation, the performance metrics agreed for Rel. 16 eURLLC SI are applied. - For link-level evaluation, at least the resource efficiency, i.e., the average number of REs used for completing the transmission of a TB, should be reported. The number of transmissions for successfully decoding a TB and the target BLER for each transmission should be reported. - For both system-level and link-level evaluations, the simulation parameters agreed for Rel. 16 eURLLC SI are the baseline. - For all aspects, the comparison reference point is Rel. 15 NR capability timing 2 for FR1 and Rel. 15 NR capability timing 1 for FR2. - For all aspects, companies should report the assumed values for the following parameters: - Alignment latency - The considered N1/N2 values - SR periodicity in case the first PUSCH Tx is based on a dynamic grant - SR reception to initial PUSCH grant processing time at the gNB - PDCCH monitoring periodicity - The number of BDs/non-overlapping CCEs per monitoring occasion should be reported. - For the purpose of this study, the possibility of enhancing the number of non-overlapping CCEs/BDs for NR eURLLC can be considered. - Type-B time-domain allocation length for PDSCH/PUSCH channels - Time-domain allocation length for PDCCH, SR and PUCCH - UE and gNB PDSCH/PUSCH decoding time - The HARQ-ACK to reTx PDCCH and PUSCH to reTx PDCCH processing time at the gNB - The maximum number of possible PUCCH transmissions carrying HARQ-ACK per slot - Companies can report operation constraints (e.g., compact DCI, TB size, #RBs, #layers, #CCs, etc.) needed to enable reducing N1/N2. - Note: If TDD is assumed, the DL/UL configurations should be reported.

In this contribution, we share our views on scheduling/HARQ/CSI enhancement to support URLLC.

- Discussion

  - Scheduling/HARQ timeline for NR URLLC

Some of URLLC requirements are extremely high and are beyond the concepts of cellular/Wi-Fi types of networks in some sense. It is important for us to know whether the system can rely on HARQ re-transmission to design air-interface. In this section, we analyze the no. of transmissions that can be completed within the latency budget, especially for the tightest 1ms requirement. In our analysis, we consider following scenarios; (1) dynamic scheduling of PDSCH, (2) SR-based scheduling of PUSCH, and (3) configured grant PUSCH, for TDD configurations agreed in RAN4. Both FR1 and FR2 are considered. For FR1, as per RAN4 agreements [2], the TDD configuration of SU (S=10D:2G:2U) with SCS 30kHz is assumed and Rel.15 NR capability#2 is used; for FR2, as per RAN4 agreements [2], the TDD configuration DSUU (S=10D:2G:2U) with SCS 120kHz is assumed and Rel.15 NR capability#1 is used.

2.1.1 Downlink HARQ timeline

We analyze the downlink achievable latency of one transmission with HARQ-ACK feedback for both FR1 and FR2. The analysis is based on the following assumptions. Specifically, we assume that the gNB processing time is twice of the UE processing time, since the gNB is required to handle extra processes such as multi-user scheduling with appropriate packet prioritizations (e.g., prioritize latency bound critical UEs, prioritize SRB, etc), adaptive scheduling taking into account CSI feedback, beam-forming, HARQ operations, etc.

| FR1 | TDD | SU (10D:2G:2U) |
| --- | --- | --- |
|  | SCS | 30kHz |
|  | PDSCH length | 2 symbols |
|  | PDCCH overlap with PDSCH | 0 |
|  | PDCCH length | 1 symbol |
|  | PUCCH length | 1 symbol |
|  | UE processing time | 4.5 |
|  | gNB processing time | 2*4.5 |
|  | PDCCH monitoring periodicity | 2 symbols |
| FR2 | TDD | DSUU (10D:2G:2U) |
|  | SCS | 120kHz |
|  | PDSCH length | 2 symbols |
|  | PDCCH overlap with PDSCH | 0 |
|  | PDCCH length | 1 symbol |
|  | PUCCH length | 1 symbol |
|  | UE processing time | 20+3 |
|  | gNB processing time | 2*(20+3) |
|  | PDCCH monitoring periodicity | 2 symbols |

For TDD, DL/UL pattern may add extra delay; half-duplex constraint with the specific TDD configuration adds latency. Therefore, the following components are considered in the analysis.

| ID | Component | Description |
| --- | --- | --- |
| 1 | DL data transfer | (1.1+1.2+1.3+1.4) |
| 1.1 | DL frame alignment (control alignment) | The time interval b/w data arrives and DL assignment is transmitted |
| 1.2 | PDCCH transmission | The time duration for PDCCH transmission |
| 1.3 | DL Frame alignment (transmission alignment) | The time interval b/w DL assignment is transmitted and PDSCH is transmitted. |
| 1.4 | PDSCH transmission | The time duration for PDSCH transmission |
| 2 | HARQ retransmission | (2.1+2.2+2.3+2.4) |
| 2.1 | UE processing time | The time interval b/w PDSCH transmission and HARQ-ACK preparation |
| 2.2 | UL frame alignment time | The time interval b/w HARQ-ACK preparation and A/N transmission |
| 2.3 | A/N transmission | A/N transmission |
| 2.4 | gNB processing delay | The time interval b/w A/N transmission and A/N decoding |

Fig.1 shows an example on the downline HARQ timeline when packet arrives at the 1st symbol of the special slot. In this case, the latency for one downlink transmission with HARQ-ACK = (2+1+0+2+4.5+2.5+1+9) * 0.5/14=0.79ms. Considering the impact of TDD configuration, the latency may be different when the packet arrives at different time. Therefore, using the same approach, the latency for one downlink transmission with HARQ-ACK when packet arrives at different time for FR1 and FR2 are analyzed and the summary is given in table.1 and table.2. The detailed analysis can be found in the Appendix.

Fig.1 Downlink transmission latency when packet arrives at 1st symbol.

Table.1 Downlink transmission latency @ FR1

| DL latency [ms] | UE capability#2 (30kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PDSCH | 0.79 | 1.25 | 0.71 | 0.68 | 0.71 | 0.68 | 1.57 | 1.54 | 1.50 | 1.46 | 1.43 | 1.39 | 1.36 | 1.32 |
| 2-OS PDSCH w/o A/N | 0.24 | 0.21 | 0.24 | 0.21 | 0.24 | 0.21 | 0.88 | 0.84 | 0.84 | 0.81 | 0.78 | 0.74 | 0.71 | 0.68 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21st OS | 22nd OS | 23rd OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PDSCH | 1.29 | 1.25 | 1.21 | 1.18 | 1.14 | 1.11 | 1.07 | 1.04 | 1.00 | 0.96 | 0.93 | 0.89 | 0.86 | 0.82 |
| 2-OS PDSCH w/o A/N | 0.64 | 0.61 | 0.58 | 0.54 | 0.51 | 0.48 | 0.44 | 0.41 | 0.38 | 0.34 | 0.31 | 0.28 | 0.24 | 0.21 |

Table.2 Downlink transmission latency @ FR2

| DL latency [ms] | UE capability#1 (120kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PDSCH | 0.68 | 0.67 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 |
| 2-OS PDSCH w/o A/N | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PDSCH | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.97 | 0.96 | 0.96 | 0.95 | 0.94 | 0.93 | 0.92 | 0.91 |
| 2-OS PDSCH w/o A/N | 0.15 | 0.14 | 0.15 | 0.14 | 0.15 | 0.14 | 0.44 | 0.43 | 0.43 | 0.42 | 0.42 | 0.41 | 0.40 | 0.39 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PDSCH | 0.90 | 0.89 | 0.88 | 0.88 | 0.87 | 0.86 | 0.85 | 0.84 | 0.83 | 0.82 | 0.81 | 0.80 | 0.79 | 0.79 |
| 2-OS PDSCH w/o A/N | 0.38 | 0.37 | 0.36 | 0.35 | 0.34 | 0.33 | 0.33 | 0.32 | 0.31 | 0.30 | 0.29 | 0.28 | 0.27 | 0.26 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PDSCH | 0.78 | 0.77 | 0.76 | 0.75 | 0.74 | 0.73 | 0.72 | 0.71 | 0.71 | 0.70 | 0.69 | 0.68 | 0.67 | 0.66 |
| 2-OS PDSCH w/o A/N | 0.25 | 0.25 | 0.24 | 0.23 | 0.22 | 0.21 | 0.20 | 0.19 | 0.18 | 0.17 | 0.17 | 0.16 | 0.15 | 0.14 |

From table 1, it can be found that for FR1, depending on the packet arrival timing, the latency for one downlink transmission with HARQ-ACK can be more than 1ms, while the latency for one downlink transmission without HARQ-ACK is less than 1ms which is calculated by (1.1+1.2+1.3+1.4+2.1/2). Therefore, for factory automation on FR1, it can be said that it is not feasible to rely on HARQ re-transmission within the latency bound for TDD with the Rel.15 UE capability.

From table 2, it can be found that for FR2, in all cases, the latency for one downlink transmission with HARQ-ACK is less than 1ms, and in some cases, the one-shot PDSCH transmission with HARQ-ACK feedback is almost close to 1ms; when a packet arrives at 21st symbol, it becomes the largest value and is 0.97ms. In this case, the latency of DL transmission including DL re-transmission based on one HARQ-ACK retransmission is larger than 1ms. Therefore, it can be said that for factory automation on FR2, it is not possible to perform HARQ retransmission for TDD with Rel.15 UE capability.

Observation 1:

- For TDD on FR1, with Rel.15 UE capability#2, it is not possible to perform HARQ retransmission within 1ms latency.

- For TDD on FR2, with Rel.15 UE capability#1, it is not possible to perform HARQ retransmission within 1ms latency.

There are two directions. One is to design URLLC for factory automation such that the requirements are met without relying on HARQ re-transmission. Another is to enable further UE processing capability to enable faster HARQ re-transmission. Considering that the URLLC packet for factory automation is basically small enough and periodic/deterministic traffic is controllable by the NW, it would be possible to design the system such that HARQ re-transmission is not taken into account for achieving the factory automation requirements.

Proposal 1:

- Design URLLC for factory automation such that requirements are met without HARQ re-transmission.

- If retransmission is considered, the PDSCH processing time on both FR1 and FR2 should be reduced for NR URLLC.

2.1.2 SR-based UL transmission timeline

In this section, we analyze the UL achievable latency of one-shot SR-based UL transmission for both FR1 and FR2. The analysis is based on the following assumptions:

| FR1 | TDD | SU (10D:2G:2U) |
| --- | --- | --- |
|  | SCS | 30kHz |
|  | gNB processing time for SR | 2 symbols |
|  | PUSCH length | 2 symbols |
|  | PDCCH length | 1 symbol |
|  | SR length | 1 symbol |
|  | UE processing time | 5.5+1 |
|  | gNB processing time | 2*(5.5+1) |
|  | PDCCH monitoring periodicity | 2 symbols |
| FR2 | TDD | DSUU (10D:2G:2U) |
|  | SCS | 120kHz |
|  | gNB processing time for SR | 2 symbols |
|  | PUSCH length | 2 symbols |
|  | PDCCH length | 1 symbol |
|  | SR length | 1 symbol |
|  | UE processing time | 36+1 |
|  | gNB processing time | 2*(36+1) |
|  | PDCCH monitoring periodicity | 2 symbols |

For TDD, considering that DL/UL is not always available due to the TDD configuration, the following components are considered in the analysis.

| ID | Component | Description |
| --- | --- | --- |
| 1 | SR transmission | (1.1+1.2+1.3+1.4) |
| 1.1 | UL Frame alignment | The time interval b/w data arrives and SR is transmitted |
| 1.2 | SR transmission | The time duration for SR transmission |
| 1.3 | gNB processing time for SR | The time interval b/w SR is received, SR is decoded and scheduling decision is made |
| 1.4 | DL Frame alignment | The time interval b/w the scheduling decision is made and UL grant is transmitted |
| 2 | UL scheduling | (2.1+2.2+2.3+2.4) |
| 2.1 | UL grant | The time duration for UL grant transmission |
| 2.2 | UE processing time | The time interval b/w the UL grant is decoded and UL data is prepared |
| 2.3 | UL frame alignment | The time interval b/w the UL data is prepared and UL data is transmitted |
| 2.4 | PUSCH transmission | The time duration for PUSCH transmission |
| 2.5 | gNB processing time | The time interval b/w PUSCH is received and PUSCH is decoded |

Fig.2 shows an example on the SR-based UL scheduling timeline when packet arrives at the 1st symbol of the special slot. In this case, the latency for one-shot SR-based UL transmission = (12+1+2+13+1+6.5+2+13) * 0.5/14=1.96ms. Considering the impact of TDD configuration, the latency may be different when the packet arrives at different time. Therefore, using the same approach as DL latency analysis, the latency for one-shot SR-based UL transmission when packet arrives at different time for FR1 and FR2 are given in table3 and table 4. The detailed analysis can be found in the Appendix.

Fig.2 SR-based UL transmission latency when packet arrives at 1st symbol.

Table.3 SR-based UL transmission latency @ FR1

| UL latency [ms] | Capability#2(30kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PUSCH | 1.96 | 1.93 | 1.89 | 1.86 | 1.82 | 1.79 | 1.75 | 1.71 | 1.68 | 1.64 | 1.61 | 1.57 | 1.54 | 1.50 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 1.46 | 1.43 | 1.39 | 1.36 | 1.32 | 1.29 | 1.25 | 1.21 | 1.18 | 1.14 | 1.11 | 1.07 | 1.04 | 2.00 |

Table.4 SR-based UL transmission latency @ FR2

| UL latency [ms] | Capability#1(120kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PUSCH | 1.52 | 1.51 | 1.50 | 1.49 | 1.48 | 1.47 | 1.46 | 1.46 | 1.45 | 1.44 | 1.43 | 1.42 | 1.41 | 1.40 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 1.39 | 1.38 | 1.38 | 1.37 | 1.36 | 1.35 | 1.34 | 1.33 | 1.32 | 1.31 | 1.30 | 1.29 | 1.29 | 1.28 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 1.27 | 1.26 | 1.25 | 1.24 | 1.23 | 1.22 | 1.21 | 1.21 | 1.20 | 1.19 | 1.18 | 1.17 | 1.16 | 1.15 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 1.14 | 1.13 | 1.13 | 1.12 | 1.11 | 1.10 | 1.09 | 1.08 | 1.07 | 1.06 | 1.05 | 1.06 | 1.05 | 1.53 |

From table 3 and table 4, it can be found that for FR1, in all cases, the latency for one-shot SR-based UL transmission is more than 1ms. Therefore, for factory automation on FR1 and FR2, the one-shot SR-based UL transmission latency cannot be satisfied with Rel.15 UE capability.

Observation 2:

- For TDD on FR1 and FR2, with Rel.15 UE capability, the one-shot SR-based UL transmission latency cannot be satisfied.

Proposal 2:

- For SR-based UL transmission, if the requirement for factory automation is to be met, the PUSCH preparation time on both FR1 and FR2 should be reduced for NR URLLC.

2.1.3 Configured grant UL transmission timeline

In this section, we analyze the UL achievable latency of one-shot configured grant UL transmission for both FR1 and FR2. The assumptions used for the analysis are the same as SR-based UL transmission in section 2.1.2.

For TDD, considering that DL/UL is not always available due to the TDD configuration, the following components are considered in the analysis.

| ID | Component | Description |
| --- | --- | --- |
|  | UL scheduling time | (1.1+1.2+1.3) |
| 1.1 | Frame alignment | The time interval b/w UL data arrives and UL data is transmitted |
| 1.2 | PUSCH transmission | PUSCH duration |
| 1.3 | gNB processing time | The time interval b/w UL data is received and UL data is decoded |

Fig.3 shows an example on the configured grant UL transmission timeline when packet arrives at the 1st symbol of the special slot. In this case, the latency for one-shot configured grant UL transmission = (12+2+13) * 0.5/14=0.96 ms. Considering the impact of TDD configuration, the latency may be different when the packet arrives at different time. Therefore, using the same approach, the latency for one-shot configured grant UL transmission when packet arrives at different time for FR1 and FR2 are given in table5 and table 6. The detailed analysis can be found in the Appendix.

Fig.3 Configured grant UL transmission latency when packet arrives at 1st symbol.

Table.5 Configured grant UL transmission latency analysis @ FR1

| UL latency [ms] | Capability#2(30kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PUSCH | 0.96 | 0.93 | 0.89 | 0.86 | 0.82 | 0.79 | 0.75 | 0.71 | 0.68 | 0.64 | 0.61 | 0.57 | 0.61 | 0.57 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 1.04 | 1.00 |

Table.6 Configured grant UL transmission latency analysis @ FR2

| UL latency [ms] | Capability1(120kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| 2-OS PUSCH | 0.91 | 0.90 | 0.89 | 0.88 | 0.88 | 0.87 | 0.86 | 0.85 | 0.84 | 0.83 | 0.82 | 0.81 | 0.80 | 0.79 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 0.79 | 0.78 | 0.77 | 0.76 | 0.75 | 0.74 | 0.73 | 0.72 | 0.71 | 0.71 | 0.70 | 0.69 | 0.70 | 0.69 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-OS PUSCH | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.93 | 0.92 |

From table 5, it can be found that for FR1, the worst-case latency for one-shot configured grant UL transmission is more than 1ms. Therefore, for factory automation on FR1, the one-shot configured grant UL transmission latency cannot be satisfied with Rel.15 UE capability.

From table 6, it can be found that for FR2, in all cases, the latency for one-shot configured grant UL transmission is less than 1ms. On the other hand, latency for HARQ re-transmission would exceed 1ms at least for some cases, e.g., packet arrival at 55th OS. Therefore, for factory automation on FR2, it can be said that it is not possible to do retransmission for configured grant UL transmission based on dynamic scheduling with Rel.15 UE capability.

Observation 3:

- For TDD on FR1, with Rel.15 UE capability, the one-shot configured grant UL transmission latency can be satisfied within 1ms latency for most cases.

- For TDD on FR2, with Rel.15 UE capability, the one-shot configured grant UL transmission latency can be satisfied within 1ms latency for all cases.

- For TDD on both FR1 and FR2, it is not possible to perform retransmission scheduled by DCI for configured grant UL within 1ms latency.

Proposal 3:

- For configured grant UL transmission, if HARQ re-transmission is used within the latency bound of factory automation, PUSCH preparation time on FR1 should be reduced for NR URLLC.

- Otherwise, configured grant UL transmission for URLLC factory automation shall consider that it cannot rely on HARQ re-transmission.

  - CSI computation timing for NR URLLC

For CSI computation timing enhancement, shorter computation time is always beneficial. However, how much critical or the usefulness of CSI computation timeline improvements compared to other essential enhancements need to be carefully investigated, taking into account the amount of available TUs and Rel.16 timeline. Open loop link adaptation, periodic CSI and/or SP-CSI can still be used. In addition, A-CSI report can be used also, although the report may not be up-to-date if CSI computation timing is not enhanced. Our expectation is that, the current CSI computation timing can be acceptable for UE with limited/moderate mobility. For UEs with high mobility, it is up to gNB implementation whether to believe CSI feedback from the UE or to determine conservative MCS value. If the UE’s reported CQI index corresponds to MCS index which cannot achieve the target BLER e.g., 10-6, and if gNB relies on the report, the initial transmission will fail. For high mobility UE, if gNB considers CSI feedback is unreliable, then the gNB can choose a conservative specific CQI index, e.g., 0, always. Then the CSI report is not useful. Therefore, it is more important to consider how much CSI feedback is reliable.

Proposal 4:

- It is more important to study the CSI feedback reliability before study solutions to enhance the CSI computation timeline.

  - Out-of-order scheduling and HARQ-ACK feedback

For UEs with the baseline processing capability, for any two HARQ process IDs A and B for a given cell, if the scheduling DCI scrambled by C-RNTI for unicast PUSCH/PDSCH transmission A comes before (in time) the scheduling DCI scrambled by C-RNTI for unicast PUSCH/PDSCH transmission B, the UE is not expected to be scheduled such that PUSCH/PDSCH for B is before the PUSCH/PDSCH for A as illustrated in Fig.4. Similarly, for any two HARQ process IDs A and B for a given cell, if scheduled unicast PDSCH transmission for A comes before the scheduled unicast PDSCH transmission for B then the (baseline capability) UE is not expected to be triggered to send the HARQ-ACK for A after the HARQ-ACK for B as illustrated in Fig.5.

Fig. 4	Out-of-order scheduling.

Fig.5	Out-of-order HARQ feedback

These restrictions are reasonable only for the case where a single service type is operated. However, if a UE supports mixed service types, such as eMBB and URLLC services, the restriction is quite non-sense. For a UE supporting eMBB and URLLC services, different traffics with totally different timelines occur. If these restrictions are kept in Rel.16 NR URLLC, when a UE is operated with eMBB and URLLC, the URLLC traffic is restricted by eMBB traffic.

Proposal 5:

- Out-of-order scheduling and HARQ-ACK feedback should be enabled for Rel.16 URLLC.

- Conclusion

In this contribution, we discuss the scheduling/HARQ/CSI processing timeline enhancement for URLLC and following is the proposal summary:

References

- 3GPP RAN1 #95 meeting, chairman’s notes.

- R4-1809555, ‘Summary of operator input on the TDD patterns for NR UE performance tests in Rel-15,’ Ericsson, Verizon, AT&T, CMCC, NTT DOCOMO, KDDI.

Appendix

Table.1 Downlink transmission latency analysis @ FR1

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL assign. | 3th | 3th | 5th | 5th | 7th | 7th | 9th | 9th | 1st | 1st | 1st | 1st | 1st | 1st |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 20 | 19 | 18 | 17 | 16 | 15 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 19 | 19 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |
| 2.2 | 2.5 | 16.5 | 0.5 | 0.5 | 0.5 | 0.5 | 5.5 | 5.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| Total (no. of symbol) | 22 | 35 | 20 | 19 | 20 | 19 | 44 | 43 | 42 | 41 | 40 | 39 | 38 | 37 |
| Total [ms] | 0.79 | 1.25 | 0.71 | 0.68 | 0.71 | 0.68 | 1.57 | 1.54 | 1.50 | 1.46 | 1.43 | 1.39 | 1.36 | 1.32 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21st OS | 22nd OS | 23rd OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL assign. | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 2nd | 3rd | 4th | 5th |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |
| 2.2 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| Total (no. of symbol) | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 |
| Total [ms] | 1.29 | 1.25 | 1.21 | 1.18 | 1.14 | 1.11 | 1.07 | 1.04 | 1.00 | 0.96 | 0.93 | 0.89 | 0.86 | 0.82 |

Table.2 Downlink transmission latency analysis @ FR2

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3th | 3th | 5th | 5th | 7th | 7th | 9th | 9th | 11th | 11th | 13th | 13th | 15th | 15th |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| 2.2 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 |
| Total (no. of symbol) | 76 | 75 | 75 | 74 | 75 | 74 | 75 | 74 | 75 | 74 | 75 | 74 | 75 | 74 |
| Total [ms] | 0.68 | 0.67 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL assign. | 17th | 17th | 19th | 19th | 21th | 21th | 23th | 23th | 1st | 1st | 1st | 1st | 1st | 1st |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 34 | 33 | 32 | 31 | 30 | 29 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 33 | 33 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| 2.2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 |
| Total (no. of symbol) | 75 | 74 | 75 | 74 | 75 | 74 | 109 | 108 | 107 | 106 | 105 | 104 | 103 | 102 |
| Total [ms] | 0.67 | 0.66 | 0.67 | 0.66 | 0.67 | 0.66 | 0.97 | 0.96 | 0.96 | 0.95 | 0.94 | 0.93 | 0.92 | 0.91 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL assign. | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| 2.2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 |
| Total (no. of symbol) | 101 | 100 | 99 | 98 | 97 | 96 | 95 | 94 | 93 | 92 | 91 | 90 | 89 | 88 |
| Total [ms] | 0.90 | 0.89 | 0.88 | 0.88 | 0.87 | 0.86 | 0.85 | 0.84 | 0.83 | 0.82 | 0.81 | 0.80 | 0.79 | 0.79 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL assign. | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st | 1st |
| 1 | DL data transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | HARQ-ACK retransmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| 2.2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.4 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 | 46 |
| Total (no. of symbol) | 87 | 86 | 85 | 84 | 83 | 82 | 81 | 80 | 79 | 78 | 77 | 76 | 75 | 74 |
| Total [ms] | 0.78 | 0.77 | 0.76 | 0.75 | 0.74 | 0.73 | 0.72 | 0.71 | 0.71 | 0.70 | 0.69 | 0.68 | 0.67 | 0.66 |

Table.3 SR-based UL transmission latency analysis @ FR1

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS |  | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 1 |  | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 |
| 1.4 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 12 |  | 11 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |
| 2.2 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 |  | 6.5 |
| 2.3 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 |
| 2.5 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |  | 13 |
| Total (no. of symbols) | 55 | 54 | 53 | 52 | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 |  | 42 |
| Total [ms] | 1.96 | 1.93 | 1.89 | 1.86 | 1.82 | 1.79 | 1.75 | 1.71 | 1.68 | 1.64 | 1.61 | 1.57 | 1.54 |  | 1.50 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 13 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.4 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 1 | 0 | 13 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.2 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 | 6.5 |
| 2.3 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 2.5 | 2.5 | 4.5 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2.5 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| Total (no. of symbols) | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | 56 |
| Total [ms] | 1.46 | 1.43 | 1.39 | 1.36 | 1.32 | 1.29 | 1.25 | 1.21 | 1.18 | 1.14 | 1.11 | 1.07 | 1.04 | 2.00 |

Table.4 SR-based UL transmission latency analysis @ FR2

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.4 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.2 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
| 2.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2.5 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 170 | 169 | 168 | 167 | 166 | 165 | 164 | 163 | 162 | 161 | 160 | 159 | 158 | 157 |
| Total [ms] | 1.52 | 1.51 | 1.50 | 1.49 | 1.48 | 1.47 | 1.46 | 1.46 | 1.45 | 1.44 | 1.43 | 1.42 | 1.41 | 1.40 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 1 | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.4 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 26 | 25 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.2 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
| 2.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2.5 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 156 | 155 | 154 | 153 | 152 | 151 | 150 | 149 | 148 | 147 | 146 | 145 | 144 | 143 |
| Total [ms] | 1.39 | 1.38 | 1.38 | 1.37 | 1.36 | 1.35 | 1.34 | 1.33 | 1.32 | 1.31 | 1.30 | 1.29 | 1.29 | 1.28 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.4 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.2 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
| 2.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2.5 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 142 | 141 | 140 | 139 | 138 | 137 | 136 | 135 | 134 | 133 | 132 | 131 | 130 | 129 |
| Total [ms] | 1.27 | 1.26 | 1.25 | 1.24 | 1.23 | 1.22 | 1.21 | 1.21 | 1.20 | 1.19 | 1.18 | 1.17 | 1.16 | 1.15 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 27 |
| 1.2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.4 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 1 | 0 | 27 |
| 2 | UL scheduling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2.2 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
| 2.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2.4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2.5 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 128 | 127 | 126 | 125 | 124 | 123 | 122 | 121 | 120 | 119 | 118 | 119 | 118 | 171 |
| Total [ms] | 1.14 | 1.13 | 1.13 | 1.12 | 1.11 | 1.10 | 1.09 | 1.08 | 1.07 | 1.06 | 1.05 | 1.06 | 1.05 | 1.53 |

Table.5 Configured grant UL transmission latency analysis @ FR1

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 2 | 1 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| Total (no. of symbols) | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 17 | 16 |
| Total [ms] | 0.96 | 0.93 | 0.89 | 0.86 | 0.82 | 0.79 | 0.75 | 0.71 | 0.68 | 0.64 | 0.61 | 0.57 | 0.61 | 0.57 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 14 | 13 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| Total (no. of symbols) | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 29 | 28 |
| Total [ms] | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 1.04 | 1.00 |

Table.6 Configured grant UL transmission latency analysis @ FR2

| Packet arrival | 1st OS | 2nd OS | 3rd OS | 4th OS | 5th OS | 6th OS | 7th OS | 8th OS | 9th OS | 10th OS | 11th OS | 12th OS | 13th OS | 14th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 102 | 101 | 100 | 99 | 98 | 97 | 96 | 95 | 94 | 93 | 92 | 91 | 90 | 89 |
| Total [ms] | 0.91 | 0.90 | 0.89 | 0.88 | 0.88 | 0.87 | 0.86 | 0.85 | 0.84 | 0.83 | 0.82 | 0.81 | 0.80 | 0.79 |

| Packet arrival | 15th OS | 16th OS | 17th OS | 18th OS | 19th OS | 20th OS | 21th OS | 22th OS | 23th OS | 24th OS | 25th OS | 26th OS | 27th OS | 28th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 2 | 1 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 88 | 87 | 86 | 85 | 84 | 83 | 82 | 81 | 80 | 79 | 78 | 77 | 78 | 77 |
| Total [ms] | 0.79 | 0.78 | 0.77 | 0.76 | 0.75 | 0.74 | 0.73 | 0.72 | 0.71 | 0.71 | 0.70 | 0.69 | 0.70 | 0.69 |

| Packet arrival | 29th OS | 30th OS | 31st OS | 32nd OS | 33rd OS | 34th OS | 35th OS | 36th OS | 37th OS | 38th OS | 39th OS | 40th OS | 41st OS | 42nd OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 |
| Total [ms] | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 |

| Packet arrival | 43rd OS | 44th OS | 45th OS | 46th OS | 47th OS | 48th OS | 49th OS | 50th OS | 51st OS | 52nd OS | 53rd OS | 54th OS | 55th OS | 56th OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 28 | 27 |
| 1.2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 1.3 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Total (no. of symbols) | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 77 | 104 | 103 |
| Total [ms] | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.69 | 0.93 | 0.92 |