3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                     R1-1901007

Taipei, Taiwan, 21st – 25th January 2019

Source:	WILUS Inc.

Title:	Channel access procedures for NR-U operation

Agenda item:	7.2.2.2.1

Document for:	Discussion/Decision

- Introduction

During NR-U study item phase, channel access mechanisms for DL/UL channels and signals have been studied. The agreements were captured in the TR38.889v16.0.0 [1]. In the TSG-RAN#82, Rel-16 NR-Unlicensed WID has been approved [2].

Section 7.2.1.3.1 in the TR38.889 v16.0.0 … For CWS adjustment procedure in NR-U, in addition to aspects considered in LTE LAA, NR-U may additionally consider at least the following aspects: CBG based HARQ-ACK operation, NR scheduling and HARQ-feedback delays and processing times, wideband (>20 MHz) operation including BWPs, Configured grant operation. For initiation of a COT by the gNB (operating as an LBE device), the channel access schemes in Table 7.2.1.3.1-1 are used. Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device Cat 2 LBT; Cat 4 LBT. DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR); When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA); When DRS duty cycle is > 1/20, or total duration > 1 ms. DRS multiplexed with unicast data; N/A except for the cases discussed in the Note below; Channel access priority class is selected according to the multiplexed data. PDCCH and PDSCH; N/A except for the cases discussed in the Note below; Channel access priority class is selected according to the multiplexed data. Note: Applicability of an LBT scheme other than Cat 4 LBT for control messages related to initial/random access, mobility, paging, reference signals only, and PDCCH-only transmissions, e.g. "RACH message 4", handover command, GC-PDCCH, or short message paging transmitted either alone or when multiplexed with DRS have been discussed. Further details related to exceptions in this note can be determined when specifications are developed. For initiation of a COT by the UE, the channel access schemes in Table 7.2.1.3.1-4 are used. Table 7.2.1.3.1-4: Channel access schemes for initiating a COT by UE as LBE device Cat 2 LBT; Cat 4 LBT. PUSCH (including at least UL-SCH with user plane data); N/A except for the cases discussed in Note 2 below; Channel access priority class is selected according to the data. SRS-only; N/A; Cat4 with lowest channel access priority class value (as in LTE eLAA). RACH-only; (see Note 2); Cat4 with lowest channel access priority class value. PUCCH-only; (see Note 2); Cat4 with lowest channel access priority class value. Note 1: If the COT includes multiple signals/channels with different channel access categories / priority classes, the highest channel access priority class value and highest channel access category among the channel access priority classes and channel access categories corresponding to the multiple signals/channels applies. Note 2: Applicability of a channel access scheme other than Cat 4 for the following signals / channels have been discussed and details are to be determined when the specifications are developed: - UL control information including UCI only on PUSCH, e.g. HARQ-ACK, Scheduling Request, and Channel State Information - Random Access

In this contribution, we discuss channel access procedure for NR-U DRS and contention window mechanism considering CBG-based transmission for NR-U operation and provide our view.

Discussion on Channel Access Procedure for NR-U operation

Channel access schemes for NR-U DRS under initiation of a COT by gNB

During the study item phase, it was agreed that NR-U should have a signal containing at least SS/PBCH block burst set transmission and, furthermore, the inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) at the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial on the perspective of several aspects. In case of NR-U DRS with RMSI-CORESET(s) and PDSCH(s) carrying RMSI in one contiguous slot, however, it is necessary to further clarify and discuss the details of the channel access type for NR-U DRS transmission. Considering the NR-U standalone operation, since the transmission of both RMSI-CORESET and RMSI from the gNB and the reception of RMSI-CORESET and RMSI at the UE would be essential, it may be desirable to apply simplified LBT, e.g. Cat-2 LBT or Cat-4 LBT with the highest priority (i.e., channel access priority class #1) at the gNB for NR-U DRS including RMSI-CORESET and PDSCH carrying RMSI.

- Proposal 1: For NR-U DRS including RMSI-CORESET and PDSCH carrying RMSI, it may be desirable to apply simplified LBT, e.g. Cat-2 LBT or Cat-4 LBT with the highest priority (i.e., channel access priority class #1) at the gNB.

In addition, it was captured in the TR38.889 [1] that the transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial. For NR-U DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR), if the NR-U DRS duty cycle is equal to or less than 1/20 and the total duration is up to 1 ms, 25 µs Cat 2 LBT is used as in LAA. Otherwise, Cat-4 LBT is used for NR-U DRS. In that case for Cat-4 LBT in NR-U DRS transmission, it is necessary to further clarify which channel access priority class and contention window size should be used when there is no HARQ-ACK feedback for NR-U DRS multiplexed with non-unicast data (e.g. OSI, paging, RAR) transmission. Considering NR-U DRS for initial access and mobility, even if the NR-U DRS duty cycle exceeds 1/20 or the total duration exceeds 1ms, it may be desirable to use Cat-4 LBT with the highest priority (e.g. channel access priority class #1) and the shortest CWS (e.g. {3}) of allowed CWp sizes such as {3, 7} of the channel access priority class #1 in LTE-LAA. For the case of NR-U DRS multiplexed with non-unicast data having HARQ-ACK feedback, it may be desirable to apply the channel access priority class according to the multiplexed non-unicast data as similar to method for NR-U DRS multiplexed with unicast data

- Proposal 2: It is necessary to further clarify which channel access priority class and contention window size should be used when there is no HARQ-ACK feedback for NR-U DRS multiplexed with non-unicast data (e.g. OSI, paging, RAR) transmission.

- Proposal 3: We propose to apply the highest priority and the shortest fixed CWS to NR-U DRS multiplexed with non-unicast data (e.g. OSI, paging, RAR) transmission without HARQ-ACK feedback.

For applicability of an LBT scheme other than Cat 4 LBT for control messages related to initial/random access, mobility, paging, reference signals only, and PDCCH-only transmissions, e.g. "RACH message 4", handover command, GC-PDCCH, or short message paging transmitted either alone or when multiplexed with DRS, similar to LTE-LAA, it should use Cat-4 LBT as channel access procedure for transmission(s) including PDSCH/PDCCH except channel access procedure for transmission including NR-U DRS and not including PDSCH.

CWS update mechanism

For NR, in contrast to LTE-LAA, CBG-based transmission and CBG-based HARQ-ACK feedback were adopted for the reduced HARQ processing delay. In addition, NR also supports the flexible DL/UL scheduling and corresponding HARQ-ACK timing according to the UE capability. Therefore, considering the NR-based scheduling and CBG transmission with relevant HARQ feedback/processing times for DL at the gNB and UL at the UE, the channel access mechanisms for DL and UL used in the LTE-LAA needs to be modified. In particular, when CBG-based transmission is used for NR-U operation, it is necessary to deal with TB- and CBG-based A/Ns, separately, in calculating the NACK ratio for updating CWS, due to the fact that the gNB can receive TB-based A/Ns and CBG-based A/Ns from the UE in a given time depending on the UE-specific transmission methods.

- Proposal 4: When CBG-based transmission is used for NR-U operation, it is necessary to deal with TB-based A/Ns and CBG-based A/Ns separately, in calculating the NACK ratio for updating CWS.

It should be also discussed how to determine the reference slot for updating CWS. Similar to LTE-LAA, the reference slot can be the starting slot of the most recent transmission on the carrier made by the gNB, for which at least some HARQ-ACK feedbacks are expected to be available. Since both TB- and CBG-based HARQ-ACK corresponding PDSCH in the reference slot can be supported for NR-U operation, the following alternatives should be further investigated when calculating the NACK ratio in the CWS adjustment procedure for NR-U.

- Alt-1: If at least one of CBG-based HARQ-ACKs corresponding PDSCH(s) in a reference slot is ACK, HARQ-ACK feedback(s) corresponding the PDSCH(s) can be counted as ACK(s), otherwise, NACK(s).

- Alt-2: If the first one of CBGs corresponding PDSCH(s) in a reference slot is NACK, HARQ-ACK feedback(s) corresponding the PDSCH(s) can be counted as NACK(s), otherwise, ACK(s).

- Alt-3: If all of CBG-based HARQ-ACKs corresponding PDSCH(s) in a reference slot is ACK(s), HARQ-ACK feedback(s) corresponding the PDSCH(s) is counted as ACK(s), otherwise, NACK(s).

Conclusion

In this contribution, we have discussed the channel access procedure for NR-DRS transmission and CWS adjustment mechanism considering NR CBG-based transmission for NR-U operation. We summarize our view as follows.

- Proposal 1: For NR-U DRS including RMSI-CORESET and PDSCH carrying RMSI, it may be desirable to apply simplified LBT, e.g. Cat-2 LBT or Cat-4 LBT with the highest priority (i.e., channel access priority class #1) at the gNB.

- Proposal 2: It is necessary to further clarify which channel access priority class and contention window size should be used when there is no HARQ-ACK feedback for NR-U DRS multiplexed with non-unicast data (e.g. OSI, paging, RAR) transmission.

- Proposal 3: We propose to apply the highest priority and the shortest fixed CWS to NR-U DRS multiplexed with non-unicast data (e.g. OSI, paging, RAR) transmission without HARQ-ACK feedback.

- Proposal 4: When CBG-based transmission is used for NR-U operation, it is necessary to deal with TB-based A/Ns and CBG-based A/Ns separately, in calculating the NACK ratio for updating CWS.

References

[1] 3GPP TR 38.889 v16.0.0, “Study on NR-based access to unlicensed spectrum (Release 16)”

[2] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”	Qualcomm