3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1901036

Taipei, Taiwan, January 21st – 25th, 2019

Title:	[DRAFT] LS on SCS of SSB and SS raster for NR-U bands

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG1

To:	RAN WG4

Cc:	-

Contact Person:

#### Name:	Hongbo Si

E-mail Address:	Hongbo_dot_si_at_samsung_dot_com

1. Overall Description:

RAN1 has agreed to support single SCS for SS/PBCH block on an unlicensed PCell per band, and has discussed the value of single SCS at least for 5 GHz unlicensed band. It is RAN1’s understanding that, at least for 5 GHz unlicensed band, supporting 30 kHz SCS for SS/PBCH block on an unlicensed PCell is beneficial in achieving better initial cell search performance and smaller time-domain overhead, comparing to 15 kHz. Hence, RAN1 recommends, at least for 5 GHz unlicensed band, supporting 30 kHz as the single SCS for SS/PBCH block on an unlicensed PCell. RAN1 respectfully asks RAN4 to provide feedback if the recommendation conflicts with RAN4’s decision.

In RAN1’s understanding, the SS raster design in RAN4 highly depends on the selection of the SCS of SS/PBCH block per band, and also depends on the channel raster design for that given band. For NR-U, the channelization shall be determined subject to the regulation of a given unlicensed band, which could be more restricted comparing to the channelization in NR Rel-15. Hence, RAN1 expects the channel raster design and SS raster design for NR-U could follow different principles from NR Rel-15. To progress work in RAN1, it is important to understand whether the potential changes to the channel raster design and SS raster design in RAN4 could impact the configuration of CORESET #0 in RAN1. Based on the above discussion, RAN1 respectfully asks RAN4 to strive for completing the channel raster design and SS raster design for NR-U in an early stage of the WI.

2. Actions:

To RAN WG4.

ACTION: RAN WG1 kindly asks RAN WG4 to take above considerations for their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #96 	   25th Feb – 1st Mar 2019	Athens, Greece.

TSG-WG1 Meeting #96bis  8th – 12th Apr 2019	China