3GPP TSG RAN WG1 AH-1901	R1-1901080

Taipei, Taiwan, 21st – 25th January 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: FD compression unit

Document for:	Discussion and Decision

# Introduction

In RAN1#95, the DFT-based frequency domain (FD) compression was agreed as the supported mechanism for Type II overhead reduction for rank 1-2 [1]. The following agreement was made about the FD compression unit.

Agreement: In RAN1 NR-AH 1901, select one of the following alternatives for precoder/PMI FD compression unit, taking into account UPT vs. overhead and complexity - Alt1. Subband (SB), wherein the SB size for precoder/PMI compression is the same as the CQI subband size - Alt2. X resource blocks (RBs), different from CQI subband size. Three sub-alternatives - Alt2.1 X = 1 - Alt2.2 X = CQI SB size / R where R>1 is a predetermined integer - Only one R value is supported. FFS: the value of R - Alt2.3 X = {2, 4} where X is higher-layer configured Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for quantization for evaluation purposes.

This contribution provides simulation results to compare the alternatives for FD compression unit and makes conclusions in support of our proposal in [2].

# Simulation results for FD compression unit

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in medium (50% target RU for 10 MHz BW) and low (30% target RU for 100 MHz BW) traffic loading scenarios, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. The results are provided in Figure 1 and Figure 2 for the following parameters.

- Spatial compression: L = 4

- Frequency compression: M = {2, 4, 6, 8}

- Coefficient quantization: (Amp, phase) = (3,3) bits

- (CQI SB size, PMI SB size) = (x, y) according to table below.

| BW (MHz) | CQI SB size (x) | PMI SB size (y) |
| --- | --- | --- |
| 10 | 4 | 4,2,1 |
| 100 | 16 | 16,8,4,2,1 |

As reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered. We can observe the following.

Observation:

- For 10 MHz BW, difference between y = 1, 2, and 4 are small.

- For 100 MHz BW, difference between y = 1, 2, 4, 8, and 16 are large.

- The performance-overhead tradeoff improves then gets worse as y decreases, which implies that there exists an optimum value for y.

- y = x/R with R = 1 or 2 achieves the best performance-overhead trade-off.

Figure 1: Performance-overhead trade-off for 10 MHz BW

Figure 2: Performance-overhead trade-off for 100 MHz BW

# Conclusions

In this contribution, simulation results are provided for FD compression unit. The observations made are summarized as follows.

Observation:

- For 10 MHz BW, difference between y = 1, 2, and 4 are small.

- For 100 MHz BW, difference between y = 1, 2, 4, 8, and 16 are large.

- The performance-overhead tradeoff improves then gets worse as y decreases, which implies that there exists and optimum value for y.

- y = x/R with R = 1 or 2 achieves the best performance-overhead trade-off.

# References

- Chairman’s notes, RAN1#95

- R1-1900566, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz, 10/100 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% for 10 MHz BW, 30% for 100 MHz BW |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |