3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		                                      R1-1901145

Taipei, Taiwan, 21st – 25th January, 2019

Agenda item:	7.2.4.1.1

Source:		ITL

Title:		Physical layer structure for NR V2X

Document for:	Discussion and Decision

Introduction

In RAN1#95, there was significant progress on the sidelink physical layer structure design with numerology, waveform, SL-BWP/resource pool, multiplexing between PSCCH/PSSCH and AGC/Tx-Rx switching time [1]. There are still remaining issues that should be discussed in RAN1 to resolve it as followings:

- Support of CP-OFDM only or both

- Support of additional CP length and SCS

- Details on PSCCH/PSSCH multiplexing

- Details on SL BWP and resource pool

- Need on gap for AGC and switching time and the details

- Sidelink physical layer channel and RS design

In this contribution, we discuss the remaining issues in above and provide our views.

Numerology and Waveform

It was agreed in RAN1#95 that:

Agreements:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

Agreements:

- For PSCCH/PSSCH in FR1, NR V2X supports normal CP for 15kHz, 30kHz, 60kHz, and extended CP for 60kHz.

  - FFS extended CP for 30 kHz in FR1.

- FFS CP for PSCCH/PSSCH in FR2

  - E.g., NR V2X supports normal CP for 60kHz and 120kHz, and extended CP for 60kHz

    - FFS extended CP for 120 kHz in FR2.

- Only one combination of CP length and SCS is used in a carrier at a given time for NR V2X UEs communicating with each other using SL

For numerology, currently NR V2X supports normal CP for 15kHz, 30kHz and 60kHz for FR1. One remaining FFS is the necessity of extended CP for 30kHz in FR1. Since the extended CP is mainly for handling larger delay spreads and tolerable larger synchronization errors with a cost of higher overhead than normal CP, its necessity should be only focused on the case where larger communication range (e.g. 1000m in extended sensor case) is required for NR V2X. When considering that the extended CP for 60kHz SCS was already supported for that purpose in FR1, it seems there was no strong motivation to introduce the extended CP for 30kHz SCS which result in the potential performance degradation on data rate due to larger CP overhead in FR1.

In addition, Extended CP for 120kHz SCS in FR2 is not needed, because it is expected that NR V2X communication in FR2 is mainly useful for unicast/groupcast transmission with the limited communication range that may have V2X data traffic with higher data rate and lower latency. Thus, it is enough to use normal CP only in FR2 similar with NR Uu.

Table 1. Possible NR SL Numerology

| Subcarrier spacing (kHz) | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- |
| OFDM symbol length (μs) | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 |
| Normal CP length (μs) | 4.69 | 2.34 | 1.17 | 0.59 | 0.29 |
| Extended CP length (μs) | - | 8.34 | 4.17 | 2.085 | - |

Proposal 1:	Support following NR V2X numerology additionally including PSFCH:

- For PSCCH/PSSCH/PSFCH in FR1, NR V2X supports normal CP for 15kHz, 30kHz, 60kHz, and extended CP for 60kHz.

- For PSCCH/PSSCH/PSFCH in FR2, NR V2X supports normal CP for 60kHz and 120kHz, and extended CP for 60kHz

Regarding waveform, it is true that DFT-s-OFDM provides a UE with lower PAPR, so a UE can have more Tx power margin translating to larger communication range. However, NR V2X UEs are generally vehicles mounted UEs that has no concern on power consumption and equips better Tx power amplifier efficiency than hand-handled UE (e.g. smart phone), DFT-s-OFDM is not necessarily needed for NR V2X. Also, DFT-s-OFDM results in less flexible resource allocation, and thus there will be some performance degradation compared to the CP-OFDM especially for 16-QAM and 64-QAM at different speeds in both FR1 and FR2 [2], despite of the larger communication range. Moreover, supporting both waveforms (i.e. CP-OFDM and DFT-s-OFDM) will cause additional signaling and configurations on which waveform is assumed at a receiver side, and thus the NR V2X UEs will suffer more complex implementation and operations than only one waveform. In terms of specification works, that would also be problem on the related physical layer designs that are separately handled between CP-OFDM and DFT-s-OFDM. Consequently, single waveform (i.e. CP-OFDM) is preferred.

Proposal 2:	Support CP-OFDM only as a single waveform

PSCCH / PSSCH multiplexing

Figure 1. Options of the PSCCH / PSSCH multiplexing

It was agreed as a working assumption that PSCCH/PSSCH multiplexing with option 3 among the multiplexing options in above figure is at least supported for CP-OFDM as seen in below [1].

Working assumption:

- Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM.

  - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3.

  - FFS how to determine the starting symbol of PSCCH and the associated PSSCH

  - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

The main motivation to support option 3 at least for option 3 is to achieve the highest resource utilization between TDM options and the lower latency than FDM (option 2), while option 1B may possibly require a transient period if there is a large power imbalance between PSCCH and PSSCH, because option 1B has a different resource utilization than other options. It was finally agreed in last RAN1 meeting to send a LS to RAN4 to ask the need of the transient period in the above-mentioned options and are waiting for their response [3].

One of remaining questions is whether to additionally support the other options on top of option 3. It has been concerned the PSCCH coverage in option 3 as a possible drawback, since in the extreme cases, the PSCCH coverage may not be enough for NR SL communication. However, it can be compensated by PSCCH power boosting and proper power control mechanism to be defined for NR V2X, while one of the other options (e.g. option 2) can be considered if above approach is not feasible for the reliable PSCCH coverage. Thus, it seems option 3 is currently sufficient to support the variable NR V2X use cases.

Regarding starting symbol of PSCCH and the associated PSSCH, it is related to the PSCCH monitoring occasion and slot/non-slot structure to be used for NR V2X, based on PSCCH/PSSCH multiplexing scheme. In order to reduce UE blind decoding efforts of PSCCH reception in time domain, the candidate starting symbols of PSCCH should be known by a receiving UE. One possible way is to configure the PSCCH monitoring occasion and a slot format during a session/RRC configuration establishment in case of unicast/groupcast, and for the other cases (e.g. broadcast), can be pre-defined or pre-configured if there is no the configuration. This kind of configuration can be associated with a resource pool.

Proposal 3:	Support option 3 only on PSCCH/PSSCH multiplexing

PSFCH format stucture

In RAN1#94bis and 95[1][4], following agreements were made:

RAN1#94bis

Agreements:

Sidelink control information (SCI) is defined.

SCI is transmitted in PSCCH.

SCI includes at least one SCI format which includes the information necessary to decode the corresponding PSSCH.

NDI, if defined, is a part of SCI.

Sidelink feedback control information (SFCI) is defined.

SFCI includes at least one SFCI format which includes HARQ-ACK for the corresponding PSSCH.

FFS whether a solution will use only one of “ACK,” “NACK,” “DTX,” or use a combination of them.

FFS how to include other feedback information (if supported) in SFCI.

FFS how to convey SFCI on sidelink in PSCCH, and/or PSSCH, and/or a new physical sidelink channel

FFS in the context of Mode 1:

whether/how to convey information for SCI on downlink

whether/how to convey information of SFCI on uplink

RAN1#95

Agreements:

- Physical sidelink feedback channel (PSFCH) is defined and it is supported to convey SFCI for unicast and groupcast via PSFCH.

Since it was agreed to define PSFCH to convey SFCI for both unicast and groupcast, RAN1 should discuss the details on PSFCH format. There can be two PSFCH structure design options that option 1 is to modify one or some of NR PUCCH formats, while option 2 is to modify PSSCH structure that is possibly based on NR PUSCH structure. It is considered that both options are basically trying to reuse the existing NR physical layer channel as much as possible. For option 1, it would be well matched to contain a smaller number of SFCI bits (e.g. HARQ-ACK 1 or 2bits only). Same channel coding scheme as NR PUCCH can be reused due to similar number of feedback information bits. Also, the multiplexing capacity is much better than option 2 which may rely on shared channel design for higher data rate. In addition, option 1 could be well supported for feedback transmission on groupcast transmission because Tx UE may be able to identify which Rx UE fails to decode the data/control by using same time/frequency resources for PSFCH transmission among Rx UEs. For option 2, it has advantage to make a common SL physical layer channel structure among SL physical layer channels (i.e. PSCCH/PSSCH/PSFCH), which is possibly based on NR PUSCH structure. One of major concerns on option 2 is whether smaller number of feedback information is supported or not since it is not desirable to use e.g. HARQ-ACK 1 or 2 bits to be contained in a PSFCH format based on option 2. Therefore, as in NR Uu, variable number of SFCI size should be considered, and thus at least two PSFCH formats should be supported for both less and equal SFCI 2bits (≤2) and more than 2bits ( >2 bits). In case of smaller number of SFCI (e.g. ≤2), NR PUCCH format 0 and 1 can be good starting point to design one of PSFCH format while for larger number of SFCH (e.g. >2), there can be multiple ways including NR PUCCH format 2 based, PSSCH based or etc. Thus, RAN1 needs to consider both options for further comparison when PSFCH details is discussed.

Observation 1:	For PSFCH formats, it is required to support multiple PSFCH formats to contain the variable SFCI sizes

Proposal 4:	For smaller number of SFCI (e.g. ≤2), NR PUCCH format 0 or 1 can be a good starting point for PSFCH format design. For larger number of SFCI (e.g. >2), it is FFS

In NR SL mode 1, gNB schedules the SL transmission resources to a Tx UE. Since for unicast/groupcast, HARQ-ACK feedback in SL is defined between V2X UEs, gNB which schedules SL transmission resources to a Tx UE needs to know whether the scheduled SL data is correctly received at a Rx UE in order to properly make a decision on the potential scheduling works on the retransmission if failed. Therefore, it is beneficial to forward the feedback information to the gNB via either Tx UE or Rx UE.

Proposal 5:	In NR V2X mode 1, it is beneficial to consider that HARQ-feedback information for SL unicast/groupcast transmission can be forwarded to a gNB via either Tx UE or Rx UE using NR-Uu

DMRS design for PSCCH/PSSCH

DMRS design is currently discussing taking into account the range of waveform, SCS, channel type, etc. for NR V2X. For PSCCH/PSSCH DMRS design, following multiplexing options can be considered as a starting point as shown in Figure 2.

Figure 2. multiplexing of PSCCH and PSSCH in NR V2X

Particularly for Option 3 that was at this time agreed at least for CP-OFDM, two types of DMRS allocation for PSCCH and PSSCH could be considered as seen in Figure 3.

    - Type 1: Separated DMRS design for PSCCH/PSSCH

      - PSCCH: demodulated with PSCCH DMRS, PSSCH: demodulated with PSSCH DMRS

    - Type 2: Common DMRS design for PSCCH/PSSCH

      - PSCCH: demodulated with PSCCH DMRS, PSSCH: demodulated with PSCCH DMRS & PSSCH DMRS

Figure 3. Two types of DMRS allocation for PSCCH and PSSCH in Option3 of Figure X

Type 2 (Common DMRS could have several benefits. For example, 1) possibility of faster demodulation of PSSCH (from first symbol), 2) possibility of less PSSCH DMRS resources (by using PSCCH DMRS resources), 3) possibility of efficient resource allocation in non-slot based allocation.

For this, common DMRS sequence mapping (to all allocated PRBs of PSCCH and PSSCH in a symbol) and common(or similar) DMRS pattern design for PSCCH DMRS and PSSCH DMRS should be further studied.

Proposal 6:	Common design for PSCCH DMRS and PSSCH DMRS should be further studied

AGC and Tx-Rx switching

In last RAN1 meeting, there are some proposals to be more efficient resource utilization for the AGC and Tx-Rx switching time assuming that AGC and Tx-Rx switching time can be much less than one symbol depending on SCS. To get more clear assumption, RAN1 sent LS to ask it. In response LS from RAN4, following answer for the questions from RAN1 has been provided especially for the AGC and Tx/Rx switching time [5].

- AGC settling time

  - At least AGC settling time = 1 OFDM symbol for 15 kHz SCS is feasible.

  - RAN4 will further study if smaller AGC time is achievable incl. 15 kHz SCS.

- TX/RX switching time

  - FR1: 13 us

  - FR2: 7us

Considering above response, it seems RAN4 needs also some more time to conclude the required AGC settling time depending on numerology. Thus, RAN1 needs to wait for RAN4 conclusion for AGC time.

Regarding potential solution for overhead reduction if smaller AGC time is assumed than 1 OFDM symbol, there are some proposals as followings:

- option 1. AGC/data and Tx-Rx switching/data in two OFDM symbols, respectively

- option 2. Merging AGC/Tx-Rx switching in one OFDM symbol

- option 3. Switching between option 1/2 and LTE-V2X way (i.e. first symbol for AGC and last symbol for Tx-Rx switching, depending on SCS

Basically, option 1 may result in potential performance impacts from smaller number of data/control REs in the OFDM symbol due to either AGC or Tx-Rx switching, and additional specification and implementation efforts may be foreseen in order to handle the different OFDM symbol structure (e.g. comb type) over general OFDM symbol in terms of Physical channel/RS design, etc. So, it is preferred to use one symbol entirely for AGC and/or Tx-Rx switching like option 2 or LTE-V2X-like.

Proposal 7:	It can be considered to adopt merging AGC/Tx-Rx switching in one OFDM symbol or LTE-V2X-like way

Conclusion

This contribution discusses the synchronization mechanism for NR V2X and makes the following proposal:

Proposal 1:	Support following NR V2X numerology including PSFCH:

- For PSCCH/PSSCH/PSFCH in FR1, NR V2X supports normal CP for 15kHz, 30kHz, 60kHz, and extended CP for 60kHz.

- For PSCCH/PSSCH/PSFCH in FR2, NR V2X supports normal CP for 60kHz and 120kHz, and extended CP for 60kHz

Proposal 2:	Support CP-OFDM only as a single waveform

Proposal 3:	Support option 3 only on PSCCH/PSSCH multiplexing

Observation 1:	For PSFCH formats, it is required to support multiple PSFCH formats to contain the variable SFCI sizes

Proposal 4:	For smaller number of SFCI (e.g. ≤2), NR PUCCH format 0 or 1 can be a good starting point for PSFCH format design. For larger number of SFCI (e.g. >2), it is FFS

Proposal 5:	In NR V2X mode 1, it is beneficial to consider that HARQ-feedback information for SL unicast/groupcast transmission can be forwarded to a gNB via either Tx UE or Rx UE using NR-Uu

Proposal 6:	Common design for PSCCH DMRS and PSSCH DMRS should be further studied

Proposal 7:	It can be considered to adopt merging AGC/Tx-Rx switching in one OFDM symbol or LTE-V2X-like way

References

- Chairman’s Note 3GPP TSG RAN WG1 #95, Spokane, USA, Nobemver 2018

- R1-1812206, " Sidelink physical layer structure for NR V2X ". Huawei, HiSilicon, Spokane, USA, Nobemver 2018

- R1-1814165, " LS on transient period in sidelink channels ". RAN WG1, Spokane, USA, Nobemver 2018

- Chairman’s Note 3GPP TSG RAN WG1 #94bis, Cheong-du, China, October 2018

- R1-1900010, " Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects ". RAN WG1, Taipei, Taiwan, January 2019