3GPP TSG RAN WG1 Meeting AH-1901			R1-1901151

Taipei, Taiwan, January 21th – 25th, 2018

Agenda item:		7.2.6.1.2

Source:		Fraunhofer HHI, Fraunhofer IIS

Title:		Enhanced HARQ-ACK for NR URLLC

Document for:	Discussion and Decision

# Introduction

In RAN#80, the following was agreed [1]:

URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified, - PDCCH enhancements. Study focus on Compact DCI, PDCCH repetition, increased PDCCH monitoring capability. - UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements. - PUSCH Enhancements. Study focus on mini-slot level hopping & retransmission/repetition enhancements. - Enhancements to scheduling/HARQ/CSI processing timeline (UE and gNB), (for existing TTI durations).

During the RAN1#95 meeting the following open issues still remain regarding the UCI enhancements.

- How to separate HARQ-ACK multiplexing windows for different PUCCHs? - How to indicate the starting symbol of different PUCCHs? - How to indicate K1, e.g. in unit of slot, half-slot, a number of symbols or symbol? - How to determine dynamic HARQ codebook? - How to determine semi-static HARQ-ACK codebook? - How to configure PUCCH resource sets, e.g. reuse R15 PUCCH resource set configurations or not? - How to determine PUCCH resource for each PUCCH? - How to do PUCCH resource overriding for HARQ-ACK multiplexing? - Maximum number of PUCCH transmissions forHARQ-ACK allowed in a slot?

In this contribution, we specifically focus on discussing potential improvements on the HARQ-ACK reporting on UCI.

# Motivation

Mixed operation of URLLC and eMBB by the same UE is expected to be one of the major modes of operation for URLLC. However, fulfilling divergent requirements for PUSCH, PDCCH and PDSCH transmissions is more straight-forward, but poses challenges for the PUCCH. PUCCH transmissions carrying HARQ-ACK/SR related to URLLC transmissions have to fulfil at least the same reliability and latency requirements as the data transmission itself.

Especially HARQ, which enables a good spectral efficiency at a high reliability, comes with the drawback of additional latency caused by HARQ-ACK reporting via a UCI either in PUSCH or PUCCH. HARQ-ACK feedback bundling (Type-I and Type-II HARQ-ACK codebooks), which was introduced in NR to lower the reporting burden on the UE and boost the spectral efficiency, poses a bottleneck for URLLC due to the HARQ-ACK waiting time until the foreseen reporting opportunity. This issue becomes more severe for UEs using mixed URLLC and eMBB services in parallel.

# HARQ-ACK Multiplexing of URLLC and eMBB

In the last meeting, it has been agreed to support multiple PUCCHs carrying HARQ-ACK per slot. One of the remaining issues is how to deal with the multiplexing between the multiple PUCCH resources. The proposed solutions can mainly be categorized into two categories which are discussed in this section.

Option 1:

Subslot granularity up to symbol granularity of K1 indication allows to address multiple PUCCHs in a slot. This approach could be realized by small spec changes and would maintain the procedures as defined in Rel. 15. However, a drawback of this scheme is the limited number of PUCCHs per slot. This is limited by the subslot granularity. Another major drawback is that the UE is not aware of the difference between URLLC and eMBB. Hence, the same PUCCH transmission configurations are applied. Furthermore, the URLLC HARQ-ACK is multiplexed with eMBB HARQ which increases the number of bits transmitted in PUCCH. This poses a major limitation regarding the reliability of the PUCCH transmissions itself and prioritization if two PUCCH resources overlap.

Observation 1: Finer K1 indication allows multiple PUCCHs per slot reporting HARQ-ACK. However, it can cause reliability issues regarding advanced URLLC use cases.

Option 2:

PDSCH-grouping considers methods to map PDSCHs based on parameters, e.g. such as the PRI, to different HARQ-ACK codebooks in different PUCCH resources in the same slot. In contrast to the first option this does not limit the number of PUCCH per slot in principle. However, if for example the PRI is used to distinguish the HARQ-ACK codebooks, the collision avoidance flexibility of the gNB is reduced. Also, the question how to apply semi-static HARQ-ACK codebooks is an open point since the DCI does not have a PRI in this case.

Observation 2: PDSCH-grouping based on PRI allows multiple PUCCHs per slot reporting HARQ-ACK. However, it decreases PUCCH multiplexing capabilities of the gNB.

To maintain the advantages of multiple PUCCHs per slot but avoiding the drawbacks, the PHY layer has to distinguish between a URLLC and eMBB transmissions and apply appropriate procedures to enable an efficient operation. This allows to separate the transmissions of URLLC and eMBB HARQ-ACK enabling a more reliable PUCCH operation. Separate HARQ-ACK codebooks working independently from each user could be used. This way URLLC and eMBB HARQ feedback are not multiplexed in the same PUCCH resource and the collision avoidance feature of dynamic HARQ-ACK codebooks is not degraded.

Observation 3: Identifying URLLC on the PHY layer enables optimizing HARQ procedures for higher reliability.

Proposal 1: URLLC and eMBB service types, at least these which are tied to URLLC-specific procedures, e.g. HARQ-ACK reporting, are distinguishable on the PHY layer.

It needs to be further studied how this identification on the PHY layer is accomplished. It can be achieved by:

- Compact DCI

- URLLC-specific DCI or DCI fields

- URLLC RNTI

- Search space

Proposal 2: Study how to identify URLLC on the PHY layer.

# Conclusion

In this contribution, we discussed potential improvements on HARQ-ACK reporting to enable NR URLLC and propose the following:

Observation 1: Finer K1 indication allows multiple PUCCHs per slot reporting HARQ-ACK. However, it can cause reliability issues regarding advanced URLLC use cases.

Observation 2: PDSCH-grouping based on PRI allows multiple PUCCHs per slot reporting HARQ-ACK. However, it decreases PUCCH multiplexing capabilities of the gNB.

Observation 3: Identifying URLLC on the PHY layer enables optimizing HARQ procedures for higher reliability.

Proposal 1: URLLC and eMBB service types, at least these which are tied to URLLC-specific procedures, e.g. HARQ-ACK reporting, are distinguishable on the PHY layer.

Proposal 2: Study how to identify URLLC on the PHY layer.

# References

[1] 	RP-181477, “SID on Physical Layer Enhancements for NR URLLC”, Nokia, Nokia 	Shanghai Bell, 3GPP RAN#80, June 2018.