3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1901162

Taipei, Taiwan, 21st – 25th January, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On QoS Management

Agenda item:	7.2.4.4

Document for:	Discussion and Decision

# Introduction

At RAN#80, a study item “Study on NR V2X” (FS_NR_V2X) was approved ‎[1].

One of the objectives deals with QoS management:

5: QoS management [RAN1, RAN2]:

- Study technical solutions for QoS management of the radio interface (including both Uu and sidelink) used for V2X operations based on input from SA2

The following agreement was reached at RAN1#94bis ‎[4]:

Agreements:

RAN1 studies further how to use

- priority,

- latency,

- reliability,

- minimum required communication range (as defined by higher layers) if agreed to use

in the physical layer aspects of at least

- resource allocation and

- congestion control and

- resolution of in-device coexistence issues and

- power control

# Discussion

## QoS Parameters/Characteristics

### Minimum communication range

In recent SA2 discussions it seems to have become uncontroversial that “minimum required communication range” is to be considered as a QoS parameter for sidelink. E.g. one of the solutions approved at the most recent SA2 meeting ‎[5] contains the following text:

V2X Layer informs the Access Stratum Layer of the communication type, and QoS parameters (including 5QI and Range) for the group communication traffic

However, it still seems unclear if minimum required communication range would be provided by the higher layer in all scenarios (the SA2 solution above applies to groupcast).

## QoS-related Metrics/Measurements

### Congestion

Regional regulation may require UEs to support sidelink congestion control. E.g. ETSI TC ITS has defined decentralised congestion control (DCC); under European regulations ITS stations need to perform DCC on ITS carriers (e.g. 5.9 GHz).

The congestion metric used in the ETSI TC ITS framework is CBR (channel busy ratio). The same metric, generalized to LTE sidelink, has been adopted for LTE sidelink-based V2X. It seems sensible to adopt the same metric for NR sidelink-based V2X to demonstrate regulatory compliance. Further refined congestion metrics can of course be studied.

Proposal 1: Support at least CBR as congestion metric.

Proposal 2: Support reporting of the congestion metric(s) to the gNB.

## Tools to achieve QoS

### Sidelink Inter-UE Preemption in UE-autonomous mode

It can happen that a UE receives a high-priority and urgent (low latency) message from higher layers, but no resources are available which meet the latency requirement. In that case it may be necessary to preempt lower priority transmissions of one or more other UEs.

Proposal 3: Support sidelink inter-UE preemption in UE-autonomous mode, taking priority and latency into account.

Specific mechanisms to provide such a preemption mechanism are discussed in our companion contribution ‎[6].

# Conclusions

In this contribution we discussed sidelink QoS management and make the following proposals:

Proposal 1: Support at least CBR as congestion metric.

Proposal 2: Support reporting of the congestion metric(s) to the gNB.

Proposal 3: Support sidelink inter-UE preemption in UE-autonomous mode, taking priority and latency into account.

# References

- RP-182491, “Revised SID: Study on NR V2X”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- RAN1#94 chairman notes

- RAN1#94bis chairman notes

- S2-1810808, Support of the group communication over NR PC5

- R1-1901158, On Sidelink Resource Allocation