3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	 R1-1901236

Taipei, 21st – 25th January, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Phase Randomization and Correction for CSI quantization in frequency domain

Document for:	Discussion and Decision

Introduction

Based on the WID of NR MIMO enhancements for Rel-16 in RAN meeting #80 [1], Rel-16 will specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead as follows:

- Extend specification support in the following areas [1]

  - Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2.

It has been agreed at RAN1 #95 [2] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

In this contribution, a problem of phase randomization and correction for DFT-based compression codebook is discussed. This is for UE implementation and affects the performance if this point is ignored.

Space-frequency compression codebook

The design of DFT-based compression codebook is introduced in [3-4]. In this design, the space-frequency matrix is compressed in both spatial domain and frequency domain, by a set of spatial basis vectors and a set of frequency basis vectors.

The space-frequency matrix W can be represented and approximated by the following formulation

W=W1W2WfH

where W1 and Wf are composed of selected basis vectors from the spatial codebook and frequency codebook, respectively. The dimension of the coefficients matrix W2 is 2L×M, with L and M as the number of selected spatial and frequency basis vectors, respectively. The UE only needs to feedback the indices of selected spatial and frequency basis vectors, i.e. W1 and Wf, as well as the combination coefficients W2.

Phase correction for eigenvectors

In the design of space-frequency compression codebook, the space-frequency matrix W is obtained by concatenating the precoding vectors of different subbands. Then the space-frequency matrix is compressed in both spatial domain and frequency domain. The effect of frequency domain compression highly depends on the smoothness of the coefficients along subbands.

In one implementation, the leading eigenvector is utilized as the precoding vector for each subband. Note that if a vector v is the eigenvector of a matrix A, ejθ*v is also its eigenvector. We should be cautious that eigenvectors, as the output of a SVD solver, may have a “random” phase rotation. Whether such phase rotation exists may depend on the numerical algorithm of the SVD solver. To achieve more efficient frequency domain compression, random phase rotation should be corrected.

To implement phase correction, the eigenvector for each subband should be multiplied by a proper phase. Then the space-frequency matrix is obtained with rotated eigenvectors. It should be noted that phase correction between adjacent subbands will provide more efficient frequency domain compression. A better phase correction will lead to the result that a larger proportion of power concentrates within the selected frequency components and then a smaller M is enough in the frequency domain compression. A potential phase correction implementation is to compensate the phase of eigenvector corresponding to the (i+1)-th subband (ei+1) with the phase of the inner product of the i-th and (i+1)-th eigenvectors (phase of ei+1H*ei), i.e., ei+1=phaseei+1H*ei*ei+1.

This procedure has no impact over codebook structure and associated PMI quantization. Whilst PMI quantization is purely a UE implementation, a proper phase correction will compress PMI coefficients in the frequency domain more efficiently.

CDF curves for power ratio

We define power ratio as the ratio of preserved power after spatial-frequency compression and the total power before the compression. The power ratio reflects the level that the power of coefficients concentrates on the selected spatial and frequency basis vectors.

Fig.1 illustrates the CDF curves of power ratio with and without phase correction. With the help of a proper phase correction, the power concentrates on the selected frequency basis more, which leads to a more efficient frequency domain compression.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

Figure 1. CDF curves for power ratio with and without phase correction.

Simulation results

System-level simulation results for the proposed space-frequency compression codebook with or without phase correction are included in this section.

The spatial basis reuses the oversampled 2D-DFT beam, whilst the frequency basis uses the oversampled DFT beam with oversampling factor of 4. Each coefficient within matrix W2 is quantized with A bits for amplitude and P bits for phase. Detailed parameters are shown in Appendix I.

The trade-off between performance and overhead with and without phase rotation is shown in Fig. 2. If phase rotation is not conducted, the performance of frequency domain compression is even worse than Type II codebook. The phase rotation procedure will significantly improve the performance and compress the coefficients in the frequency domain more efficiently.

Figure 2. The trade-off between performance and overhead with and without phase rotation (L=4).

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observation and proposal are made.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

- References

RP-181453, Samsung, “WI Proposal on NR MIMO Enhancements”, 3GPP RAN#80, La Jolla, USA, 11-14 June, 2018.

“RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

R1-1812242, Huawei, HiSilicon, “Discussion on CSI enhancement”, Spokane, USA, November 12-16, 2018.

R1-1813002, Samsung, “Summary of CSI enhancement for MU-MIMO support”, Spokane, USA, November 12-16, 2018.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz |
| System bandwidth | 10MHz (13 subbands, 4 PRB for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) for overhead reduction; (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2) for higher rank of Type II; the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; SU-MIMO for higher rank of Type II |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |