3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1901239

Taipei, 21st – 25th January, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Evaluation results of sequence design for Pi/2-BPSK DFT-S-OFDM

Document for:	Discussion and Decision

Introduction

And for DFT-s-OFDM, it has been agreed that:

Agreement Metrics to consider for new sequence design for the pi/2 BPSK DMRS are the gNB receiver complexity, PAPR relative to data and link level throughput/BLER performance considering frequency domain flatness and autocorrelation properties, interference considering cross correlation properties and when applicable (e.g. PUCCH), orthogonality of sequences Agreement - For sequences with length 30 or larger, DMRS for ππ/2 BPSK modulation for PUSCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding resulting in a DMRS Type 1 comb structure. - For sequences with length 30 or larger, DMRS for π/2 BPSK modulation for PUCCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding. - For sequences with allocation length 6,12,18 and 24 CGS is used for DMRS for π/2 BPSK modulation in case of PUSCH and PUCCH

Based on the agreed metrics and principles, we provided proposed sequence and related simulation results in this contribution.

Simulation results on DM-RS sequence for Pi/2-BPSK DFT-S-OFDM

In [1], we have been discussed our detailed sequence design and new sets of length-12, length-18 and length-24 CGS sequences are provide in table 1-3.

Table 1 Length-12 CGS sequences

| Group ID | Sequence | PAPR(dB) with α=0.28 |
| --- | --- | --- |
| 0 | 1, 0, 1, 1, 0, 1, 1, 0, 1, 0, 0, 0 | 1.32301 |
| 1 | 1, 0, 1, 0, 0, 1, 1, 1, 0, 0, 1, 0 | 1.03083 |
| 2 | 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 1, 0 | 1.36099 |
| 3 | 1, 0, 1, 1, 0, 0, 0, 1, 1, 0, 1, 1 | 1.03928 |
| 4 | 1, 0, 1, 0, 0, 1, 0, 0, 1, 0, 1, 0 | 1.36099 |
| 5 | 1, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0, 1 | 1.20599 |
| 6 | 1, 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1 | 1.03928 |
| 7 | 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 0 | 1.15831 |
| 8 | 1, 0, 1, 1, 0, 1, 0, 1, 1, 0, 1, 1 | 1.15832 |
| 9 | 1, 0, 1, 1, 0, 1, 0, 0, 0, 1, 0, 1 | 1.32299 |
| 10 | 1, 0, 1, 0, 1, 0, 1, 1, 0, 0, 0, 1 | 1.03083 |
| 11 | 1, 0, 1, 0, 1, 1, 0, 0, 1, 0, 0, 1 | 1.20599 |
| 12 | 1, 0, 1, 0, 0, 0, 1, 0, 0, 0, 0, 0 | 1.51707 |
| 13 | 1, 1, 1, 1, 0, 1, 1, 1, 0, 1, 0, 1 | 1.32447 |
| 14 | 1, 0, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1 | 1.15496 |
| 15 | 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 0, 1 | 1.19169 |
| 16 | 1, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 0 | 1.19169 |
| 17 | 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 1, 1 | 1.15495 |
| 18 | 1, 0, 1, 0, 0, 1, 0, 1, 0, 0, 1, 1 | 0.75327 |
| 19 | 1, 0, 1, 0, 0, 1, 0, 1, 1, 1, 0, 0 | 0.75327 |
| 20 | 1, 0, 1, 1, 0, 1, 0, 1, 1, 0, 0, 0 | 0.75327 |
| 21 | 1, 0, 1, 0, 1, 1, 0, 1, 0, 0, 0, 1 | 0.75328 |
| 22 | 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 0, 0 | 1.38737 |
| 23 | 1, 0, 1, 1, 0, 1, 0, 1, 0, 0, 0, 1 | 1.38737 |
| 24 | 1, 0, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0 | 0.98191 |
| 25 | 1, 0, 1, 0, 1, 0, 0, 0, 1, 1, 0, 1 | 0.98191 |
| 26 | 1, 0, 1, 1, 0, 0, 1, 1, 1, 0, 0, 0 | 1.34262 |
| 27 | 1, 0, 1, 1, 0, 1, 1, 0, 0, 1, 1, 1 | 0.97738 |
| 28 | 1, 0, 1, 1, 0, 0, 1, 1, 1, 1, 0, 1 | 1.34262 |
| 29 | 1, 0, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1 | 0.97739 |

Table 2 Length-18 CGS sequences

| Group ID | Sequence | PAPR(dB) with α=0.28 |
| --- | --- | --- |
| 0 | 1, 0, 0, 0, 1, 0, 1, 0, 1, 0, 0, 0, 1, 1, 0, 1, 0, 1 | 1.19001 |
| 1 | 1, 0, 1, 1, 1, 0, 0, 1, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0 | 1.19002 |
| 2 | 1, 1, 1, 1, 0, 0, 0, 1, 1, 0, 1, 1, 1, 0, 1, 0, 0, 0 | 1.18207 |
| 3 | 1, 0, 0, 1, 1, 1, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 0 | 1.18207 |
| 4 | 1, 0, 0, 0, 0, 1, 1, 1, 0, 0, 1, 0, 0, 0, 1, 0, 1, 1 | 1.23463 |
| 5 | 1, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1 | 1.23461 |
| 6 | 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 1, 1, 1, 0, 0, 0 | 1.23405 |
| 7 | 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 0, 1, 1, 0, 1 | 1.23405 |
| 8 | 1, 1, 1, 0, 0, 1, 0, 1, 1, 1, 1, 0, 1, 1, 0, 0, 1, 0 | 1.22978 |
| 9 | 1, 0, 1, 1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0 | 1.22981; |
| 10 | 1, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 1, 0, 1, 1, 1, 0, 0 | 1.24242 |
| 11 | 1, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 0, 0, 0, 0, 0, 1 | 1.00993 |
| 12 | 1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 1, 0, 0, 1, 1, 0, 1, 0 | 1.11076 |
| 13 | 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 0, 0, 1, 1, 0, 1, 1 | 1.24242 |
| 14 | 1, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 1, 0, 0, 0, 1, 1, 1 | 1.00993 |
| 15 | 1, 1, 0, 1, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0, 1, 0, 0, 0 | 1.11077 |
| 16 | 1, 0, 1, 1, 0, 0, 0, 1, 0, 1, 1, 1, 0, 0, 0, 1, 0, 1 | 0.99604 |
| 17 | 1, 0, 1, 1, 0, 1, 1, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 0 | 1.00837 |
| 18 | 1, 0, 1, 1, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0 | 1.12880 |
| 19 | 1, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 1, 1, 0, 1, 0, 0 | 1.12881 |
| 20 | 1, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0 | 1.04749 |
| 21 | 1, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 1, 0 | 1.04750 |
| 22 | 1, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0, 1, 1, 1, 1, 0, 0, 0 | 1.21670 |
| 23 | 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 1, 0, 1 | 1.21671 |
| 24 | 1, 0, 1, 1, 0, 0, 0, 1, 1, 0, 1, 1, 1, 1, 0, 1, 1, 0 | 1.11015 |
| 25 | 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 1, 1, 0, 1, 0, 1, 1, 0 | 1.22320 |
| 26 | 1, 0, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1, 1, 0, 1, 1 | 1.22319 |
| 27 | 1, 0, 1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 1, 1, 1, 0, 0 | 1.11017 |
| 28 | 1, 1, 0, 0, 1, 1, 1, 1, 1, 0, 1, 0, 0, 1, 1, 0, 1, 0 | 1.17611 |
| 29 | 1, 1, 1, 1, 0, 0, 1, 1, 0, 1, 0, 1, 1, 0, 0, 1, 0, 1 | 1.17610 |

Table 3 Length-24 CGS sequences

| Group ID | Sequence | PAPR(dB) with α=0.28 |
| --- | --- | --- |
| 0 | 1, 0, 1, 1, 0, 1, 1, 1, 1, 1, 0, 0, 0, 0, 1, 0, 0, 1, 1, 1, 0, 1, 1, 1 | 1.01204 |
| 1 | 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 1, 0, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 0 | 1.45039 |
| 2 | 1, 1, 1, 1, 0, 0, 1, 0, 1, 0, 0, 0, 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1, 1 | 1.31348 |
| 3 | 1, 0, 1, 0, 0, 1, 0, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1 | 1.21667 |
| 4 | 1, 1, 1, 1, 0, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0, 1, 1, 0 | 1.01202 |
| 5 | 1, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1, 1, 0, 0, 0, 0, 1, 0, 1, 0, 0 | 1.31348 |
| 6 | 1, 0, 1, 0, 0, 0, 1, 1, 0, 1, 1, 1, 0, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 1 | 1.21667 |
| 7 | 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1, 1, 0, 0, 0, 0 | 1.45036 |
| 8 | 1, 0, 1, 0, 1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 1, 1, 1, 0, 1, 0, 1, 0, 0, 1 | 1.52323 |
| 9 | 1, 1, 1, 1, 1, 0, 0, 1, 0, 0, 0, 1, 1, 1, 1, 0, 1, 1, 1, 1, 1, 1, 0, 0 | 1.31464 |
| 10 | 1, 0, 1, 0, 0, 1, 1, 0, 1, 0, 1, 0, 0, 0, 1, 0, 1, 1, 0, 1, 1, 1, 0, 0 | 1.52323 |
| 11 | 1, 0, 1, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 0, 0, 0, 1, 1, 1, 1, 1, 0 | 0.88391 |
| 12 | 1, 1, 1, 1, 0, 0, 1, 0, 1, 0, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 1 | 1.30165 |
| 13 | 1, 1, 1, 1, 0, 0, 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1, 0, 1, 0, 0, 0, 1, 1 | 1.30172 |
| 14 | 1, 0, 1, 0, 0, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 0, 1, 1, 0 | 0.88386 |
| 15 | 1, 0, 1, 0, 1, 1, 1, 0, 0, 1, 0, 1, 1, 1, 0, 0, 1, 0, 1, 1, 0, 1, 1, 0 | 1.37272 |
| 16 | 1, 1, 1, 1, 0, 0, 1, 1, 1, 1, 1, 1, 0, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 1 | 1.31464 |
| 17 | 1, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 1, 0, 0, 1, 1 | 1.37268 |
| 18 | 1, 0, 1, 0, 0, 0, 1, 1, 0, 1, 0, 0, 0, 1, 1, 0, 1, 0, 0, 1, 0, 0, 1, 0 | 0.92387 |
| 19 | 1, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 1, 1, 0, 1 | 0.92386 |
| 20 | 1, 0, 1, 0, 0, 1, 0, 0, 0, 1, 1, 1, 0, 0, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1 | 1.21167 |
| 21 | 1, 0, 1, 0, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 1 | 1.21166 |
| 22 | 1, 0, 1, 0, 0, 0, 0, 1, 1, 0, 0, 1, 0, 0, 1, 1, 0, 1, 0, 1, 0, 1, 0, 0 | 1.47071 |
| 23 | 1, 1, 1, 1, 0, 1, 0, 0, 1, 1, 0, 0, 0, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 1 | 1.12587 |
| 24 | 1, 0, 1, 1, 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 1, 1, 0 | 1.37872 |
| 25 | 1, 1, 1, 1, 0, 1, 0, 1, 1, 0, 1, 0, 1, 0, 1, 0, 0, 1, 1, 0, 1, 1, 0, 0 | 1.12587 |
| 26 | 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 0, 0, 0, 1, 0, 0, 1, 0, 1, 1 | 1.37867 |
| 27 | 1, 0, 1, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 0, 1 | 1.47071 |
| 28 | 1, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 1, 1, 1, 0, 0, 1, 0, 0, 0, 1, 0, 1, 1 | 1.18821 |
| 29 | 1, 1, 1, 1, 0, 1, 1, 1, 0, 0, 1, 0, 0, 1, 0, 1, 0, 0, 0, 1, 1, 1, 1, 0 | 1.18818 |

The evaluation results for cross-correlation, PAPR and channel estimation performance of CGSs in Table 1-3 is shown in Figure 1-3. The CGSs in [2], [3], [4], and first set of CGS in [5] is used for comparison. The detail of simulation assumptions can be found in Appendix. From the simulation results, we can see that the PAPR of CGSs is reduced to the same level as for data symbols, but the maximum cross-correlation of length 12, length 18 and length 24 CGSs from [5] and length 12, length 18 CGSs from [4] is 1, which means the CGS sets contain equivalent sequences. The link level simulation results show that the channel estimation performance loss of CGSs from our proposed sequence and [2] and [3] is no more than 0.5 dB, but for [4], the performance loss is more than 1 dB.

Figure 1 cross-correlation, PAPR and channel estimation performance of length 12 CGS

Figure 2 cross-correlation, PAPR and channel estimation performance of length 18 CGS

Figure 3 cross-correlation, PAPR and channel estimation performance of length 24 CGS

From Figure 1~ Figure 3, we have the following observations

Observation 1: The maximum cross correlation for pi/2 BPSK based DMRS sequence proposed in [4] and [5] is up to 1, e.g., the probability of maximum cross correlation coefficient being 1 reaches up to 44% for length-18 DMRS sequence.

Observation 2: The channel estimation of the pi/2 BPSK based DMRS sequence proposed in [4] degrades significantly, e.g., more than 1dB for length-24 DMRS sequence.

Observation 3: The performance of proposed sequence for length-12/18/24 can be the same performance on PAPR with pi/2 BPSK data, and also with good performance in Channel estimation and cross-correlation.

For length 6, we have been proposed that two separate groups of sequence design for even subcarrier s(0) and odd subcarrier s(1). The sequence can use 8PSK or 16PSK based CGS sequence as shown in table 4 and 5.

Table 4.  PAPR for s(0) and s(1) of length-6 DMRS sequence with 8PSK

| u | ϕ0,…, ϕ(5) for s(0) | PAPR of s(0) | ϕ0,…, ϕ(5) for s(1) | PAPR of s(1) |
| --- | --- | --- | --- | --- |
| 0 | 1 3 1 -5 1 7 | 2.0792 | 1 5 1 -5 3 3 | 2.1005 |
| 1 | 1 -3 3 1 7 -7 | 2.1155 | 1 -5 1 3 -3 7 | 1.7244 |
| 2 | 1 -5 5 5 -5 1 | 2.0359 | 1 7 1 7 -3 -5 | 1.7714 |
| 3 | 1 7 1 -1 1 -5 | 2.0793 | 1 5 5 -5 3 -1 | 2.0083 |
| 4 | 1 7 1 -1 -7 -1 | 1.8853 | 1 7 1 1 -3 5 | 1.8877 |
| 5 | 1 5 1 -7 -3 -5 | 1.9572 | 1 7 1 -1 5 -5 | 1.7244 |
| 6 | 1 7 1 -5 -3 3 | 1.7901 | 1 7 1 -5 -3 -1 | 1.7715 |
| 7 | 1 5 1 -1 3 -7 | 2.1070 | 1 -1 5 -7 -1 -1 | 1.9848 |
| 8 | 1 5 1 -5 7 -1 | 1.7675 | 1 7 1 -5 -3 7 | 1.9289 |
| 9 | 1 3 1 7 -3 -7 | 2.0214 | 1 -3 1 1 -5 3 | 1.8619 |
| 10 | 1 5 1 -1 3 -3 | 1.7817 | 1 1 7 -7 3 -1 | 1.9849 |
| 11 | 1 -3 1 5 -1 3 | 1.7817 | 1 5 1 1 7 -1 | 1.8619 |
| 12 | 1 -5 1 3 -7 7 | 1.8864 | 1 -5 1 7 5 -5 | 1.9289 |
| 13 | 1 -3 1 -7 7 -5 | 1.9665 | 1 -5 1 7 -3 -5 | 1.9701 |
| 14 | 1 -3 5 -7 -5 5 | 2.0290 | 1 7 3 -1 5 5 | 2.0881 |
| 15 | 1 5 1 -5 -1 -3 | 1.4418 | 1 5 1 3 -1 5 | 2.0755 |
| 16 | 1 7 5 -1 -7 -5 | 1.7657 | 1 -3 1 -5 3 -7 | 1.8619 |
| 17 | 1 -3 1 5 3 -7 | 2.0932 | 1 -7 5 -1 3 -7 | 1.9151 |
| 18 | 1 -5 5 3 -7 -1 | 1.7657 | 1 5 1 7 -1 -7 | 1.8619 |
| 19 | 1 5 1 5 -5 -7 | 1.6489 | 1 5 1 -5 -5 3 | 2.1006 |
| 20 | 1 3 1 -5 5 -7 | 1.9297 | 1 -5 1 -1 5 -5 | 1.7245 |
| 21 | 1 5 1 -3 1 5 | 1.5333 | 1 -5 1 3 -3 -1 | 1.9289 |
| 22 | 1 7 1 -5 -7 -1 | 1.8863 | 1 -3 1 5 -1 -5 | 2.0887 |
| 23 | 1 5 1 5 -5 5 | 1.5200 | 1 -3 1 -1 3 -3 | 2.0755 |
| 24 | 1 5 1 -5 -1 3 | 1.8358 | 1 7 1 -5 5 7 | 1.9701 |
| 25 | 1 -1 1 -7 -3 7 | 1.9298 | 1 7 1 3 5 -1 | 1.7715 |
| 26 | 1 -3 1 5 -7 7 | 1.9853 | 1 7 3 -1 -1 5 | 1.8384 |
| 27 | 1 5 1 7 -1 -3 | 2.1115 | 1 7 1 7 5 3 | 1.7714 |
| 28 | 1 -3 1 -5 -1 5 | 1.8003 | 1 5 1 -3 3 7 | 2.0887 |
| 29 | 1 -7 5 -1 -5 -3 | 1.8788 | 1 -5 3 7 -3 -3 | 2.084 |

Table 5.  PAPR for s(0) and s(1) of length-6 DMRS sequence with 16PSK

| u | ϕ0,…, ϕ(5) for s(0) | PAPR of s(0) | ϕ0,…, ϕ(5) for s(1) | PAPR of s(1) |
| --- | --- | --- | --- | --- |
| 0 | 1 -5 5 11 -13 11 | 1.5266 | 1 -7 13 -13 -11 -3 | 1.8551 |
| 1 | 1 -5 3 13 3 -5 | 1.7337 | 1 -7 -9 -15 -3 5 | 1.8549 |
| 2 | 1 -5 5 13 5 11 | 1.8202 | 1 5 15 -15 5 -3 | 1.8895 |
| 3 | 1 -9 -5 5 15 11 | 1.5718 | 1 13 11 1 -3 9 | 1.8895 |
| 4 | 1 9 -15 11 -13 11 | 1.4234 | 1 11 3 15 11 5 | 1.8709 |
| 5 | 1 9 -15 11 3 11 | 1.5758 | 1 -11 -3 3 -9 -5 | 1.7423 |
| 6 | 1 11 -11 -9 13 3 | 1.8733 | 1 -11 -3 3 -9 13 | 1.7806 |
| 7 | 1 -7 7 15 11 15 | 1.8914 | 1 -7 3 15 11 5 | 1.8514 |
| 8 | 1 -9 -1 -5 -15 -7 | 1.6919 | 1 -3 7 -13 9 5 | 1.8332 |
| 9 | 1 -13 -9 -15 -5 7 | 1.7162 | 1 11 7 -13 9 5 | 1.8716 |
| 10 | 1 -1 7 15 3 11 | 1.432 | 1 13 -9 1 -9 -15 | 1.8763 |
| 11 | 1 9 -15 15 -9 11 | 1.4795 | 1 -9 13 1 1 7 | 1.8763 |
| 12 | 1 15 7 -5 -11 -9 | 1.8878 | 1 3 11 -1 -11 -3 | 1.8214 |
| 13 | 1 11 15 -3 -13 5 | 1.8748 | 1 3 11 -1 7 -3 | 1.7856 |
| 14 | 1 9 -15 15 7 15 | 1.7096 | 1 9 -1 7 9 -3 | 1.9068 |
| 15 | 1 9 -15 9 7 15 | 1.8449 | 1 11 -11 13 15 -7 | 1.8635 |
| 16 | 1 -11 -3 11 -15 13 | 1.7534 | 1 -7 3 -5 -3 7 | 1.5049 |
| 17 | 1 11 1 5 -9 -9 | 1.839 | 1 9 7 -3 5 -5 | 1.8634 |
| 18 | 1 -3 9 -1 -15 -11 | 1.8617 | 1 13 15 7 -3 5 | 1.8965 |
| 19 | 1 15 -13 7 -5 -9 | 1.767 | 1 -7 3 11 9 -3 | 1.8966 |
| 20 | 1 11 -3 3 1 -9 | 1.9052 | 1 13 -7 -5 -15 -7 | 1.9074 |
| 21 | 1 -11 -13 9 -13 -3 | 1.9006 | 1 -7 13 15 -3 3 | 1.8752 |
| 22 | 1 -11 -7 3 13 3 | 1.9006 | 1 -13 -15 -3 5 -9 | 1.8753 |
| 23 | 1 -11 11 -11 -7 3 | 1.85 | 1 15 11 -1 11 7 | 1.8943 |
| 24 | 1 -11 -15 -9 3 11 | 1.8371 | 1 -3 11 7 -5 5 | 1.8703 |
| 25 | 1 15 5 -9 -7 -9 | 1.8506 | 1 -13 -9 3 -7 -3 | 1.8343 |
| 26 | 1 11 15 9 -1 -11 | 1.8196 | 1 7 7 -5 -15 -3 | 1.8124 |
| 27 | 1 -11 -1 -5 5 11 | 1.6235 | 1 11 1 11 -11 -9 | 1.6131 |
| 28 | 1 7 -5 5 15 11 | 1.6895 | 1 -5 5 -7 -11 9 | 1.8519 |
| 29 | 1 11 3 13 -13 15 | 1.8407 | 1 -9 1 3 -3 7 | 1.8959 |

Figure 4 provide the cross-correlation, PAPR and channel estimation comparison of our proposed 8PSK and 16PSK based length-6 sequence, sequence proposed in [2] and R15 sequence.

Figure 4 cross-correlation, PAPR and channel estimation performance of length 6 CGS

From Figure 4, we have the following observations

Observation 4: The PAPR for  s(0) and s(1)of the length-6 sequence proposed in [2] differs significantly, where up to 4dB for  s(1)and it cannot fulfill the low PAPR requirement.

Observation 5: Our proposed two separate groups of length-6 sequence for s0 and s1 achieve similar PAPR and both can be reduced to the same level as data.

Observation 6: 16PSK based length-6 sequence has lower PAPR than 8PSK based sequence.

Observation 7: Our proposed 8PSK and 16PSK based length-6 DMRS sequence achieve similar channel estimation and cross-correlation performance with that proposed in [2].

Conclusions

According to the provided evaluation, we have the following observations:

Observation 1: The maximum cross correlation for pi/2 BPSK based DMRS sequence proposed in [4] and [5] is up to 1, e.g., the probability of maximum cross correlation coefficient being 1 reaches up to 44% for length-18 DMRS sequence.

Observation 2: The channel estimation of the pi/2 BPSK based DMRS sequence proposed in [4] degrades significantly, e.g., more than 1dB for length-24 DMRS sequence.

Observation 3: The performance of proposed sequence for length-12/18/24 can be the same performance on PAPR with pi/2 BPSK data, and also with good performance in Channel estimation and cross-correlation.

Observation 4: The PAPR for  s(0) and s(1)of the length-6 sequence proposed in [2] differs significantly, where up to 4dB for  s(1)and it cannot fulfill the low PAPR requirement.

Observation 5: Our proposed two separate groups of length-6 sequence for s0 and s1 achieve similar PAPR and both can be reduced to the same level as data.

Observation 6: 16PSK based length-6 sequence has lower PAPR than 8PSK based sequence.

Observation 7: Our proposed 8PSK and 16PSK based length-6 DMRS sequence achieve similar channel estimation and cross-correlation performance with that proposed in [2].

- References

R1-1900020, “Discussion on DMRS sequence design for low PAPR”, Huawei, HiSilicon.

R1-1813445, “Lower PAPR reference signals”, Qualcomm.

R1-1812510, “Low PAPR Reference Signals”, Intel.

R1-1812259, “Discussion on Low PAPR RS”, ZTE.

R1-1813086, “Low PAPR Reference Signals”, IITH, CEWiT, Reliance Jio, IITM, Tejas Networks.

Appendix: Simulation assumptions

| System bandwidth | 20 MHz |
| --- | --- |
| Numerology | 30 KHz SCS |
| Channel | CDL-A 1000ns |
| Number of Antennas | UE Tx=1; gNB Rx =2 |
| Number of UEs | 1 UE |
| PUSCH duration | 12 OFDM symbols with one front loaded DMRS symbol |
| # RBs for PUSCH | 1,2,3,4 RBs |
| Carrier Frequency | 4 GHz |
| Coding rate | 0.2 |
| UE mobility | 3 Km/h |
| Receiver | Practical Time Domain Channel Estimation Ideal Noise Estimation |